DS1216

SmartWatch RAM (DS1216B/C/D/H); SmartWatch ROM (DS1216E/F)

www.maxim-ic.com

GENERAL DESCRIPTION

The DS1216 SmartWatch RAM and SmartWatch ROM sockets are 600-mil-wide DIP sockets with a built-in CMOS watch function, an NV RAM controller circuit, and an embedded lithium energy source. The sockets provide an NV RAM solution for memory sized from 2k x 8 to 512k x 8 with package sizes from 26 pins to 32 pins. When a socket is mated with a CMOS SRAM, it provides a complete solution to problems associated with memory volatility and uses a common energy source to maintain time and date. The SmartWatch ROM sockets use the embedded lithium source to maintain the time and date only. A key feature of the SmartWatch is that the watch function remains transparent to the RAM. The SmartWatch monitors VCC for an out-of-tolerance condition. When such a condition occurs, an internal lithium energy source is automatically switched on and write protection is unconditionally enabled to prevent loss of watch and RAM data.

TYPICAL OPERATING CIRCUIT

FEATURES

	Keeps Track of Hundredths of Seconds, Seconds, Minutes, Hours, Days, Date of the Month, Months, and Years

	Converts Standard 2k x 8 Up to 512k x 8

CMOS Static RAMs into Nonvolatile Memory

	Embedded Lithium Energy Cell Maintains

Watch Information and Retains RAM Data

	Watch Function is Transparent to RAM Operation

	Month and Year Determine the Number of Days in Each Month; Leap-Year Compensation Valid Up to 2100

	Lithium Energy Source is Electrically Disconnected to Retain Freshness Until Power is Applied for the First Time

	Proven Gas-Tight Socket Contacts

	Full 10% Operating Range

	Operating Temperature Range: 0C to +70°C

	Accuracy Better Than 1 Minute/Month at +25C

ORDERING INFORMATION

PART TEMP RANGE PIN-PACKAGE

DS1216B 0°C to +70°C 28 SmartWatch Socket

DS1216C 0°C to +70°C 28 SmartWatch Socket

DS1216D 0°C to +70°C 32 SmartWatch Socket

DS1216E 0°C to +70°C 28 SmartWatch Socket

DS1216F 0°C to +70°C 32 SmartWatch Socket

DS1216H 0°C to +70°C 32 SmartWatch Socket

(See Figure 2 for letter suffix marking identification.)

Selector Guide appears on page 2.

Note: Some revisions of this device may incorporate deviations from published specifications known as errata. Multiple revisions of any device may be simultaneously available through various sales channels. For information about device errata, click here:  www.maxim-ic.com/errata.

1 of 14	REV: 031704

PIN DESCRIPTION

RST	- Reset, Active Low

OE	- Output Enable, Active Low

DQ0	- Data Input/Output 0 (RAM)

WE	- Write Enable, Active Low

A2	- Address Bit 2 (Read/Write [ROM])

VCC

- Switched VCC for 28-/32-Pin RAM

A0	- Address Bit 0 (Data Input [ROM]) GND	- Ground

CE	- Conditioned Chip Enable, Active Low

VCCB	- Switched VCC for 24-Pin RAM VCCD	- Switched VCC for 28-Pin RAM

PIN CONFIGURATIONS

| TOP VIEW RST 1 | 28 | VCC |
| --- | --- | --- |
| 2 | 27 | WE |
| 3 | 26 | VCCB |
| 4 | 25 |  |
| 5 | 24 |  |
| 6 | 23 |  |
| 7 | 22 | OE |
| [A2] 8 | 21 |  |
| 9 | 20 | CE |
| [A0] 10 | 19 |  |
| DQ0 11 | 18 |  |
| 12 | 17 |  |
| 13 | 16 |  |
| GND 14 | 15 |  |

RST	1

2

32  VCC

31

3	30  VCCD

4

5

6

7

8

9 [A2]  10

11 [A0]  12

29  WE

28

27

26

25

24  OE

23

22   CE

21

DQ0   13	20

14	19

15	18

GND	16	17

DS1216B/C/E

28-Pin Intelligent Socket

DS1216D/F/H

32-Pin Intelligent Socket

SELECTOR GUIDE

PC BOARD MODIFICATION PART RAM/ROM RAM DENSITY REQUIRED FOR DENSITY UPGRADE?

DS1216B RAM 16k/64k No/Yes

DS1216C RAM 64k/256k No

DS1216D RAM 256k/1M No/Yes

DS1216E ROM 64k/256k No

DS1216F ROM 64k/256k/1M No

DS1216H RAM 1M/4M No

DETAILED DESCRIPTION

The DS1216 SmartWatch RAM and SmartWatch ROM Sockets are 600-mil-wide DIP sockets with a built-in CMOS watch function, an NV RAM controller circuit, and an embedded lithium energy source. The sockets provide an NV RAM solution for memory sized from 2k x 8 to 512k x 8 with package sizes from 26 pins to 32 pins. When a socket is mated with a CMOS SRAM, it provides a complete solution to problems associated with memory volatility and uses a common energy source to maintain time and date. The SmartWatch ROM sockets use the embedded lithium source to maintain the time and date only. A key feature of the SmartWatch is that the watch function remains transparent to the RAM. The SmartWatch monitors VCC for an out-of-tolerance condition. When such a condition occurs, an internal lithium energy source is automatically switched on and write protection is unconditionally enabled to prevent loss of watch and RAM data.

Using the SmartWatch saves PC board space since the combination of SmartWatch and the mated RAM take up no more area than the memory alone. The SmartWatch uses the VCC, data I/O 0, CE, OE, and WE for RAM and watch control. All other pins are passed straight through to the socket receptacle.

The SmartWatch provides timekeeping information including hundredths of seconds, seconds, minutes,

hours, days, date, months, and years. The date at

the end of the month is automatically adjusted for

months with fewer than 31 days, including correction for leap years. The SmartWatch operates in either

24-hour or 12-hour format with an AM/PM indicator.

OPERATION

Communication with the SmartWatch RAM is established by pattern recognition on a serial bit stream of

64 bits that must be matched by executing 64 consecutive write cycles containing the proper data on DQ0. On the SmartWatch ROM, communication with the clock is established using A2 and A0, and either OE or CE. All accesses that occur prior to recognition of the 64-bit pattern are directed to memory. After the pattern match, the next 64 reads and/or writes are directed to the clock, and the RAM is

disabled. Once the pattern is established, the next

64 read/write cycles will be directed to the RTC

registers. When power is cycled, 64 reads should be executed prior to any writes to ensure that the RTC registers are not written. A pattern match is ignored if the RST bit is zero and the RST pin goes low during the match sequence. A pattern match is also terminated if a read occurs during the 64-bit match sequence.

PATTERN MATCH—RAM

Data transfer to and from the timekeeping registers is accomplished with a serial bit stream under control of chip enable (CE), output enable (OE), and write enable (WE). Initially, a read cycle to any memory location using the CE and OE control of the SmartWatch starts the pattern recognition sequence by moving a pointer to the first bit of the 64-bit comparison register. Next, 64 consecutive write cycles are executed using the CE and WE control of the SmartWatch. These 64 write cycles are used only to gain access to the SmartWatch. Therefore, any address to the memory in the socket is acceptable. However, the write cycles generated to gain access to the SmartWatch are also writing data to a location in the mated RAM. The preferred way to manage this requirement is to set aside just one address location in RAM as a SmartWatch scratch pad. When the first write cycle is executed, it is compared to bit 0 of the

64-bit comparison register. If a match is found, the pointer increments to the next location of the

comparison register and awaits the next write cycle. If a match is not found, the pointer does not advance and all subsequent write cycles are ignored. If a read cycle occurs at any time during pattern recognition, the present sequence is aborted and the comparison register pointer is reset. Pattern recognition continues for 64 write cycles as described above until all the bits in the comparison register have been matched (this bit pattern is shown in Figure 1). With a correct match for 64 bits, the SmartWatch is enabled and data transfer to or from the timekeeping registers can proceed. The next 64 cycles will cause the SmartWatch to either receive or transmit data on DQ0, depending on the level of the OE pin or the WE pin. Cycles to other locations outside the memory block can be interleaved with CE cycles without interrupting the pattern recognition sequence or data transfer sequence to the SmartWatch.

PATTERN MATCH—ROM

Communication with the SmartWatch is established by pattern recognition of a serial bit stream of 64 bits that must be matched by executing 64 consecutive write cycles, placing address bit A2 low with the proper data on address bit A0. The 64 write cycles are used only to gain access to the SmartWatch. Prior to executing the first of 64 write cycles, a read cycle should be executed by holding A2 high. The read cycle will reset the comparison register pointer within the SmartWatch, ensuring the pattern recognition starts with the first bit of the sequence. When the first write cycle is executed, it is compared to bit 0 of the 64-bit comparison register. If a match is found, the pointer increments to the next location of the comparison register and awaits the next write cycle. If a match is not found, the pointer does not advance and all subsequent write cycles are ignored. If a read cycle occurs at any time during pattern recognition, the present sequence is aborted and the comparison register pointer is reset. Pattern recognition continues for a total of 64 write cycles as described above, until all the bits in the comparison register have been matched (this bit pattern is shown in Figure 1). With a correct match for 64 bits, the SmartWatch is enabled and data transfer to or from the timekeeping registers can proceed. The next 64 cycles will cause the SmartWatch to either receive data on data in (A0) or transmit data on data out (DQ0), depending on the level of /WRITE READ (A2).

After  power-up,  the  controller  could  be  in  the  64-bit  clock  register  read/write  sequence  (from  an

incomplete access prior to power-down). Therefore, it is recommended that a 64-bit read be performed upon power-up to prevent accidental writes to the clock, and to prevent reading clock data when access to the RAM would otherwise be expected.

Figure 1. SmartWatch Comparison Register Definition

BYTE 0

BYTE 1

7	0

1	1	0	0	0	1	0	1

0	0	1	1	1	0	1	0

HEX VALUE

C5

3A

BYTE 2

1	0	1	0	0	0	1	1	A3

BYTE 3

BYTE 4

BYTE 5

BYTE 6

BYTE 7

0	1	0	1	1	1	0	0	5C

1	1	0	0	0	1	0	1	C5

0	0	1	1	1	0	1	0	3A

1	0	1	0	0	0	1	1	A3

0	1	0	1	1	1	0	0	5C

NOTE: THE PATTERN RECOGNITION IN HEX IS C5, 3A, 5C, C5, 3A, A3, 5C. THE ODDS OF THIS PATTERN ACCIDENTALLY  DUPLICATING AND CAUSING INADVERTENT  ENTRY TO THE SMARTWATCH  ARE LESS THAN 1 IN 1019. THIS PATTERN IS SENT TO THE SMARTWATCH LSB TO MSB.

NONVOLATILE CONTROLLER OPERATION

The DS1216 SmartWatch performs circuit functions required to make a CMOS RAM nonvolatile. First, a switch is provided to direct power from the battery or VCC supply, depending on which voltage is greater. This switch has a voltage drop of less than 0.2V. The second function that the SmartWatch provides is power-fail detection, which occurs at VTP. The DS1216 constantly monitors the VCC supply. When VCC goes out of tolerance, a comparator outputs a power-fail signal to the chip-enable logic. The third function accomplishes write protection by holding the chip-enable signal to the memory within 0.2V of VCC  or battery. During nominal power-supply conditions, the memory chip-enable signal will track the chip- enable signal sent to the socket with a maximum propagation delay of 7ns for the 5V and 12ns for the

3.3V version.

FRESHNESS SEAL

Each  DS1216  is  shipped  from  Dallas  Semiconductor  with  its  lithium  energy  source  disconnected, ensuring full energy capacity. When VCC is first applied at a level greater than the lithium energy source is enabled for battery-backup operation.

SMARTWATCH REGISTER INFORMATION

The SmartWatch information is contained in eight registers of 8 bits, each of which is sequentially

accessed

one bit at

a time after the 64-bit pattern

recognition sequence has been completed. When

updating

the SmartWatch registers, each

must be handled in groups of 8 bits. Writing and reading

individual bits within a register could produce erroneous results. These read/write registers are defined in

Figure 3.

Data contained in the SmartWatch

registers is in binary-coded decimal (BCD) format. Reading and

writing the registers is always accomplished by stepping through all eight registers, starting with bit 0 of register 0 and ending with bit 7 of register 7.

AM-PM/12-/24-MODE

Bit 7 of the hours register is defined as the 12-hour or 24-hour mode-select bit. When high, the 12-hour mode is selected. In the 12-hour mode, bit 5 is the AM/PM bit with logic high being PM. In the 24-hour mode, bit 5 is the second 10-hour bit (20 to 23 hours).

OSCILLATOR AND RESET BITS

Bits 4 and 5 of the day register are used to control the RST and oscillator functions. Bit 4 controls the RST (pin 1). When the RST bit is set to logic 1, the RST input pin is ignored.  When the RST bit is set to logic 0, a low input on the RST pin will cause the SmartWatch to abort data transfer without changing data in the watch registers. Bit 5 controls the oscillator. When set to logic 1, the oscillator is off. When set to logic 0, the oscillator turns on and the watch becomes operational. These bits are shipped from the factory set to logic 1.

ZERO BITS

Registers 1 to 6 contain one or more bits that always read logic 0. When writing these locations, a logic 1 or 0 is acceptable.

ADDITIONAL INFORMATION

Refer to Application Note 52: Using the Dallas Phantom Real-Time Clocks (available on our website at www.maxim-ic.com/RTCapps) for information about using regarding optional modifications and the phantom clock contained within the SmartWatch.

Figure 2. Reset and Memory Density Options

NOTE: THE LETTER SUFFIX OF THE SMARTWATCH IS LOCATED ON THE PC BOARD.

The RST pin on the controller has an internal pullup resistor. To disable the RST function, the trace between pin 1 on the socket and pin 13 on the controller can be cut. In this case, the socket will ignore the RST input, preventing address transitions from resetting the pattern match, even if the RST bit is enabled.

On the DS1216B and DS1216D, the two VCC pins are connected together on the PC board. The switched VCC from the controller is connected to the two VCC pins that connect to the inserted RAM. No modifications are required if the lower density RAM is used. To use the higher density RAM, the trace by the lower density RAM VCC  pin, identified by a hash mark labeled “U,” must be cut. The two square- metal pads, labeled “G,” must be shorted together. This disconnects switched VCC from the pin going to the inserted RAM, and connects it to the corresponding address input pin for the higher density RAM.

Figure 3. SmartWatch Register Definition

REGISTER	7	0

RANGE (BCD)

0	0.1 SEC	0.01 SEC

7	0

1	0	10 SEC	SECONDS

7	0

2	0	10 MIN	MINUTES

00-99

00-59

00-59

7

3	12/24	0  10	A/P	HR	HOUR

7

4	0	0	OSC	RST	0	DAY

7

5	0	0	10	DATE	DATE

7

6	0	0	0   10 MONTH	MONTH

7

0

01-12

0

01-07

00-23

0

01-31

0

01-12

0

7	10 YEAR

YEAR

00-99

ABSOLUTE MAXIMUM RATINGS

Voltage Range on any Pin Relative to Ground…………………………………………….…………-0.3V to +7.0V for 5V Operating Temperature Range……………………………………………………………………………..…...0°C to +70°C

Storage Temperature Range……………………………………………………………………………..…...-40°C to +70°C

Soldering Temperature Range………………………………………………...….See J-STD-020A Specification (Note 6)

This is a stress rating only and functional operation of the device at these or any other conditions beyond those indicated in the operation sections of this specification is not implied. Exposure to absolute maximum rating conditions for extended periods of time can affect reliability.

RECOMMENDED DC OPERATING CONDITIONS

| (TA = 0°C to +70°C) PARAMETER | SYMBOL | MIN TYP MAX | UNITS | NOTES |
| --- | --- | --- | --- | --- |
| VCC Pin, 5V Supply | VCC | 4.5 5.0 5.5 | V | 1 |
| Logic 1 | VIH | 2.2 VCC + 0.3 | V | 8 |
| Logic 0 | VIL | -0.3 +0.8 | V | 8 |

DC ELECTRICAL CHARACTERISTICS

| (VCC = 5.0V 10%, TA = 0°C to +70°C.) PARAMETER | SYMBOL | MIN TYP MAX | UNITS | NOTES |
| --- | --- | --- | --- | --- |
| VCC Supply | ICCI | 5 | mA | 1, 2, 3 |
| VCC Supply Voltage (ICCO = 80mA) | VCCO1(U) | VCC - 0.2 | V | 1, 6 |
| Input Leakage | IIL | -1.0 +1.0 | A | 2, 8, 13 |
| Output Logic 1 Voltage (IOUT = -1.0mA) | VOH | 2.4 | V |  |
| Output Logic 1 Voltage (IOUT = -1.0mA) | VOL | 0.4 | V |  |
| Write-Protection Voltage | VTP | 4.25 4.5 | V |  |

BACKUP POWER CHARACTERISTICS

(VCC < VTP; TA = 0°C to +70°C.)

| CCO2(U) PARAMETER | SYMBOL | MIN TYP MAX | UNITS | NOTES |
| --- | --- | --- | --- | --- |
| CE Output | VOH(L) | VBAT - 0.2 | V | 1 |
| VCC Supply Voltage (ICCO = 10µA) | V | V - 0.2 | V | 1, 6, 14 |
| RAM VCC (Battery) Voltage | VBAT | 2 3 3.6 | V | 1, 15 |
| Recovery at Power-Up | tREC | 2 | ms |  |
| VCC Slew Rate Power-Down VPF(MAX) to VPF(MIN) | tF | 300 | s |  |
| VCC Slew Rate Power-Down VPF(MIN) to VBAT(MIN) | tFB | 10 | s |  |
| CE Pulse Width | tCE | 1.5 | s | 5 |

BAT

CAPACITANCE

| (TA = +25C) PARAMETER | SYMBOL | MIN TYP MAX | UNITS | NOTES |
| --- | --- | --- | --- | --- |
| Input Capacitance | CIN | 5 | pF |  |
| Output Capacitance | COUT | 7 | pF |  |
| Expected Data Retention | tDR | 10 | years | 14 |

AC ELECTRICAL CHARACTERISTICS

| (VCC = 4.5V to 5.5V, TA = 0°C to +70°C.) PARAMETER | SYMBOL | MIN TYP MAX | UNITS | NOTES |
| --- | --- | --- | --- | --- |
| Read Cycle Time | tRC | 75 | ns |  |
| CE Access Time | tCO | 65 | ns |  |
| OE Access Time | tOE | 65 | ns |  |
| CE to Output Low-Z | tCOE | 6 | ns |  |
| OE to Output Low-Z | tOEE | 6 | ns |  |
| CE to Output High-Z | tOD | 30 | ns |  |
| OE to Output High-Z | tODO | 30 | ns |  |
| Address Setup Time (ROM) | tAS | 20 |  | 11 |
| Address Hold Time (ROM) | tAH | 10 |  | 12 |
| Read Recovery | tRR | 15 | ns |  |
| Write Cycle Time | tWC | 75 | ns |  |
| Write Pulse Width | tWP | 75 | ns |  |
| Write Recovery | tWR | 15 | ns | 9 |
| Data Setup Time | tDS | 35 | ns | 10 |
| Data Hold Time | tDH | 0 | ns | 10 |
| CE Pulse Width | tCW | 65 | ns |  |
| RST Pulse Width | tRST | 75 | ns |  |
| CE Propagation Delay | tPD | 6 | ns | 7 |
| CE High to Power-Fail | tPF | 0 | ns |  |

Timing Diagram: Read Cycle to SmartWatch

Timing Diagram: Write Cycle to SmartWatch

Timing Diagram: Reset for SmartWatch

Timing Diagram: Power-Down

tCE

CE (L)*

CE (U )

V IL

tPD

tCE

VIH

VBAT  -0.2V

V CC  (L)

*N ote 1

VIL

VPF (m ax)

tFB

tF

V PF (m in)

VBAT (m in)

Timing Diagram: Power-Up

WARNING:

Under no circumstances should negative undershoots of any amplitude be allowed when the device is in battery-backup mode. Water washing for flux removal will discharge internal lithium source because exposed voltage pins are present.

NOTES:

1)  Pin locations are designated “U” when a parameter definition refers to the socket receptacle and “L”

when a parameter definition refers to the socket pin.

2)  No memory inserted in the socket.

3)  Pin 26L can be connected to VCC or left disconnected at the PC board.

4) SmartWatch sockets can be successfully processed through some conventional wave-soldering techniques as long as temperature exposure to the lithium energy source contained within does not exceed +85C. However, post-solder cleaning with water-washing techniques is not permissible. Discharge to the lithium energy source can result, even if deionized water is used. It is equally imperative that ultrasonic vibration is not used in order to avert damage to the quartz crystal resonator employed by the oscillator circuit.

5)  tCE max must be met to ensure data integrity on power loss.

6)  VCCO1 is the maximum voltage drop from VCC(L) to VCC(U) while Vcc(L) is supplying power. VCCO2 is the maximum voltage drop from VBAT to VCC(U) while the part is in battery backup.

7)  Input pulse rise and fall times equal 10ns.

8)  Applies to pins RST L, A2 L, A0 L, CE L, OE L, and WE L.

9)  tWR is a functions of the latter occurring edge of WE or CE.

10) tDH and tDS are a function of the first occurring edge of WE or CE.

11) tAS is a function of the first occurring edge of OE or CE.

12) tAH is a function of the latter occurring edge of OE or CE.

13) RST (Pin 1) has an internal pullup resistor.

14) Expected data retention is based on using an external SRAM with a data retention current of less than

0.5A at +25°C. Expected data-retention time (time while on battery) for a given RAM battery current can be calculated using the following formula:

0.045 / (current in amps) = data-retention time in hours

15) The DS1216 products are shipped with the battery-backup power off. First power-up switches backup battery on to clock and RAM VCC pin upon power-down.

PACKAGE INFORMATION

(The package drawing(s) in this data sheet may not reflect the most current specifications. For the latest package outline information and land patterns, go to www.maxim-ic.com/packages.)

| PKG | 28-PIN |  | 32-PIN |  |
| --- | --- | --- | --- | --- |
| DIM | MIN | MAX | MIN | MAX |
| A IN. MM | 1.390 35.31 | 1.420 36.07 | 1.580 40.13 | 1.620 41.14 |
| B IN. MM | 0.690 17.53 | 0.720 18.29 | 0.690 17.53 | 0.720 18.29 |
| C IN. MM | 0.420 10.67 | 0.470 11.94 | 0.400 10.16 | 0.470 11.94 |
| D IN. MM | 0.035 0.89 | 0.065 1.65 | 0.035 0.89 | 0.065 1.65 |
| E IN. MM | 0.055 1.39 | 0.075 1.90 | 0.055 1.39 | 0.075 1.90 |
| F IN. MM | 0.120 3.04 | 0.160 4.06 | 0.120 3.04 | 0.160 4.06 |
| G IN. MM | 0.090 2.29 | 0.110 2.79 | 0.090 2.29 | 0.110 2.79 |
| H IN. MM | 0.590 14.99 | 0.630 16.00 | 0.590 14.99 | 0.630 16.00 |
| J IN. MM | 0.008 0.20 | 0.012 0.30 | 0.008 0.20 | 0.012 0.30 |
| K IN. MM | 0.015 0.38 | 0.021 0.53 | 0.015 0.38 | 0.021 0.53 |
| L IN. MM | 0.380 9.65 | 0.420 10.67 | 0.380 9.65 | 0.420 10.67 |

Maxim/Dallas Semiconductor cannot assume responsibility for use of any circuitry other than circuitry entirely embodied in a Maxim/Dallas Semiconductor product. No circuit patent licenses are implied. Maxim/Dallas Semiconductor reserves the right to change the circuitry and specifications without notice at any time.

M a x i m  In tegra t ed  P r oduc ts ,  1 2 0  S a n  Ga brie l  D r iv e ,  Sun ny vale ,  CA  94 086  40 8 - 7 3 7 - 76 00

© 2004 Maxim Integrated Products