19-4621; 5/09

DS1254

2M x 8 NV SRAM with Phantom Clock

GENERAL DESCRIPTION

The DS1254 is a fully nonvolatile static RAM (NV SRAM) (organized as 2M words by 8 bits) with built-in real-time

clock. It has a self-contained lithium energy source and

control circuitry that constantly monitors VCC  for an out-of- tolerance condition. When such a condition occurs, the DS1254 makes use of an attached DS3800 battery cap to maintain clock information and preserve stored data while protecting that data by disallowing all memory accesses. Additionally, the DS1254 has dedicated circuitry for monitoring the status of an attached DS3800 battery cap.

FEATURES

	Real-Time Clock (RTC) Keeps Track of Hundredths of Seconds, Seconds, Minutes, Hours, Days, Date, Months, and Years with Automatic Leap-Year Compensation Valid Up to the Year 2100

	2M x 8 NV SRAM

	Watch Function is Transparent to RAM Operation

	Automatic Data Protection During Power Loss

	Unlimited Write-Cycle Endurance

	Surface-Mountable BGA Module Construction

	Over 10 Years of Data Retention in the Absence of

Power

	Battery Monitor Checks Remaining Capacity Daily

	+3.3V or +5V Operation

PACKAGE OUTLINE

Side -A- Shown

(For Reference Only, Not to Scale) Component placement may vary.

TYPICAL OPERATING CIRCUIT

	Underwriters Laboratory (UL) Recognized

(www.maxim-ic.com/qa/info/ul/)

APPLICATIONS Telecom Switches Routers

RAID Systems

ORDERING INFORMATION

PART TEMP RANGE PIN-PACKAGE VOLTAGE TOP MARK RANGE (V)

DS1254WB-150 0°C to +70°C 40mm BGA 3.3 DS1254W-150

DS1254WB2-150 0°C to +70°C 40mm BGA 3.3 DS1254W-150

DS1254YB-100 0°C to +70°C 40mm BGA 5.0 DS1254Y-100

DS1254YB2-100 0°C to +70°C 40mm BGA 5.0 DS1254Y-100

Note: Some revisions of this device may incorporate deviations from published specifications known as errata. Multiple revisions of any device may be simultaneously available through various sales channels. For information about device errata, click here:  www.maxim-ic.com/errata.

1 of 18

DETAILED DESCRIPTION

The DS1254 is a fully nonvolatile static RAM (NV SRAM) (organized as 2M words by 8 bits) with built-in real-time clock. It has a self-contained lithium energy source and control circuitry that constantly monitors VCC for an out-of- tolerance condition. When such a condition occurs, the DS1254 makes use of an attached DS3800 battery cap to maintain clock information and preserve stored data while protecting that data by disallowing all memory accesses. Additionally, the DS1254 has dedicated circuitry for monitoring the status of an attached DS3800 battery cap.

A13

The phantom clock provides timekeeping information including hundredths of seconds, seconds, minutes, hours, day, date, month, and year information. The date at the end of the month is automatically adjusted for months with fewer than 31 days, including correction for leap years. The phantom clock operates in either 24-hour or 12-hour

34

33

A14

format with an AM/PM indicator.

37

36

35

32

31

A16

30

VCC

Because the DS1254 has a total of 168 balls and only 35 active signals, balls are wired together into groups, thus providing redundant connections for every signal.

VCC

A8

A9

38

A10

A11

GND

A12

A15

Figure 1. Pin Configuration

41

40

39

VBAT

VCC	1

A7	2

A6	3

A5	4

GND	5

A4	6

A3	7

A2	8

A0

10

DQ0

11

12

DQ1

DQ2

DQ3

A1	9

Dallas Semiconductor

DS1254

13

14

GND

DQ4

16

17

DQ5

18

19

RECEPTACLES FOR DS3800 BATTERY CAP PINS

29	VCC

28	A17

27	A18

26	A19

25	GND

24	A20

23	CE

22	OE

20

21	WE

15

DQ6

DQ7

BW

GND

RAM READ MODE

The DS1254 executes a read cycle whenever WE is inactive (high) and CE is active (low). The unique address specified by the 21 address inputs (A0–A20) defines which of the 2MB of data is to be accessed. Valid data will be available to the eight data-output drivers within tACC (access time) after the last address input is stable, providing that CE and OE access times and states are also satisfied. If OE and CE access times are not satisfied, then data

access must be measured from the later occurring signal (CE or OE) and the limiting parameter is either tCO for CE

or tOE for OE rather than address access.

RAM WRITE MODE

The DS1254 is in the write mode whenever WE and CE are in their active (low) state after address inputs are stable. The later occurring falling edge of CE or WE will determine the start of the write cycle. The write cycle is terminated by the earlier rising edge of CE or WE. All address inputs must be kept valid throughout the write cycle.

WE must return to the high state for a minimum recovery time (tWR) before another cycle can be initiated. The OE

control signal should be kept inactive (high) during write cycles to avoid bus contention. However, if the output bus has been enabled (CE and OE active), then WE will disable the outputs in tODW from its falling edge.

DATA RETENTION MODE

The device is fully accessible and data can be written and read only when VCC is greater than VPF. However, when VCC  falls below the power-fail point, VPF  (point at which write protection occurs), the internal clock registers and SRAM are blocked from any access. When VCC  falls below VBAT, device power is switched from the VCC  to VBAT. RTC operation and SRAM data are maintained from the battery until VCC is returned to nominal levels. All signals

must be powered down when VCC is powered down.

PHANTOM CLOCK OPERATION

Communication with the phantom clock is established by pattern recognition on a serial bit stream of 64 bits that must be matched by executing 64 consecutive write cycles containing the proper data on DQ0. All accesses that occur prior to recognition of the 64-bit pattern are directed to memory.

After recognition is established, the next 64 read or write cycles either extract or update data in the phantom clock, and memory access is inhibited.

Data transfer to and from the timekeeping function is accomplished with a serial bit stream under control of chip enable (CE), output enable (OE), and write enable (WE). Initially, a read cycle to any memory location using the CE and OE control of the phantom clock starts the pattern-recognition sequence by moving a pointer to the first bit of the 64-bit comparison register. Next, 64 consecutive write cycles are executed using the CE and WE signals of the device. These 64 write cycles are used only to gain access to the phantom clock. Therefore, any address within the first 512kB of memory, (00h to 7FFFFh) is acceptable. However, the write cycles generated to gain access to the phantom clock are also writing data to a location in the memory. The preferred way to manage this requirement is to set aside just one address location in memory as a phantom clock scratch pad. When the first write cycle is executed, it is compared to bit 0 of the 64-bit comparison register. If a match is found, the pointer increments to the next location of the comparison register and awaits the next write cycle. If a match is not found, the pointer does not advance and all subsequent write cycles are ignored. If a read cycle occurs at any time during pattern recognition, the present sequence is aborted and the comparison register pointer is reset. Pattern recognition continues for a total of 64 write cycles as described above until all the bits in the comparison register have been matched (this bit pattern is shown in Figure 2). With a correct match for 64-bits, the phantom clock is enabled and

data transfer to or from the timekeeping registers can proceed. The next 64 cycles will cause the phantom clock to either receive or transmit data on DQ0, depending on the level of the OE pin or the WE pin. Cycles to other locations outside the memory block can be interleaved with CE cycles without interrupting the pattern-recognition sequence or data-transfer sequence to the phantom clock.

PHANTOM CLOCK REGISTER INFORMATION

The phantom clock information is contained in eight registers of 8 bits, each of which is sequentially accessed one bit at a time after the 64-bit pattern-recognition sequence has been completed. When updating the phantom clock registers, each register must be handled in groups of 8 bits. Writing and reading individual bits within a register could produce erroneous results. These read/write registers are defined in Figure 3.

Figure 2. Phantom Clock Protocol Definition

NOTE:  THE  PATTERN RECOGNITION IN HEX  IS  C5,  3A,  A3,  5C, C5,  3A,  A3,  5C.  THE  ODDS  OF  THIS  PATTERN  BEING ACCIDENTALLY DUPLICATED AND CAUSING INADVERTENT ENTRY TO THE PHANTOM CLOCK IS LESS THAN 1 IN 1019. THIS PATTERN IS SENT TO THE PHANTOM CLOCK LSB TO MSB

Figure 3. Phantom Clock Register Definition

AM/PM/12/24 MODE

Bit 7 of the hours register is defined as the 12-hour or 24-hour mode select bit. When high, the 12-hour mode is selected. In the 12-hour mode, bit 5 is the AM/PM bit with logic high being PM. In the 24-hour mode, bit 5 is the second 10-hour bit (20–23 hours).

OSCILLATOR BIT

Bit 5 of the day register controls the oscillator. When set to logic 1, the oscillator is off. When set to logic 0, the oscillator turns on and the watch becomes operational.

ZERO BITS

Registers 1, 2, 3, 4, 5, and 6 contain one or more bits that will always read logic 0. When writing these locations, either a logic 1 or logic 0 is acceptable.

BATTERY MONITORING

The DS1254 automatically monitors the battery in an attached DS3800 battery cap on a 24-hour time interval. Such monitoring begins within tREC after VCC rises above VPF and is suspended when power failure occurs.

After each 24-hour period has elapsed, the battery is connected to an internal 1M test resistor for one second. During this one second, if the battery voltage falls below the battery-voltage trip point (~2.6V), the battery warning output BW is asserted. Once asserted, BW remains active until the attached DS3800 battery cap is replaced. However, the battery is still retested after each VCC power-up, even if it was active on power-down. If the battery voltage is found to be higher than ~2.6V during such testing, BW is de-asserted and regular testing resumes. BW has an open-drain output driver.

ABSOLUTE MAXIMUM RATINGS

Voltage Range on Any Pin Relative to Ground	-0.3V to +6.0V Operating Temperature Range	  0C to +70C

Storage Temperature Range

-40C to +70C

Soldering Temperature	See IPC/JEDEC J-STD-020

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions beyond those indicated in the operational sections of the specifications is not implied. Exposure to the absolute maximum rating conditions for extended periods may affect device.

RECOMMENDED DC OPERATING CONDITIONS

| (TA = 0C to +70C) PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS | NOTES |
| --- | --- | --- | --- | --- | --- |
| Power-Supply Voltage (5V Operation) | VCC |  | 4.5 5.0 5.5 | V | 1 |
| Power-Supply Voltage (3.3V Operation) | VCC |  | 3.0 3.3 3.7 | V | 1 |
| Logic 1 Voltage (All Inputs) | VIH | VCC = 5V 10% | 2.2 VCC + 0.3 | V | 1 |
|  |  | VCC = 3.3V 10% | VCC + 2.0 0.3 |  |  |
| Logic 0 Voltage (All Inputs) | VIL | VCC = 5V 10% | -0.3 0.8 | V | 1 |
|  |  | VCC = 3.3V 10% | -0.3 0.6 |  |  |

DC ELECTRICAL CHARACTERISTICS

| (VCC = 5.0V 10%, TA = 0C to +70C.) PARAMETER | SYMBOL | MIN TYP MAX | UNITS | NOTES |
| --- | --- | --- | --- | --- |
| Input Leakage Current | IIL | -4.0 +4.0 | A |  |
| I/O Leakage Current | IIO | -4.0 +4.0 | A |  |
| Output Current at 2.4V | IOH | -1.0 | mA | 3 |
| Output Current at 0.4V | IOL | 2.0 | mA | 3 |
| Standby Current (CE = 2.2V) | ICCS1 | 5.0 10 | mA |  |
| Standby Current (CE = VCC - 0.5V) | ICCS2 | 3.0 5.0 | mA |  |
| Operating Current, tCYC = 100ns | ICCO1 | 85 | mA |  |
| Write Protection Voltage | VPF | 4.25 4.50 | V | 1 |

DC ELECTRICAL CHARACTERISTICS

| (VCC = 3.3V 10%, TA = 0C to +70C.) PARAMETER | SYMBOL | MIN TYP MAX | UNITS | NOTES |
| --- | --- | --- | --- | --- |
| Input Leakage Current | IIL | -4.0 +4.0 | A |  |
| I/O Leakage Current | IIO | -4.0 +4.0 | A |  |
| Output Current at 2.4V | IOH | -1.0 | mA | 3 |
| Output Current at 0.4V | IOL | 2.0 | mA | 3 |
| Standby Current (CE = 2.2V) | ICCS1 | 5.0 7 | mA |  |
| Standby Current (CE = VCC - 0.5V) | ICCS2 | 2.0 3.0 | mA |  |
| Operating Current, tCYC = 100ns | ICCO1 | 50 | mA |  |
| Write Protection Voltage | VPF | 2.8 2.97 | V | 1 |

CAPACITANCE

| (TA = +25C) PARAMETER | SYMBOL | MIN TYP MAX | UNITS | NOTES |
| --- | --- | --- | --- | --- |
| Input Capacitance: A0 to A18, OE, WE, CE | CIN | 25 50 | pF |  |
| Input Capacitance: A19 to A20 | CIN | 5 10 | pF |  |
| I/O Capacitance: DQ0 to DQ7 | CIO | 25 50 | pF |  |
| Output Capacitance: BW | COUT | 5 10 | pF |  |

AC ELECTRICAL CHARACTERISTICS

| (VCC = 5.0V 10%, TA = 0°C to +70°C.) PARAMETER | SYMBOL | MIN MAX | UNITS | NOTES |
| --- | --- | --- | --- | --- |
| Read Cycle Time | tRC | 100 | ns |  |
| Address Access Time | tAAC | 100 | ns |  |
| OE to Output Valid | tOE | 55 | ns |  |
| CE to Output Valid | tCO | 100 | ns |  |
| CE or OE to Output Active | tCOE | 0 | ns | 2 |
| Output High-Z from Deselection | tOD | 35 | ns | 2 |
| Output Hold from Address Change | tOH | 5 | ns |  |
| Write Cycle Time | tWC | 100 | ns |  |
| WE, CE Pulse Width | tWP | 70 | ns | 5 |
| Address Setup Time | tAW | 0 | ns |  |
| Address Hold Time | tAH1 | 5 | ns | 6 |
|  | tAH2 | 25 | ns | 7 |
| Output High-Z from WE | tODW | 35 | ns | 2 |
| Output Active from WE | tOEW | 5 | ns | 2 |
| Data Setup Time | tDS | 40 | ns | 8 |
| Data Hold Time | tDH1 | 0 | ns | 6 |
|  | tDH2 | 20 | ns | 8 |
| Read Recovery (Clock Access Only) | tRR | 20 | ns |  |
| Write Recovery (Clock Access Only) | tWR | 20 | ns |  |

AC ELECTRICAL CHARACTERISTICS

| (VCC = 3.3V 10%, TA = 0C to +70C) PARAMETER | SYMBOL | MIN MAX | UNITS | NOTES |
| --- | --- | --- | --- | --- |
| Read Cycle Time | tRC | 150 | ns |  |
| Address Access Time | tAAC | 150 | ns |  |
| OE to Output Valid | tOE | 75 | ns |  |
| CE to Output Valid | tCO | 150 | ns |  |
| CE or OE to Output Active | tCOE | 0 | ns | 2 |
| Output High-Z from Deselection | tOD | 70 | ns | 2 |
| Output Hold from Address Change | tOH | 5 | ns |  |
| Write Cycle Time | tWC | 150 | ns |  |
| WE, CE Pulse Width | tWP | 100 | ns | 5 |
| Address Setup Time | tAW | 10 | ns |  |
| Address Hold Time | tAH1 | 5 | ns | 6 |
|  | tAH2 | 25 | ns | 7 |
| Output High-Z from WE | tODW | 70 | ns | 2 |
| Output Active from WE | tOEW | 5 | ns | 2 |
| Data Setup Time | tDS | 60 | ns | 8 |
| Data Hold Time | tDH1 | 0 | ns | 6 |
|  | tDH2 | 20 | ns | 8 |
| Read Recovery (Clock Access Only) | tRR | 20 | ns |  |
| Write Recovery (Clock Access Only) | tWR | 20 | ns |  |

POWER-UP/POWER-DOWN CHARACTERISTICS

(VCC = 5V 10%)

| PARAMETER | SYMBOL | MIN TYP MAX | UNITS | NOTES |
| --- | --- | --- | --- | --- |
| CE and WE at VIH Before Power-Down | tPD | 0 | s |  |
| VCC Fall Time: VPF(MAX) to VPF(MIN) | tF | 300 | s |  |
| VCC Fall Time: VPF(MIN) to VBAT | tFB | 10 | s |  |
| VCC Rise Time: 0V to VPF(MIN) | tR | 150 | s |  |
| VCC Valid to End of Write Protection | tREC | 125 | ms |  |
| VCC Valid to BW Valid | tBPU | 1 | s | 3 |

| (TA = +25C) PARAMETER | SYMBOL | MIN TYP MAX | UNITS | NOTES |
| --- | --- | --- | --- | --- |
| Expected Data-Retention Time (Oscillator On) | tDR | 10 | years | 4 |

WARNING: Under no circumstances are negative undershoots, of any amplitude, allowed when device is in battery-backup mode.

BATTERY WARNING TIMING

| (VCC = 5.0V 10%, TA = 0C to +70C) PARAMETER | SYMBOL | MIN TYP MAX | UNITS | NOTES |
| --- | --- | --- | --- | --- |
| Battery Test Cycle | tBTC | 24 | hour |  |
| Battery Test Pulse Width | tBTPW | 1 | seconds |  |
| Battery Test to BW Active | tBW | 1 | seconds |  |
| VCC Valid to BW Valid | tBPU | 1 | seconds | 3 |

AC TEST CONDITIONS

Output Load:

100pF + 1 TTL Gate

Input Pulse Levels:	0V to 3.0V Timing Measurement Reference Levels:

Input: 1.5V Output: 1.5V

Input Pulse Rise and Fall Times: 5ns

Figure 4. Memory Read Cycle Timing (Note 9)

tRC

ADDRESS

tACC

CE	tCO

OE

tOE

tOD

tOH

DQ0–DQ7

tCOE

tCOE

tOD

OUTPUT DATA VALID

Figure 5. Memory Write Cycle Timing, Write-Enable Controlled (Notes 5, 6, 8, 10, 11, 12, and 13)

tWC

ADDRESS

tAW

CE

tAH1

WE	tWP

tODW

tOEW

DQ0–DQ7

tDS

tDH1

DATA IN STABLE

Figure 6. Memory Write Cycle Timing, Chip-Enable Controlled (Notes 5, 7, 8, 10, 11, 12, and 13)

tWC

ADDRESS

tAW

CE

WE

tWP

tAH2

tCOE	tODW

DQ0–DQ7

tDS	tDH2

DATA IN STABLE

Figure 7. Read Cycle to Phantom Clock

WE = VIH

CE

OE

tCO

tRC

tOE

tRR

tOD

DQ0

tCOE

tCOE

tOD

OUTPUT DATA VALID

Figure 8. Write Cycle to Phantom Clock

OE = VIH

tWC

tWR

tWP

WE

TAH2

CE	tWP

tDS

tDH1

tDH2

DQ0	DATA IN STABLE

Figure 9. Power-Up/Power-Down Waveform Timing (Note 14)

VCC

VPF(max)

VPF(min)

VBAT

CE ,

WE

tPD

tF

tFB

tDR

tR

SLEWS WITH VCC

tREC

SLEWS WITH VCC

tBPU

BW

Figure 10. Battery Warning Detection (Note 3)

VCC

VBAT

tBPU

tBTPW

2.6V

tBTC

BATTERY TEST ACTIVE

BW

tBW

NOTES:

1)   Voltage referenced to ground.

2)   These parameters are sampled with a 50pF load and are not 100% tested.

3)   BW is an open-drain output and, as such, cannot

source current. An external pullup resistor should be

connected to this pin for proper operation. BW can sink 10mA.

4)   The DS3800 battery cap is a one-time use part, but can be removed and replaced. By design, DS3800 removal will mechanically damage the battery cap, which eliminates the accidental use of a previously attached and possibly low-capacity battery cap.

5)   tWP specified as the logical AND of CE and WE, tWP is measured from the latter of CE or WE going low to the earlier of CE or WE going high.

6)   tAH1, tDH1 are measured from WE going high.

7)   tAH2, tDH2 are measured from CE going high.

8)   tDS is measured from the earlier of CE or WE going high.

9)   WE is high for a read cycle.

10)  OE = VIH or VIL. If OE = VIH during write cycle, the output buffers remain in a high-impedance state.

11) If the CE low transition occurs simultaneously with or later than the WE low transition in a write-enable- controlled write cycle, the output buffers remain in a high-impedance state during this period.

12) If the CE high transition occurs prior to or simultaneously with the WE high transition, the output buffers remain in a high-impedance state during this period.

13) If WE is low or the WE low transition occurs prior to or simultaneously with the CE low transition, the output buffers remain in a high-impedance state during this period.

14) In a power-down condition, the voltage on any pin cannot exceed the voltage on VCC.

PACKAGE INFORMATION

(The package drawing(s) in this data sheet may not reflect the most current specifications. For the latest package outline information, go to  www.maxim-ic.com/packages.)

| PKG |  | MIN | MAX |
| --- | --- | --- | --- |
| A | IN MM | 1.570 39.88 | 1.580 40.13 |
| B | IN MM | 1.570 39.88 | 1.580 40.13 |
| C | IN MM | 0.033 0.84 | 0.043 1.09 |
| D | IN MM | 1.497 38.02 | 1.503 38.18 |
| E | IN MM | 0.047 1.19 | 0.053 1.35 |
| F | IN MM | 0.033 0.84 | 0.043 1.09 |
| G | IN MM | 0.047 1.19 | 0.053 1.35 |
| H | IN MM | 0.234 5.94 | 0.240 6.10 |
| I | IN MM | 0.160 4.00 | 0.200 5.10 |
| K | IN MM | 0.025 0.64 | 0.032 0.82 |

PACKAGE INFORMATION (continued)

(The package drawing(s) in this data sheet may not reflect the most current specifications. For the latest package outline information, go to  www.maxim-ic.com/packages.)

DS1254 with Attached DS3800 Battery Cap

| PKG |  | MIN | MAX |
| --- | --- | --- | --- |
| A | IN MM | 1.656 42.06 | 1.668 42.37 |
| B | IN MM | 1.656 42.06 | 1.668 42.37 |
| C | IN MM | — — | 0.485 12.32 |

PACKAGE INFORMATION (continued)

(The package drawing(s) in this data sheet may not reflect the most current specifications. For the latest package outline information, go to  www.maxim-ic.com/packages.)

Recommended Land Pattern (with Overlaid Package Outline)

The DS1254 BGA is a subset of the industry-standard 40mm BGA format, with all balls on a 50-mil grid. Corner balls have been removed to provide space for the electrical and mechanical interface features that facilitate

attachment of the DS3800 battery cap.

NOTE

0.250

0.500

0.150

NOTE: GROUND SHIELD TO ISOLATE RTC XTAL FROM EMI.

REVISION HISTORY

| REVISION DATE | DESCRIPTION | PAGES CHANGED |
| --- | --- | --- |
| 051209 | Changed TAW MIN = 0ns to TAW MIN = 10ns in the Electrical Characteristics table. | 8, 9 |

18 of 18

Maxim/Dallas Semiconductor cannot assume responsibility for use of any circuitry other than circuitry entirely embodied in a Maxim/Dallas Semiconductor product. No circuit patent licenses are implied. Maxim/Dallas Semiconductor reserves the right to change the circuitry and specifications without notice at any time.

M a x i m  In tegra t ed  P r oduc ts ,  1 2 0  S a n  Ga brie l  D r iv e ,  Sun ny vale ,  CA  94 086  40 8 - 7 3 7 - 76 00

© 2009 Maxim Integrated Products

The Maxim logo is a registered trademark of Maxim Integrated Products, Inc. The Dallas logo is a registered trademark of Dallas Semiconductor Corporation.