DS1305

Serial Alarm Real-Time Clock

FEATURES

Real-Time Clock (RTC) Counts Seconds,

PIN CONFIGURATIONS

Minutes, Hours, Date of the Month, Month, Day of the Week, and Year with Leap-Year

TOP VIEW

VCC2              1 	20 	VCC1

Compensation Valid Up to 2100

VBAT              2

DS1305

19 	N.C.

96-Byte, Battery-Backed NV RAM for Data Storage

Two Time-Of-Day Alarms, Programmable

on Combination of Seconds, Minutes, Hours,

and Day of the Week

Supports Motorola SPIﾙ (Serial Peripheral Interface) Modes 1 and 3 or Standard 3-Wire

X1                 3 	18 	PF N.C.              4 	17	VCCIF X2                 5 	16 	SD0 N.C.              6 	15	SDI INT0             7 	14	SCLK N.C.              8 	13	 N.C. INT1             9 	12	CE

Interface

Burst Mode for Reading/Writing Successive Addresses in Clock/RAM

GND 1

10 	11	SERMODE

TSSOP (4.4mm)

Dual-Power Supply Pins for Primary and Backup Power Supplies

VCC2              1 	16 	VCC1

VBAT              2 	15 	PF

Optional Trickle Charge Output to Backup

X1                 3

DS1305

14 	VCCIF

Supply

2.0V to 5.5V Operation

Optional Industrial Temperature Range:

-   C to +85 C

Available in Space-Efficient, 20-Pin TSSOP Package

Underwriters Laboratory (UL) Recognized

SPI is a trademark of Motorola, Inc.

X2                 4 	13 	SDO N.C.              5 	12 	SDI INT0             6 	11 	SCLK INT1             7 	10 	CE

GND             8 	9 	SERMODE

DIP (300 mils)

TYPICAL OPERATING CIRCUIT

1 of 21 	   REV: 070705

ORDERING INFORMATION

DS1305

PART TEMP RANGE PIN-PACKAGE TOP MARK*

DS1305 0ﾰC to +70ﾰC 16 DIP (300 mils) DS1305

DS1305E+ 0ﾰC to +70ﾰC 20 TSSOP (173 mils) DS1305

DS1305EN+ -40ﾰC to +85ﾰC 20 TSSOP (173 mils) DS1305N

DS1305EN+T&R -40ﾰC to +85ﾰC 20 TSSOP (173), Tape/Reel DS1305

DS1305E+T&R 0ﾰC to +70ﾰC 20 TSSOP (173 mils), Tape/Reel DS1305

DS1305E 0ﾰC to +70ﾰC 20 TSSOP (173) DS1305

DS1305EN -40ﾰC to +85ﾰC 20 TSSOP (173 mils) DS1305

DS1305EN/T&R -40ﾰC to +85ﾰC 20 TSSOP (173 mils), Tape/Reel DS1305

DS1305E/T&R 0ﾰC to +70ﾰC 20 TSSOP (173 mils), Tape/Reel DS1305

DS1305N -40ﾰC to +85ﾰC 16 DIP (300 mils) DS1305N

+ Denotes a lead-free/RoHS-compliant device.

* An ﾓNﾔ on the top mark denotes an industrial device.

DESCRIPTION

The DS1305 serial alarm real-time clock provides a full binary coded decimal (BCD) clock calendar that is accessed by  a simple  serial interface. The  clock/calendar provides seconds,  minutes, hours, day, date,  month, and year information. The end of the month date is automatically adjusted for months with fewer than 31 days, including  corrections for leap  year. The  clock operates in  either the 24-hour or 12-hour  format with AM/PM indicator.  In addition, 96  bytes of  NV RAM are  provided for data storage. The  DS1305 will maintain the time and date, provided the oscillator is enabled, as long as at least one supply is at a valid level.

An interface logic power-supply input pin (VCCIF) allows the DS1305 to drive SDO and  PF pins to a level that is compatible with the interface logic. This allows an easy interface to 3V logic in mixed supply systems.

The DS1305 offers dual-power supplies as well as a battery input pin. The dual power supplies support a

programmable  trickle charge circuit  that allows a  rechargeable energy source  (such  as a super  cap or  rechargeable battery) to be used for a backup supply. The VBAT pin allows the device to be backed up by a non-rechargeable battery. The DS1305 is fully operational from 2.0V to 5.5V.

Two  programmable  time-of-day  alarms  are  provided  by  the  DS1305.  Each  alarm  can  generate  an

interrupt on a programmable combination of seconds, minutes, hours, and day. ﾓDonﾒt careﾔ states can be inserted into one or more fields if it is desired for them to be ignored for the alarm condition. The time-of- day alarms can be programmed to assert two different interrupt outputs or to assert one common interrupt output. Both interrupt outputs operate when the device is powered by VCC1, VCC2, or VBAT.

The  DS1305  supports  a  direct  interface  to  SPI  serial  data  ports  or  standard  3-wire  interface.  A  straightforward address and data format is implemented in which data transfers can occur 1 byte at a time or in multiple-byte-burst mode.

DS1305

PIN DESCRIPTION

| PIN |  | NAME | FUNCTION |
| --- | --- | --- | --- |
| DIP | TSSOP |  |  |
| 1 | 1 | VCC2 | Backup Power Supply. This is the secondary power supply pin. In systems using the trickle charger, the rechargeable energy source is connected to this pin. |
| 2 | 2 | VBAT | Battery Input for Standard +3V Lithium Cell or Other Energy Source. If not used, VBAT must be connect to ground. Diodes must not be placed in series between VBAT and the battery, or improper operation will result. UL recognized to ensure against reverse charging current when used in conjunction with a lithium battery. See “Conditions of Acceptability” at www.maxim-ic.com/TechSupport/QA/ntrl.htm. |
| 3 | 3 | X1 | Connections for Standard 32.768kHz Quartz Crystal. The internal oscillator is designed for operation with a crystal having a specified load capacitance of 6pF. For more information on crystal selection and crystal layout considerations, refer to Application Note 58: Crystal Considerations with Dallas Real-Time Clocks. The DS1305 can also be driven by an external 32.768kHz oscillator. In this configuration, the X1 pin is connected to the external oscillator signal and the X2 pin is floated. |
| 4 | 5 | X2 |  |
| 5 | 4, 6, 8, 13, 19 | N.C. | No Connection |
| 6 | 7 | INT0 | Active-Low Interrupt 0 Output. The INT0 pin is an active low output of the DS1305 that can be used as an interrupt input to a processor. The INT0 pin can be programmed to be asserted by only Alarm 0 or can be programmed to be asserted by either Alarm 0 or Alarm 1. The INT0 pin remains low as long as the status bit causing the interrupt is present and the corresponding interrupt enable bit is set. The INT0 pin operates when the DS1305 is powered by VCC1, VCC2, or VBAT. The INT0 pin is an open-drain output and requires an external pullup resistor. |
| 7 | 9 | INT1 | Active-Low Interrupt 1 Output. The INT1 pin is an active-low output of the DS1305 that can be used as an interrupt input to a processor. The INT1 pin can be programmed to be asserted by Alarm 1 only. The INT1 pin remains low as long as the status bit causing the interrupt is present and the corresponding interrupt enable bit is set. The INT1 pin operates when the DS1305 is powered by VCC1, VCC2, or VBAT. The INT1 pin is an open-drain output and requires an external pullup resistor. Both INT0 and INT1 are open- drain outputs. The two interrupts and the internal clock continue to run regardless of the level of VCC (as long as a power source is present). |
| 8 | 10 | GND | Ground |
| 9 | 11 | SERMODE | Serial Interface Mode. The SERMODE pin offers the flexibility to choose between two serial interface modes. When connected to GND, standard 3-wire communication is selected. When connected to VCC, SPI communication is selected. |
| 10 | 12 | CE | Chip Enable. The chip-enable signal must be asserted high during a read or a write for both 3-wire and SPI communication. This pin has an internal 55k pulldown resistor (typical). |
| 11 | 14 | SCLK | Serial Clock Input. SCLK is used to synchronize data movement on the serial interface for either the SPI or 3-wire interface. |

PIN DESCRIPTIONS (continued)

| PIN |  | NAME | FUNCTION |
| --- | --- | --- | --- |
| DIP | TSSOP |  |  |
| 12 | 15 | SDI | Serial Data Input. When SPI communication is selected, the SDI pin is the serial data input for the SPI bus. When 3-wire communication is selected, this pin must be tied to the SDO pin (the SDI and SDO pins function as a single I/O pin when tied together). |
| 13 | 16 | SDO | Serial Data Output. When SPI communication is selected, the SDO pin is the serial data output for the SPI bus. When 3-wire communication is selected, this pin must be tied to the SDI pin (the SDI and SDO pins function as a single I/O pin when tied together). |
| 14 | 17 | VCCIF | Interface Logic Power-Supply Input. The VCCIF pin allows the DS1305 to drive SDO and PF output pins to a level that is compatible with the interface logic, thus allowing an easy interface to 3V logic in mixed supply systems. This pin is physically connected to the source connection of the p-channel transistors in the output buffers of the SDO and PF pins. |
| 15 | 18 | PF | Active-Low Power-Fail Output. The PF pin is used to indicate loss of the primary power supply (VCC1). When VCC1 is less than VCC2 or is less than VBAT, the PF pin is driven low. |
| 16 | 20 | VCC1 | Primary Power Supply. DC power is provided to the device on this pin. |

OPERATION

The  block  diagram  in  Figure  1  shows  the  main  elements  of  the  serial  alarm  RTC.  The  following

paragraphs describe the function of each pin.

Figure 1. BLOCK DIAGRAM

OSCILLATOR AND COUNTDOWN CHAIN

1Hz

RECOMMENDED LAYOUT FOR CRYSTAL

Local ground plane (Layer 2)

crystal

X1

X2

GND

CLOCK ACCURACY

The accuracy of the  clock is dependent upon the  accuracy of the  crystal  and the accuracy of the  match

between the capacitive load of the oscillator circuit and the capacitive load for which the crystal was trimmed. Additional error is added by  crystal  frequency  drift caused by  temperature shifts. External circuit noise  coupled into the oscillator circuit  can result in the clock running  fast. Refer to  Application Note 58, ﾓCrystal Considerations with Dallas Real-Time Clocksﾔ for detailed information.

CLOCK, CALENDAR, AND ALARM

The time and calendar  information is obtained by reading the appropriate register bytes. The RTC

registers and user RAM are illustrated in Figure 2. The time, calendar, and alarm are set or initialized by writing the appropriate register bytes. Note that some bits are set to 0. These bits always read 0 regardless of how they are written. Also note that registers 12h to 1Fh (read) and registers 92h to 9Fh are reserved. These registers always read 0 regardless of how they are written. The contents of the time, calendar, and alarm registers are in the BCD format. The day register increments at midnight. Values that correspond to the day of week are user-defined but must be sequential (e.g., if 1 equals Sunday, 2 equals Monday and so on). Illogical time and date entries result in undefined operation.

Except where otherwise  noted, the initial power  on state of all registers is not defined. Therefore, it is important  to  enable  the  oscillator  (EOSC  =  0)  and  disable  write  protect  (WP  =  0)  during  initial configuration.

WRITING TO THE CLOCK REGISTERS

The  internal  time  and  date  registers  continue  to  increment  during  write  operations.  However,  the

countdown chain is reset when the seconds register is written. Writing the time and date registers within

one second after writing the seconds register ensures consistent data.

Terminating a write before the last bit is sent aborts the write for that byte.

READING FROM THE CLOCK REGISTERS

Buffers are used to copy  the time  and date  register at the beginning of  a read. When reading in burst

mode, the user copy is static while the internal registers continue to increment.

Figure 2. RTC REGISTERS AND ADDRESS MAP

| HEX ADDRESS |  | Bit7 | Bit6 | Bit5 | Bit4 | Bit3 | Bit2 | Bit1 | Bit0 | RANGE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| READ | WRITE |  |  |  |  |  |  |  |  |  |
| 00H | 80H | 0 | 10 Seconds |  |  | Seconds |  |  |  | 00ﾖ59 |
| 01H | 81H | 0 | 10 Minutes |  |  | Minutes |  |  |  | 00ﾖ59 |
| 02H | 82H | 0 | 12 | P | 10 Hour | Hours |  |  |  | 01ﾖ12 + P/A |
|  |  |  |  | A |  |  |  |  |  | 00ﾖ23 |
|  |  |  | 24 | 10 |  |  |  |  |  |  |
| 03H | 83H | 0 | 0 | 0 | 0 | Day |  |  |  | 1ﾖ7 |
| 04H | 84H | 0 | 0 | 10 Date |  | Date |  |  |  | 1ﾖ31 |
| 05H | 85H | 0 | 0 | 10 Month |  | Month |  |  |  | 01ﾖ12 |
| 06H | 86H | 10 Year |  |  |  | Year |  |  |  | 00ﾖ99 |
| ﾗ | ﾗ | Alarm 0 |  |  |  |  |  |  |  | ﾗ |
| 07H | 87H | M | 10 Seconds Alarm |  |  | Seconds Alarm |  |  |  | 00ﾖ59 |
| 08H | 88H | M | 10 Minutes Alarm |  |  | Minutes Alarm |  |  |  | 00ﾖ59 |
| 09H | 89H | M | 12 | P | 10 Hour | Hour Alarm |  |  |  | 01ﾖ12 + P/A |
|  |  |  |  | A |  |  |  |  |  |  |
|  |  |  | 24 | 10 |  |  |  |  |  | 00ﾖ23 |
| 0AH | 8AH | M | 0 | 0 | 0 | Day Alarm |  |  |  | 01ﾖ07 |
| ﾗ | ﾗ | Alarm 1 |  |  |  |  |  |  |  | ﾗ |
| 0BH | 8BH | M | 10 Seconds Alarm |  |  | Seconds Alarm |  |  |  | 00ﾖ59 |
| 0CH | 8CH | M | 10 Minutes Alarm |  |  | Minutes Alarm |  |  |  | 00ﾖ59 |
| 0DH | 8DH | M | 12 | P | 10 Hour | Hour Alarm |  |  |  | 01ﾖ12 + P/A |
|  |  |  |  | A |  |  |  |  |  |  |
|  |  |  | 24 | 10 |  |  |  |  |  | 00ﾖ23 |
| 0EH | 8EH | M | 0 | 0 | 0 | Day Alarm |  |  |  | 01ﾖ07 |
| 0FH | 8FH | Control Register |  |  |  |  |  |  |  | ﾗ |
| 10H | 90H | Status Register |  |  |  |  |  |  |  | ﾗ |
| 11H | 91H | Trickle Charger Register |  |  |  |  |  |  |  | ﾗ |
| 12ﾖ1FH | 92ﾖ9FH | Reserved |  |  |  |  |  |  |  | ﾗ |
| 20ﾖ7FH | A0ﾖFFH | 96 Bytes User RAM |  |  |  |  |  |  |  | 00ﾖFF |

Note: Range for alarm registers does not include maskﾒmﾒ bits.

The DS1305 can be run  in either 12-hour or 24-hour mode. Bit 6 of the  hours register is defined as the

12- or 24-hour mode select bit. When high, the 12-hour mode is selected. In the 12-hour mode, bit 5 is the AM/PM bit with logic high being PM.  In the 24-hour mode, bit 5 is the second 10-hour bit (20 to 23  hours).

The DS1305 contains two time-of-day alarms. Time-of-day Alarm 0 can be set by writing to registers 87h

to  8Ah.  Time-of-day  Alarm  1  can  be  set  by  writing  to  registers  8Bh  to  8Eh.  The  alarms  can  be programmed (by the INTCN bit of the control register) to operate in two different modes; each alarm can drive its own separate interrupt output or both alarms can drive a common interrupt output. Bit 7 of each of the time-of-day alarm registers are mask bits (Table 1). When all of the mask bits are logic 0, a time- of-day alarm only occurs once per week when the  values stored in timekeeping registers 00h to 03h match the values stored in the time-of-day alarm registers. An alarm is generated every day when bit 7 of the day alarm register is set to a logic 1. An alarm is generated every hour when bit 7 of the day and hour alarm registers is set to  a logic 1. Similarly, an  alarm is  generated every minute when bit 7 of the day, hour, and minute alarm registers is set to a logic 1.  When bit 7 of the day, hour, minute, and seconds  alarm registers is set to a logic 1, alarm occurs every second.

During each clock update, the RTC compares the Alarm 0 and Alarm 1 registers with the corresponding clock registers. When a match occurs, the corresponding alarm flag bit in the status register is set to a 1. If the corresponding alarm interrupt enable bit is enabled, an interrupt output is activated.

Table 1. TIME-OF-DAY ALARM MASK BITS

| ALARM REGISTER MASK BITS (BIT 7) |  |  |  | FUNCTION |
| --- | --- | --- | --- | --- |
| SECONDS | MINUTES | HOURS | DAYS |  |
| 1 | 1 | 1 | 1 | Alarm once per second |
| 0 | 1 | 1 | 1 | Alarm when seconds match |
| 0 | 0 | 1 | 1 | Alarm when minutes and seconds match |
| 0 | 0 | 0 | 1 | Alarm hours, minutes, and seconds match |
| 0 | 0 | 0 | 0 | Alarm day, hours, minutes and seconds match |

SPECIAL PURPOSE REGISTERS

The DS1305 has three additional registers (control register, status register, and trickle charger register)

that control the RTC, interrupts, and trickle charger.

CONTROL REGISTER (READ 0FH, WRITE 8FH)

| BIT7 | BIT6 | BIT5 | BIT4 | BIT3 | BIT2 | BIT1 | BIT0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EOSC | WP | 0 | 0 | 0 | INTCN | AIE1 | AIEO |

EOSC  (Enable  Oscillator) ﾖ  This bit when set  to logic 0 starts the oscillator.  When this bit is set to a logic 1, the oscillator is stopped and the DS1305 is placed into a low-power standby mode with a current drain of less than 100nA when power is supplied by VBAT or VCC2. On initial application of power, this bit will be set to a logic 1.

WP (Write  Protect) ﾖ  Before any write operation to the clock  or RAM, this bit must be logic 0. When high, the write protect bit prevents a write operation to any register, including bits 0, 1, 2, and 7  of the control register. Upon initial power-up, the state of the WP bit is undefined. Therefore, the WP bit should be cleared before attempting to write to the device.

INTCN (Interrupt Control) ﾖ This bit controls the relationship between the two time-of-day alarms and

the interrupt output pins. When the INTCN bit is set to a logic 1, a match between the timekeeping registers and the Alarm 0 registers  activates the  INT0  pin (provided that the alarm is enabled)  and  a match between the timekeeping registers and the Alarm 1 registers activate the  INT1   pin (provided that

the alarm is enabled). When the INTCN bit is set to a logic 0, a match between the timekeeping registers

and either Alarm 0 or Alarm 1 activate the  INT0  pin (provided that the alarms are enabled).  INT1  has no

function when INTCN is set to a logic 0.

AIE0 (Alarm Interrupt Enable 0) ﾖ When set to a logic 1, this bit permits the interrupt 0 request flag (IRQF0) bit in the status register to assert  INT0 . When the AIE0 bit is set to logic 0, the IRQF0 bit does not initiate the  INT0  signal.

AIE1 (Alarm Interrupt Enable 1) ﾖ When set to a logic 1, this bit permits the interrupt 1 request flag

(IRQF1) bit in the status register to assert  INT1  (when INTCN = 1) or to assert  INT0  (when INTCN = 0). When the AIE1 bit is set to logic 0, the IRQF1 bit does not initiate an interrupt signal.

STATUS REGISTER (READ 10H)

| BIT7 | BIT6 | BIT5 | BIT4 | BIT3 | BIT2 | BIT1 | BIT0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | IRQF1 | IRQF0 |

IRQF0 (Interrupt 0 Request Flag) ﾖ A logic 1 in the interrupt request flag bit indicates that the current

time has matched the Alarm 0 registers. If the AIE0 bit is also a logic 1, the  INT0  pin goes low. IRQF0 is

cleared when the address pointer goes to any of the Alarm 0 registers during a read or write.

IRQF1 (Interrupt 1 Request Flag) ﾖ A logic 1 in the interrupt request flag bit indicates that the current time has matched the Alarm 1 registers. This flag can be used to generate an interrupt on either  INT0  or INT1  depending on the status of the INTCN bit in the control register. If the INTCN bit is set to a logic 1 and IRQF1 is at a logic 1 (and AIE1 bit is also a logic 1), the  INT1  pin goes low. If the INTCN bit is set to a logic 0 and  IRQF1 is at a logic 1 (and AIE1 bit is also a logic 1), the  INT0  pin goes low.  IRQF1 is

cleared when the address pointer goes to any of the Alarm 1 registers during a read or write.

TRICKLE CHARGE REGISTER (READ 11H, WRITE 91H)

This register controls the trickle charge characteristics of the DS1305. The simplified schematic of Figure

3 shows the basic components of the trickle charger. The trickle-charge select (TCS) bits (bits 4ﾖ7) control the selection of the trickle charger. To prevent accidental enabling, only a pattern of 1010 enables the trickle charger. All other patterns disable the trickle charger. On the initial application of power, the DS1305 powers up with the trickle charger disabled. The diode select (DS) bits (bits 2ﾖ3) select whether one diode or two diodes  are connected between VCC1 and VCC2. The resistor select (RS) bits select the  resistor that is connected between VCC1 and VCC2. The resistor and diodes are selected by the RS and DS bits, as shown in Table 2.

Figure 3. PROGRAMMABLE TRICKLE CHARGER

Table 2. TRICKLE CHARGER RESISTOR AND DIODE SELECT

| TCS Bit 7 | TCS Bit 6 | TCS Bit 5 | TCS Bit 4 | DS Bit 3 | DS Bit 2 | RS Bit 1 | RS Bit 0 | FUNCTION |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X | X | X | X | X | X | 0 | 0 | Disabled |
| X | X | X | X | 0 | 0 | X | X | Disabled |
| X | X | X | X | 1 | 1 | X | X | Disabled |
| 1 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 1 Diode, 2kΩ |
| 1 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 1 Diode, 4kΩ |
| 1 | 0 | 1 | 0 | 0 | 1 | 1 | 1 | 1 Diode, 8kΩ |
| 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 2 Diodes, 2kΩ |
| 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 2 Diodes, 4kΩ |
| 1 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 2 Diodes, 8kΩ |
| 0 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | Initial power-on state |

The user determines diode and resistor selection according to the maximum current desired for battery or super cap charging. The maximum charging current can be calculated as illustrated in the following  example. Assume that a  system power supply of  5V is applied to VCC1 and a super cap is connected to VCC2. Also assume that the trickle charger has been  enabled with 1 diode and  resister R1 between  VCC1 and VCC2. The maximum current IMAX would, therefore, be calculated as follows:

IMAX = (5.0V - diode drop) / R1   (5.0V - 0.7V) / 2k     2.2mA

As the super  cap charges, the voltage drop between VCC1 and VCC2 decreases and, therefore, the charge

current decreases.

POWER CONTROL

Power is provided through the VCC1, VCC2, and  VBAT pins. Three different power-supply  configurations are illustrated in Figure  4. Configuration 1 shows the DS1305 being backed up by  a nonrechargeable  energy source such as a lithium battery.  In this configuration, the system power supply is connected to

VCC1 and VCC2 is grounded. The DS1305 is write-protected if VCC1 is less than VBAT. The DS1305 is fully

accessible when VCC1 is greater than VBAT + 0.2V.

Configuration 2 illustrates the DS1305 being backed up by a rechargeable energy source. In this case, the VBAT pin is grounded,  VCC1 is connected to the primary power supply, and VCC2 is connected to the secondary  supply  (the rechargeable energy source). The DS1305 operates from the larger of  VCC1 or VCC2. When VCC1 is greater than VCC2 + 0.2V (typical), VCC1 powers the DS1305. When VCC1 is less than VCC2, VCC2 powers the DS1305. The DS1305 does not write-protect itself in this configuration.

Configuration 3 shows the DS1305 in battery operate mode where the device is powered only by a single

battery. In this case, the VCC1 and VBAT pins are grounded and the battery is connected to the VCC2 pin.

Only these three configurations are allowed. Unused supply pins must be grounded.

Figure 4. POWER SUPPLY CONFIGURATIONS

CONFIGURATION 1: BACKUP SUPPLY IS NONRECHARGEABLE LITHIUM BATTERY

NOTE: DEVICE IS WRITE-PROTECTED IF VCC < VCCTP.

CONFIGURATION 2: BACKUP SUPPLY IS A RECHARGEABLE BATTERY OR SUPER CAPACITOR

NOTE: DEVICE DOES NOT PROVIDE AUTOMATIC WRITE PROTECTION.

CONFIGURATION 3: BATTERY OPERATE MODE

SERIAL INTERFACE

The  DS1305  offers  the  flexibility  to  choose  between  two  serial  interface  modes.  The  DS1305  can  communicate with the SPI interface or with a standard 3-wire interface. The interface method used is determined by the SERMODE pin. When  this pin is connected to VCC,  SPI communication is selected.  When this pin is connected to ground, standard 3-wire communication is selected.

SERIAL PERIPHERAL INTERFACE (SPI)

The serial peripheral interface (SPI) is a synchronous bus for address and data transfer, and is used when

interfacing  with the SPI bus on specific Motorola microcontrollers such as the 68HC05C4 and the

68HC11A8. The SPI mode of serial communication is selected by tying the SERMODE pin to VCC. Four pins are used for the SPI. The  four pins are the SDO  (serial data out),  SDI  (serial data in), CE (chip  enable), and SCLK (serial clock). The DS1305 is the slave device in an SPI application, with the microcontroller being the master.

The SDI  and SDO pins  are the serial data input  and output pins for the  DS1305, respectively. The CE input  is  used  to  initiate  and  terminate  a  data  transfer.  The  SCLK  pin  is  used  to  synchronize  data movement between the master (microcontroller) and the slave (DS1305) devices.

The shift clock (SCLK), which is generated by the microcontroller, is active only during address and data transfer  to  any  device  on  the  SPI  bus.  The  inactive  clock  polarity  is  programmable  in  some microcontrollers. The DS1305 determines the clock polarity by sampling SCLK when CE becomes  active. Therefore, either SCLK polarity can be accommodated. Input data (SDI) is latched on the internal strobe edge and output data (SDO) is shifted out on the shift edge (Figure 5). There is one clock for each bit transferred. Address and data bits are transferred in groups of eight.

Figure 5. SERIAL CLOCK AS A FUNCTION OF MICROCONTROLLER CLOCK POLARITY (CPOL)

CPOL = 1

CPOL = 0

CE

SCLK

SHIFT DATA OUT (READ)

SCLK

SHIFT DATA OUT (READ)

DATA LATCH (WRITE)

DATA LATCH (WRITE)

NOTE 1: CPHA BIT POLARITY (IF APPLICABLE) MAY NEED TO BE SET ACCORDINGLY. NOTE 2: CPOL IS A BIT THAT IS SET IN THE MICROCONTROLLERﾒS CONTROL REGISTER.

NOTE 3: SDO REMAINS AT HIGH-Z UNTIL 8 BITS OF DATA ARE READY TO BE SHIFTED OUT DURING A READ.

ADDRESS AND DATA BYTES

Address and data bytes are  shifted MSB first into the serial data  input (SDI) and out of the serial data output (SDO). Any transfer requires the address of the byte to specify a write or read to either a RTC or RAM location, followed by one or more bytes  of data. Data is transferred out of the SDO for  a read  operation and into the SDI for a write operation (Figures 6 and 7).

Figure 6. SPI SINGLE-BYTE WRITE

* SCLK CAN BE EITHER POLARITY.

Figure 7. SPI SINGLE-BYTE READ

* SCLK CAN BE EITHER POLARITY.

The address byte is always the first byte entered after CE is driven high. The most significant bit (A7) of this byte determines if a read or write takes place. If A7 is 0, one or more read cycles occur. If A7 is 1, one or more write cycles occur.

Data transfers can occur one byte  at a time or in multiple-byte burst mode. After CE is driven high an

address is written to the DS1305. After the address, one or more data bytes can be written or read. For a single-byte transfer, one byte is read or written and then CE is driven low. For a multiple-byte transfer, however, multiple bytes  can be read or  written to  the DS1305 after the  address has been written. Each  read or write cycle causes the RTC register or  RAM address to  automatically increment.  Incrementing continues  until  the  device  is  disabled.  When  the  RTC  is  selected,  the  address  wraps  to  00h  after  incrementing to 1Fh  (during  a read)  and wraps to 80h after incrementing to 9Fh (during  a write). When the RAM is selected, the address  wraps to 20h after incrementing to 7Fh (during  a read) and wraps to  A0h after incrementing to FFh (during a write).

Figure 8. SPI MULTIPLE-BYTE BURST TRANSFER

READING AND WRITING IN BURST MODE

Burst mode is similar to a single-byte  read or write, except that CE is kept high and additional  SCLK cycles are sent until the end of the burst. The clock registers and the user RAM can be read or written in burst mode. When accessing the clock registers  in  burst mode, the address pointer wraps  around after reaching 1Fh  (9Fh for  writes). When accessing the user RAM in burst mode, the address pointer  wraps around after reaching 7Fh (FFh for writes).

3-WIRE INTERFACE

The 3-wire interface mode operates similarly to the SPI mode. However, in 3-wire mode there is one I/O instead of separate data in and data out signals. The 3-wire interface consists of the  I/O (SDI and  SDO  pins tied together), CE, and SCLK pins. In 3-wire mode, each byte is shifted in LSB first unlike SPI mode where each byte is shifted in MSB first.

As is the case with the SPI mode, an address byte is written to the device followed by a single data byte

or multiple data bytes.  Figure 9 illustrates a read and  write  cycle.  In  3-wire mode, data is input on the

rising edge of SCLK and output on the falling edge of SCLK.

Figure 9. 3-WIRE SINGLE-BYTE TRANSFER

CE SCLK

I/O*

CE SCLK

| I/O* A0 | A1 | A2 | A3 | A4 | A5 | A6 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |

A0      A1     A2    A3    A4    A5   A6   0

SINGLE-BYTE READ

D0        D1        D2        D3         D4        D5         D6         D7

| SINGLE-BYTE WRITE D0 | D1 | D2 | D3 | D4 | D5 | D6 | D7 |
| --- | --- | --- | --- | --- | --- | --- | --- |

NOTE: IN BURST MODE, CE IS KEPT HIGH AND ADDITIONAL SCLK CYCLES ARE SENT UNTIL THE END OF THE BURST.

*I/O IS SDI AND SDO TIED TOGETHER.

ABSOLUTE MAXIMUM RATINGS

Voltage Range on Any Pin Relative to Groundﾅﾅﾅﾅﾅﾅﾅﾅﾅﾅﾅﾅﾅﾅﾅﾅﾅ..-0.5V to +7.0V Storage Temperature Rangeﾅﾅﾅﾅﾅﾅﾅﾅﾅﾅﾅﾅﾅﾅﾅﾅﾅﾅﾅﾅﾅﾅﾅﾅ.-55ﾰC to +125ﾰC Soldering Temperatureﾅﾅﾅﾅﾅﾅﾅﾅﾅﾅﾅﾅﾅﾅﾅﾅ.See IPC/JEDEC J-STD-020 Specification

This is a stress rating only and functional operation of the device at these or any other conditions beyond those indicated in the operation sections of  this specification is  not  implied. Exposure to  absolute maximum rating conditions for extended  periods  of time can  affect reliability.

OPERATING RANGE

| RANGE | TEMP RANGE | VCC (V) |
| --- | --- | --- |
| Commercial | 0ﾰC to +70ﾰC | 2.0 to 5.5 VCC1 or VCC2 |
| Industrial | -40ﾰC to +85ﾰC | 2.0 to 5.5 VCC1 or VCC2 |

RECOMMENDED DC OPERATING CONDITIONS

(Over the operating range, unless otherwise specified.)

| 2 PARAMETER | SYMBOL |  | MIN TYP MAX |  | UNITS | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| Supply Voltage VCC1, VCC2 | VCC1, VCC2 |  | .0 5.5 |  | V | 7 |
| Logic 1 Input | VIH |  | 2.0 VCC + 0.3 |  | V |  |
| Logic 0 Input | V | VCC = 2.0V | -0.3 | +0.3 | V |  |
|  |  | VCC = 5V |  | +0.8 |  |  |
| VBAT Battery Voltage | VBAT |  | 2.0 5.5 |  | V |  |
| VCCIF Supply Voltage | VCCIF |  | 2.0 5.5 |  | V | 11 |

IL

DC ELECTRICAL CHARACTERISTICS

(Over the operating range, unless otherwise specified.)

CC1

CC1A

| CC2 PARAMETER |  | SYMBOL |  | MIN TYP MAX | UNITS | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| Input Leakage |  | ILI |  | -100 +500 | ﾵA |  |
| Output Leakage |  | ILO |  | -1 1 | ﾵA |  |
| Logic 0 Output | IOL= 1.5mA | VOL | VCC = 2.0V | 0.4 | V |  |
|  | IOL = 4.0mA |  | VCC = 5V | 0.4 |  |  |
| Logic 1 Output | IOH = -0.4mA | VOH | VCCIF = 2.0V | 1.6 | V |  |
|  | IOH = -1.0mA |  | VCCIF = 5V | 2.4 |  |  |
| V Active Supply Current |  | I | VCC1 = 2.0V | 0.425 | mA | 2, 8 |
|  |  |  | VCC1 = 5V | 1.28 |  |  |
| VCC1 Timekeeping Current (Osc on) |  | ICC1T | VCC1 = 2.0V | 25.3 | ﾵA | 1, 8, 12 |
|  |  |  | VCC1 = 5V | 81 |  |  |
| VCC1 Standby Current (Osc off) |  | ICC1S | VCC1 = 2.0V | 25 | ﾵA | 6, 8, 12 |
|  |  |  | VCC1 = 5V | 80 |  |  |
| V Active Supply Current |  | I | VCC2 = 2.0V | 0.4 | mA | 2, 9 |
|  |  |  | VCC2 = 5V | 1.2 |  |  |
| VCC2 Timekeeping Current (Osc on) |  | ICC2T | VCC2 = 2.0V | 0.3 | ﾵA | 1, 9, 12 |
|  |  |  | VCC2 = 5V | 1 |  |  |
| VCC2 Standby Current (Osc off) |  | ICC2S | VCC2 = 2.0V | 200 | nA | 6, 9, 12 |
|  |  |  | VCC2 = 5V | 200 |  |  |
| Battery Timekeeping Current |  | IBAT | VBAT = 3V | 400 | nA | 10, 12 |
| Battery Standby Current |  | IBATS | VBAT = 3V | 200 | nA | 10, 12 |
| VCC Trip Point |  | VCCTP |  | VBAT + VBAT - 50 200 | mV |  |
| Trickle Charge Resistors |  | R1 |  | 2 | k |  |
|  |  | R2 |  | 4 |  |  |
|  |  | R3 |  | 8 |  |  |
| Trickle Charge Diode Voltage Drop |  | VTD |  | 0.7 | V |  |

CC2A

CAPACITANCE

| (TA = +25 C) PARAMETER | SYMBOL | MIN TYP MAX | UNITS | NOTES |
| --- | --- | --- | --- | --- |
| Input Capacitance | CI | 10 | pF |  |
| Output Capacitance | CO | 15 | pF |  |
| Crystal Capacitance | CX | 6 | pF |  |

3-WIRE AC ELECTRICAL CHARACTERISTICS

(Over the operating range, unless otherwise specified.) (Figure 10 and Figure 11)

DC

CDH

CDD

CL

CH

| CLK PARAMETER | SYMBOL |  | MIN TYP MAX | UNITS | NOTES |
| --- | --- | --- | --- | --- | --- |
| Data to CLK Setup | t | VCC = 2.0V | 200 | ns | 3,4 |
|  |  | VCC = 5V | 50 |  |  |
| CLK to Data Hold | t | VCC = 2.0V | 280 | ns | 3,4 |
|  |  | VCC = 5V | 70 |  |  |
| CLK to Data Delay | t | VCC = 2.0V | 800 | ns | 3,4,5 |
|  |  | VCC = 5V | 200 |  |  |
| CLK Low Time | t | VCC = 2.0V | 1000 | ns | 4 |
|  |  | VCC = 5V | 250 |  |  |
| CLK High Time | t | VCC = 2.0V | 1000 | ns | 4 |
|  |  | VCC = 5V | 250 |  |  |
| CLK Frequency | t | VCC = 2.0V | 0.6 | MHz | 4 |
|  |  | VCC = 5V | DC 2.0 |  |  |
| CLK Rise and Fall | t t | VCC = 2.0V | 2000 | ns |  |
|  |  | VCC = 5V | 500 |  |  |
| CE to CLK Setup | t | VCC = 2.0V | 4 | ﾵs | 4 |
|  |  | VCC = 5V | 1 |  |  |
| CLK to CE Hold | t | VCC = 2.0V | 240 | ns | 4 |
|  |  | VCC = 5V | 60 |  |  |
| CE Inactive Time | t | VCC = 2.0V | 4 | ﾵs | 4 |
|  |  | VCC = 5V | 1 |  |  |
| CE to Output High-Z | t | VCC = 2.0V | 280 | ns | 3,4 |
|  |  | VCC = 5V | 70 |  |  |
| SCLK to Output High-Z | t | VCC = 2.0V | 280 | ns | 3,4 |
|  |  | VCC = 5V | 70 |  |  |

R,  F

CC

CCH

CWH

CDZ

CCZ

Figure 10. TIMING DIAGRAM: 3-WIRE READ DATA TRANSFER

* I/O IS SDI AND SDO TIED TOGETHER.

Figure 11. TIMING DIAGRAM: 3-WIRE WRITE DATA TRANSFER

* I/O IS SDI AND SDO TIED TOGETHER.

SPI AC ELECTRICAL CHARACTERISTICS

(Over the operating range, unless otherwise specified.) (Figure 12 and Figure 13)

DC

CDH

CDD

CL

CH

| CLK PARAMETER | SYMBOL |  | MIN TYP MAX | UNITS | NOTES |
| --- | --- | --- | --- | --- | --- |
| Data to CLK Setup | t | VCC = 2.0V | 200 | ns | 5,6 |
|  |  | VCC = 5V | 50 |  |  |
| CLK to Data Hold | t | VCC = 2.0V | 280 | ns | 5,6 |
|  |  | VCC = 5V | 70 |  |  |
| CLK to Data Delay | t | VCC = 2.0V | 800 | ns | 5,6,7 |
|  |  | VCC = 5V | 200 |  |  |
| CLK Low Time | t | VCC = 2.0V | 1000 | ns | 6 |
|  |  | VCC = 5V | 250 |  |  |
| CLK High Time | t | VCC = 2.0V | 1000 | ns | 6 |
|  |  | VCC = 5V | 250 |  |  |
| CLK Frequency | t | VCC = 2.0V | 0.6 | MHz | 6 |
|  |  | VCC = 5V | DC 2.0 |  |  |
| CLK Rise and Fall | t t | VCC = 2.0V | 2000 | ns |  |
|  |  | VCC = 5V | 500 |  |  |
| CE to CLK Setup | t | VCC = 2.0V | 4 | ﾵs | 6 |
|  |  | VCC = 5V | 1 |  |  |
| CLK to CE Hold | t | VCC = 2.0V | 240 | ns | 6 |
|  |  | VCC = 5V | 60 |  |  |
| CE Inactive Time | t | VCC = 2.0V | 4 | ﾵs | 6 |
|  |  | VCC = 5V | 1 |  |  |
| CE to Output High-Z | t | VCC = 2.0V | 280 | ns | 5,6 |
|  |  | VCC = 5V | 70 |  |  |

R,  F

CC

CCH

CWH

CDZ

Figure 12. TIMING DIAGRAM: SPI READ DATA TRANSFER

* SCLK CAN BE EITHER POLARITY, TIMING SHOWN FOR CPOL = 1.

Figure 13. TIMING DIAGRAM: SPI WRITE DATA TRANSFER

* SCLK CAN BE EITHER POLARITY, TIMING SHOWN FOR CPOL = 1.

NOTES:

1)  ICC1T and ICC2T are specified with CE set to a logic 0 and  EOSC  bit = 0 (oscillator enabled).

2)  ICC1A  and  ICC2A  are  specified  with  CE  =  VCC,  SCLK=2MHz  at  VCC  =  5V;  SCLK  =  500kHz

at VCC = 2.0V, VIL = 0V, VIH = VCC, and  EOSC  bit = 0 (oscillator enabled).

3)  Measured at VIH = 2.0V or VIL = 0.8V and 10ms maximum rise and fall time.

4)   Measured with 50pF load.

5)  Measured at VOH = 2.4V or VOL = 0.4V.

6)  ICC1S and ICC2S are specified with CE set to a logic 0. The  EOSC  bit must be set to logic 1 (oscillator

disabled).

7)  VCC = VCC1, when VCC1 > VCC2 + 0.2V (typical); VCC = VCC2, when VCC2 > VCC1.

8)  VCC2 = 0V.

9)  VCC1 = 0V.

10) VCC1 < VBAT.

11) VCCIF must be less than or equal to the largest of VCC1, VCC2, and VBAT.

12) Using a crystal on X1 and X2, rated for 6pF load.

21 of 21

Maxim/Dallas Semiconductor cannot assume responsibility for use of any circuitry other than circuitry entirely embodied in a Maxim/Dallas Semiconductor product. No circuit patent licenses are implied. Maxim/Dallas Semiconductor reserves the right to change the circuitry and specifications without notice at any time.

M a x i m  In teg r a t ed  P r od uc ts , 120   San   Ga b r i e l D r iv e ,  Su nny va l e ,  CA  9 4086  4 0 8 - 73 7 - 76 00

ﾩ 2005 Maxim Integrated Products   Printed USA

The Maxim logo is a registered trademark of Maxim Integrated Products, Inc. The Dallas logo is a registered trademark of Dallas Semiconductor Corporation.