FEATURES

	Real-Time Clock (RTC) Counts Seconds, Minutes, Hours, Date of the Month, Month,

DS1306

Serial Alarm Real-Time Clock

PIN CONFIGURATIONS

Day of the Week, and Year with Leap-Year

Compensation Valid Up to 2100

	96-Byte, Battery-Backed NV RAM for Data

Storage

	Two Time-of-Day Alarms, Programmable on

Combination of Seconds, Minutes, Hours, and Day of the Week

	1Hz and 32.768kHz Clock Outputs

	Supports Motorola SPI™ (Serial Peripheral

Interface) Modes 1 and 3 or Standard 3-Wire

VCC2

VBAT

X1

N.C. X2

N.C.

INT0

INT1

1Hz

GND

1	20

2	19

3	18

4	17

5	16

6	15

7	14

8	13

9	12

10	11

TSSOP (4.4mm)

VCC1

N.C.

32kHz VCCIF SDO SDI SCLK N.C. CE

SERMODE

Interface

	Burst Mode for Reading/Writing Successive

Addresses in Clock/RAM

	Dual-Power Supply Pins for Primary and

Backup Power Supplies

	Optional Trickle Charge Output to Backup

Supply

	2.0V to 5.5V Operation

	Optional Industrial Temperature Range:

VCC2	1

VBAT	2

X1	3

X2	4

INT0	5

INT1	6

1Hz	7

GND	8

16	VCC1

15	32kHz

14	VCCIF

13	SDO

12	SDI

11	SCLK

10	CE

9	SERMODE

-40°C to +85°C

	Available in Space-Efficient, 20-Pin TSSOP Package

	Underwriters Laboratory (UL) Recognized

DIP (300 mils)

TYPICAL OPERATING CIRCUIT

SPI is a trademark of Motorola, Inc.

1 of 21	REV: 071105

ORDERING INFORMATION

| PART | TEMP RANGE | PIN-PACKAGE | TOP MARK* |
| --- | --- | --- | --- |
| DS1306 | 0°C to +70°C | 16 DIP (300 mils) | DS1306 |
| DS1306+ | 0°C to +70°C | 16 DIP (300 mils) | DS1306 + |
| DS1306N | -40°C to +85°C | 16 DIP (300 mils) | DS1306N |
| DS1306N+ | 0°C to +70°C | 16 DIP (300 mils) | DS1306N + |
| DS1306E+ | 0°C to +70°C | 20 TSSOP (173 mils) | DS1306 + |
| DS1306EN+ | -40°C to +85°C | 20 TSSOP (173 mils) | DS1306N + |
| DS1306EN+T&R | -40°C to +85°C | 20 TSSOP (173 mils) | DS1306N + |
| DS1306E+T&R | 0°C to +70°C | 20 TSSOP (173 mils) | DS1306 + |
| DS1306E | 0°C to +70°C | 20 TSSOP (173 mils) | DS1306 |
| DS1306EN | -40°C to +85°C | 20 TSSOP (173 mils) | DS1306N |
| DS1306EN/T&R | -40°C to +85°C | 20 TSSOP (173 mils) | DS1306N |
| DS1306E/T&R | 0°C to +70°C | 20 TSSOP (173 mils) | DS1306 |

+Denotes a lead(Pb)-free/RoHS-compliant device.

* An “N” on the top mark indicates an industrial device.

| PIN DESCRIPTION PIN |  | NAME | FUNCTION |
| --- | --- | --- | --- |
| TSSOP | DIP |  |  |
| 1 | 1 | VCC2 | Backup Power Supply. This is the secondary power supply pin. In systems using the trickle charger, the rechargeable energy source is connected to this pin. |
| 2 | 2 | VBAT | Battery Input for Any Standard +3V Lithium Cell or Other Energy Source. If not used, VBAT must be connected to ground. Diodes must not be placed in series between VBAT and the battery, or improper operation will result. UL recognized to ensure against reverse charging current when used in conjunction with a lithium battery. See “Conditions of Acceptability” at www.maxim-ic.com/TechSupport/QA/ntrl.htm. |
| 3 | 3 | X1 | Connections for Standard 32.768kHz Quartz Crystal. The internal oscillator is designed for operation with a crystal having a specified load capacitance of 6pF. For more information on crystal selection and crystal layout considerations, refer to Application Note 58, “Crystal Considerations with Dallas Real-Time Clocks.” The DS1306 can also be driven by an external 32.768kHz oscillator. In this configuration, the X1 pin is connected to the external oscillator signal and the X2 pin is floated. |
| 5 | 4 | X2 |  |
| 7 | 5 | INT0 | Active-Low Interrupt 0 Output. The INT0 pin is an active-low output of the DS1306 that can be used as an interrupt input to a processor. The INT0 pin can be programmed to be asserted by Alarm 0. The INT0 pin remains low as long as the status bit causing the interrupt is present and the corresponding interrupt enable bit is set. The INT0 pin operates when the DS1306 is powered by VCC1, VCC2, or VBAT. The INT0 pin is an open-drain output and requires an external pullup resistor. |
| 8 | 6 | INT1 | Interrupt 1 Output. The INT1 pin is an active high output of the DS1306 that can be used as an interrupt input to a processor. The INT1 pin can be programmed to be asserted by Alarm 1. When an alarm condition is present, the INT1 pin generates a 62.5ms active-high pulse. The INT1 pin operates only when the DS1306 is powered by VCC2 or VBAT. When active, the INT1 pin is internally pulled up to VCC2 or VBAT. When inactive, the INT1 pin is internally pulled low. |

PIN DESCRIPTION (continued)

| PIN |  | NAME | FUNCTION |
| --- | --- | --- | --- |
| TSSOP | DIP |  |  |
| 9 | 7 | 1Hz | 1Hz Output. The 1Hz pin provides a 1Hz square wave output. This output is active when the 1 Hz bit in the control register is a logic 1. Both INT0 and 1Hz pins are open-drain outputs. The interrupt, 1Hz signal, and the internal clock continue to run regardless of the level of VCC (as long as a power source is present). |
| 10 | 8 | GND | Ground |
| 11 | 9 | SERMODE | Serial Interface Mode. The SERMODE pin offers the flexibility to choose between two serial interface modes. When connected to GND, standard 3-wire communication is selected. When connected to VCC, SPI communication is selected. |
| 12 | 10 | CE | Chip Enable. The chip enable signal must be asserted high during a read or a write for both 3-wire and SPI communication. This pin has an internal 55k pulldown resistor (typical). |
| 14 | 11 | SCLK | Serial Clock. SCLK is used to synchronize data movement on the serial interface for either the SPI or 3-wire interface. |
| 15 | 12 | SDI | Serial Data In. When SPI communication is selected, the SDI pin is the serial data input for the SPI bus. When 3-wire communication is selected, this pin must be tied to the SDO pin (the SDI and SDO pins function as a single I/O pin when tied together). |
| 16 | 13 | SDO | Serial Data Out. When SPI communication is selected, the SDO pin is the serial data output for the SPI bus. When 3-wire communication is selected, this pin must be tied to the SDI pin (the SDI and SDO pins function as a single I/O pin when tied together). VCCIF provides the logic high level. |
| 17 | 14 | VCCIF | Interface Logic Power-Supply Input. The VCCIF pin allows the DS1306 to drive SDO and 32kHz output pins to a level that is compatible with the interface logic, thus allowing an easy interface to 3V logic in mixed supply systems. This pin is physically connected to the source connection of the p- channel transistors in the output buffers of the SDO and 32kHz pins. |
| 18 | 15 | 32kHz | 32.768kHz Output. The 32kHz pin provides a 32.768kHz output. This signal is always present. VCCIF provides the logic high level. |
| 20 | 16 | VCC1 | Primary Power Supply. DC power is provided to the device on this pin. VCC1 is the primary power supply. |
| 4, 6, 13, 19 | — | N.C. | No Connection |

DESCRIPTION

The DS1306 serial alarm real-time clock (RTC) provides a full binary coded decimal (BCD) clock calendar that is accessed by a simple serial interface. The clock/calendar provides seconds, minutes, hours, day, date, month, and year information. The end of the month date is automatically adjusted for months with fewer than 31 days, including corrections for leap year. The clock operates in either the 24- hour or 12-hour format with AM/PM indicator. In addition, 96 bytes of NV RAM are provided for data storage.

An interface logic-power supply input pin (VCCIF) allows the DS1306 to drive SDO and 32kHz pins to a level that is compatible with the interface logic. This allows an easy interface to 3V logic in mixed supply systems. The DS1306 offers dual-power supplies as well as a battery-input pin. The dual-power supplies support a programmable trickle charge circuit that allows a rechargeable energy source (such as a super cap or rechargeable battery) to be used for a backup supply. The VBAT pin allows the device to be backed up by a non-rechargeable battery. The DS1306 is fully operational from 2.0V to 5.5V.

Two  programmable  time-of-day  alarms  are  provided  by  the  DS1306.  Each  alarm  can  generate  an interrupt on a programmable combination of seconds, minutes, hours, and day. “Don’t care” states can be inserted into one or more fields if it is desired for them to be ignored for the alarm condition. A 1Hz and a

32kHz clock output are also available.

The DS1306 supports a direct interface to SPI serial data ports or standard 3-wire interface. An easy-to- use address and data format is implemented in which data transfers can occur 1 byte at a time or in multiple-byte burst mode.

OPERATION

The block diagram in Figure 1 shows the main elements of the serial alarm RTC. The following paragraphs describe the function of each pin.

Figure 1. BLOCK DIAGRAM

1Hz

RECOMMENDED LAYOUT FOR CRYSTAL

Local ground plane (Layer 2)

crystal

X1

X2

GND

CLOCK ACCURACY

The accuracy of the clock is dependent upon the accuracy of the crystal and the accuracy of the match between the capacitive load of the oscillator circuit and the capacitive load for which the crystal was trimmed. Additional error is added by crystal frequency drift caused by temperature shifts. External circuit noise coupled into the oscillator circuit can result in the clock running fast. Refer to Application Note 58: Crystal Considerations with Dallas Real-Time Clocks for detailed information.

CLOCK, CALENDAR, AND ALARM

The time and calendar information is obtained by reading the appropriate register bytes. The RTC registers are illustrated in Figure 2. The time, calendar, and alarm are set or initialized by writing the appropriate register bytes. Note that some bits are set to 0. These bits always read 0 regardless of how they are written. Also note that registers 12h to 1Fh (read) and registers 92h to 9Fh are reserved. These registers always read 0 regardless of how they are written. The contents of the time, calendar, and alarm registers are in the BCD format..  Values in the day register that correspond to the day of the week are user-defined, but must be sequential (e.g. if 1 equals Sunday, 2 equals Monday and so on). The day register increments at midnight. Illogical time and date entries result in undefined operation.

WRITING TO THE CLOCK REGISTERS

The internal time and date registers continue to increment during write operations. However, the countdown chain is reset when the seconds register is written. Writing the time and date registers within one second after writing the seconds register ensures consistent data.

Terminating a write before the last bit is sent aborts the write for that byte.

READING FROM THE CLOCK REGISTERS

Buffers are used to copy the time and date register at the beginning of a read. When reading in burst mode, the user copy is static while the internal registers continue to increment.

Figure 2. RTC REGISTERS AND ADDRESS MAP

| HEX ADDRESS |  | Bit 7 | Bit6 | Bit5 |  | Bit4 | Bit3 | Bit2 | Bit1 | Bit0 |  | RANGE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| READ | WRITE |  |  |  |  |  |  |  |  |  |  |  |
| 00H | 80H | 0 | 10 SEC |  |  |  | SEC |  |  |  |  | 00–59 |
| 01H | 81H | 0 | 10 MIN |  |  |  | MIN |  |  |  |  | 00–59 |
| 02H | 82H | 0 | 12 | P |  | 10-HR | HOURS |  |  |  |  | 01–12 + P/A |
|  |  |  |  | A |  |  |  |  |  |  |  |  |
|  |  |  | 24 | 10 |  |  |  |  |  |  |  | 00–23 |
| 03H | 83H | 0 | 0 | 0 |  | 0 | 0 | DAY |  |  |  | 01–07 |
| 04H | 84H | 0 | 0 | 10-DATE |  |  | DATE |  |  |  |  | 1–31 |
| 05H | 85H | 0 | 0 | 10-MONTH |  |  | MONTH |  |  |  |  | 01–12 |
| 06H | 86H | 10-YEAR |  |  |  |  | YEAR |  |  |  |  | 00–99 |
| 07H | 87H | M | 10-SEC ALARM 0 |  |  |  | SEC ALARM 0 |  |  |  |  | 00–59 |
| 08H | 88H | M | 10-MIN ALARM 0 |  |  |  | MIN ALARM 0 |  |  |  |  | 00–59 |
| 09H | 89H | M | 12 | P | 10-HR |  | HOUR ALARM 0 |  |  |  |  | 01–12 + P/A |
|  |  |  |  | A |  |  |  |  |  |  |  |  |
|  |  |  | 24 | 10 |  |  |  |  |  |  |  | 00–23 |
| 0AH | 8AH | M | 0 | 0 | 0 |  | 0 | DAY ALARM 0 |  |  |  | 01–07 |
| 0BH | 8BH | M | 10 SEC ALARM 1 |  |  |  | SEC ALARM 1 |  |  |  |  | 00–59 |
| 0CH | 8CH | M | 10 MIN ALARM 1 |  |  |  | MIN ALARM 1 |  |  |  |  | 00–59 |
| 0DH | 8DH | M | 12 | P | 10-HR |  | HOUR ALARM 1 |  |  |  |  | 01–12 + P/A |
|  |  |  |  | A |  |  |  |  |  |  |  |  |
|  |  |  | 24 | 10 |  |  |  |  |  |  |  | 00–23 |
| 0EH | 8EH | M | 0 | 0 | 0 |  | 0 | DAY ALARM 1 |  |  |  | 01–07 |
| — |  |  |  |  |  |  |  |  |  |  |  |  |
| 0FH | 8FH | CONTROL REGISTER |  |  |  |  |  |  |  |  | — |  |
| 10H | 90H | STATUS REGISTER |  |  |  |  |  |  |  |  | — |  |
| 11H | 91H | TRICKLE CHARGER REGISTER |  |  |  |  |  |  |  |  | — |  |
| 12–1FH | 92–9FH | RESERVED |  |  |  |  |  |  |  |  | — |  |
| 20–7FH | A0– FFH | 96-BYTES USER RAM |  |  |  |  |  |  |  |  | — |  |

Note: Range for alarm registers does not include mask’m’ bits.

The DS1306 can be run in either 12-hour or 24-hour mode.  Bit 6 of the hours register is defined as the

12- or 24-hour mode select bit. When high, the 12-hour mode is selected.  In the 12-hour mode, bit 5 is the AM/PM bit with logic high being PM. In the 24-hour mode, bit 5 is the second 10-hour bit (20 to 23 hours).

The DS1306 contains two time-of-day alarms. Time-of-day alarm 0 can be set by writing to registers 87h to 8Ah. Time-of-day Alarm 1 can be set by writing to registers 8 Bh to 8 Eh. Bit 7 of each of the time-of- day alarm registers are mask bits (Table 1). When all of the mask bits are logic 0, a time-of-day alarm only occurs once per week when the values stored in timekeeping registers 00h to 03h match the values stored in the time-of-day alarm registers. An alarm is generated every day when bit 7 of the day alarm register is set to a logic 1. An alarm is generated every hour when bit 7 of the day and hour alarm registers is set to a logic 1. Similarly, an alarm is generated every minute when bit 7 of the day, hour, and

minute alarm registers is set to a logic 1. When bit 7 of the day, hour, minute, and seconds alarm registers is set to a logic 1, an alarm occurs every second.

During each clock update, the RTC compares the Alarm 0 and Alarm 1 registers with the corresponding clock registers. When a match occurs, the corresponding alarm flag bit in the status register is set to a 1. If the corresponding alarm interrupt enable bit is enabled, an interrupt output is activated.

Table 1. TIME-OF-DAY ALARM MASK BITS

| ALARM REGISTER MASK BITS (BIT 7) |  |  |  | FUNCTION |
| --- | --- | --- | --- | --- |
| SECONDS | MINUTES | HOURS | DAYS |  |
| 1 | 1 | 1 | 1 | Alarm once per second |
| 0 | 1 | 1 | 1 | Alarm when seconds match |
| 0 | 0 | 1 | 1 | Alarm when minutes and seconds match |
| 0 | 0 | 0 | 1 | Alarm hours, minutes, and seconds match |
| 0 | 0 | 0 | 0 | Alarm day, hours, minutes and seconds match |

SPECIAL PURPOSE REGISTERS

The DS1306 has three additional registers (control register, status register, and trickle charger register)

that control the real-time clock, interrupts, and trickle charger.

CONTROL REGISTER (READ 0FH, WRITE 8FH)

| BIT7 | BIT6 | BIT5 | BIT4 | BIT3 | BIT2 | BIT1 | BIT0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | WP | 0 | 0 | 0 | 1Hz | AIE1 | AIE0 |

WP (Write Protect) – Before any write operation to the clock or RAM, this bit must be logic 0. When high, the write protect bit prevents a write operation to any register, including bits 0, 1, and 2 of the control register. Upon initial power-up, the state of the WP bit is undefined. Therefore, the WP bit should be cleared before attempting to write to the device. When WP is set, it must be cleared before any other control register bit can be written.

1Hz (1Hz Output Enable) – This bit controls the 1Hz output. When this bit is a logic 1, the 1Hz output is enabled. When this bit is a logic 0, the 1Hz output is high-Z.

AIE0 (Alarm Interrupt Enable 0) – When set to a logic 1, this bit permits the interrupt 0 request flag (IRQF0) bit in the status register to assert INT0 . When the AIE0 bit is set to logic 0, the IRQF0 bit does not initiate the INT0 signal.

AIE1 (Alarm Interrupt Enable 1) – When set to a logic 1, this bit permits the interrupt 1 request flag (IRQF1) bit in the status register to assert INT1. When the AIE1 bit is set to logic 0, the IRQF1 bit does not initiate an interrupt signal, and the INT1 pin is set to a logic 0 state.

STATUS REGISTER (READ 10H)

| BIT7 | BIT6 | BIT5 | BIT4 | BIT3 | BIT2 | BIT1 | BIT0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | IRQF1 | IRQF0 |

IRQF0 (Interrupt 0 Request Flag) – A logic 1 in the interrupt request flag bit indicates that the current time has matched the Alarm 0 registers. If the AIE0 bit is also a logic 1, the INT0 pin goes low. IRQF0 is cleared when the address pointer goes to any of the Alarm 0 registers during a read or write. IRQF0 is

activated when the device is powered by VCC1, VCC2, or VBAT.

IRQF1 (Interrupt 1 Request Flag) – A logic 1 in the interrupt request flag bit indicates that the current time has matched the Alarm 1 registers. If the AIE1 bit is also a logic 1, the INT1 pin generates a 62.5ms active-high pulse. IRQF1 is cleared when the address pointer goes to any of the alarm 1 registers during a read or write. IRQF1 is activated only when the device is powered by VCC2 or VBAT.

TRICKLE CHARGE REGISTER (READ 11H, WRITE 91H)

This register controls the trickle charge characteristics of the DS1306. The simplified schematic of Figure

3 shows the basic components of the trickle charger. The trickle charge select (TCS) bits (bits 4–7) control the selection of the trickle charger. In order to prevent accidental enabling, only a pattern of 1010 enables the trickle charger. All other patterns disable the trickle charger. The DS1306 powers up with the trickle charger disabled. The diode select (DS) bits (bits 2–3) select whether one diode or two diodes are connected between VCC1 and VCC2. The diode select (DS) bits (bits 2–3) select whether one diode or two diodes are connected between VCC1  and VCC2. The resistor select (RS) bits select the resistor that is connected between VCC1 and VCC2. The resistor and diodes are selected by the RS and DS bits as shown in Table 2.

Figure 3. PROGRAMMABLE TRICKLE CHARGER

Table 2. TRICKLE CHARGER RESISTOR AND DIODE SELECT

| TCS Bit 7 | TCS Bit 6 | TCS Bit 5 | TCS Bit 4 | DS Bit 3 | DS Bit 2 | RS Bit 1 | RS Bit 0 | FUNCTION |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X | X | X | X | X | X | 0 | 0 | Disabled |
| X | X | X | X | 0 | 0 | X | X | Disabled |
| X | X | X | X | 1 | 1 | X | X | Disabled |
| 1 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 1 Diode, 2kΩ |
| 1 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 1 Diode, 4kΩ |
| 1 | 0 | 1 | 0 | 0 | 1 | 1 | 1 | 1 Diode, 8kΩ |
| 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 2 Diodes, 2kΩ |
| 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 2 Diodes, 4kΩ |
| 1 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 2 Diodes, 8kΩ |
| 0 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | Initial power-on state |

If RS is 00, the trickle charger is disabled independently of TCS.

Diode and resistor selection is determined by the user according to the maximum current desired for battery or super cap charging. The maximum charging current can be calculated as illustrated in the following example. Assume that a system power supply of 5V is applied to VCC1  and a super cap is connected to VCC2. Also assume that the trickle charger has been enabled with one diode and resister R1 between VCC1 and VCC2. The maximum current IMAX would, therefore, be calculated as follows:

IMAX = (5.0V - diode drop) / R1  (5.0V - 0.7V) / 2kΩ  2.2mA

As the super cap charges, the voltage drop between VCC1 and VCC2 decreases and, therefore, the charge current decreases.

POWER CONTROL

Power is provided through the VCC1, VCC2, and VBAT  pins. Three different power supply configurations are illustrated in Figure 4. Configuration 1 shows the DS1306 being backed up by a non-rechargeable energy source such as a lithium battery. In this configuration, the system power supply is connected to VCC1  and VCC2  is grounded. When VCC  falls below VBAT the device switches into a low-current battery backup  mode.  Upon  power-up,  the  device  switches  from  VBAT   to  VCC   when  VCC   is  greater  than VBAT + 0.2V. The device is write-protected whenever it is switched to VBAT.

Configuration 2 illustrates the DS1306 being backed up by a rechargeable energy source. In this case, the VBAT  pin is grounded, VCC1  is connected to the primary power supply, and VCC2  is connected to the secondary supply (the rechargeable energy source). The DS1306 operates from the larger of VCC1  or VCC2. When VCC1 is greater than VCC2 + 0.2V (typical), VCC1 powers the DS1306. When VCC1 is less than VCC2, VCC2 powers the DS1306. The DS1306 does not write-protect itself in this configuration.

Configuration 3 shows the DS1306 in battery-operate mode, where the device is powered only by a single battery. In this case, the VCC1 and VBAT pins are grounded and the battery is connected to the VCC2 pin.

Only these three configurations are allowed. Unused supply pins must be grounded.

Figure 4. POWER SUPPLY CONFIGURATIONS

CONFIGURATION 1: BACKUP SUPPLY IS NONRECHARGEABLE LITHIUM BATTERY

NOTE: DEVICE IS WRITE-PROTECTED IF VCC < VCCTP.

CONFIGURATION 2: BACKUP SUPPLY IS A RECHARGEABLE BATTERY OR SUPER CAPACITOR

NOTE: DEVICE DOES NOT PROVIDE AUTOMATIC WRITE PROTECTION.

CONFIGURATION 3: BATTERY OPERATE MODE

SERIAL INTERFACE

The DS1306 offers the flexibility to choose between two serial interface modes. The DS1306 can communicate with the SPI interface or with a standard 3-wire interface. The interface method used is determined by the SERMODE pin. When this pin is connected to VCC, SPI communication is selected. When this pin is connected to ground, standard 3-wire communication is selected.

SERIAL PERIPHERAL INTERFACE (SPI)

The serial peripheral interface (SPI) is a synchronous bus for address and data transfer and is used when interfacing with the SPI bus on specific Motorola microcontrollers such as the 68HC05C4 and the

68HC11A8. The SPI mode of serial communication is selected by tying the SERMODE pin to VCC.

Four pins are used for the SPI. The four pins are the SDO (serial data out), SDI (serial data in), CE (chip enable), and SCLK (serial clock). The DS1306 is the slave device in an SPI application, with the microcontroller being the master.

The SDI and SDO pins are the serial data input and output pins for the DS1306, respectively. The CE input  is  used  to  initiate  and  terminate  a  data  transfer.  The  SCLK  pin  is  used  to  synchronize  data movement between the master (microcontroller) and the slave (DS1306) devices.

The shift clock (SCLK), which is generated by the microcontroller, is active only during address and data transfer to any device on the SPI bus. The inactive clock polarity is programmable in some microcontrollers. The DS1306 determines on the clock polarity by sampling SCLK when CE becomes active. Therefore either SCLK polarity can be accommodated. Input data (SDI) is latched on the internal strobe edge and output data (SDO) is shifted out on the shift edge (Figure 5). There is one clock for each bit transferred. Address and data bits are transferred in groups of eight.

Figure 5. SERIAL CLOCK AS A FUNCTION OF MICROCONTROLLER CLOCK POLARITY (CPOL)

CPOL = 1

CPOL = 0

CE SCLK

SCLK

SHIFT DATA OUT (READ)	DATA LATCH (WRITE)

SHIFT DATA OUT (READ)	DATA LATCH (WRITE)

NOTE 1: CPHA BIT POLARITY (IF APPLICABLE) MAY NEED TO BE SET ACCORDINGLY.

NOTE 2: CPOL IS A BIT THAT IS SET IN THE MICROCONTROLLER’S CONTROL REGISTER.

NOTE 3: SDO REMAINS AT HIGH-Z UNTIL 8 BITS OF DATA ARE READY TO BE SHIFTED OUT DURING A READ.

ADDRESS AND DATA BYTES

Address and data bytes are shifted MSB first into the serial data input (SDI) and out of the serial data output (SDO). Any transfer requires the address of the byte to specify a write or read to either a RTC or RAM location, followed by one or more bytes of data. Data is transferred out of the SDO for a read operation and into the SDI for a write operation (Figures 6 and 7).

Figure 6. SPI SINGLE-BYTE WRITE

* SCLK CAN BE EITHER POLARITY.

Figure 7. SPI SINGLE-BYTE READ

* SCLK CAN BE EITHER POLARITY.

The address byte is always the first byte entered after CE is driven high. The most significant bit (A7) of this byte determines if a read or write takes place. If A7 is 0, one or more read cycles occur. If A7 is 1, one or more write cycles occur.

Data transfers can occur one byte at a time or in multiple-byte burst mode. After CE is driven high an address is written to the DS1306. After the address, 1 or more data bytes can be written or read. For a single-byte transfer, one byte is read or written and then CE is driven low. For a multiple-byte transfer, however, multiple bytes can be read or written to the DS1306 after the address has been written. Each read or write cycle causes the RTC register or RAM address to automatically increment. Incrementing continues until the device is disabled. When the RTC is selected, the address wraps to 00h after incrementing to 1Fh (during a read) and wraps to 80h after incrementing to 9Fh (during a write). When the RAM is selected, the address wraps to 20h after incrementing to 7Fh (during a read) and wraps to A0h after incrementing to FFh (during a write).

Figure 8. SPI MULTIPLE-BYTE BURST TRANSFER

READING AND WRITING IN BURST MODE

Burst mode is similar to a single-byte read or write, except that CE is kept high and additional SCLK cycles are sent until the end of the burst. The clock registers and the user RAM may be read or written in burst mode. When accessing the clock registers in burst mode, the address pointer will wrap around after reaching 1Fh (9Fh for writes). When accessing the user RAM in burst mode, the address pointer wraps around after reaching 7Fh (FFh for writes).

3-WIRE INTERFACE

The 3-wire interface mode operates similar to the SPI mode. However, in 3-wire mode there is one I/O

instead of separate data in and data out signals. The 3-wire interface consists of the I/O (SDI and SDO pins tied together), CE, and SCLK pins. In 3-wire mode, each byte is shifted in LSB first, unlike SPI mode, where each byte is shifted in MSB first.

As is the case with the SPI mode, an address byte is written to the device followed by a single data byte or multiple data bytes. Figure 9 illustrates a read and write cycle. In 3-wire mode, data is input on the rising edge of SCLK and output on the falling edge of SCLK.

Figure 9. 3-WIRE SINGLE BYTE TRANSFER

SINGLE-BYTE READ

CE

SCLK

I/O*

A0	A1	A2

A3	A4

A5   A6   0

D0	D1	D2	D3	D4	D5	D6	D7

| SINGLE-BYTE WRITE D0 | D1 | D2 | D3 | D4 | D5 | D6 | D7 |
| --- | --- | --- | --- | --- | --- | --- | --- |

CE

| SCLK I/O* A0 | A1 | A2 | A3 | A4 | A5 | A6 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |

NOTE: IN BURST MODE, CE IS KEPT HIGH AND ADDITIONAL SCLK CYCLES ARE SENT UNTIL THE END OF THE BURST.

*I/O IS SDI AND SDO TIED TOGETHER.

ABSOLUTE MAXIMUM RATINGS

Voltage Range on Any Pin Relative to Ground……………………………………………..-0.5V to +7.0V

Storage Temperature Range……………………………………………………………….-55C to +125C Soldering Temperature.……………………………….Refer to the IPC/JEDEC Standard J-STD-020

Specification

This is a stress rating only and functional operation of the device at these or any other conditions beyond those indicated in the operation sections of this specification is not implied. Exposure to absolute maximum rating conditions for extended periods of time can affect reliability.

OPERATING RANGE

| RANGE | TEMP RANGE | VCC (V) |
| --- | --- | --- |
| Commercial | 0°C to +70°C | 2.0 to 5.5 VCC1 or VCC2 |
| Industrial | -40°C to +85°C | 2.0 to 5.5 VCC1 or VCC2 |

RECOMMENDED DC OPERATING CONDITIONS

(TA = Over the operating range, unless otherwise specified.)

| IL PARAMETER | SYMBOL |  | MIN TYP MAX |  | UNITS | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| Supply Voltage VCC1, VCC2 | VCC1, VCC2 |  | 2.0 5.5 |  | V | 1, 8 |
| Logic 1 Input | VIH |  | 2.0 VCC + 0.3 |  | V |  |
| Logic 0 Input | V | VCC = 2.0V | -0.3 | +0.3 | V |  |
|  |  | VCC = 5V |  | +0.8 |  |  |
| VBAT Battery Voltage | VBAT |  | 2.0 5.5 |  | V |  |
| VCCIF Supply Voltage | VCCIF |  | 2.0 5.5 |  | V | 10 |

DC ELECTRICAL CHARACTERISTICS

| (TA = Over the operating range, unless otherwise specified.) PARAMETER |  | SYMBOL |  | MIN TYP MAX | UNITS | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| Input Leakage |  | ILI |  | -100 +500 | A |  |
| Output Leakage |  | ILO |  | -1 +1 | A |  |
| Logic 0 Output | IOL = 1.5mA | V | VCC = 2.0 | 0.4 | V |  |
|  | IOL = 4.0mA |  | VCC = 5V | 0.4 |  |  |
| Logic 1 Output | IOH = -0.4mA | V | VCCIF = 2.0V | 1.6 | V |  |
|  | IOH = -1.0mA |  | VCCIF = 5V | 2.4 |  |  |
| Logic 1 Output Current (INT1 pin) |  | IOH, INT1 | (VCC2, VBAT) -0.3V | -100 | A |  |
| V Active Supply Current |  | I | VCC1 = 2.0V | 0.425 | mA | 2, 7 |
|  |  |  | VCC1 = 5V | 1.28 |  |  |
| V Timekeeping Current |  | I | VCC1 = 2.0V | 25.3 | A | 1, 7 |
|  |  |  | VCC1 = 5V | 81 |  |  |
| V Active Supply Current |  | I | VCC2 = 2.0V | 0.4 | mA | 2, 8 |
|  |  |  | VCC2 = 5V | 1.2 |  |  |
| V Timekeeping Current |  | I | VCC2 = 2.0V | 0.4 | A | 1, 8 |
|  |  |  | VCC2 = 5V | 1 |  |  |
| Battery Timekeeping Current |  | IBAT | VBAT = 3V | 550 | nA | 9 |
| Battery Timekeeping Current (IND) |  | IBAT | VBAT = 3V | 800 | nA | 9 |
| VCC Trip Point |  | VCCTP |  | VBAT - VBAT + 50 200 | mV |  |
| Trickle Charge Resistors |  | R1 |  | 2 | k |  |
|  |  | R2 |  | 4 |  |  |
|  |  | R3 |  | 8 |  |  |
| Trickle Charger Diode Voltage Drop |  | VTD |  | 0.7 | V |  |

OL

OH

CC1

CC1

CC2

CC1A CC1T

CC2A

CC2

CC2T

CAPACITANCE

(TA = +25C)

| PARAMETER | SYMBOL | MIN TYP MAX | UNITS | NOTES |
| --- | --- | --- | --- | --- |
| Input Capacitance | CI | 10 | pF |  |
| Output Capacitance | CO | 15 | pF |  |
| Crystal Capacitance | CX | 6 | pF |  |

3-WIRE AC ELECTRICAL CHARACTERISTICS

| (TA = Over the operating range, unless otherwise specified.) (Figure 10 and Figure 11) PARAMETER | SYMBOL |  | MIN TYP MAX | UNITS | NOTES |
| --- | --- | --- | --- | --- | --- |
| Data to CLK Setup | t | VCC = 2.0V | 200 | ns | 3, 4 |
|  |  | VCC = 5V | 50 |  |  |
| CLK to Data Hold | t | VCC = 2.0V | 280 | ns | 3, 4 |
|  |  | VCC = 5V | 70 |  |  |
| CLK to Data Delay | t | VCC = 2.0V | 800 | ns | 3, 4, 5 |
|  |  | VCC = 5V | 200 |  |  |
| CLK Low Time | t | VCC = 2.0V | 1000 | ns | 4 |
|  |  | VCC = 5V | 250 |  |  |
| CLK High Time | t | VCC = 2.0V | 1000 | ns | 4 |
|  |  | VCC = 5V | 250 |  |  |
| CLK Frequency | t | VCC = 2.0V | 0.6 | MHz | 4 |
|  |  | VCC = 5V | DC 2.0 |  |  |
| CLK Rise and Fall | t , t | VCC = 2.0V | 2000 | ns |  |
|  |  | VCC = 5V | 500 |  |  |
| CE to CLK Setup | t | VCC = 2.0V | 4 | s | 4 |
|  |  | VCC = 5V | 1 |  |  |
| CLK to CE Hold | t | VCC = 2.0V | 240 | ns | 4 |
|  |  | VCC = 5V | 60 |  |  |
| CE Inactive Time | t | VCC = 2.0V | 4 | s | 4 |
|  |  | VCC = 5V | 1 |  |  |
| CE to Output High-Z | t | VCC = 2.0V | 280 | ns | 3, 4 |
|  |  | VCC = 5V | 70 |  |  |
| SCLK to Output High-Z | t | VCC = 2.0V | 280 | ns | 3, 4 |
|  |  | VCC = 5V | 70 |  |  |

DC CDH CDD CL CH

CLK

R    F

CC CCH CWH CDZ CCZ

Figure 10. TIMING DIAGRAM: 3-WIRE READ DATA TRANSFER

* I/O IS SDI AND SDO TIED TOGETHER.

Figure 11. TIMING DIAGRAM: 3-WIRE WRITE DATA TRANSFER

* I/O IS SDI AND SDO TIED TOGETHER.

SPI AC ELECTRICAL CHARACTERISTICS

| (TA = Over the operating range, unless otherwise specified.) PARAMETER | SYMBOL |  | MIN TYP MAX | UNITS | NOTES |
| --- | --- | --- | --- | --- | --- |
| Data to CLK Setup | t | VCC = 2.0V | 200 | ns | 3, 4 |
|  |  | VCC = 5V | 50 |  |  |
| CLK to Data Hold | t | VCC = 2.0V | 280 | ns | 3, 4 |
|  |  | VCC = 5V | 70 |  |  |
| CLK to Data Delay | t | VCC = 2.0V | 800 | ns | 3, 4, 5 |
|  |  | VCC = 5V | 200 |  |  |
| CLK Low Time | t | VCC = 2.0V | 1000 | ns | 4 |
|  |  | VCC = 5V | 250 |  |  |
| CLK High Time | t | VCC = 2.0V | 1000 | ns | 4 |
|  |  | VCC = 5V | 250 |  |  |
| CLK Frequency | t | VCC = 2.0V | 0.6 | MHz | 4 |
|  |  | VCC = 5V | DC 2.0 |  |  |
| CLK Rise and Fall | t , t | VCC = 2.0V | 2000 | ns |  |
|  |  | VCC = 5V | 500 |  |  |
| CE to CLK Setup | t | VCC = 2.0V | 4 | s | 4 |
|  |  | VCC = 5V | 1 |  |  |
| CLK to CE Hold | t | VCC = 2.0V | 240 | ns | 4 |
|  |  | VCC = 5V | 60 |  |  |
| CE Inactive Time | t | VCC = 2.0V | 4 | s | 4 |
|  |  | VCC = 5V | 1 |  |  |
| CE to Output High-Z | t | VCC = 2.0V | 280 | ns | 3, 4 |
|  |  | VCC = 5V | 70 |  |  |

DC

CDH

CDD

CL

CH

CLK

R    F

CC

CCH

CWH

CDZ

Figure 12. TIMING DIAGRAM: SPI READ DATA TRANSFER

* SCLK CAN BE EITHER POLARITY, TIMING SHOWN FOR CPOL = 1.

Figure 13. TIMING DIAGRAM: SPI WRITE DATA TRANSFER

* SCLK CAN BE EITHER POLARITY, TIMING SHOWN FOR CPOL = 1.

NOTES:

1)  ICC1T and ICC2T are specified with CE set to a logic 0.

2)  ICC1A and ICC2A are specified with CE = VCC, SCLK = 2MHz at VCC = 5V; SCLK = 500kHz at VCC =

2.0V, VIL = 0V, VIH = VCC.

3)  Measured at VIH = 2.0V or VIL = 0.8V and 10ms maximum rise and fall time.

4)  Measured with 50pF load.

5)  Measured at VOH = 2.4V or VOL = 0.4V.

6)  VCC = VCC1, when VCC1 > VCC2 + 0.2V (typical); VCC = VCC2, when VCC2 > VCC1.

7)  VCC2 = 0V.

8)  VCC1 = 0V.

9)  VCC1 < VBAT.

10) VCCIF must be less than or equal to the largest of VCC1, VCC2, and VBAT.

PACKAGE INFORMATION

For the latest package outline information and land patterns, go to www.maxim-ic.com/packages.

| PACKAGE TYPE | PACKAGE CODE | DOCUMENT NO. |
| --- | --- | --- |
| 16 PDIP | P16+1 | 21-0043 |
| 20 TSSOP | U20+1 | 21-0066 |

21 of 21

Maxim/Dallas Semiconductor cannot assume responsibility for use of any circuitry other than circuitry entirely embodied in a Maxim/Dallas Semiconductor product. No circuit patent licenses are implied. Maxim/Dallas Semiconductor reserves the right to change the circuitry and specifications without notice at any time.

M a x i m  In tegra t ed  P r oduc ts ,  1 2 0  S a n  Ga brie l  D r iv e ,  Sun ny vale ,  CA  94 086  40 8 - 7 3 7 - 76 00

© 2005 Maxim Integrated Products

The Maxim logo is a registered trademark of Maxim Integrated Products, Inc. The Dallas logo is a registered trademark of Dallas Semiconductor Corporation.