19-1728; Rev 0; 7/00

10-Bit Serial-Output Temperature Sensors with 5-Channel ADC

General Description

The MAX1098/MAX1099 implement both local and remote temperature sensing with 10-bit resolution, using +5V and +3V supply  voltages, respectively. Accuracy  is ±1°C from 0°C to +70°C, with no calibra- tion needed.  The devices  feature an algorithmic switched-capacitor  analog-to-digital  converter (ADC), on-chip clock, and 3-wire serial interface compatible with SPI™, QSPI™, and MICROWIRE™.

The MAX1098/MAX1099 also perform fully differential voltage measurements with 10-bit resolution and sepa- rate track-and-hold  (T/N) for positive and negative inputs. Both devices accept versatile input modes con- sisting of two 3-channel signal pairs, five 1-channel sig- nals relative to a floating common, or VDD/4 relative to ground. An external reference may be used for more accurate voltage measurements.

Typical power consumption is only 1.3mW (MAX1099). A shutdown mode and two standby modes provide multiple strategies for prolonging battery life in portable applications that require limited sampling throughput.

The MAX1098/MAX1099 are available in 16-pin SSOP

packages.

Applications

Temperature/Voltage Supervision of

Workstations and Communications Equipment

Hand-Held Instruments Medical Equipment Industrial Process Control

Pin  Configuration

TOP VIEW

Features

♦ Local and Remote Temperature Sensing

♦ 12-Bit Resolution for Temperature and

10-Bit Resolution for Voltage Inputs

♦ ±1°C Accuracy from -40°C to +85°C

♦ Fully Differential Inputs

♦ Single-Supply Operation

+4.75V to +5.25V (MAX1098)

+2.7V to +3.6V (MAX1099)

♦ 3-Wire SPI/QSPI/MICROWIRE-Compatible

Interface

♦ Internal Precision Voltage Reference

2.50V (MAX1098)

1.20V (MAX1099)

♦ Space-Saving 16-Pin SSOP Package

Ordering Information

TEMP. PIN- TEMP. SENSE PART RANGE PACKAGE ACCURACY (°C)

MAX1098AEAE* -40°C to +85°C 16 SSOP ±0.75

MAX1098BEAE* -40°C to +85°C 16 SSOP ±1.0

MAX1098CEAE -40°C to +85°C 16 SSOP ±4.0

MAX1099AEAE* -40°C to +85°C 16 SSOP ±0.75

MAX1099BEAE* -40°C to +85°C 16 SSOP ±1.0

MAX1099CEAE -40°C to +85°C 16 SSOP ±4.0

| *Future product—contact factory for availability. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

Temperature Error

AIN1   1

SHO   2

AIN2    3

AIN3    4

AIN4    5

GND   6

SSTRB   7

CS    8

MAX1098

MAX1099

16   AIN0

15   AIN5

14    REF

13   GND

12   VDD

11   SCLK

10   DIN

9    COUT

1.0

0.5

TEMPERATURE  ERROR (°C)

0

MAX1098

TEMPERATURE ERROR

MAX1098/9-29

vs. INTERNAL DIODE TEMPERATURE

SSOP

-0.5

Typical Operating Circuit appears at end of data sheet.

SPI and QSPI are trademarks of Motorola, Inc.

-1.0

-60

-40

-20    0

20      40

60      80     100

MAX1098/MAX1099

MICROWIRE is a trademark of National Semiconductor Corp.

TEMPERATURE (°C)

ABSOLUTE MAXIMUM RATINGS

VDD to GND.……………………………………………-0.3V to +6V SHO to GND ................................................-0.3V to (VDD +0.3V) Analog Inputs to GND

(AIN0–AIN5, REF)...................................-0.3V  to (VDD +0.3V) Digital Inputs to GND (DIN, SCLK, CS).......-0.3V to (VDD +0.3V) Digital Outputs to GND (DOUT, SSTRB) .....-0.3V to (VDD +0.3V) Digital Output Sink Current ..…………………………………25mA Maximum Current into Any Pin……………………………….50mA

Continuous Power Dissipation (TA = +70°C)

16-Pin SSOP (derate 8.00mW/°C above +70°C) ........667mW

Operating Temperature Range

MAX109_ _EAE ...............................................-40°C to +85°C Junction Temperature....……………………………………+150°C Storage Temperature Range .............................-65°C to +150°C Lead Temperature (soldering,  10s) .................................+300°C

MAX1098/MAX1099

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS

(VDD = +4.75V to +5.25V (MAX1098), VDD = +2.7V to +3.6V (MAX1099), external reference, VREF = +2.5V (MAX1098), VREF = +1.2V (MAX1099), fSCLK = 2.5MHz, TA = TMIN to TMAX, unless otherwise noted. Typical values are at TA = +25°C.)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| DC ACCURACY (Note 1) |  |  |  |  |
| Resolution | RES |  | 10 | Bits |
| Relative Accuracy (Note 2) | INL |  | ±1 | LSB |
| Differential Nonlinearity | DNL |  | ±1 | LSB |
| Offset Error |  | Inputs AIN0−AIN5 | ±1 | LSB |
| Offset Temperature Coefficient |  |  | ±10 | µV/°C |
| Gain Error |  | Inputs AIN0−AIN5, offset nulled | ±1 | LSB |
| VDD/4 Absolute Error |  |  | ±1 | LSB |
| Gain Temperature Coefficient |  |  | ±2 | ppm/°C |
| Channel-to-Channel Offset Matching |  |  | ±0.25 | LSB |
| CONVERSION RATE |  |  |  |  |
| Conversion Time (Note 3) | tCONV | Voltage measurement | 1.1 | ms |
|  |  | Temperature measurement | 2.2 |  |
| Track/Hold Acquisition Time | tACQ |  | 16 | µs |
| Aperture Delay | tAPR |  | 30 | ns |
| Internal Clock Frequency | fCLK |  | 57.6 62.3 65.5 | kHz |
| ANALOG INPUTS (AIN0−AIN5) |  |  |  |  |
| Input Voltage Range (Note 4) |  | Measurement with respect to IN-, Figure 1 | -2VREF +2VREF | V |
| Common-Mode Range |  |  | 0 VDD | V |
| Input Current (Note 5) |  |  | 0.1 5 | µA |
| Input Capacitance |  |  | 16 | pF |

MAX1098/MAX1099

ELECTRICAL CHARACTERISTICS  (continued)

(VDD = +4.75V to +5.25V (MAX1098), VDD = +2.7V to +3.6V (MAX1099), external reference, VREF = +2.5V (MAX1098), VREF = +1.2V (MAX1099), fSCLK = 2.5MHz, TA = TMIN to TMAX, unless otherwise noted. Typical values are at TA = +25°C.)

| PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| DIGITAL INPUTS |  |  |  |  |  |
| Input Voltage Low | VIL |  |  | 0.8 | V |
| Input Voltage High | VIH |  |  | VDD - 0.8 | V |
| Input Hysteresis | VHYST |  |  | 0.2 | V |
| Input Leakage Current | IIN |  |  | 1 | µA |
| Input Capacitance |  |  |  | 16 | pF |
| DIGITAL OUTPUTS |  |  |  |  | V |
| Output Low Voltage | VOL | ISINK = 5mA |  | 0.6 | V |
| Output High Voltage | VOH | ISOURCE = 0.5mA |  | VDD - 0.6 | V |
| Three-State Output Leakage Current | IOUT |  |  | ±10 | µA |
| Three-State Output Capacitance |  |  |  | 15 | pF |
| POWER REQUIREMENTS |  |  |  |  |  |
| Positive Supply Voltage | VDD | MAX1098 |  | 4.75 5.25 | V |
|  |  | MAX1099 |  | 2.7 3.6 |  |
| Positive Supply Current (Note 6) | IDD | Full-on, voltage measurements, internal reference | MAX1098 | 390 | µA |
|  |  |  | MAX1099 | 350 |  |
|  |  | Full-on, voltage measurements, external reference | MAX1098 | 310 |  |
|  |  |  | MAX1099 | 280 |  |
|  |  | Full-on, temperature measurements, internal reference | MAX1098 | 440 500 |  |
|  |  |  | MAX1099 | 400 500 |  |
|  |  | Full-on, temperature measurements, external reference | MAX1098 | 360 |  |
|  |  |  | MAX1099 | 330 |  |
|  |  | Standby, SCLK = GND |  | 120 |  |
|  |  | Standby-plus, SCLK = GND |  | 190 |  |
|  |  | Shutdown, SCLK = GND |  | 2 10 |  |
| Power-Supply Rejection | PSRR | (Note 7) |  | 50 65 | dB |
| INTERNAL VOLTAGE REFERENCE CHARACTERISTICS |  |  |  |  |  |
| Reference Voltage | VREF | VDD = 5V | MAX1098 | 2.494 2.50 2.506 | V |
|  |  | VDD = 3V | MAX1099 | 1.197 1.20 1.203 |  |
| Reference Tempco | TC VREF |  |  | ±20 | ppm/°C |
| Output Short-Circuit Current |  |  |  | 1.25 | mA |
| Capacitive Bypass at REF |  |  |  | 0.1 | µF |
| REF Output Noise |  | fN = 10Hz to 10kHz | MAX1098 | 130 | µVRMS |
|  |  |  | MAX1099 | 65 |  |
| REF Line Regulation |  |  | MAX1098 | +3.0 | mV/V |
|  |  |  | MAX1099 | +0.2 |  |
| REF Load Regulation |  | 0 to 100µA output current (Note 8) | MAX1098 | 4 10 | µV/µA |
|  |  |  | MAX1099 | 2 10 |  |

MAX1098/MAX1099

ELECTRICAL CHARACTERISTICS  (continued)

(VDD = 4.75V to 5.25V (MAX1098),  VDD = 2.7V to 3.6V (MAX1099),  external  reference,  VREF = +2.5V  (MAX1098),  VREF = +1.2V (MAX1099), fSCLK = 2.5MHz, TA = TMIN to TMAX, unless otherwise noted. Typical values are at TA = +25°C.)

| PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| EXTERNAL VOLTAGE REFERENCE CHARACTERISTICS |  |  |  |  |  |
| Reference Voltage Range | VREF | MAX1098 |  | 0.8 2.5 | V |
|  |  | MAX1099 |  | 0.8 1.2 |  |
| REF Input Resistance |  | Converting |  | 10 | MΩ |
|  |  | Shutdown |  | 25 |  |
| REF Input Capacitance |  |  |  | 24 | pF |
| INTERNAL TEMPERATURE MEASUREMENT CHARACTERISTICS |  |  |  |  |  |
| Resolution |  |  |  | 0.13 | °C |
| Output Error (Notes 1, 9) |  | TA = +85°C, PD = 1mW | MAX109_A | ±0.75 | °C |
|  |  |  | MAX109_B | ±1 |  |
|  |  |  | MAX109_C | ±1 |  |
|  |  | TA = 0°C to +70°C | MAX109_A | ±0.75 |  |
|  |  |  | MAX109_B | ±1 |  |
|  |  |  | MAX109_C | ±2 |  |
|  |  | TA = -40°C to 0°C, TA = +70°C to +85°C | MAX109_A | ±0.75 |  |
|  |  |  | MAX109_B | ±1 |  |
|  |  |  | MAX109_C | ±4 |  |
| Power-Supply Rejection Ratio | PSRR | (Note 7) |  | 0.2 | °C/V |
| Noise |  |  |  | 0.18 | °CRMS |
| EXTERNAL TEMPERATURE MEASUREMENT CHARACTERISTICS |  |  |  |  |  |
| Output Error |  | 2N3904 (Note 10) |  | ±2 ±4 | °C |
| Remote Diode Excitation (1X) |  |  |  | 10 | µA |
| Remote Diode Excitation (10X) |  |  |  | 100 | µA |

MAX1098/MAX1099

TIMING CHARACTERISTICS

(VDD = +4.75V to +5.25V (MAX1098), VDD = +2.7V to +3.6V (MAX1099), external reference, VREF = +2.5V (MAX1098), VREF = +1.2V (MAX1099), fSCLK = 2.5MHz, TA = TMIN to TMAX, unless otherwise noted. Typical values are at TA = +25°C.) (Figures 4, 6)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| SCLK Frequency | fSCLK |  | 2.5 | MHz |
| SCLK Pulse Width Low | tCL |  | 200 | ns |
| SCLK Pulse Width High | tCH |  | 200 | ns |
| CS Low to SCLK High | tCSS |  | 100 | ns |
| SCLK High to CS Setup | tCSH |  | 100 | ns |
| CS Pulse Width | tCS |  | 100 | ns |
| SCLK High to CS Low Setup | tCS0 |  | 50 | ns |
| SCLK High to CS High Setup | tCS1 |  | 100 | ns |
| DIN Setup to SCLK High Time | tDS |  | 100 | ns |
| DIN Hold Time | tDH |  | 0 | ns |
| SCLK Fall to Output Data Valid | tDO | RL = 100kΩ, CL = 50pF | 150 | ns |
| CS Fall to Output Enable | tDV | RL = 100kΩ, CL = 50pF | 150 | ns |
| CS Rise to Output Disable | tTR | RL = 100kΩ, CL = 50pF | 50 | ns |
| SSTRB Rise to SCLK Rise | tSCLK |  | 0 | ns |
| SCLK Fall to SSTRB Fall | tSSTRB |  | 200 | ns |

Note 1:  Tested at VDD = +5.0V (MAX1098) and VDD = +3.0V (MAX1099).

Note 2:  Relative accuracy  is the deviation of the analog value at any code from its theoretical value after the full-scale range has been calibrated.

Note 3:  Conversion time is defined as the number of clock cycles (64 for voltage measurements,  125 for temperature  measure-

ments) multiplied by the internal clock period.

Note 4:  Individual analog input voltages cannot extend beyond the power-supply rails.

Note 5:  Input resistance is typically 250MΩ; 5µA limit reflects limitations in production  testing.

Note 6:  Specifications for full-on status assume continuous conversions.  Power modes are software selected (Table 3).

Note 7:  Measured at VFS(+4.75V) - VFS(+5.25V) for the MAX1098 and at VFS(+2.7V) - VFS(+3.6V) for the MAX1099.

Note 8:  External load should not change during conversions  for specified  accuracy.

Note 9:  Excludes noise and self-heating effects. Output error for MAX109_C guaranteed  by design.

Note 10: External temperature  sensing over -40°C to +85°C range, device at +25°C. Guaranteed by design.

Typical Operating Characteristics

(TA = +25°C, unless otherwise noted.)

0.20

MAX1098

INTEGRAL NONLINEARITY

vs. OUTPUT CODE

0.20

MAX1099

INTEGRAL NONLINEARITY

vs. OUTPUT CODE

MAX1098/9-02

0.20

MAX1098

MAX1098/9-03

DIFFERENTIAL NONLINEARITY

vs. OUTPUT CODE

0.15

INTEGRAL NONLINEARITY (LSB)

0.15

DIFFERENTIAL NONLINEARITY (LSB)

0.15

0.10

0.10

0.10

0.05

MAX1098/9-01

0.05

0.05

0	0	0

-0.05

-0.10

-0.15

-0.05

-0.10

-0.15

-0.05

-0.10

-0.15

-0.20

-625	0	625

OUTPUT CODE

MAX1099

MAX1098/9-04

DIFFERENTIAL NONLINEARITY

vs. OUTPUT CODE

-0.20

-625	0	625

OUTPUT CODE

MAX1098

MAX1098/9-05

| SUPPLY CURRENT vs. SUPPLY VOLTAGE (VOLTAGE MEASUREMENT MODE) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | INTERNAL REFERENCE |  |  |  |  |

-0.20

-625	0	625

OUTPUT CODE

MAX1099

MAX1098/9-06

SUPPLY CURRENT vs. SUPPLY VOLTAGE (VOLTAGE MEASUREMENT MODE)

0.20

DIFFERENTIAL NONLINEARITY (LSB)

0.15

0.10

0.05

0

-0.05

-0.10

-0.15

-0.20

500

450

300

250

200

SUPPLY CURRENT (µA)

150

100

50

0

500

450

300

250

200

SUPPLY CURRENT (µA)

150

100

50

0

INTERNAL REFERENCE EXTERNAL REFERENCE

-625	0	625

4.7

4.8

4.9

5.0	5.1	5.2

2.7

2.9

3.1

3.3	3.5

500

400

OUTPUT CODE

MAX1098

MAX1098/9-07

| SUPPLY CURRENT vs. SUPPLY VOLTAGE (TEMPERATURE MEASUREMENT MODE) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | INTERNAL REFERENCE |  |  |  |  |
|  | EXTERNAL REFERENCE |  |  |  |  |

500

400

SUPPLY VOLTAGE (V)

MAX1099

MAX1098/9-08

SUPPLY CURRENT vs. SUPPLY VOLTAGE (TEMPERATURE MEASUREMENT MODE)

INTERNAL  REFERENCE

500

400

SUPPLY VOLTAGE (V)

MAX1098

SUPPLY CURRENT vs. TEMPERATURE (VOLTAGE MEASUREMENT MODE)

MAX1098/9-09

INTERNAL  REFERENCE

300

250

200

SUPPLY CURRENT (µA)

150

100

50

0

4.7

4.8

4.9

5.0	5.1	5.2

300

250

200

SUPPLY CURRENT (µA)

150

100

50

0

2.7

2.9

3.1

3.3	3.5

300

250

SUPPLY CURRENT (µA)

200

150

100

50

0

EXTERNAL REFERENCE

-40	-20	0	20	40	60	80

MAX1098/MAX1099

INTEGRAL NONLINEARITY (LSB)

SUPPLY VOLTAGE (V)

SUPPLY VOLTAGE (V)

TEMPERATURE (°C)

500

450

400

350

300

250

200

150

100

50

0

MAX1099

MAX1098/9-10

SUPPLY CURRENT vs. TEMPERATURE (VOLTAGE MEASUREMENT MODE)

INTERNAL  REFERENCE EXTERNAL REFERENCE

-40	-20	0	20	40	60	80

TEMPERATURE (°C)

500

450

400

350

300

250

200

SUPPLY CURRENT (µA)

150

100

50

0

MAX1098

SUPPLY CURRENT vs. TEMPERATURE (TEMPERATURE MEASUREMENT MODE)

INTERNAL  REFERENCE EXTERNAL REFERENCE

-40	-20	0	20	40	60	80

TEMPERATURE (°C)

MAX1298/9-11

500

450

400

350

300

SUPPLY CURRENT (µA)

250

200

150

100

50

0

MAX1099

SUPPLY CURRENT vs. TEMPERATURE (TEMPERATURE MEASUREMENT MODE)

MAX1098/9-12

INTERNAL  REFERENCE EXTERNAL REFERENCE

-40	-20	0	20	40	60	80

TEMPERATURE (°C)

500

450

400

350

300

250

200

150

100

50

0

MAX1098

MAX1098/9-13

POWER-DOWN SUPPLY CURRENT

vs. SUPPLY VOLTAGE

STANDBY

500

450

400

350

300

250

200

SUPPLY CURRENT (µA)

150

100

50

0

MAX1099

MAX1098/9-14

POWER-DOWN SUPPLY CURRENT

vs. SUPPLY VOLTAGE

STANDBY+ STANDBY

500

450

SUPPLY CURRENT (µA)

400

350

300

250

200

150

100

50

0

MAX1098

MAX1098/9-15

POWER-DOWN SUPPLY CURRENT

vs. TEMPERATURE

STANDBY+ STANDBY

4.7

4.8

4.9

5.0	5.1	5.2

2.7

2.9

3.1

3.3	3.5

-40    -20	0	20	40	60	80

500

450

400

350

300

250

200

150

100

50

0

SUPPLY VOLTAGE (V)

MAX1099

MAX1098/9-16

POWER-DOWN SUPPLY CURRENT

vs. TEMPERATURE

STANDBY+ STANDBY

| 2.52 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

REFERENCE VOLTAGE (V)

2.51

2.50

2.49

2.48

SUPPLY VOLTAGE (V)

MAX1098

MAX1098/9-17

INTERNAL REFERENCE VOLTAGE

vs. SUPPLY VOLTAGE

| 1.22 |  |  |  |  |
| --- | --- | --- | --- | --- |

REFERENCE VOLTAGE (V)

1.21

1.20

1.19

1.18

TEMPERATURE (°C)

MAX1099

MAX1098/9-18

INTERNAL REFERENCE VOLTAGE

vs. SUPPLY VOLTAGE

-40	-20	0	20	40	60	80

4.7

4.8	4.9

5.0

5.1	5.2

2.7

2.9

3.1

3.3	3.5

MAX1098/MAX1099

SUPPLY CURRENT (µA)

SUPPLY CURRENT (µA)

SUPPLY CURRENT (µA)

TEMPERATURE (°C)

SUPPLY VOLTAGE (V)

SUPPLY VOLTAGE (V)

Typical Operating Characteristics (continued)

(TA = +25°C, unless otherwise noted.)

2.52

2.51

2.50

2.49

MAX1098

MAX1098/9-19

INTERNAL REFERENCE VOLTAGE

vs. TEMPERATURE

1.22

REFERENCE VOLTAGE (V)

1.21

1.20

1.19

MAX1099

MAX1098/9-20

INTERNAL REFERENCE VOLTAGE

vs. TEMPERATURE

0.250

0.125

0

-0.125

MAX1098

MAX1098/9-21

OFFSET vs. SUPPLY VOLTAGE

2.48

| -40 |  |  |  |  |
| --- | --- | --- | --- | --- |

-20	0

20	40

60	80

1.18

OFFSET (LSB)

| -40 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

-20	0

20	40

60	80

-0.250

| 4.7 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

4.8	4.9

5.0

5.1	5.2

TEMPERATURE (°C)

TEMPERATURE (°C)

SUPPLY VOLTAGE (V)

0.250

0.125

OFFSET (LSB)

0

-0.125

MAX1099

MAX1098/9-22

OFFSET vs. SUPPLY VOLTAGE

0.250

0.125

OFFSET (LSB)

0

-0.125

MAX1098

MAX1098/9-23

| OFFSET vs. TEMPERATURE |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

0.250

0.125

OFFSET (LSB)

0

-0.125

MAX1099

MAX1098/9-24

OFFSET vs. TEMPERATURE

-0.250

2.7

2.9

3.1

3.3	3.5

-0.250

-40

-20	0

20	40

60	80

-0.250

-40

-20	0

20	40

60	80

SUPPLY VOLTAGE (V)

TEMPERATURE (°C)

TEMPERATURE (°C)

0.25

MAX1098

MAX1098/9-27

GAIN ERROR vs. TEMPERATURE

0.25

MAX1099

MAX1098/9-28

GAIN ERROR vs. TEMPERATURE

1.0

MAX1098

TEMPERATURE ERROR

MAX1098/9-29

vs. INTERNAL DIODE TEMPERATURE

0.5

GAIN ERROR (LSB)

GAIN ERROR (LSB)

TEMPERATURE  ERROR (°C)

| 0 0 0 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

-0.5

-0.25

-40

-20	0

20	40

60	80

-0.25

| -40 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

-20	0

20	40

60	80

-1.0

| -60 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

-40

-20    0

20     40

60     80     100

MAX1098/MAX1099

REFERENCE VOLTAGE (V)

TEMPERATURE (°C)

TEMPERATURE (°C)

TEMPERATURE (°C)

1.0

MAX1099

TEMPERATURE ERROR

MAX1098/9-30

vs. INTERNAL DIODE TEMPERATURE

2.0

1.5

MAX1098

TEMPERATURE ERROR

MAX1098/9-31

vs. REMOTE DIODE TEMPERATURE

2.0

1.5

MAX1099

TEMPERATURE ERROR

MAX1098/9-32

vs. REMOTE DIODE TEMPERATURE

0.5

1.0

1.0

0.5

0.5

TEMPERATURE  ERROR (°C)

| 0 0 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

-0.5

0

-0.5

-0.5

TEMPERATURE  ERROR (°C)

-1.0

-1.5

-1.0

-1.5

-1.0

-60

-40

-20    0

20     40

60     80     100

-2.0

| -60 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

-40

-20    0

20      40      60      80     100

-2.0

| -60 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

-40

-20    0

20      40      60      80     100

MAX1098/MAX1099

TEMPERATURE  ERROR (°C)

TEMPERATURE (°C)

TEMPERATURE (°C)

TEMPERATURE (°C)

MAX1098/MAX1099

Pin  Description

| PIN | NAME | FUNCTION |
| --- | --- | --- |
| 1 | AIN1 | Analog Input 1. Negative differential input relative to AIN0 or positive differential input relative to AIN5 (Table 4). Connect to the cathode of external diode 1 for remote temperature sensing. |
| 2 | SHO | Shield Output. Used to suppress leakage currents at the anodes of remote temperature sensors (see Remote Diode Shielding). May also be connected to the shields of twisted-pair input cables used for remote temperature measurements. Leave unconnected for other applications. |
| 3 | AIN2 | Analog Input 2. Positive differential input relative to AIN3 or positive differential input relative to AIN5 (Table 4). Connect to the anode of external diode 2 for remote temperature sensing. |
| 4 | AIN3 | Analog Input 3. Negative differential input relative to AIN2 or positive differential input relative to AIN5 (Table 4). Connect to the cathode of external diode 2 for remote temperature sensing. |
| 5 | AIN4 | Analog Input 4. Positive differential input relative to AIN5 (Table 4). |
| 6 | GND | Ground. Connect to pin 13. |
| 7 | SSTRB | Serial Strobe Output. SSTRB goes low at the beginning of an A/D conversion, and it goes high when the conversion is finished. |
| 8 | CS | Active-Low Chip Select. Data will not be clocked into DIN unless CS is low. When CS is high, DOUT is at high impedance. |
| 9 | DOUT | Serial Data Output. DOUT transitions on the falling edge of SCLK. |
| 10 | DIN | Serial Data Input. DIN latches data on the rising edge of SCLK. |
| 11 | SCLK | Serial Clock Input. Clocks data in and out of the serial interface. |
| 12 | VDD | Positive Supply Voltage. Bypass with a 0.1µF capacitor to GND (pin 13). |
| 13 | GND | Ground (star ground) |
| 14 | REF | Reference-Buffer Output/ADC Reference Input. Reference voltage for A/D conversion. Bypass to GND (pin 13) with a 0.1µF capacitor. Select reference mode by writing to configuration byte (Table 1). |
| 15 | AIN5 | Analog Input 5. Negative differential input relative to AIN0–AIN4 (Table 4). |
| 16 | AIN0 | Analog Input 0. Positive differential input relative to AIN1 or positive differential input relative to AIN5 (Table 4). Connect to the anode of external diode 1 for remote temperature sensing. |

CS

SCLK

DIN

INPUT REGISTER

OUTPUT REGISTER

DOUT

DIODE BIAS

CONTROL	CONTROL LOGIC

AIN0

AIN1

AIN2

AIN3

AIN4

INPUT MUX

T/H	IN+

ADC

CLOCK

AIN5

T/H

IN- 	VDD

GND

VDD/4

SHIELD OUTPUT

REF

SHO 	REF

Figure 1. MAX1098/MAX1099 Functional Diagram

MAX1098/MAX1099

Detailed Description

The MAX1098/MAX1099 are low-power, serial-output, multichannel ADCs with temperature-sensing capability for thermostatic, process-control, and monitoring appli- cations. An algorithmic switched-capacitor  converter with T/H circuitry for both positive and negative inputs supports fully differential 10-bit conversions from an internal temperature sensor, two external temperature sensors, or voltage sources in a variety of channel con-

figurations. Microprocessor  (µP) control is made easy through a flexible 3-wire serial interface.

Figure 1 shows a simplified functional diagram of the MAX1098/MAX1099 internal architecture. In tempera- ture-sensing mode, the multiplexer (mux) steers bias currents through internal or external diodes while the ADC computes their temperature in relation to changes in forward voltage. Channels not used for temperature measurement can be configured to measure other sys- tem voltages.

TIMING/CONTROL LOGIC

IN+

IN-

RIN

40k

RIN

40k

T/H

T/H

CHOLDP

4pF

CHOLDN

4pF

FULLY DIFFERENTIAL A/D

OUTPUT

TRACK AND HOLD

REF

RR

30k

CREF

4pF

GAIN OF 2

MAX1098/MAX1099

Figure 2. Converter Input Structure

Converter Operation Figure 2 shows a simplified model of the converter input structure. Once initiated, a voltage conversion requires 64 fCLK  periods, where fCLK  is the internal master clock. Each conversion is preceded by 13 fCLK periods of warm-up time, performed in twelve 4 fCLK period cycles, and followed by three fCLK  periods to load the output register. SSTRB falls at the beginning of a conversion and rises at the end of a conversion.

Inputs IN+ and IN- charge capacitors  CHOLDP and CHOLDN, respectively, during the acquisition interval that occurs during the first fCLK period of the first con- version cycle.  In the second  fCLK  period,  the T/H switches open so that charge is retained on CHOLDP and CHOLDN  as a sample of the differential voltage between IN+ and IN-. This charge is transferred to the ADC during the third and fourth fCLK periods.

The reference sampling process begins in the second conversion cycle and continues until the conversion is complete. Sampling occurs  during  the second and fourth fCLK periods to yield an effective doubling of the reference voltage. The reference sampling requirement

is signal dependent and may or may not occur in every subsequent conversion cycle.

Temperature conversion is nothing more than subtract- ing the results of two sequential voltage conversions. The only difference is that output registers are not loaded at the end of the first conversion.  Thus, temperature con- versions require 2 x 64 - 3 = 125 fCLK periods. Figures

3a and 3b show timing diagrams for voltage and tem- perature conversions, respectively.

Track/Hold The T/H stage for the MAX1098/MAX1099 is a simple switched-capacitor  sampling  operation. The time required for the T/H stage to acquire an input signal is a function of how fast its input capacitance is charged. If the signal source impedance is high, the acquisition time lengthens  and  more time must be  allowed between conversions. The acquisition time (tACQ) is the maximum time the device takes to acquire the signal. Calculate this with the following equation:

tACQ = 7 (Rs + RIN) CIN

where Rs is the source impedance of the input signal, RIN is the T/H input impedance (40kΩ), and CIN is the

SSTRB

FCLK

13 fCLKs

WARMUP

INPUT ACQUISITION FCLKS

CONVERSION CYCLE 1

REF

ACQUISITION 1 	REF ACQUISITION 2

CONVERSION CYCLES 2–12

REFERENCE SAMPLING

3 fCLKs

WRITE TO OUTPUT

REGISTER

Figure 3a. Voltage Conversion Timing Diagram

SSTRB

FCLK

13 fCLKs

WARMUP

INPUT ACQUISITION

4 fCLKs

CONVERSION CYCLE 1

44 fCLKs CONVERSION CYCLES 2–12

REFERENCE SAMPLING

13 fCLKs

WARMUP

INPUT ACQUISITION

48 fCLKs

CONVERSION CYCLES 1–12

3 fCLKs SUBTRACTION AND WRITE TO

OUTPUT REGISTER

FIRST CONVERSION	SECOND CONVERSION

Figure 3b. Temperature Conversion Timing Diagram

MAX1098/MAX1099

input sampling capacitance  of the ADC (4pF). Source impedances below 100kΩ have no significant effect on MAX1098/MAX1099 AC performance.

Analog Input Protection Internal protection diodes clamp the analog inputs to VDD and GND so channels can swing within GND -

0.3V and VDD + 0.3V without damage.  However, for accurate  conversions, the inputs should not extend beyond the supply rails.

If an off-channel analog input extends beyond the supply rails, limit the input current to 2mA.

Serial Digital Interface The MAX1098/MAX1099 feature a serial interface that is fully compatible  with SPI, QSPI, and MICROWIRE devices. For SPI/QSPI, ensure that the CPU serial inter- face runs in master mode so it generates the serial clock signal. Select a 2.5MHz clock frequency or less, and set zero values for clock  polarity (CPOL) and phase (CPHA) in the µP control registers. Figure 4 shows detailed serial interface timing information. See Tables 1–4 for programming information.

CS

SCLK

DIN

tCS0

X

tDS

tCSS

VALID

tCH

tDH

X

tCL

VALID

VALID

tCSH

tCS

tCS1

X

DOUT

tDV

tDO 	tTR

Figure 4. Detailed Serial Interface Timing

OUTPUT CODE

Output Data Format

Output data from the MAX1098/MAX1099 are clocked

0111111111

0111111110

0000000010

0000000001

0000000000

1111111111

1111111110

1111111101

1000000010

1000000001

+FS = + 2VREF

-FS = - 2VREF

1LSB = 2VREF

512

- FS + 1LSB

0 	+ FS - 1LSB IN+ - IN - (LSB)

onto DOUT on the falling edge of SCLK in the form of two

8-bit words, MSB first (Table 5). For temperature conver- sions, the output is 12-bit binary (D8–S2) padded with 2 leading extraneous bits and two trailing zeros. For volt- age conversions, the output is 10-bit two’s-complement binary (D9–D0) with 3 sub-bits and two trailing zeros. Figure 5 shows the bipolar transfer function.

Performing a Conversion On power-up, the MAX1098/MAX1099 default to shut- down mode. Start a conversion by transferring a configu- ration byte and a conversion byte into DIN with the control formats shown in Tables 1 and 2, respectively. (See Power Modes for a related discussion.)

Figure 5. Bipolar Transfer Function

Input Data Format

SSTRB goes low on the falling edge of the last bit of the conversion byte, and it returns high when the conversion is complete. For best noise performance, SCLK should

MAX1098/MAX1099

Input data (configuration  and conversion bytes) are

clocked into the MAX1098/MAX1099 at DIN on the ris- ing edge of SCLK when CS is low. The start bit (MSB) of an input data byte is the first logic 1 bit that arrives:

After CS falls

OR

After receipt of a complete configuration byte with no conversion in progress

OR

After 16 bits have been clocked onto DOUT following a conversion.

remain low while SSTRB is low. Typical conversion times are 2.2ms for temperature measurements and 1.1ms for voltage measurements. The MSB of the 2 output bytes is present at DOUT starting at the rising edge of SSTRB. Successive SCLK falling edges shift the two 8-bit data bytes out from an internal register. Additional (>16) SCLK edges will result in zeros on DOUT.

SSTRB does not go into a high-impedance  state when CS goes high. Pulling CS high prevents data from being clocked in or out, but it does not adversely affect a conversion in progress. Figure 6 shows SSTRB timing details.

Subsequent conversions with the same reference mode do not require a configuration byte.

Table 1. Configuration-Byte  Format

| BIT 7 (MSB) | BIT 6 | BIT 5 BIT 4 | BIT 3 | BIT 2 | BIT 1 | BIT 0 (LSB) |
| --- | --- | --- | --- | --- | --- | --- |
| Start | 0 | 0 0 | 0 | PM1 | PM0 | REF |
| BIT | NAME | DESCRIPTION |  |  |  |  |
| 7 (MSB) | Start | First logic 1 after CS goes low. (See Input Data Format.) |  |  |  |  |
| 6, 5, 4, 3 |  | Must be 0000 to load a configuration byte. |  |  |  |  |
| 2, 1 | PM1, PM0 | These 2 bits select the desired power mode (Table 3). |  |  |  |  |
| 0 | REF | A logic high enables the internal reference. A logic low disables the internal reference and selects the external reference mode. |  |  |  |  |

Table 2. Conversion-Byte Format

| BIT 7 (MSB) | BIT 6 | BIT 5 | BIT 4 | BIT 3 | BIT 2 | BIT 1 | BIT 0 (LSB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Start | 0 | 1 | 0 | SEL3 | SEL2 | SEL1 | SEL0 |
| BIT | NAME | DESCRIPTION |  |  |  |  |  |
| 7 (MSB) | Start | First logic 1 after CS goes low. (See Input Data Format.) |  |  |  |  |  |
| 6, 5, 4 |  | Must be 010 to load a conversion byte. |  |  |  |  |  |
| 3, 2, 1, 0 | SEL3, SEL2, SEL1, SEL0 | These 4 bits select the input configuration (Table 4). |  |  |  |  |  |

CSB SSTRB

tCSH

tCONV

tSCK

tCSS

Internal Reference The MAX1098 has a 2.50V internal reference, while the MAX1099 has a 1.20V internal reference. Both are fac- tory trimmed for accuracy.  When internal reference is selected, REF can be used to drive an external load

SCLK DOUT

tSSTRB

PDO CLOCKED IN

SSTRB TIMING

tDO

with 100µA capability. Bypass REF to GND with a 0.1µF minimum capacitance.  Wake-up time is C x 2.5 x 104s for the MAX1098 and C x 1.2 x 104s for the MAX1099.

External Reference The MAX1098 can directly accept reference voltages at REF from 0.8V to 2.5V, while the MAX1099 can directly

Figure 6. Detailed SSTRB Timing

Reference Selection

accept  reference voltages from 0.8V to 1.2V. Bypass REF to GND with a 0.1µF capacitor. Temperature mea- surements always use internal reference.

Power Modes

MAX1098/MAX1099

Select between internal and external voltage modes

through bit REF of the configuration byte. Set REF = 1 for internal reference mode and REF = 0 for external reference mode.

The MAX1098/(MAX1099) typically requires supply cur- rents of 380µA (350µA) or 310µA (280µA) when per- forming voltage conversions at 100% duty cycle with internal or external references, respectively. The differ-

ence reflects the power requirement of an internal refer- ence buffer amplifier that can accommodate external loads. Temperature conversions at 100% duty cycle increase supply  currents to 440µA (400µA) through additional amplification, buffer, and bias circuitry that is otherwise inactive.

Place the MAX1098/MAX1099 in a low-current power- down state between conversions to conserve power. Select standby, standby plus, or shutdown through bits PM1 and PM0 of the initialization byte (Table 3).

The MAX1098/MAX1099 assume the shutdown power mode when VDD is first applied.

Standby Mode Standby mode turns off the MAX1098/MAX1099 ADC, internal clock, and reference buffer amplifier. Special circuitry for temperature conversions is also deactivat- ed. Wake-up time is limited by the reference buffer amplifier and the associated bypass capacitor  (see Internal Reference). When an external reference is used, wake-up time is 0.1ms.

Standby-Plus Mode Standby-plus mode is similar to the standby mode, but the internal reference output buffer remains active to shorten the wake-up time to 0.1ms for internal refer- ence mode. When using an external reference, stand- by-plus mode is equivalent to standby mode.

Shutdown Mode Shutdown mode turns off all functions other than start- up circuitry, thereby reducing typical supply current to

2µA. Data registers are cleared. Use this power mode when interconversion times are no less than 5ms.

Table 3. Power-Mode Selection

See Power Requirements in Electrical Characteristics.

| PM1 | PM0 | MODE |
| --- | --- | --- |
| 0 | 0 | Shutdown |
| 0 | 1 | Standby plus |
| 1 | 0 | Standby |
| 1 | 1 | Normal operation |

achieve a digital output that is proportional to absolute temperature in degrees Kelvin.

The reference voltage used in conjunction with tempera- ture measurements is derived from the internal reference source to ensure that 1LSB corresponds  to 1/8 of a degree. To convert to degrees Celsius, subtract 273.15 from the temperature inferred from the ADC output.

Temperature measurements require a conversion time of 2.2ms.

Shield Output Buffer The MAX1098/MAX1099 provide a shield output buffer voltage at SHO that is approximately 0.6V (one diode drop) above VDD/2. When performing  temperature measurements with an external diode, use this voltage to suppress error-producing  leakage currents (see Remote Diode Shielding). Figure 7 shows the SHO out- put circuit.

5µA

SHO

MAX1098/MAX1099

Monitoring VDD

This mode of operation samples and converts the sup-

ply voltage, VDD/4, which is internally generated. The reference voltage must be larger than VDD/8 for the operation to work properly. From the result of a conver- sion (CODE), CODE = 64 VDD / VREF.

Temperature Measurements The MAX1098/MAX1099 perform temperature measure- ments with internal or external diode-connected transis- tors through a three-step process. First, the diode bias current changes  from 31.6µA to 10µA to produce  a temperature-dependent  bias voltage difference, which is amplified by a factor of 20 and converted to digital format. Second, the bias current changes from 31.6µA to 100µA, and the bias voltage difference is similarly amplified by a factor of 20 and converted to digital for- mat. Third, the intermediate results are subtracted  to

VDD

2

Figure 7. SHO Output Circuit

Applications Information

Remote Diode Selection Temperature accuracy  depends  on having a good- quality, diode-connected,  small-signal transistor. Accuracy has been experimentally verified for 2N3904 devices. CPUs and other ICs having on-board temper- ature-sensing diodes can also be monitored if the diode connections are floating.

| SEL3 | SEL2 | SEL1 | SEL0 | POSITIVE INPUT (IN+) | NEGATIVE INPUT (IN-) |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | AIN0 | AIN5 |
| 0 | 0 | 0 | 1 | AIN1 | AIN5 |
| 0 | 0 | 1 | 0 | AIN2 | AIN5 |
| 0 | 0 | 1 | 1 | AIN3 | AIN5 |
| 0 | 1 | 0 | 0 | AIN4 | AIN5 |
| 0 | 1 | 0 | 1 | — | — |
| 0 | 1 | 1 | 0 | AIN5 | AIN5 |
| 0 | 1 | 1 | 1 | Internal diode anode* | Internal diode cathode |
| 1 | 0 | 0 | 0 | AIN0 | AIN1 |
| 1 | 0 | 0 | 1 | AIN2 | AIN3 |
| 1 | 0 | 1 | 0 | — | — |
| 1 | 0 | 1 | 1 | VDD/4 | GND |
| 1 | 1 | 0 | 0 | External diode 1 anode* (AIN0) | External diode 1 cathode (AIN1) |
| 1 | 1 | 0 | 1 | External diode 2 anode* (AIN2) | External diode 2 cathode (AIN3) |
| 1 | 1 | 1 | 0 | — | — |
| 1 | 1 | 1 | 1 | — | — |

*Temperature-measurement mode

Table 5. Output Data Format

| D9 | D9 | D8 | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | S0 | S1 | S2 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

MAX1098/MAX1099

The transistor must be a small-signal type with a base resistance less than 100Ω. Tight specifications  for for- ward current gain (+50 to +150, for example) indicate that the manufacturer has good process controls and that the devices have consistent Vbe characteristics. (See Table 6 for recommended devices.)

For heatsink mounting, the 500-32BT02-000 thermal sensor from Fenwal Electronics is a good choice.  This device consists of a diode-connected transistor, an alu- minum plate with screw hole, and twisted-pair cable (Fenwal Inc., Milford MA, 508-478-6000).

Twisted-Pair and Shielded Cables For remote-sensor distances greater than 8 inches, or in particularly noisy environments, use a twisted-pair cable. A practical length is 6 feet to 12 feet. For longer distances, the best solution is a shielded twisted-pair cable such as that used for audio microphones.  For

Table 6. Remote-Sensor Transistor

| Manufacturers MANUFACTURER | MODEL NUMBER |
| --- | --- |
| Central Semiconductor (USA) | CMPT3904 |
| Fairchild Semiconductor (USA) | MMBT3904 |
| Motorola (USA) | MMBT3904 |
| Rohm Semiconductor (Japan) | SST3904 |
| Siemens (Germany) | SMB3904 |
| Zetex (England) | FMMT3904CT-ND |

SHIELD

ANODE

CATHODE

Definitions

Relative Accuracy Relative accuracy  is the deviation of the values on an actual transfer function from a straight line. This straight line can be either a best-straight-line fit or a line drawn between the endpoints of the transfer function, once offset and gain errors have been nullified. The static lin- earity parameters for the MAX1098/MAX1099 are mea- sured using the best-straight-line fit method.

Differential Nonlinearity (DNL) Differential nonlinearity is the difference between an actual step width and the ideal value of 1LSB. A DNL error specification  of less than 1LSB guarantees no missing codes and a monotonic transfer function.

Offset Error

MAX1098/MAX1099

Figure 8. Remote Diode Shielding for PC Boards

example, the Belden 8451 works well for distances up to 100 feet in a noisy environment. Connect the shield to SH0-0H.

Cable resistances affect remote-sensor accuracy;  1Ω

series resistance introduces +0.45°C error.

Remote Diode Shielding Temperature measurements will reflect significant error if a portion of the bias current supplied  to the diode anode is allowed to flow through parallel paths to ground. If the diode-connected  transistor is mounted on a PC board, suppress error-producing  “leakage” current by surrounding the collector/base leads with a metal trace that is connected to the SHO shield output (Figure 8).

Layout, Grounding, and Bypassing For best performance, use PC boards. Do not use wire- wrap boards. Board layout should ensure that digital and analog signal lines are separated from each other. Do not run analog and digital (especially clock) signals parallel to one another or run digital lines underneath the ADC package.

High-frequency  noise in the VDD power supply  may affect ADC performance.  Bypass the supply  with a

0.1µF capacitor  close to pin VDD. Minimize capacitor lead lengths for best supply-noise  rejection. If the power supply is very noisy, connect a 10Ω resistor in series with the supply to provide lowpass filtering.

Offset error is the difference between the ideal and the actual offset points. For an ADC, the offset point is the midstep value when the digital output is zero.

Gain Error Gain or full-scale error is the difference between the ideal and actual gain points on the transfer function, after the offset error has been canceled  out. For an ADC, the gain point is the midstep value when the digi- tal output is full scale.

Aperture Delay Aperture delay (tAD) is the time defined between the rising edge of the sampling clock and the instant when an actual sample is taken.

Chip Information

TRANSISTOR COUNT: 13,669

PROCESS: BiCMOS

0.1µF 	+5V

VDD

AIN0

2N3904

2N3904 	(SHIELD)

AIN1

AIN2

AIN3

SHO AIN4

AIN5

MAX1098

CS SCLK

DIN DOUT

SSTRB

MAX1098/MAX1099

GND	GND

MAX1098/MAX1099

SSOP.EPS

Package Information

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses are implied. Maxim reserves the right to change the circuitry and specifications without notice at any time.

20 	Maxim Integrated Products, 120 San Gabriel Drive, Sunnyvale, CA   94086 408-737-7600

©  2000 Maxim Integrated  Products 	Printed USA 	  is a registered  trademark of Maxim Integrated  Products.