19-0065; Rev 6; 10/03

+5V, RS-232 Transceivers with 0.1µF External Capacitors

General Description 	  Next-Generation Device Features

The MAX200–MAX211/MAX213 transceivers are designed  for RS-232 and V.28 communication inter- faces where ±12V supplies are not available. On-board charge pumps convert the +5V input to the ±10V need- ed for RS-232 output levels. The MAX201 and MAX209 operate from +5V and +12V, and contain a +12V to

-12V charge-pump voltage converter.

The MAX200–MAX211/MAX213 drivers and receivers meet all EIA/TIA-232E and CCITT V.28 specifications at a data rate of 20kbps. The drivers maintain the ±5V EIA/TIA-232E output signal levels at data rates in excess of 120kbps when loaded in accordance with the EIA/TIA-232E specification.

The 5µW shutdown mode of the MAX200, MAX205, MAX206, and MAX211 conserves energy in battery- powered systems. The MAX213 has an active-low shut- down and an active-high receiver enable control. Two receivers of the MAX213 are active, allowing ring indica- tor (RI) to be monitored easily using only 75µW power.

The MAX211 and MAX213 are available in a 28-pin wide small-outline (SO) package  and a 28-pin shrink small-outline (SSOP) package,  which occupies  only

40% of the area of the SO. The MAX207 is now avail- able in a 24-pin SO package and a 24-pin SSOP. The MAX203 and MAX205 use no external components, and are recommended  for applications  with limited circuit board space.

Applications

Computers

Laptops, Palmtops, Notebooks

Battery-Powered Equipment

Hand-Held Equipment

♦ For Low-Cost Applications:

MAX221E: ±15kV ESD-Protected, +5V, 1µA, Single RS-232 Transceiver with AutoShutdown™

♦ For Low-Voltage and Space-Constrained Applications: MAX3222E/MAX3232E/MAX3237E/MAX3241E/ MAX3246E: ±15kV ESD-Protected, Down to 10nA,

+3.0V to +5.5V, Up to 1Mbps, True RS-232

Transceivers (MAX3246E Available in UCSP™ Package)

♦ For Space-Constrained Applications: MAX3228E/MAX3229E: ±15kV ESD-Protected,

+2.5V to +5.5V, RS-232 Transceivers in UCSP

♦ For Low-Voltage or Data Cable Applications: MAX3380E/MAX3381E: +2.35V TO +5.5V, 1µA,

2Tx/2Rx RS-232 Transceivers with ±15kV ESD-

Protected I/O and Logic Pins

♦ For Low-Power Applications: MAX3224E–MAX3227E/MAX3244E/MAX3245E:

±15kV ESD-Protected, 1µA, 1Mbps, +3.0V to

+5.5V, RS-232 Transceivers with AutoShutdown

Plus™

Ordering Information appears at end of data sheet

AutoShutdown, AutoShutdown Plus, and UCSP are trademarks of Maxim Integrated Products, Inc.

MAX200–MAX211/MAX213

Selector Guide

| Part Number | Power-Supply Voltage (V) | No. of RS- 232 Drivers | No. of RS-232 Receivers | No. of Receivers Active in Shutdown | No. of External Capacitors (0.1µF) | Low-Power Shutdown/ TTL Three-State |
| --- | --- | --- | --- | --- | --- | --- |
| MAX200 MAX201 MAX202 | +5 +5 and +9.0 to +13.2 +5 | 5 2 2 | 0 2 2 | 0 0 0 | 4 2 4 | Yes/No No/No No/No |
| MAX203 MAX204 MAX205 | +5 +5 +5 | 2 4 5 | 2 0 5 | 0 0 0 | None 4 None | No/No No/No Yes/Yes |
| MAX206 MAX207 MAX208 | +5 +5 +5 | 4 5 4 | 3 3 4 | 0 0 0 | 4 4 4 | Yes/Yes No/No No/No |
| MAX209 MAX211 MAX213 | +5 and +9.0 to +13.2 +5 +5 | 3 4 4 | 5 5 5 | 0 0 2 | 2 4 4 | No/Yes Yes/Yes Yes/Yes |

Maxim Integrated Products    1

For pricing, delivery, and ordering information, please contact Maxim/Dallas Direct! at

1-888-629-4642, or visit Maxim’s website at www.maxim-ic.com.

MAX200–MAX211/MAX213

+5V, RS-232 Transceivers with 0.1µF External Capacitors

Ordering Information

PART TEMP RANGE PIN-PACKAGE

MAX206EWG -40°C to +85°C 24 Wide SO

MAX206EAG -40°C to +85°C 24 SSOP

MAX207CNG 0°C to +70°C 24 Narrow Plastic Dip

MAX207CWG 0°C to +70°C 24 Wide SO

MAX207CAG 0°C to +70°C 24 SSOP

MAX207ENG -40°C to +85°C 24 Narrow Plastic Dip

MAX207EWG -40°C to +85°C 24 Wide SO

MAX207EAG -40°C to +85°C 24 SSOP

MAX208CNG 0°C to +70°C 24 Narrow Plastic Dip

MAX208CWG 0°C to +70°C 24 Wide SO

MAX208CAG 0°C to +70°C 24 SSOP

MAX208C/D 0°C to +70°C Dice*

MAX208ENG -40°C to +85°C 24 Narrow Plastic Dip

MAX208EWG -40°C to +85°C 24 Wide SO

MAX208EAG -40°C to +85°C 24 SSOP

MAX209CNG 0°C to +70°C 24 Narrow Plastic Dip

MAX209CWG 0°C to +70°C 24 Wide SO

MAX209C/D 0°C to +70°C Dice*

MAX209ENG -40°C to +85°C 24 Narrow Plastic Dip

MAX209EWG -40°C to +85°C 24 Wide SO

MAX211CWI 0°C to +70°C 28 Wide SO

MAX211CAI 0°C to +70°C 28 SSOP

MAX211C/D 0°C to +70°C Dice*

MAX211EWI -40°C to +85°C 28 Wide SO

MAX211EAI -40°C to +85°C 28 SSOP

MAX213CWI 0°C to +70°C 28 Wide SO

MAX213CAI 0°C to +70°C 28 SSOP

MAX213C/D 0°C to +70°C Dice*

MAX213EWI -40°C to +85°C 28 Wide SO

MAX213EAI -40°C to +85°C 28 SSOP

*Contact factory for dice specifications. PART TEMP RANGE PIN-PACKAGE

MAX200CPP 0°C to +70°C 20 Plastic Dip

MAX200CWP 0°C to +70°C 20 Wide SO

MAX200EPP -40°C to +85°C 20 Plastic Dip

MAX200EWP -40°C to +85°C 20 Wide SO

MAX201CPD 0°C to +70°C 14 Plastic Dip

MAX201CWE 0°C to +70°C 16 Wide SO

MAX201C/D 0°C to +70°C Dice*

MAX201EPD -40ºC to +85°C 14 Plastic Dip

MAX201EWE -40ºC to +85°C 16 Wide SO

MAX202CPE 0°C to +70°C 16 Plastic Dip

MAX202CSE 0°C to +70°C 16 Narrow SO

MAX202CWE 0°C to +70°C 16 Wide SO

MAX202C/D 0°C to +70°C Dice*

MAX202EPE -40ºC to +85°C 16 Plastic Dip

MAX202ESE -40ºC to +85°C 16 Narrow SO

MAX202EWE -40ºC to +85°C 16 Wide SO

MAX203CPP 0°C to +70°C 20 Plastic Dip

MAX203CWP 0°C to +70°C 20 Wide SO

MAX203EPP -40°C to +85°C 20 Plastic Dip

MAX203EWP -40°C to +85°C 20 Wide SO

MAX204CPE 0°C to +70°C 16 Plastic Dip

MAX204CWE 0°C to +70°C 16 Wide SO

MAX204C/D 0°C to +70°C Dice*

MAX204EPE -40ºC to +85°C 16 Plastic Dip

MAX204EWE -40ºC to +85°C 16 Wide SO

MAX205CPG 0°C to +70°C 24 Wide Plastic Dip

MAX205EPG -40°C to +85°C 24 Wide Plastic Dip

MAX206CNG 0°C to +70°C 24 Narrow Plastic Dip

MAX206CWG 0°C to +70°C 24 Wide SO

MAX206CAG 0°C to +70°C 24 SSOP

MAX206ENG -40°C to +85°C 24 Narrow Plastic Dip

19

MAX200–MAX211/MAX213

+5V, RS-232 Transceivers

| with 0.1µF External Capacitors | INCHES |  | MILLIMETERS |  |
| --- | --- | --- | --- | --- |
| DIM | MIN | MAX | MIN | MAX |
| A | 0.068 | 0.078 | 1.73 | 1.99 |
| A1 | 0.002 | 0.008 | 0.05 | 0.21 |
| B | 0.010 | 0.015 | 0.25 | 0.38 |
| C | 0.004 | 0.008 | 0.09 | 0.20 |
| D | SEE VARIATIONS |  |  |  |
| E | 0.205 | 0.212 | 5.20 | 5.38 |
| e | 0.0256 BSC |  | 0.65 BSC |  |
| H | 0.301 | 0.311 | 7.65 | 7.90 |
| L | 0.025 | 0.037 | 0.63 | 0.95 |
|  | 0∞ | 8∞ | 0∞ | 8∞ |

SSOP.EPS

| Package Information (The package drawing(s) in this data sheet may not reflect the most current specifications. For the latest package outline information, go to www.maxim-ic.com/packages.) | INCHES |  | MILLIMETERS |  |  |
| --- | --- | --- | --- | --- | --- |
|  | MIN | MAX | MIN | MAX | N |
| D | 0.239 | 0.249 | 6.07 | 6.33 | 14L |
| D | 0.239 | 0.249 | 6.07 | 6.33 | 16L |
| D | 0.278 | 0.289 | 7.07 | 7.33 | 20L |
| D | 0.317 | 0.328 | 8.07 | 8.33 | 24L |
| D | 0.397 | 0.407 | 10.07 | 10.33 | 28L |

| 2 1 E H N A C B e A1 L D NOTES: |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. D&E DO NOT INCLUDE MOLD FLASH. 2. MOLD FLASH OR PROTRUSIONS NOT TO EXCEED .15 MM (.006"). 3. CONTROLLING DIMENSION: MILLIMETERS. 4. MEETS JEDEC MO150. 5. LEADS TO BE COPLANAR WITHIN 0.10 MM. | PROPRIETARY INFORMATION |  |  |  |
|  | TITLE: PACKAGE OUTLINE, SSOP, 5.3 MM |  |  |  |
|  | APPROVAL | DOCUMENT CONTROL NO. 21-0056 | REV. C | 1 1 |

Maxim cannot assume responsibility for use of any circuitry other than circuitry entirely embodied in a Maxim product.  No circuit patent licenses are implied. Maxim reserves the right to change the circuitry and specifications without notice at any time.

20    	Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA   94086 408-737-7600

©  2003 Maxim Integrated Products 	Printed USA	is a registered trademark of Maxim Integrated Products.