19-0175; Rev 6; 3/05

±15kV ESD-Protected, +5V  RS-232 Transceivers

General Description

The MAX202E–MAX213E, MAX232E/MAX241E line drivers/receivers  are designed  for RS-232 and V.28 communications in harsh environments. Each transmitter output and receiver input is protected  against ±15kV electrostatic  discharge  (ESD) shocks, without latchup. The various combinations of features are outlined in the Selector Guide. The drivers and receivers for all ten devices  meet  all  EIA/TIA-232E and  CCITT V.28 specifications at data rates up to 120kbps, when loaded in accordance with the EIA/TIA-232E specification.

The MAX211E/MAX213E/MAX241E are available in 28- pin SO packages,  as well as a 28-pin SSOP that uses

60% less board space. The MAX202E/MAX232E come

in 16-pin  TSSOP, narrow SO, wide  SO, and  DIP packages.  The MAX203E comes in a 20-pin DIP/SO package, and needs no external charge-pump capacitors.  The MAX205E comes in a 24-pin wide DIP package,  and also eliminates external charge-pump capacitors. The MAX206E/MAX207E/MAX208E come in

24-pin  SO, SSOP, and narrow DIP packages.  The MAX232E/MAX241E operate with four 1µF capacitors, while the MAX202E/MAX206E/MAX207E/MAX208E/

Next-Generation Device Features

♦ For Low-Voltage Applications MAX3222E/MAX3232E/MAX3237E/MAX3241E/ MAX3246E: ±15kV ESD-Protected Down to

10nA, +3.0V to +5.5V, Up to 1Mbps, True RS-232

Transceivers (MAX3246E Available in a UCSP™ Package)

♦ For Low-Power Applications MAX3221/MAX3223/MAX3243: 1µA Supply Current, True +3V to +5.5V RS-232 Transceivers with Auto-Shutdown™

♦ For Space-Constrained Applications

MAX3233E/MAX3235E: ±15kV ESD-Protected,

1µA, 250kbps, +3.0V/+5.5V, Dual RS-232

Transceivers with Internal Capacitors

♦ For Low-Voltage or Data Cable Applications

MAX3380E/MAX3381E: +2.35V to +5.5V, 1µA,

2Tx/2Rx RS-232 Transceivers with ±15kV ESD- Protected I/O and Logic Pins

Ordering Information

MAX211E/MAX213E operate with four 0.1µF capacitors, further reducing cost and board space.

Applications

PART MAX202ECPE

MAX202ECSE

TEMP RANGE

0°C to +70°C

0°C to +70°C

PIN-PACKAGE

16 Plastic DIP

16 Narrow SO

Notebook, Subnotebook, and Palmtop Computers

Battery-Powered Equipment

Hand-Held Equipment

Ordering Information continued at end of data sheet.

Pin Configurations and Typical Operating Circuits appear at end of data sheet.

AutoShutdown  and UCSP are trademarks  of Maxim Integrated

Products, Inc.

MAX202E–MAX213E, MAX232E/MAX241E

Selector Guide

| PART | NO. OF RS-232 DRIVERS | NO. OF RS-232 RECEIVERS | RECEIVERS ACTIVE IN SHUTDOWN | NO. OF EXTERNAL CAPACITORS (µF) | LOW-POWER SHUTDOWN | TTL TRI- STATE |
| --- | --- | --- | --- | --- | --- | --- |
| MAX202E | 2 | 2 | 0 | 4 (0.1) | No | No |
| MAX203E | 2 | 2 | 0 | None | No | No |
| MAX205E | 5 | 5 | 0 | None | Yes | Yes |
| MAX206E | 4 | 3 | 0 | 4 (0.1) | Yes | Yes |
| MAX207E | 5 | 3 | 0 | 4 (0.1) | No | No |
| MAX208E | 4 | 4 | 0 | 4 (0.1) | No | No |
| MAX211E | 4 | 5 | 0 | 4 (0.1) | Yes | Yes |
| MAX213E | 4 | 5 | 2 | 4 (0.1) | Yes | Yes |
| MAX232E | 2 | 2 | 0 | 4 (1) | No | No |
| MAX241E | 4 | 5 | 0 | 4 (1) | Yes | Yes |

ABSOLUTE MAXIMUM RATINGS

VCC..........................................................................-0.3V to +6V

V+ ................................................................(VCC - 0.3V) to +14V V- ............................................................................-14V to +0.3V Input Voltages

T_IN ............................................................-0.3V to (V+ + 0.3V) R_IN ...................................................................................±30V

Output Voltages

T_OUT.................................................(V- - 0.3V) to (V+ + 0.3V) R_OUT ......................................................-0.3V to (VCC + 0.3V) Short-Circuit  Duration, T_OUT....................................Continuous

Continuous Power Dissipation  (TA = +70°C)

16-Pin Plastic DIP (derate 10.53mW/°C above +70°C)....842mW

16-Pin Narrow SO (derate 8.70mW/°C above +70°C) .....696mW

16-Pin Wide SO (derate 9.52mW/°C above +70°C) ......762mW

16-Pin TSSOP (derate 9.4mW/°C above +70°C) ...........755mW

20-Pin Plastic DIP (derate 11.11mW/°C above +70°C)...889mW

20-Pin SO (derate 10.00mW/°C above +70°C).............800mW

24-Pin Narrow Plastic DIP

(derate 13.33mW/°C above +70°C)  ...............................1.07W

24-Pin Wide Plastic DIP

(derate 14.29mW/°C above +70°C) ................................1.14W

24-Pin SO (derate 11.76mW/°C above +70°C).............941mW

24-Pin SSOP (derate 8.00mW/°C above +70°C) ..........640mW

28-Pin SO (derate 12.50mW/°C above +70°C)....................1W

28-Pin SSOP (derate 9.52mW/°C above +70°C) ..........762mW Operating  Temperature  Ranges

MAX2_ _EC_ _ .....................................................0°C to +70°C MAX2_ _EE_ _...................................................-40°C to +85°C Storage Temperature  Range .............................-65°C to +165°C Lead Temperature  (soldering,  10s) .................................+300°C

MAX202E–MAX213E, MAX232E/MAX241E

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS

(VCC = +5V ±10%  for MAX202E/206E/208E/211E/213E/232E/241E; VCC = +5V ±5%  for MAX203E/205E/207E;  C1–C4 = 0.1µF for MAX202E/206E/207E/208E/211E/213E; C1–C4 = 1µF for MAX232E/241E; TA = TMIN to TMAX; unless otherwise  noted. Typical values are at TA = +25°C.)

| PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| DC CHARACTERISTICS |  |  |  |  |  |
| VCC Supply Current | ICC | No load, TA = +25°C | MAX202E/203E | 8 15 | mA |
|  |  |  | MAX205E–208E | 11 20 |  |
|  |  |  | MAX211E/213E | 14 20 |  |
|  |  |  | MAX232E | 5 10 |  |
|  |  |  | MAX241E | 7 15 |  |
| Shutdown Supply Current |  | TA = +25°C, Figure 1 | MAX205E/206E | 1 10 | µA |
|  |  |  | MAX211E/241E | 1 10 |  |
|  |  |  | MAX213E | 15 50 |  |
| LOGIC |  |  |  |  |  |
| Input Pullup Current |  | T_IN = 0V (MAX205E–208E/211E/213E/241E) |  | 15 200 | µA |
| Input Leakage Current |  | T_IN = 0V to VCC (MAX202E/203E/232E) |  | ±10 | µA |
| Input Threshold Low | VIL | T_IN; EN, SHDN (MAX213E) or EN, SHDN (MAX205E–208E/211E/241E) |  | 0.8 | V |
| Input Threshold High | VIH | T_IN |  | 2.0 | V |
|  |  | EN, SHDN (MAX213E) or EN, SHDN (MAX205E–208E/211E/241E) |  | 2.4 |  |
| Output-Voltage Low | VOL | R_OUT; IOUT = 3.2mA (MAX202E/203E/232E) or IOUT = 1.6mA (MAX205E/208E/211E/213E/241E) |  | 0.4 | V |
| Output-Voltage High | VOH | R_OUT; IOUT = -1.0mA |  | 3.5 VCC - 0.4 | V |
| Output Leakage Current |  | EN = VCC, EN = 0V, 0V ≤ ROUT ≤ VCC, MAX205E–208E/211E/213E/241E outputs disabled |  | ±0.05 ±10 | µA |

MAX202E–MAX213E, MAX232E/MAX241E

ELECTRICAL CHARACTERISTICS (continued)

(VCC = +5V ±10%  for MAX202E/206E/208E/211E/213E/232E/241E; VCC = +5V ±5%  for MAX203E/205E/207E;  C1–C4 = 0.1µF for MAX202E/206E/207E/208E/211E/213E; C1–C4 = 1µF for MAX232E/241E; TA = TMIN to TMAX; unless otherwise  noted. Typical values are at TA = +25°C.)

sted with VCC = +5V ±5%.

213EE_ _ te

Note 1: MAX211EE_ _ and MAX

| PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| EIA/TIA-232E RECEIVER INPUT | S |  |  |  |  |
| Input Voltage Range |  |  |  | -30 30 | V |
| Input Threshold Low |  | TA = +25°C, VCC = 5V | All parts, normal operation | 0.8 1.2 | V |
|  |  |  | MAX213E, SHDN = 0V, EN = VCC | 0.6 1.5 |  |
| Input Threshold High |  | TA = +25°C, VCC = 5V | All parts, normal operation | 1.7 2.4 | V |
|  |  |  | MAX213E (R4, R5), SHDN = 0V, EN = VCC | 1.5 2.4 |  |
| Input Hysteresis |  | VCC = 5V, no hysteresis in shutdown |  | 0.2 0.5 1.0 | V |
| Input Resistance |  | TA = +25°C, VCC = 5V |  | 3 5 7 | kΩ |
| EIA/TIA-232E TRANSMITTER O | UTPUTS |  |  |  |  |
| Output Voltage Swing |  | All drivers loaded with 3kΩ to ground (Note 1) |  | ±5 ±9 | V |
| Output Resistance |  | VCC = V+ = V- = 0V, VOUT = ±2V |  | 300 | Ω |
| Output Short-Circuit Current |  |  |  | ±10 ±60 | mA |
| TIMING CHARACTERISTICS |  |  |  |  |  |
| Maximum Data Rate |  | RL = 3kΩ to 7kΩ, CL = 50pF to 1000pF, one transmitter switching |  | 120 | kbps |
| Receiver Propagation Delay | tPLHR, tPHLR | CL = 150pF | All parts, normal operation | 0.5 10 | µs |
|  |  |  | MAX213E (R4, R5), SHDN = 0V, EN = VCC | 4 40 |  |
| Receiver Output Enable Time |  | MAX205E/206E/211E/213E/241E normal operation, Figure 2 |  | 600 | ns |
| Receiver Output Disable Time |  | MAX205E/206E/211E/213E/241E normal operation, Figure 2 |  | 200 | ns |
| Transmitter Propagation Delay | tPLHT, tPHLT | RL = 3kΩ, CL = 2500pF, all transmitters loaded |  | 2 | µs |
| Transition-Region Slew Rate |  | TA = +25°C, VCC = 5V, RL = 3kΩ to 7kΩ, CL = 50pF to 1000pF, measured from -3V to +3V or +3V to -3V, Figure 3 |  | 3 6 30 | V/µs |
| ESD PERFORMANCE: TRANS | ITTER OU | TPUTS, RECEIVER INPUTS |  |  |  |
| ESD-Protection Voltage |  | Human Body Model |  | ±15 | kV |
|  |  | IEC1000-4-2, Contact Discharge |  | ±8 |  |
|  |  | IEC1000-4-2, Air-Gap Discharge |  | ±15 |  |

Note 1: MAX211EE_ _ tested with VCC = +5V ±5%.

Typical Operating Characteristics

(Typical Operating  Circuits, VCC = +5V, TA = +25°C, unless otherwise noted.)

MAX202E-TOC1

MAX232E TRANSMITTER  OUTPUT VOLTAGE

MAX202E/MAX203E TRANSMITTER  OUTPUT VOLTAGE

MAX241E TRANSMITTER  OUTPUT VOLTAGE

7.5

ALL TRANSMITTERS LOADED DATA RATE = 120kbps

RL = 3kΩ

7.5

ALL TRANSMITTERS LOADED DATA RATE = 120kbps

RL = 3kΩ

7.5

ALL TRANSMITTERS LOADED DATA RATE = 120kbps

MAX202E-TOC3

RL = 3kΩ

7.0

6.5

6.0

VCC = 5.0V

VCC = 5.5V

VOH, -VOL (V)

MAX202E-TOC2

7.0

6.5

VOH, -VOL (V)

6.0

VCC = 5.0V

VCC = 5.5V

7.0

6.5

6.0

VCC = 5.0V

5.0

0

VCC = 4.5V

1000	2000	3000	4000    5000

LOAD CAPACITANCE (pF)

5.0

0

VCC = 4.5V

1000	2000	3000	4000    5000

LOAD CAPACITANCE (pF)

5.0

0

VCC = 4.5V

1000	2000	3000	4000    5000

LOAD CAPACITANCE (pF)

7.5

MAX211E/MAX213E TRANSMITTER  OUTPUT VOLTAGE vs. LOAD CAPACITANCE

MAX202E-TOC4

ALL TRANSMITTERS LOADED DATA RATE = 120kbps

RL = 3kΩ

MAX211E/MAX213E/MAX241E TRANSMITTER SLEW RATE vs. LOAD CAPACITANCE

MAX202E-TOC5

ALL TRANSMITTERS LOADED DATA RATE = 120kbps

25	RL = 3kΩ

VOH, -VOL (V)

SLEW RATE ( V/µs)

7.0	20

6.5

15

VCC = 5.5V

6.0

5.5

5.0

10

5

+SLEW RATE

0

-SLEW RATE

MAX202E–MAX213E,

MAX232E/MAX241E

VOH, -VOL (V)

0	1000	2000	3000	4000    5000

LOAD CAPACITANCE (pF)

0	1000	2000	3000	4000    5000

LOAD CAPACITANCE (pF)

Typical Operating Characteristics (continued)

(Typical Operating  Circuits, VCC = +5V, TA = +25°C, unless otherwise noted.)

MAX202E/MAX203E/MAX232E TRANSMITTER SLEW RATE

MAX205E–MAX208E TRANSMITTER  OUTPUT VOLTAGE

MAX205E–MAX208E TRANSMITTER SLEW RATE

ALL TRANSMITTERS LOADED DATA RATE = 120kbps

MAX202E-TOC6

OUTPUT VOLTAGE (V)

12	RL = 3kΩ	5.0

SLEW RATE ( V/µs)

10	2.5

240kbps

20kbps

MAX202E TOC-07

120kbps

18	VCC = +4.5V, RL = 3kΩ

1 TRANSMITTER AT FULL DATA RATE

MAX202E TOC-08

16	4 TRANSMITTERS AT 1/8 DATA RATE

8

-SLEW  RATE

6

0

-2.5

1 TRANSMITTER AT FULL DATA RATE

4 TRANSMITTERS AT 1/8 DATA RATE

RISE

10

8

SLEW RATE (V/µs)

6

+SLEW RATE

4

2

0	1000	2000	3000	4000    5000

-5.0

-7.5

0

1000	2000

240kbps

3000

120kbps

4000	5000

4

2

0

0	1000	2000

3000

4000	5000

LOAD CAPACITANCE (pF)

LOAD CAPACITANCE (pF)

LOAD CAPACITANCE (pF)

MAX205E–MAX208E SUPPLY CURRENT

vs. LOAD CAPACITANCE

50

MAX202E TOC-09

240kbps

45

10.0

7.5

OUTPUT VOLTAGE (V)

5.0

MAX205E–MAX208E OUTPUT VOLTAGE vs. DATA RATE

MAX202E TOC-10

V+

VOUT+

SUPPLY CURRENT (mA)

35	0

VCC = +4.5V, RL = 3kΩ

1 TRANSMITTER AT FULL DATA RATE

4 TRANSMITTERS AT 1/8 DATA RATE

-5.0

VOUT-

1 TRANSMITTER AT FULL DATA RATE

4 TRANSMITTERS AT 1/8 DATA RATE

20

-7.5	V-

-10.0

0	1000	2000

3000

4000	5000

0     30    60

90   120 150  180

210  240

MAX202E–MAX213E, MAX232E/MAX241E

LOAD CAPACITANCE (pF)

DATA RATE (kbps)

MAX202E–MAX213E, MAX232E/MAX241E

Pin  Descriptions

MAX202E/MAX232E

| PIN |  | NAME | FUNCTION |
| --- | --- | --- | --- |
| DIP/SO/TSSOP | LCC |  |  |
| 1, 3 | 2, 4 | C1+, C1- | Terminals for Positive Charge-Pump Capacitor |
| 2 | 3 | V+ | +2VCC Voltage Generated by the Charge Pump |
| 4, 5 | 5, 7 | C2+, C2- | Terminals for Negative Charge-Pump Capacitor |
| 6 | 8 | V- | -2VCC Voltage Generated by the Charge Pump |
| 7, 14 | 9, 18 | T_OUT | RS-232 Driver Outputs |
| 8, 13 | 10, 17 | R_IN | RS-232 Receiver Inputs |
| 9, 12 | 12, 15 | R_OUT | RS-232 Receiver Outputs |
| 10, 11 | 13, 14 | T_IN | RS-232 Driver Inputs |
| 15 | 19 | GND | Ground |
| 16 | 20 | VCC | +4.5V to +5.5V Supply-Voltage Input |
|  | 1, 6, 11, 16 | N.C. | No Connection—Not Internally Connected |

MAX203E

| PIN |  | NAME | FUNCTION |
| --- | --- | --- | --- |
| DIP | SO |  |  |
| 1, 2 | 1, 2 | T_IN | RS-232 Driver Inputs |
| 3, 20 | 3, 20 | R_OUT | RS-232 Receiver Outputs |
| 4,19 | 4, 19 | R_IN | RS-232 Receiver Inputs |
| 5,18 | 5, 18 | T_OUT | RS-232 Transmitter Outputs |
| 6, 9 | 6, 9 | GND | Ground |
| 7 | 7 | VCC | +4.5V to +5.5V Supply-Voltage Input |
| 8 | 13 | C1+ | Make no connection to this pin. |
| 10, 16 | 11, 16 | C2- | Connect pins together. |
| 12, 17 | 10, 17 | V- | -2VCC Voltage Generated by the Charge Pump. Connect pins together. |
| 13 | 14 | C1- | Make no connection to this pin. |
| 14 | 8 | V+ | +2VCC Voltage Generated by the Charge Pump |
| 11, 15 | 12, 15 | C2+ | Connect pins together. |

MAX205E

| PIN | NAME | FUNCTION |
| --- | --- | --- |
| 1–4, 19 | T_OUT | RS-232 Driver Outputs |
| 5, 10, 13, 18, 24 | R_IN | RS-232 Receiver Inputs |
| 6, 9, 14, 17, 23 | R_OUT | TTL/CMOS Receiver Outputs. All receivers are inactive in shutdown. |
| 7, 8, 15, 16, 22 | T_IN | TTL/CMOS Driver Inputs. Internal pullups to VCC. |
| 11 | GND | Ground |
| 12 | VCC | +4.75V to +5.25V Supply Voltage |
| 20 | EN | Receiver Enable—Active Low |
| 21 | SHDN | Shutdown Control—Active High |

MAX202E–MAX213E, MAX232E/MAX241E

Pin  Descriptions (continued)

MAX206E

| PIN | NAME | FUNCTION |
| --- | --- | --- |
| 1, 2, 3, 24 | T_OUT | RS-232 Driver Outputs |
| 4, 16, 23 | R_IN | RS-232 Receiver Inputs |
| 5, 17, 22 | R_OUT | TTL/CMOS Receiver Outputs. All receivers are inactive in shutdown. |
| 6, 7, 18, 19 | T_IN | TTL/CMOS Driver Inputs. Internal pullups to VCC. |
| 8 | GND | Ground |
| 9 | VCC | +4.5V to +5.5V Supply Voltage |
| 10, 12 | C1+, C1- | Terminals for Positive Charge-Pump Capacitor |
| 11 | V+ | +2VCC Generated by the Charge Pump |
| 13, 14 | C2+, C2- | Terminals for Negative Charge-Pump Capacitor |
| 15 | V- | -2VCC Generated by the Charge Pump |
| 20 | EN | Receiver Enable—Active Low |
| 21 | SHDN | Shutdown Control—Active High |

MAX207E

| PIN | NAME | FUNCTION |
| --- | --- | --- |
| 1, 2, 3, 20, 24 | T_OUT | RS-232 Driver Outputs |
| 4, 16, 23 | R_IN | RS-232 Receiver Inputs |
| 5, 17, 22 | R_OUT | TTL/CMOS Receiver Outputs. All receivers are inactive in shutdown. |
| 6, 7, 18, 19, 21 | T_IN | TTL/CMOS Driver Inputs. Internal pullups to VCC. |
| 8 | GND | Ground |
| 9 | VCC | +4.75V to +5.25V Supply Voltage |
| 10, 12 | C1+, C1- | Terminals for Positive Charge-Pump Capacitor |
| 11 | V+ | +2VCC Generated by the Charge Pump |
| 13, 14 | C2+, C2- | Terminals for Negative Charge-Pump Capacitor |
| 15 | V- | -2VCC Generated by the Charge Pump |

MAX208E

| PIN | NAME | FUNCTION |
| --- | --- | --- |
| 1, 2, 20, 24 | T_OUT | RS-232 Driver Outputs |
| 3, 7, 16, 23 | R_IN | RS-232 Receiver Inputs |
| 4, 6, 17, 22 | R_OUT | TTL/CMOS Receiver Outputs. All receivers are inactive in shutdown. |
| 5, 18, 19, 21 | T_IN | TTL/CMOS Driver Inputs. Internal pullups to VCC. |
| 8 | GND | Ground |
| 9 | VCC | +4.5V to +5.5V Supply Voltage |
| 10, 12 | C1+, C1- | Terminals for Positive Charge-Pump Capacitor |
| 11 | V+ | +2VCC Generated by the Charge Pump |
| 13, 14 | C2+, C2- | Terminals for Negative Charge-Pump Capacitor |
| 15 | V- | -2VCC Generated by the Charge Pump |

Pin  Descriptions (continued)

MAX211E/MAX213E/MAX241E

| PIN | NAME | FUNCTION |
| --- | --- | --- |
| 1, 2, 3, 28 | T_OUT | RS-232 Driver Outputs |
| 4, 9, 18, 23, 27 | R_IN | RS-232 Receiver Inputs |
| 5, 8, 19, 22, 26 | R_OUT | TTL/CMOS Receiver Outputs. For the MAX213E, receivers R4 and R5 are active in shutdown mode when EN = 1. For the MAX211E and MAX241E, all receivers are inactive in shutdown. |
| 6, 7, 20, 21 | T_IN | TTL/CMOS Driver Inputs. Only the MAX211E, MAX213E, and MAX241E have internal pullups to VCC. |
| 10 | GND | Ground |
| 11 | VCC | +4.5V to +5.5V Supply Voltage |
| 12, 14 | C1+, C1- | Terminals for Positive Charge-Pump Capacitor |
| 13 | V+ | +2VCC Voltage Generated by the Charge Pump |
| 15, 16 | C2+, C2- | Terminals for Negative Charge-Pump Capacitor |
| 17 | V- | -2VCC Voltage Generated by the Charge Pump |
| 24 | EN | Receiver Enable—Active Low (MAX211E, MAX241E) |
|  | EN | Receiver Enable—Active High (MAX213E) |
| 25 | SHDN | Shutdown Control—Active High (MAX211E, MAX241E) |
|  | SHDN | Shutdown Control—Active Low (MAX213E) |

+5.5V

0.1µF*

0.1µF*

0.1µF*

ISHDN

C1+ C1-

C2+  VCC

C2-

VCC

MAX206E MAX211E MAX213E MAX241E

0.1µF*

V+

0.1µF*

V-

EN INPUT

+3V

RECEIVER OUTPUT

CL  = 150pF

0V

OUTPUT ENABLE TIME

+3.5V

+0.8V

+5.5V

T_IN

400kΩ

R1 TO R5

T1 TO T5

T_OUT

3kΩ

EN	0V INPUT

+3V

0V OR +5.5V DRIVE

+5.5V (0V)

( ) ARE FOR MAX213E

R_OUT

EN (EN)

SHDN (SHDN)

GND

R_IN

5kΩ

+5.5V

CAPACITORS MAY BE

NOTE:

POLARITY OF EN IS REVERSED FOR THE MAX213E

RECEIVER OUTPUTS

VOH

VOL

OUTPUT DISABLE TIME

VOH - 0.1V

+2.5V RL = 1kΩ

VOL + 0.1V

* 1µF FOR MAX241E

POLARIZED OR UNPOLARIZED

MAX202E–MAX213E, MAX232E/MAX241E

Figure 1. Shutdown-Current Test Circuit (MAX206E, MAX211E/MAX213E/MAX241E)

Figure 2. Receiver Output Enable and Disable Timing

(MAX205E/MAX206E/MAX211E/MAX213E/MAX241E)

+5V

0.1µF*

0.1µF*

+5V

0.1µF*

0.1µF*

0.1µF*

0.1µF*

C1+ C1-

C2+   VCC

VCC

MAX2_ _E

V+

0.1µF*

V-

0.1µF*

0.1µF*

C1+ C1- C2+

VCC

VCC

MAX2_ _E

V+

0.1µF*

V-

C2-

T_IN

400kΩ

T_

T_OUT

C2-

T_IN

400kΩ

T_

T_OUT

0V (+5V)

0V (+5V)

R_ R_OUT

EN (EN)

SHDN (SHDN)

R_IN

5kΩ

3kΩ

2500pF

0V (+5V)

0V (+5V)

R_ R_OUT

EN (EN)

SHDN (SHDN)

R_IN

5kΩ

7kΩ

50pF

MINIMUM SLEW-RATE TEST CIRCUIT

( ) ARE FOR MAX213E

* 1µF FOR MAX232E/MAX241E

MAXIMUM SLEW-RATE TEST CIRCUIT

TRANSMITTER INPUT PULL-UP  RESISTORS, ENABLE,  AND SHUTDOWN ARE NOT PROVIDED ON THE MAX202E, MAX203E, AND MAX232E. ENABLE AND SHUTDOWN ARE NOT PROVIDED ON THE MAX207E AND MAX208E.

Figure 3.  Transition Slew-Rate Circuit

MAX202E–MAX213E, MAX232E/MAX241E

Detailed Description

The MAX202E–MAX213E, MAX232E/MAX241E consist of three sections:   charge-pump  voltage converters, drivers (transmitters), and receivers. These E versions provide  extra protection  against ESD. They survive

±15kV discharges  to the RS-232 inputs and outputs, tested using the Human Body Model. When tested according  to IEC1000-4-2, they survive ±8kV contact- discharges and ±15kV air-gap discharges. The rugged E versions are intended for use in harsh environments or applications  where the RS-232 connection  is frequently changed (such as notebook computers). The standard  (non-“E”)  MAX202, MAX203, MAX205– MAX208, MAX211, MAX213, MAX232, and MAX241 are recommended for applications where cost is critical.

+5V  to ±10V Dual Charge-Pump

Voltage Converter The +5V to ±10V conversion  is performed  by dual charge-pump  voltage converters (Figure 4). The first charge-pump  converter uses capacitor  C1 to double the +5V into +10V, storing the +10V on the output filter capacitor,  C3. The second uses C2 to invert the +10V

into -10V, storing the -10V on the V- output  filter capacitor, C4.

In shutdown mode, V+ is internally connected to VCC by a 1kΩ pull-down resistor, and V- is internally connected to ground by a 1kΩ pull up resistor.

RS-232 Drivers

With VCC = 5V, the typical driver output voltage swing

is  ±8V  when  loaded  with  a  nominal  5kΩ  RS-232

receiver. The output  swing is guaranteed  to meet EIA/TIA-232E and V.28 specifications  that call for ±5V minimum output levels under worst-case conditions. These  include  a  3k Ω  load,  minimum  V CC ,  and maximum operating  temperature.  The open-circuit output voltage swings from (V+ - 0.6V) to V-.

Input  thresholds  are CMOS/TTL compatible.  The unused drivers’ inputs on the MAX205E–MAX208E, MAX211E, MAX213E, and  MAX241E can  be  left unconnected  because  400kΩ pull up resistors to VCC are included on-chip. Since all drivers invert, the pull up resistors force the unused drivers’ outputs low. The MAX202E, MAX203E, and MAX232E do not have pull up resistors on the transmitter inputs.

S1

VCC

C1+

V+

S2	S5

C2+   S6

GND

GND

C1	C3	C2

IL +	RL +	C4

S3	S4	S7	S8

VCC

IL- 	RL-

| V - PART | fCLK (kHz) |
| --- | --- |
| MAX202E MAX203E MAX205E–208E MAX211E/213E MAX232E MAX241E | 230 230 200 200 140 30 |

C1-

C2-

fCLK

Figure 4.  Charge-Pump  Diagram

MAX202E–MAX213E, MAX232E/MAX241E

When in low-power shutdown mode, the MAX205E/ MAX206E/MAX211E/MAX213E/MAX241E driver outputs are turned off and draw only leakage currents—even if they are back-driven  with voltages between 0V and

12V. Below -0.5V in shutdown, the transmitter output is diode-clamped    to   ground   with   a   1k Ω   series impedance.

RS-232 Receivers The receivers convert the RS-232 signals to CMOS-logic output levels. The guaranteed  0.8V and 2.4V receiver input thresholds are significantly  tighter than the ±3V thresholds required by the EIA/TIA-232E specification. This allows the receiver inputs to respond to TTL/CMOS- logic levels, as well as RS-232 levels.

The guaranteed  0.8V input low threshold ensures that receivers shorted to ground have a logic 1 output. The

5kΩ input resistance to ground ensures that a receiver with its input left open will also have a logic 1 output.

Receiver inputs have approximately  0.5V hysteresis. This provides clean output transitions, even with slow rise/fall-time signals with moderate amounts of noise and ringing.

In shutdown, the MAX213E’s R4 and R5 receivers have no hysteresis.

Shutdown and Enable Control (MAX205E/MAX206E/MAX211E/ MAX213E/MAX241E)

In shutdown mode, the charge pumps are turned off,

V+ is pulled down to VCC, V- is pulled to ground, and the transmitter outputs are disabled.  This reduces supply current typically to 1µA (15µA for the MAX213E). The time required  to exit shutdown is under 1ms, as shown in Figure 5.

Receivers All MAX213E receivers, except R4 and R5, are put into a high-impedance  state in shutdown mode (see Tables

1a and 1b). The MAX213E’s R4 and R5 receivers still function in shutdown  mode.  These two awake-in- shutdown receivers can monitor external activity while maintaining minimal power consumption.

The enable control is used to put the receiver outputs into a high-impedance  state, to allow wire-OR connection of two EIA/TIA-232E ports (or ports of different types) at the UART. It has no effect on the RS-232 drivers or the charge pumps.

Note:   The enable control pin is active low for the MAX211E/MAX241E (EN), but is active high for the MAX213E (EN). The shutdown control pin is active high for the MAX205E/MAX206E/MAX211E/MAX241E (SHDN), but is active low for the MAX213E (SHDN).

The MAX213E’s receiver propagation  delay is typically

0.5µs in normal operation. In shutdown mode, propagation  delay increases to 4µs for both rising and falling transitions. The MAX213E’s receiver inputs have approximately  0.5V hysteresis, except  in shutdown, when receivers R4 and R5 have no hysteresis.

When entering shutdown with receivers active, R4 and R5 are not valid until 80µs after SHDN is driven low. When coming out of shutdown, all receiver outputs are invalid until the charge pumps reach nominal voltage levels (less than 2ms when using 0.1µF capacitors).

±15kV ESD Protection As with all Maxim devices,  ESD-protection structures are incorporated on all pins to protect against electrostatic  discharges  encountered  during  handling and assembly. The driver outputs and receiver inputs have extra protection against static electricity. Maxim’s engineers developed  state-of-the-art structures to

3V

0V

10V

5V

0V

-5V

-10V

MAX211E

200µs/div

SHDN

V+

V-

protect  these pins against ESD of ±15kV  without damage. The ESD structures withstand high ESD in all states:   normal operation, shutdown, and powered down. After an ESD event, Maxim’s E versions keep working without latchup,  whereas competing  RS-232 products  can latch and must be powered  down to remove latchup.

ESD protection  can be tested in various ways; the transmitter outputs and receiver inputs of this product family are characterized  for protection to the following limits:

1)  ±15kV using the Human Body Model

2)  ±8kV using the contact-discharge  method specified in IEC1000-4-2

3)  ±15kV using IEC1000-4-2’s air-gap method.

ESD Test Conditions ESD performance  depends  on a variety of conditions. Contact Maxim for a reliability report that documents test set-up, test methodology, and test results.

Human Body Model

Figure 6a shows the Human Body Model, and Figure

6b shows the current waveform it generates when discharged  into a low impedance.  This model consists of a 100pF capacitor  charged  to the ESD voltage of interest, which is then discharged  into the test device through a 1.5kΩ resistor.

Figure 5.  MAX211E V+ and V- when Exiting Shutdown (0.1µF

capacitors)

Table 1a. MAX205E/MAX206E/MAX211E/ MAX241E Control Pin Configurations

| SHDN | EN | OPERATION STATUS | Tx | Rx |
| --- | --- | --- | --- | --- |
| 0 | 0 | Normal Operation | All Active | All Active |
| 0 | 1 | Normal Operation | All Active | All High-Z |
| 1 | X | Shutdown | All High-Z | All High-Z |

X = Don't care.

Table 1b. MAX213E Control Pin

| Configurations SHDN | EN | OPERATION STATUS | Tx 1–4 | Rx |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1–3 | 4, 5 |
| 0 | 0 | Shutdown | All High-Z | High-Z | High-Z |
| 0 | 1 | Shutdown | All High-Z | High-Z | Active* |
| 1 | 0 | Normal Operation | All Active | High-Z | High-Z |
| 1 | 1 | Normal Operation | All Active | Active | Active |

MAX202E–MAX213E, MAX232E/MAX241E

*Active = active with reduced performance

RC 1MΩ	RD 1500Ω

IP 100%

Ir 	PEAK-TO-PEAK RINGING

HIGH-

CHARGE-CURRENT-

LIMIT RESISTOR

DISCHARGE

RESISTANCE

DEVICE

AMPERES

90%

(NOT DRAWN TO SCALE)

VOLTAGE DC SOURCE

Cs

100pF

STORAGE

CAPACITOR

UNDER TEST

36.8%

10%

0

0

tRL	TIME

tDL

CURRENT WAVEFORM

Figure 6a.  Human Body ESD Test Model

Figure 6b.  Human Body Model Current Waveform

RC 50MΩ to 100MΩ	RD 330Ω

I

100%

CHARGE-CURRENT- LIMIT RESISTOR

IPEAK

DISCHARGE RESISTANCE

90%

HIGH- VOLTAGE DC SOURCE

Cs

150pF

STORAGE CAPACITOR

DEVICE UNDER TEST

10%

Figure 7a.  IEC1000-4-2 ESD Test Model

IEC1000-4-2

tr = 0.7ns to 1ns

30ns

t

60ns

MAX202E–MAX213E,

MAX232E/MAX241E

The IEC1000-4-2 standard  covers ESD testing and performance  of finished  equipment;  it does  not specifically refer to integrated circuits. The MAX202E/MAX203E–MAX213E, MAX232E/MAX241E help you design  equipment  that meets level 4 (the highest level) of IEC1000-4-2, without the need for additional ESD-protection components.

The major difference  between tests done using the Human Body Model and IEC1000-4-2 is higher peak current in IEC1000-4-2, because  series resistance is lower in the IEC1000-4-2 model. Hence, the ESD

withstand voltage measured to IEC1000-4-2 is generally lower than that measured using the Human Body Model. Figure 7b shows the current waveform for the 8kV IEC1000-4-2 level-four ESD contact-discharge test.

Figure 7b.  IEC1000-4-2 ESD Generator Current Waveform

The air-gap test involves approaching  the device with a charged  probe.  The contact-discharge   method connects the probe  to the device before the probe  is energized.

Machine Model

The Machine Model for ESD tests all pins using a

200pF storage capacitor and zero discharge resistance. Its objective is to emulate the stress caused by contact  that occurs  with handling  and assembly during  manufacturing.  Of course, all pins require this protection during manufacturing, not just RS-232 inputs and outputs. Therefore, after PC board assembly, the Machine Model is less relevant to I/O ports.

Applications Information

Capacitor Selection The capacitor  type used for C1–C4 is not critical  for proper operation. The MAX202E, MAX206–MAX208E, MAX211E, and MAX213E require 0.1µF capacitors, and  the  MAX232E  and  MAX241E  require  1µ F capacitors, although in all cases capacitors up to 10µF can  be  used  without harm. Ceramic,  aluminum- electrolytic,  or tantalum capacitors  are suggested  for the 1µF capacitors,  and  ceramic  dielectrics  are suggested  for the 0.1µF capacitors.  When using the minimum recommended  capacitor  values, make sure the capacitance  value does not degrade  excessively as the operating  temperature varies. If in doubt,  use capacitors  with a larger (e.g., 2x) nominal value. The capacitors’  effective series resistance (ESR), which usually rises at low temperatures,  influences  the amount of ripple on V+ and V-.

Use larger capacitors (up to 10µF) to reduce the output impedance  at V+ and V-. This can be useful when “stealing” power from V+ or from V-. The MAX203E and MAX205E have internal charge-pump capacitors.

Bypass  V CC  to  ground  with  at  least  0.1 µF.  In applications sensitive to power-supply noise generated by the charge pumps, decouple  VCC to ground with a capacitor the same size as (or larger than) the charge- pump capacitors (C1–C4).

V+ and V- as  Power Supplies A small amount of power can be drawn from V+ and V-, although this will reduce both driver output swing and noise margins. Increasing the value of the charge-pump capacitors  (up to 10µF) helps maintain performance when power is drawn from V+ or V-.

Driving Multiple Receivers Each transmitter is designed to drive a single receiver. Transmitters can  be  paralleled  to drive  multiple receivers.

Driver Outputs when Exiting Shutdown The driver outputs display  no ringing  or undesirable transients as they come out of shutdown.

High Data Rates These transceivers  maintain  the  RS-232 ±5.0V minimum driver output voltages at data rates of over

120kbps. For data rates above 120kbps, refer to the Transmitter Output  Voltage vs. Load Capacitance graphs  in the Typical Operating  Characteristics. Communication at these high rates is easier if the capacitive  loads on the transmitters are small; i.e., short cables are best.

MAX202E–MAX213E, MAX232E/MAX241E

Table 2. Summary of EIA/TIA-232E, V.28 Specifications

| PARAMETER |  | CONDITIONS | EIA/TIA-232E, V.28 SPECIFICATIONS |
| --- | --- | --- | --- |
| Driver Output Voltage | 0 Level | 3kΩ to 7kΩ load | +5V to +15V |
|  | 1 Level | 3kΩ to 7kΩ load | -5V to -15V |
| Driver Output Level, Max |  | No load | ±25V |
| Data Rate |  | 3kΩ ≤ RL ≤ 7kΩ, CL ≤ 2500pF | Up to 20kbps |
| Receiver Input Voltage | 0 Level |  | +3V to +15V |
|  | 1 Level |  | -3V to -15V |
| Receiver Input Level |  |  | ±25V |
| Instantaneous Slew Rate, Max |  | 3kΩ ≤ RL ≤ 7kΩ, CL ≤ 2500pF | 30V/µs |
| Driver Output Short-Circuit Current, Max |  |  | 100mA |
| Transition Rate on Driver Output |  | V.28 | 1ms or 3% of the period |
|  |  | EIA/TIA-232E | 4% of the period |
| Driver Output Resistance |  | -2V < VOUT < +2V | 300Ω |

Table 3. DB9 Cable Connections Commonly Used for EIA/TIAE-232E and V.24 Asynchronous Interfaces

| PIN | CONNECTION |  |
| --- | --- | --- |
| 1 | Received Line Signal Detector (sometimes called Carrier Detect, DCD) | Handshake from DCE |
| 2 | Receive Data (RD) | Data from DCE |
| 3 | Transmit Data (TD) | Data from DTE |
| 4 | Data Terminal Ready | Handshake from DTE |
| 5 | Signal Ground | Reference point for signals |
| 6 | Data Set Ready (DSR) | Handshake from DCE |
| 7 | Request to Send (RTS) | Handshake from DTE |
| 8 | Clear to Send (CTS) | Handshake from DCE |
| 9 | Ring Indicator | Handshake from DCE |

Pin  Configurations and Typical Operating Circuits (continued)

TOP VIEW

+5V INPUT

0.1µF*

0.1µF

1     C1+

3

16

VCC

+5V TO +10V

0.1µF*

6.3V

V+     2

+10V

C1+    1

16   VCC

6.3V

0.1µF*

C1-  VOLTAGE DOUBLER

4     C2+

+10V TO -10V

V-   6

-10V

0.1µF*

V+    2

C1-  3

C2+    4

C2-  5

MAX202E MAX232E

15   GND

14   T1OUT

13    R1IN

12    R1OUT

16V

TTL/CMOS INPUTS

5     C2-

11     T1IN

VOLTAGE INVERTER

T1	T1OUT   14

16V

RS-232

OUTPUTS

V- T2OUT R2IN

6	11

7	10

8	9

DIP/SO/TSSOP

T1IN T2IN R2OUT

TTL/CMOS OUTPUTS

10     T2IN

12     R1OUT

9     R2OUT

T2

R1

5kΩ

R2

5kΩ

T2OUT    7

R1IN   13

R2IN     8

RS-232

INPUTS

MAX202E–MAX213E, MAX232E/MAX241E

PIN NUMBERS  ON TYPICAL  OPERATING  CIRCUIT  REFER TO DIP/SO/TSSOP PACKAGE, NOT LCC.

* 1.0µF CAPACITORS, MAX232E ONLY.

GND

15

Pin  Configurations and Typical Operating Circuits (continued)

TOP VIEW

0.1µF

+5V INPUT

T2IN    1

T1IN    2

R1OUT   3

R1IN   4

20    R2OUT

19    R2IN

18    T2OUT

17   V-

TTL/CMOS INPUTS

2     T1IN

1     T2IN

+5V

+5V

400kΩ

400kΩ

7

VCC

T1

T2

T1OUT    5

T2OUT    18

RS-232

OUTPUTS

T1OUT   5

GND    6

VCC    7

C1+ (V+)    8

GND   9

C2- (V-) 10

MAX203E

DIP/SO

16   C2-

15   C2+

14    V+ (C1-)

13   C1- (C1+)

12   V- (C2+)

11   C2+ (C2-)

TTL/CMOS OUTPUTS

DO NOT MAKE CONNECTION TO THESE PINS

INTERNAL -10V POWER SUPPLY

INTERNAL +10V

3

20

8(13)

13(14)

12(10)

17

14(8)

R1OUT

R2OUT

C1+ C1- V-

V-

R1IN R1

5kΩ

R2IN R2

5kΩ

C2+ C2+ C2- C2-

4

19

11 (12)

15

16

10  (11)

RS-232

INPUTS

POWER SUPPLY	V+

GND	GND

MAX202E–MAX213E, MAX232E/MAX241E

6	9

PIN NUMBERS IN () ARE FOR SO PACKAGE.

Pin  Configurations and Typical Operating Circuits (continued)

TOP VIEW

0.1µF

+5V INPUT

+5V

12

VCC

400kΩ

8   T1IN	T1

+5V

T1OUT  3

T2IN

400kΩ

T2OUT

7

+5V

T2	4

400kΩ

T4OUT    1

T3OUT    2

T1OUT    3

T2OUT    4

R2IN     5

R2OUT    6

MAX205E

24    R3IN

23    R3OUT

22    T5IN

21    SHDN

20    EN

19    T5OUT

TTL/CMOS INPUTS

15    T3IN 	T3

+5V

400kΩ

16    T4IN 	T4

+5V

400kΩ

T5IN

T3OUT  2

T4OUT  1

T5OUT

RS-232

OUTPUTS

T2IN     7

T1IN     8

R1OUT    9

R1IN    10

GND    11

VCC     12

18    R4IN

17    R4OUT

16    T4IN

15    T3IN

14    R5OUT

13    R5IN

22	T5	19

9   R1OUT	R1	R1IN   10

5kΩ

6   R2OUT	R2	R2IN   5

DIP

TTL/CMOS OUTPUTS

5kΩ

23   R3OUT	R3	R3IN   24

5kΩ

RS-232

INPUTS

17    R4OUT

R4

R4IN   18

5kΩ

14   R5OUT	R5	R5IN   13

5kΩ

MAX202E–MAX213E, MAX232E/MAX241E

20    EN

GND

11

SHDN    21

Pin  Configurations and Typical Operating Circuits (continued)

TOP VIEW

0.1µF

+5V INPUT

0.1µF

6.3V

0.1µF

6.3V

0.1µF

10   C1+

12   C1-

13   C2+

14

9

VCC

+5V TO +10V VOLTAGE DOUBLER

+10V TO -10V

V+  11

15

V-

0.1µF

16V

16V

C2-

VOLTAGE INVERTER

+5V

T3OUT    1

24    T4OUT

T1IN

400kΩ

T1OUT

T1OUT    2

T2OUT    3

R1IN     4

R1OUT    5

T2IN     6

MAX206E

23    R2IN

22    R2OUT

21    SHDN

20    EN

19    T4IN

TTL/CMOS INPUTS

7

+5V

6    T2IN

+5V

T1	2

400kΩ

T2OUT T2

3

400kΩ

RS-232

OUTPUTS

T1IN     7

GND     8

VCC      9

18    T3IN

17    R3OUT

16    R3IN

18    T3IN 	T3

+5V

400kΩ

19    T4IN

T3OUT  1

T4OUT

C1+    10

V+    11

C1-   12

15    V-

14    C2-

13    C2+

T4	24

5   R1OUT	R1	R1IN   4

DIP/SO/SSOP

TTL/CMOS OUTPUTS

5kΩ

22   R2OUT	R2	R2IN   23

5kΩ

17   R3OUT	R3	R3IN   16

RS-232

INPUTS

5kΩ

MAX202E–MAX213E, MAX232E/MAX241E

20    EN

GND

8

SHDN    21

Pin  Configurations and Typical Operating Circuits (continued)

TOP VIEW

0.1µF

+5V INPUT

0.1µF

6.3V

0.1µF

6.3V

0.1µF

10   C1+

12   C1-

13   C2+

14

9

VCC

+5V TO +10V VOLTAGE DOUBLER

+10V TO -10V

V+  11

15

V-

0.1µF

16V

16V

C2-

VOLTAGE INVERTER

T3OUT    1

T1OUT    2

T2OUT    3

24    T4OUT

23    R2IN

22    R2OUT

+5V

7   T1IN

+5V

6   T2IN

+5V

400kΩ

T1

400kΩ

T2

400kΩ

T1OUT  2

T2OUT  3

R1IN     4

R1OUT    5

T2IN     6

MAX207E

21    T5IN

20    T5OUT

19    T4IN

TTL/CMOS INPUTS

18    T3IN 	T3

+5V

400kΩ

T4IN

T3OUT  1

T4OUT

RS-232

OUTPUTS

T1IN     7

GND     8

VCC      9

C1+    10

V+    11

C1-   12

18    T3IN

17    R3OUT

16    R3IN

15    V-

14    C2-

13    C2+

19

+5V

21   T5IN

5    R1OUT

T4	24

400kΩ

T5OUT T5

20

R1	R1IN   4

5kΩ

DIP/SO/SSOP

TTL/CMOS OUTPUTS

22   R2OUT	R2	R2IN   23

5kΩ

17   R3OUT	R3	R3IN   16

RS-232

INPUTS

MAX202E–MAX213E, MAX232E/MAX241E

5kΩ

GND

8

Pin  Configurations and Typical Operating Circuits (continued)

TOP VIEW

0.1µF

+5V INPUT

0.1µF

6.3V

0.1µF

6.3V

0.1µF

10   C1+

12   C1-

13   C2+

14

9

VCC

+5V TO +10V VOLTAGE DOUBLER

+10V TO -10V

V+  11

15

V-

0.1µF

16V

16V

C2-

VOLTAGE INVERTER

+5V

T2OUT    1

24    T3OUT

400kΩ

5    T1IN	T1  Ω

+5V

T1OUT  2

T1OUT    2

R2IN     3

R2OUT    4

T1IN     5

23    R3IN

22    R3OUT

21    T4IN

20    T4OUT

TTL/CMOS INPUTS

18    T2IN

+5V

T3IN

400kΩ

T2

400kΩ

T2OUT  1

T3OUT

RS-232

OUTPUTS

R1OUT    6

R1IN     7

MAX208E

19    T3IN

18    T2IN

19

+5V

T3	24

400kΩ

GND     8

VCC      9

17    R4OUT

16    R4IN

21    T4IN	T4

T4OUT  20

C1+ V+ C1-

10	15

11	14

12	13

DIP/SO/SSOP

V- C2- C2+

6   R1OUT	R1	R1IN   7

5kΩ

4   R2OUT	R2	R2IN   3

TTL/CMOS

OUTPUTS

5kΩ

22   R3OUT	R3	R3IN   23

RS-232

INPUTS

MAX202E–MAX213E, MAX232E/MAX241E

5kΩ

17   R4OUT	R4	R4IN   16

5kΩ

GND

8

Pin  Configurations and Typical Operating Circuits (continued)

TOP VIEW

0.1µF

0.1µF*

6.3V

12   C1+

14   C1-

15   C2+

+5V INPUT

11

VCC

+5V TO +10V VOLTAGE DOUBLER

V+  13

17

0.1µF*

6.3V

0.1µF*

16V

0.1µF*	16

+10V TO -10V 	V-

16V

C2-

+5V

VOLTAGE INVERTER

400kΩ

TTL/CMOS INPUTS

7    T1IN

+5V

6    T2IN

+5V

T1OUT T1

2

400kΩ

T2OUT T2

3

400kΩ

RS-232

OUTPUTS

20   T3IN	T3

+5V

T3OUT  1

21    T4IN

400kΩ

T4OUT

T4	28

8   R1OUT	R1	R1IN   9

5kΩ

5   R2OUT	R2	R2IN   4

| SO/SSOP T3OUT | 1 |  | 28 | T4OUT |
| --- | --- | --- | --- | --- |
| T1OUT | 2 |  | 27 | R3IN |
| T2OUT | 3 |  | 26 | R3OUT |
| R2IN | 4 |  | 25 | SHDN (SHDN) |
| R2OUT | 5 |  | 24 | EN (EN) |
| T2IN | 6 | 23 R4IN MAX211E MAX213E 22 R4OUT MAX241E 21 T4IN 20 T3IN 19 R5OUT 18 R5IN 17 V- 16 C2- 15 C2+ |  |  |
| T1IN | 7 |  |  |  |
| R1OUT | 8 |  |  |  |
| R1IN | 9 |  |  |  |
| GND | 10 |  |  |  |
| VCC | 11 |  |  |  |
| C1+ | 12 |  |  |  |
| V+ | 13 |  |  |  |
| C1- | 14 |  |  |  |

TTL/CMOS OUTPUTS

5kΩ

26   R3OUT	R3	R3IN   27

5kΩ

RS-232

INPUTS

22    R4OUT

R4

R4IN   23

5kΩ

19   R5OUT	R5	R5IN   18

( ) ARE FOR MAX213E ONLY

* 1.0µF CAPACITORS, MAX241E ONLY

24    EN (EN)

GND

5kΩ

SHDN (SHDN)    25

MAX202E–MAX213E, MAX232E/MAX241E

10

MAX202E–MAX213E, MAX232E/MAX241E

Ordering Information (continued) PART TEMP RANGE PIN-PACKAGE

MAX208ECNG 0°C to +70°C 24 Narrow Plastic DIP

MAX208ECWG 0°C to +70°C 24 SO

MAX208ECAG 0°C to +70°C 24 SSOP

MAX208EENG -40°C to +85°C 24 Narrow Plastic DIP

MAX208EEWG -40°C to +85°C 24 SO

MAX208EEAG -40°C to +85°C 24 SSOP

MAX211ECWI 0°C to +70°C 28 SO

MAX211ECAI 0°C to +70°C 28 SSOP

MAX211EEWI -40°C to +85°C 28 SO

MAX211EEAI -40°C to +85°C 28 SSOP

MAX213ECWI 0°C to +70°C 28 SO

MAX213ECAI 0°C to +70°C 28 SSOP

MAX213EEWI -40°C to +85°C 28 SO

MAX213EEAI -40°C to +85°C 28 SSOP

MAX232ECPE 0°C to +70°C 16 Plastic DIP

MAX232ECSE 0°C to +70°C 16 Narrow SO

MAX232ECWE 0°C to +70°C 16 Wide SO

MAX232EC/D 0°C to +70°C Dice*

MAX232EEPE -40°C to +85°C 16 Plastic DIP

MAX232EESE -40°C to +85°C 16 Narrow SO

MAX232EEWE -40°C to +85°C 16 Wide SO

MAX241ECWI 0°C to +70°C 28 SO

MAX241ECAI 0°C to +70°C 28 SSOP

MAX241EEWI -40°C to +85°C 28 SO

MAX241EEAI -40°C to +85°C 28 SSOP

PART TEMP RANGE PIN-PACKAGE

MAX202ECUE 0°C to +70°C 16 TSSOP

MAX202ECWE 0°C to +70°C 16 Wide SO

MAX202EC/D 0°C to +70°C Dice*

MAX202EEPE -40°C to +85°C 16 Plastic DIP

MAX202EESE -40°C to +85°C 16 Narrow SO

MAX202EEUE -40°C to +85°C 16 TSSOP

MAX202EEWE -40°C to +85°C 16 Wide SO

MAX203ECPP 0°C to +70°C 20 Plastic DIP

MAX203ECWP 0°C to +70°C 20 SO

MAX203EEPP -40°C to +85°C 20 Plastic DIP

MAX203EEWP -40°C to +85°C 20 SO

MAX205ECPG 0°C to +70°C 24 Wide Plastic DIP

MAX205EEPG -40°C to +85°C 24 Wide Plastic DIP

MAX206ECNG 0°C to +70°C 24 Narrow Plastic DIP

MAX206ECWG 0°C to +70°C 24 SO

MAX206ECAG 0°C to +70°C 24 SSOP

MAX206EENG -40°C to +85°C 24 Narrow Plastic DIP

MAX206EEWG -40°C to +85°C 24 SO

MAX206EEAG -40°C to +85°C 24 SSOP

MAX207ECNG 0°C to +70°C 24 Narrow Plastic DIP

MAX207ECWG 0°C to +70°C 24 SO

MAX207ECAG 0°C to +70°C 24 SSOP

MAX207EENG -40°C to +85°C 24 Narrow Plastic DIP

MAX207EEWG -40°C to +85°C 24 SO

MAX207EEAG -40°C to +85°C 24 SSOP

*Dice are specified at TA = +25°C.

Chip Topographies

MAX202E/MAX232E 	MAX211E/MAX213E/MAX241E

T1OUT

T4OUT

V+  C1+	V CC

GND

T2OUT

T3OUT

R3IN

C1- C2+

R3OUT

SHDN  (SHDN)

C2- V-

T1OUT R1IN

0.117"

(2.972mm)

R2IN R2OUT

T2IN

T1IN R1OUT

R1IN GND

EN (EN) R4IN

R4OUT T4IN

T3IN

0.174"

(4.420mm)

R1OUT

R5OUT

T1IN

R5IN

R2IN

T2IN

V CC

C1+

C1-

C2-

T2OUT	R2OUT

0.080"

(2.032mm)

V+     C2+	V-

0.188"

(4.775mm)

( ) ARE FOR MAX213E ONLY

TRANSISTOR COUNT: 123

SUBSTRATE CONNECTED TO GND

TRANSISTOR COUNT: 542

SUBSTRATE CONNECTED TO GND

MAX202E–MAX213E, MAX232E/MAX241E

Chip Information

MAX205E/MAX206E/MAX207E/MAX208E

TRANSISTOR COUNT: 328

SUBSTRATE CONNECTED TO GND

MAX202E–MAX213E, MAX232E/MAX241E

PDIPN.EPS

Package Information (The package drawing(s)  in this data sheet may not reflect the most current specifications. For the latest package outline information go to  www.maxim-ic.com/packages.)

MAX202E–MAX213E,

MAX232E/MAX241E

SSOP.EPS

| Package Information (continued) (The package drawing(s) in this data sheet may not reflect the most current specifications. For the latest package outline information go to www.maxim-ic.com/packages.) | INCHES |  | MILLIMETERS |  |
| --- | --- | --- | --- | --- |
| DIM | MIN | MAX | MIN | MAX |
| A | 0.068 | 0.078 | 1.73 | 1.99 |
| A1 | 0.002 | 0.008 | 0.05 | 0.21 |
| B | 0.010 | 0.015 | 0.25 | 0.38 |
| C | 0.004 | 0.008 | 0.09 | 0.20 |
| D | SEE VARIATIONS |  |  |  |
| E | 0.205 | 0.212 | 5.20 | 5.38 |
| e | 0.0256 BSC |  | 0.65 BSC |  |
| H | 0.301 | 0.311 | 7.65 | 7.90 |
| L | 0.025 | 0.037 | 0.63 | 0.95 |
|  | 0 | 8 | 0 | 8 |

|  | INCHES |  | MILLIMETERS |  |  |
| --- | --- | --- | --- | --- | --- |
|  | MIN | MAX | MIN | MAX | N |
| D | 0.239 | 0.249 | 6.07 | 6.33 | 14L |
| D | 0.239 | 0.249 | 6.07 | 6.33 | 16L |
| D | 0.278 | 0.289 | 7.07 | 7.33 | 20L |
| D | 0.317 | 0.328 | 8.07 | 8.33 | 24L |
| D | 0.397 | 0.407 | 10.07 | 10.33 | 28L |

| 2 1 E H N A C B e A1 L D NOTES: |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. D&E DO NOT INCLUDE MOLD FLASH. 2. MOLD FLASH OR PROTRUSIONS NOT TO EXCEED .15 MM (.006"). 3. CONTROLLING DIMENSION: MILLIMETERS. 4. MEETS JEDEC MO150. 5. LEADS TO BE COPLANAR WITHIN 0.10 MM. | PROPRIETARY INFORMATION |  |  |  |
|  | TITLE: PACKAGE OUTLINE, SSOP, 5.3 MM |  |  |  |
|  | APPROVAL | DOCUMENT CONTROL NO. 21-0056 | REV. C | 1 1 |

| Package Information (continued) (The package drawing(s) in this data sheet may not reflect the most current specifications. For the latest package outline information go to www.maxim-ic.com/packages.) | INCHES |  | MILLIMETERS |  |
| --- | --- | --- | --- | --- |
| DIM | MIN | MAX | MIN | MAX |
| A | 0.093 | 0.104 | 2.35 | 2.65 |
| A1 | 0.004 | 0.012 | 0.10 | 0.30 |
| B | 0.014 | 0.019 | 0.35 | 0.49 |
| C | 0.009 | 0.013 | 0.23 | 0.32 |
| e | 0.050 |  | 1.27 |  |
| E | 0.291 | 0.299 | 7.40 | 7.60 |
| H | 0.394 | 0.419 | 10.00 | 10.65 |
| L | 0.016 | 0.050 | 0.40 | 1.27 |

SOICW.EPS

N

E 	H

| VARIATIONS: | INCHES |  | MILLIMETERS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| DIM | MIN | MAX | MIN | MAX | N | MS013 |
| D | 0.398 | 0.413 | 10.10 | 10.50 | 16 | AA |
| D | 0.447 | 0.463 | 11.35 | 11.75 | 18 | AB |
| D | 0.496 | 0.512 | 12.60 | 13.00 | 20 | AC |
| D | 0.598 | 0.614 | 15.20 | 15.60 | 24 | AD |
| D | 0.697 | 0.713 | 17.70 | 18.10 | 28 | AE |

1

TOP VIEW

D

A 	C

e 	B 	A1

FRONT VIEW

L

SIDE VIEW

0 -8

PROPRIETARY INFORMATION TITLE:

PACKAGE OUTLINE, .300" SOIC

APPROVAL

DOCUMENT CONTROL NO.

REV.        1

MAX202E–MAX213E, MAX232E/MAX241E

TSSOP4.40mm.EPS

21-0042	B     1

PACKAGE OUTLINE, TSSOP 4.40mm BODY

1

21-0066	G      1

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses  are implied. Maxim reserves the right to change the circuitry and specifications  without notice at any time.

Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA  94086 408-737-7600  	25

©  2005 Maxim Integrated  Products 	Printed USA 	is a registered  trademark  of Maxim Integrated  Products, Inc.