19-0199; Rev 1; 8/95

Programmable DTE/DCE,

+5V  RS-232 Transceiver

General Description

The MAX214 +5V RS-232 transceiver provides a com- plete, 8-line, software-configurable, DTE or DCE port RS-232 interface.  Tx, Rx, RTS, CTS, DTR, DSR, DCD, and RI circuits  can be configured  as either Data Terminal Equipment (DTE) or Data Circuit-Terminating Equipment (DCE) using the DTE/DCE control pin.  The MAX214 eliminates the need to swap cables when switching between DTE and DCE configurations.

Applications

AT-Compatible Laptop Computers AT-Compatible Desktop Computers Modems, Printers, and Other Peripherals

Features

♦ Eliminates Null Modem Cables

♦ Programmable DTE or DCE Serial Port

♦ 1µF Charge-Pump Capacitors

♦ 116kbps Data Rate—Guaranteed

♦ 20µA Shutdown Mode

♦ Receivers Active in Shutdown

Ordering Information PART TEMP. RANGE PIN-PACKAGE

MAX214CPI 0°C to +70°C 28 Plastic DIP

MAX214CWI 0°C to +70°C 28 Wide SO

MAX214C/D 0°C to +70°C Dice*

MAX214EPI -40°C to +85°C 28 Plastic DIP

MAX214EWI -40°C to +85°C 28 Wide SO

Typical Operating Circuit

1  F

27

C1+

1  F

28   C1-

1

C2+

1  F

2

C2-

+5V

16

VCC	15

V+

MAX214

14

V-

* Dice are specified at TA = +25°C.

1  F

Pin  Configuration

TOP VIEW

1  F

5   TA

DTE

DCE

TRA  9

C2+     1

28   C1-

6    RA

DTE

RTA   10

C2-     2

HI-Z     3

27   C1+

26   SHDN

25   TB

23    RB

24     TC

DTE

DTE DTE

DCE

DCE

TRB  20

RTB  19

TRC   18

N.C.     4

TA      5

RA      6

RDC      7

25   TB

24   TC

MAX214

23   RB

22   RC

22    RC

DCE

DTE DCE

RTC   17

RE      8

TRA      9

21   DTE/DCE

20   TRB

7    RDC

DTE

DTE

RDTC   11

RTA   10

19   RTB

8     RE

21

DTE/DCE

26

SHDN

3

HI-Z

GND

13

RRE  12

RDTC   11

RRE   12

GND   13

V-   14

DIP/SO

18   TRC

17   RTC

16   VCC

15   V+

MAX214

Maxim Integrated Products    1

ABSOLUTE MAXIMUM RATINGS

VCC ...........................................................................-0.3V to +6V Input Voltages

TIN, DTE/DCE, SHDN HI-Z ......................-0.3V to (VCC + 0.3V) RIN ....................................................................................±15V

Output Voltages:

TOUT..................................................................................±15V ROUT ........................................................-0.3V to (VCC + 0.3V)

Short-Circuit (one output at a time)

TOUT to GND ...........................................................Continuous

ROUT to GND...........................................................Continuous

Continuous Power Dissipation (TA = +70°C)

Plastic DIP (derate 9.09mW/°C above +70°C) .............727mW

Wide SO (derate 12.50mW/°C above +70°C) ............1000mW Operating Temperature Ranges:

MAX214C–I ..............................................0°C to +70°C MAX214E–I ..........................................-40°C to +85°C Storage Temperature Range ......................-65°C to +150°C

Lead Temperature (soldering, 10sec) .......................+300°C

MAX214

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions beyond those indicated in the operational sections of the specifications is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS

(VCC = 4.5V to 5.5V, C1 to C4 = 1F, TA = TMIN to TMAX, unless otherwise noted.)

| PARAMETER | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| RS-232 TRANSMITTERS |  |  |  |  |
| Logic Input Threshold Low |  |  | 0.8 1.4 | V |
| Logic Input Threshold High |  |  | 1.4 2.0 | V |
| Logic Input Pull-Up Current | Normal operation |  | 1 10 50 | A |
|  | Shutdown |  | ±0.01 ±1 |  |
| Output Voltage Swing | All transmitter outputs loaded with 3kΩ to ground |  | ±5.0 ±7.5 | V |
| Transmitter Output Resistance | VCC = V+ = V- = 0V, VOUT = ±2V (Note 1) |  | 300 300k | Ω |
| Output Short-Circuit Current | VOUT = 0V |  | ±7 ±25 | mA |
| RS-232 RECEIVERS |  |  |  |  |
| Input Voltage Operating Range |  |  | ±15 | V |
| Positive Threshold Input Low | TA = +25°C, VCC = 5V, normal operation, SHDN = 0V |  | 0.8 1.3 | V |
| Positive Threshold Input High | TA = +25°C, VCC = 5V, normal operation, SHDN = 0V |  | 1.8 2.4 | V |
| Positive Threshold Input Hysteresis | VCC = 5V, normal operation, SHDN = 0V (no hysteresis in shutdown) |  | 0.2 0.5 1.0 | V |
| Negative Threshold Input Low | TA = +25°C, VCC = 5V | Normal operation, SHDN = 0V | -2.6 -1.9 | V |
|  |  | Shutdown, SHDN = 5V | 0.8 1.3 |  |
| Negative Threshold Input High | TA = +25°C, VCC = 5V | Normal operation, SHDN = 0V | -1.5 -0.2 | V |
|  |  | Shutdown, SHDN = 5V | 1.3 2.4 |  |
| Negative Threshold Input Hysteresis | VCC = 5V, normal operation, SHDN = 0V (no hysteresis in shutdown) |  | 0.2 0.4 1.0 | V |
| Input Resistance | HI-Z = 0V and SHDN = 0V |  | 3 5 7 | kΩ |
|  | HI-Z = 5V or SHDN = 5V |  | 100 300 |  |
| TTL/CMOS Output Voltage Low | IOUT = 3.2mA |  | 0.2 0.4 | V |
| TTL/CMOS Output Voltage High | IOUT = -1.0mA |  | 3.5 VCC - 0.2 | V |

MAX214

ELECTRICAL CHARACTERISTICS (continued)

(VCC = 4.5V to 5.5V, C1 to C4 = 1F, TA = TMIN to TMAX, unless otherwise noted.)

| CC , Figure 1 PARAMETER | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| POWER SUPPLY |  |  |  |  |
| VCC Supply Current | No load, TA = +25°C |  | 9 20 | mA |
| Shutdown Supply Current | DTE/DCE = 0V, SHDN = HI-Z = V | TA = +25°C | 4 20 | A |
|  |  | TA = TMIN to TMAX | 50 |  |
| CONTROL LOGIC (DTE/DCE, SHDN, H | I-Z) |  |  |  |
| Logic Input Threshold Low |  |  | 0.8 1.3 | V |
| Logic Input Threshold High |  |  | 1.3 2.0 | V |
| Input Leakage Current |  |  | ±1 | A |
| AC CHARACTERISTICS |  |  |  |  |
| Data Rate | Normal operation, transmitters and receivers |  | 200 116 | kbps |
|  | Receivers in shutdown mode |  | 20 |  |
| Transition-Region Slew Rate | TA = +25°C, VCC = 5V, RL = 3kΩ to 7kΩ, CL = to 2500pF, measured from 3V to -3V or -3V to 3V |  | 6 12 30 | V/s |
| Transmitter Propagation Delay, TTL to RS-232 (Normal Operation) | tPHLT |  | 1.3 3.5 | s |
|  | tPLHT |  | 1.4 3.5 |  |
| Transmitter + to - Propagation-Delay Difference (Normal Operation) | tPHLT - tPLHT |  | 100 | ns |
| Receiver Propagation Delay, RS-232 to TTL (Normal Operation) | tPHLR, tPLHR |  | 0.4 1.5 | s |
| Receiver Propagation Delay, RS-232 to TTL (Shutdown Mode) | tPHLR |  | 0.4 10 | s |
|  | tPLHR |  | 1.5 10 |  |
| Receiver Propagation-Delay Difference (Normal Operation) | tPHLT - tPLHT |  | 100 | ns |
| MODE-CHANGE TIMING (DTE/DCE) |  |  |  |  |
| Transmitter Enable Time | tTEN (includes charge-pump start-up time) |  | 250 | s |
| Transmitter Disable Time | tTTR |  | 600 | ns |
| Transmitter DTE/DCE Switch Time | tTSW |  | 600 | ns |
| Receiver DTE/DCE Switch Time | tRSW |  | 300 | ns |
| Receiver Termination-Resistor Connect/Disconnect Time | (SHDN = 0V) |  | 300 | ns |
| Receiver Termination-Resistor Connect Entering SHDN Time |  |  | 250 | s |
| Receiver Termination-Resistor Disconnect Exiting SHDN Time |  |  | 300 | ns |

Note 1:   The 300 minimum is the EIA/TIA-232E specification, but the actual resistance when in shutdown mode or when VCC = 0V

is typically 300k.

Typical Operating Characteristics

(VCC = 5V, C1 to C4 = 1F, all transmitters loaded with 3k in parallel with 2.5nF, TA = +25°C, unless otherwise noted.)

TRANSMITTER OUTPUT VOLTAGE (VOL)

vs. LOAD CAPACITANCE

MAX214-01

AT DIFFERENT DATA RATES

-5.5

-6.0

-6.5

VOL (V)

-7.0

-7.5

-8.0

-8.5

7.6

7.4

VOH (V)

7.2

7.0

6.8

6.6

6.4

6.2

6.0

TRANSMITTER  OUTPUT VOLTAGE (VOH)

vs. LOAD CAPACITANCE

AT DIFFERENT DATA RATES

20kbps

64kbps

120kbps

9.0

MAX214-02

8.5

VOH (V)

8.0

7.5

7.0

6.5

TRANSMITTER  OUTPUT VOLTAGE (VOH)

vs. VCC POSITIVE SUPPLY VOLTAGE

MAX214-03

1 TRANSMITTER LOADED

2 TRANSMITTERS LOADED

3 TRANSMITTERS LOADED

DATA RATE = 0kbps

| 0 1000 2000 3000 4000 5000 6000 7000 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 120kbps |  |  |
|  |  |  |  | 64kbps |  |  |
|  |  |  |  | 20kbps |  |  |

LOAD CAPACITANCE (pF)

TRANSMITTER  OUTPUT VOLTAGE (VOL)

vs. POSITIVE SUPPLY VOLTAGE

MAX214-04

3 TRANSMITTERS LOADED

0      1000   2000  3000   4000   5000  6000  7000

LOAD CAPACITANCE (pF)

MAX214-05

TRANSMITTER  OUTPUT VOLTAGES

vs. V+, V- LOAD CURRENT

8

4.5   4.6  4.7   4.8   4.9  5.0  5.1  5.2  5.3  5.4  5.5

VCC (V)

MAX214-06

SLEW RATE vs. LOAD CAPACITANCE

SLEW+

VOL (V)

-8.0

-8.5

-9.0

2 TRANSMITTERS LOADED	6

T OUT +,  T OUT- (V)

1 TRANSMITTER LOADED	4

2

0

-2

-4

-6

18

SLEW RATE (V/ s)

16

14

12

10

SLEW-

3 TRANSMITTERS LOADED

SLEW-

3 TRANSMITTERS LOADED

SLEW+

1 TRANSMITTER LOADED

MAX214

-9.5

DATA RATE = 0kbps

4.5   4.6  4.7   4.8   4.9  5.0  5.1  5.2  5.3  5.4  5.5

VCC (V)

-8

-10

| 0 |  |  |  |  |
| --- | --- | --- | --- | --- |
| V+ AND V- EQUALLY LOADED |  | V- LOADED, V+ LOADED, NO LOAD NO LOAD ON V+ ON V- |  |  |
| ALL TRANSMITTERS UNLOADED, 0kbps |  |  |  |  |

20 	40 	60 	80

LOAD CURRENT (mA)

6    1 TRANSMITTER LOADED

4

0	2000 	4000 	6000

LOAD CAPACITANCE (pF)

Pin  Description

| PIN | NAME | FUNCTION |
| --- | --- | --- |
| 1, 2 | C2+, C2- | Terminals for negative charge-pump capacitor |
| 3 | HI-Z | RS-232 receiver impedance control. Take high to disconnect the termination resistor. |
| 4 | N.C. | No connect—not internally connected |
| 5, 24, 25 | TA, TC, TB | TTL/CMOS driver A, C, B inputs |
| 6, 8, 22, 23 | RA, RE, RC, RB | TTL/CMOS receiver A, E, C, B outputs |
| 7 | RDC | TTL/CMOS DTE receiver output D for DTE/DCE = 0V, or TTL/CMOS DCE receiver output C for DTE/DCE = +5V |
| 9, 18, 20 | TRA, TRC, TRB | RS-232 DTE driver output for DTE/DCE= 0V, or RS-232 DCE receiver input for DTE/DCE = +5V |
| 10, 17, 19 | RTA, RTC, RTB | RS-232 DTE receiver input for DTE/DCE = 0V, or RS-232 DCE driver output for DTE/DCE = +5V |
| 11 | RDTC | RS-232 DTE receiver input D for DTE/DCE = 0V, or RS-232 DCE driver output C for DTE/DCE = +5V |
| 12 | RRE | RS-232 receiver input |
| 13 | GND | Ground |
| 14 | V- | -2VCC voltage generated by the charge pump |
| 15 | V+ | +2VCC voltage generated by the charge pump |
| 16 | VCC | +4.5V to +5.5V supply voltage |
| 21 | DTE/DCE | Data terminal equipment (DTE) and data circuit-terminating equipment (DCE) control pin. DCE active high and DTE active low. |
| 26 | SHDN | Shutdown control; shutdown high, normal operation low |
| 27, 28 | C1+, C1- | Terminals for positive charge-pump capacitor |

Detailed Description

The MAX214 RS-232 transceiver provides a complete,

8-line, software-configurable, DTE or DCE port RS-232 interface.   Tx, Rx, RTS, CTS, DTR, DSR, DCD, and RI circuits can be configured  as either Data Terminal

DTE/DCE Operation The DTE/DCE pin allows circuit configuration under software control.  Tables 1a and 1b show the pin defini- tions of the MAX214 in both DTE and DCE modes.  The Function columns show the direction of data flow from the input pin to the output pin of the MAX214, and onto

Equipment   (DTE)   or   D a  ta

Circuit-Terminating

the corresponding DB-25 connector’s pin.

MAX214

Equipment (DCE) using the DTE/DCE control pin.  The

MAX214 eliminates the need to swap cables when switching between DTE and DCE configurations.   This is useful when, for example, a portable computer is required to communicate with printers, modems, and other computers without carrying multiple cables.

The MAX214 runs from a single +5V supply and incor- porates a dual charge-pump voltage converter to gener- ate the necessary voltages for the RS-232 transmitters. A shutdown mode is provided  to save power when transmission is not required, but the receivers always stay active for simple detection of ring indicator signals.

+5V  to ±10V Dual Charge-Pump

Voltage Converter The +5V to ±10V conversion is performed  by two charge-pump  voltage converters (Figure 2).   The first uses capacitor  C1 to double the +5V to +10V, storing the +10V on the output filter capacitor, C3.  The second charge-pump  voltage converter uses C2 to invert the

+10V to -10V, storing the -10V on the V- output filter capacitor, C4.

In shutdown mode, V+ is pulled to VCC by an internal resistor, and V- falls to GND.

MAX214

MAX214

Table 1a.  DTE-Operation Pin Configurations

| TTL/CMOS I/O LABEL | MAX214 PIN | FUNCTION | MAX214 PIN | RS-232 I/O LABEL | DB-25 PIN | INPUT THRESHOLD |
| --- | --- | --- | --- | --- | --- | --- |
| Transmitter (TxD) | 5 |  | 9 | TxD | 2 |  |
| Receiver (RxD) | 6 |  | 10 | RxD | 3 | + |
| Request to Send (RTS) | 25 |  | 20 | RTS | 4 |  |
| Clear to Send (CTS) | 23 |  | 19 | CTS | 5 | - |
| Data Terminal Ready (DTR) | 24 |  | 18 | DTR | 20 |  |
| Data Set Ready (DSR) | 22 |  | 17 | DSR | 6 | - |
| Detector Carrier Data (DCD) | 7 |  | 11 | DCD | 8 | + |
| Ring Indicator (RI) | 8 |  | 12 | RI | 22 | + |

Table 1b.  DCE-Operation Pin Configurations

| MAX214 PIN | FUNCTION | MAX214 PIN | RS-232 I/O LABEL | DB-25 PIN | INPUT THRESHOLD |
| --- | --- | --- | --- | --- | --- |
| 5 |  | 10 | RxD | 3 |  |
| 6 |  | 9 | TxD | 2 | + |
| 25 |  | 19 | CTS | 5 |  |
| 23 |  | 20 | RTS | 4 | - |
| 24 |  | 17 | DSR | 6 |  |
|  |  | 11 | DCD | 8 |  |
| 22 |  | 18 | DTR | 20 | - |
| 7 |  |  |  |  |  |
| 8 |  | 12 | RI | 22 | + |

+5.5V

1µF

1µF

1µF

ISHDN

C1+ C1-

C2+   +5V

VCC 	V+

MAX214	V-

1µF

1µF

RS-232 Drivers

With VCC = 5V, the typical driver output voltage swing

is  ±8V  when  loaded  with  a  nominal  5k  RS-232

receiver.   Under worst-case operating  conditions (including  116kbps data rate, 3k  2500pF load, VCC = 4.5V, maximum rated temperature) the output swing is guaranteed to meet the ±5V minimum speci- fied by EIA/TIA-232 and V.28.  The open-circuit output voltage swing ranges from (V+ - 0.6V) to V-.

Input thresholds are both CMOS and TTL compatible. The inputs of unused drivers can be left unconnected

+5.5V

0V OR +5.5V DRIVE

+5.5V

C2-

TIN

ROUT

DTE/DCE HI-Z

400k

Tx

Rx

TOUT

RIN

5k

3k

+5.5V

because 400k pull-up resistors to VCC are included

on-chip.   Since all drivers invert, the pull-up resistors

force the outputs of unused drivers low. The input pull-up resistors typically source 10µA; in shutdown mode, they are disconnected to reduce supply current.

When in low-power shutdown mode, the driver outputs are turned off and their leakage current is less than

1µA, even if the transmitter output is back-driven  with voltages up to ±15V.

+5.5V

CAPACITORS MAY BE

SHDN

GND

RS-232 Receivers

The   receivers   convert   the   RS-232   signals   to

POLARIZED OR UNPOLARIZED

Figure 1. MAX214 Shutdown-Current Test Circuit

CMOS-logic levels.  They invert, to match the inversion of RS-232 drivers.   The guaranteed  receiver input thresholds are significantly  tighter than the ±3V thresholds required by the EIA/TIA-232E specification,

S1

VCC

C1+

V+

S2 	S5

C2+   S6

GND

GND

C1 	C3 	C2

IL +	RL +

S3 	S4 	S7 	S8

VCC

C4      IL -	RL -

V -

C1-

C2-

MAX214

120kHz

Figure 2. Charge-Pump Diagram

Table 2.  Control Pin Configurations

| CONTROL INPUTS |  |  | RS-232 PINS |  |  |
| --- | --- | --- | --- | --- | --- |
| SHUTDOWN | HI-Z | DTE/DCE | TRA, TRB, TRC | RTA, RTB, RTC, RDTC | RRE |
| 0 | 0 | 0 | Transmit Mode | Receive Mode/5kΩ | Receive Mode/5kΩ |
| 0 | 0 | 1 | Receive Mode/5kΩ | Transmit Mode | Receive Mode/5kΩ |
| 0 | 1 | 0 | Transmit Mode | Receive Mode/HI-Z | Receive Mode/HI-Z |
| 0 | 1 | 1 | Receive Mode/HI-Z | Transmit Mode | Receive Mode/HI-Z |
| 1 | 0 | 0 | Disabled/HI-Z | Slow Receive/HI-Z | Slow Receive/HI-Z |
| 1 | 0 | 1 | Slow Receive/HI-Z | Disabled/HI-Z | Slow Receive/HI-Z |
| 1 | 1 | 0 | Disabled/HI-Z | Slow Receive/HI-Z | Slow Receive/HI-Z |
| 1 | 1 | 1 | Slow Receive/HI-Z | Disabled/HI-Z | Slow Receive/HI-Z |

MAX214

which improves noise margins.   The polarity of each receiver’s input threshold is shown in Tables 1a and 1b. In normal operating mode, receiver inputs are internally connected  to ground with 5k resistors.   So uncon- nected receivers with positive input thresholds have high outputs, and those with negative input thresholds have low outputs.

When shut down, all receivers have positive thresholds. This  allows  the  receiver  inputs  to  respond  to TTL-/CMOS-logic levels, as well as RS-232 levels.  The guaranteed 0.8V input threshold ensures that receivers shorted to ground will have a logic 1 output.  Also, the

300k input resistance to ground ensures that a receiv- er with its input left open will also have a logic 1 output.

The receiver’s 0.5V of hysteresis provides clean output transitions, even with slow rise-time and fall-time sig- nals with moderate amounts of noise and ringing.  The receivers have no hysteresis in shutdown mode.

HI-Z Control The receiver inputs are terminated with 5k resistors, to comply  with the requirements of EIA/TIA-232E. However, these internal resistors can be disconnected by taking the HI-Z control pin to a logic high.   This makes all of the MAX214’s receiver inputs high imped- ance, and facilitates the transmission of RS-232 data from a single transmitter to multiple receivers.   In this case, all but one of the receiving ICs should be put into the high input-impedance state.

Shutdown Control In shutdown mode, the charge pumps are turned off, V+ is pulled down to VCC, V- is pulled to ground, and the transmitter outputs are disabled.  This reduces sup- ply current typically to 4µA.    The time required to exit shutdown is about 250µs, as shown in Figure 3.

Receivers Receiver outputs never go into a high-impedance state; they are always active, even in shutdown mode (see Table 2).  These awake-in-shutdown receivers are use- ful for monitoring external activity (for example, on RI), while maintaining minimal power consumption. Receivers in shutdown mode are slower (20kbps) than when not shut down (116kbps), and lack the hysteresis present in normal operation.

Drivers The driver outputs are high impedance  in shutdown mode, even when back-driven  with voltages up to ±15V.

Applications Information

Capacitor Selection The type of capacitor (C1 to C4) used is not critical for proper operation.   The MAX214 requires 1µF capaci- tors, although in all cases capacitors  of up to 10µF can be used without harm.   Ceramic dielectrics  are suggested for the 1µF capacitors.

When using the minimum recommended  capacitor values, make sure the capacitance  value does not degrade  excessively as the operating temperature varies.	If  in  doubt,   use  capacitors   with  a

larger nominal value (for example, 2 times larger). The effective series resistance (ESR) of the capaci- tors may vary over temperature and increase when below 0°C.  ESR influences the amount of rip- ple on V+ and V-, so if low ripple  is required over wide temperature ranges, use larger capacitors  or

low-ESR types.

To reduce the output impedance  at V+ and V-, use larger capacitors  (up to 10µF).  This can be useful when “stealing” power from V+ or from V-.

Driver Outputs when Exiting Shutdown Figure 3 shows the MAX214 driver outputs when exiting shutdown.  As they become active, the two driver out- puts are shown going to opposite RS-232 levels (one

pump capacitor  sizes up to 10µF reduces the imped- ance of the V+ and V- outputs.

High Data Rates The MAX214 maintains the RS-232 ±5.0V minimum driver output voltage even at high data rates.   The Typical Operating Characteristics show transmitter out- put voltage levels driving 3k in parallel with various capacitive loads at data rates up to 120kbps.

+10V A

+5V

0V

-5V

B

driver input is high, the other is low).   Each driver is loaded with 3k in parallel with 2.5nF.

Power-Supply Bypassing Decouple VCC to ground with a capacitor of the same value as the charge-pump capacitors.

V+ and  V- as  Power Supplies A small amount of power can be drawn from V+ and V-, although this will reduce  noise margins.   See the Output Voltage vs. Load Current graph in the Typical

+5V

0V

A = TRANSMITTER OUTPUT HIGH, +5V/div B = TRANSMITTER OUTPUT LOW, +5V/div C = SHDN INPUT, +5V/div

HORIZONTAL = 200  s

-10V

C

MAX214

Operating Characteristics.   Increasing the charge-

Figure 3.  Transmitter Outputs When Exiting Shutdown

PC1

(DTE MODE)

STANDARD RS-232

CABLE

PC2

(DCE MODE)

5     TA

6     RA

25     TB

23     RB

24     TC

22     RC

7     RDC

TRA    9	3

RTA    10 	2

TRB    20 	7

RTB    19 	8

TRC    18 	4

RTC    17 	6

RDTC    11 	1

TxD RxD RTS CTS DTR DSR

DCD

3	9

2	10

7	20

8	19

4	18

6	17

1	11

TRA RTA TRB RTB TRC RTC

RDTC

TA    5

RA    6

TB    25

RB    23

TC    24

RC    22

RDC    7

8     RE

MAX214

RRE    12 	9 	RI

9	12

RRE

MAX214

RE   8

HIGH 	HIGH

HI-Z 	SHDN     DTE/DCE

HI-Z 	SHDN     DTE/DCE

3	26 	21

DB-9 CONNECTORS

3	26

21 	VCC

MAX214

POWER CONNECTIONS  OMITTED  FOR CLARITY.

Figure 4.  Typical Application Circuit Showing 2 PCs with Both DTE and DCE Operation

Table 3.  Summary of EIA/TIA-232E, V.28

| Specifications PARAMETER | CONDITIONS | EIA/TIA-232E, V.28 SPECIFICATIONS |
| --- | --- | --- |
| Driver Output Voltage 0 Level 1 Level Output Level, Max | 3kΩ to 7kΩ load | +5.0V to +15V |
|  | 3kΩ to 7kΩ load | -5.0V to -15V |
|  | No load | ±25V |
| Data Rate | 3kΩ ≤ RL ≤ 7kΩ CL ≤ 2500pF | Up to 20kbits/sec |
| Receiver Input Voltage 0 Level 1 Level Input Level, Max |  | +3.0V to +15V |
|  |  | -3.0V to -15V |
|  |  | ±25V |
| Instantaneous Slew Rate, Max | 3kΩ ≤ RL ≤ 7kΩ CL ≤ 2500pF | 30V/s |
| Driver Output Short-Circuit Current, Max |  | 100mA |
| Transition Rate on Driver Output | V.28 | 1ms or 3% of the period |
|  | EIA/TIA-232E | 4% of the period |
| Driver Output Resistance | -2V < VOUT < +2V | 300Ω |

Chip Topography

C2-   C1-

HI-Z	C2+	C1+

SHDN TB

TA 	TC

RA 	RB

RDC

RE

RC	  0.178"

4.52mm

DTE/DCE

TRA RTA

TRB RTB

RRE	GND	V+

TRC

RDTC

V-    VCC 	RTC

MAX214

0.156"

3.96mm

TRANSISTOR COUNT: 6 94; SUBSTRATE CONNECTED TO V+.

Table 4.  DB9/DB25 Cable Connections Commonly Used for EIA/TIA-232 and V.24

Asynchronous Interfaces

| DB9 PIN | DB25 PIN | NAME | SYMBOL | FUNCTION |
| --- | --- | --- | --- | --- |
| 1 | 8 | Received Line Signal Detector, sometimes called Data Carrier Detect | DCD | Handshake from DCE |
| 2 | 3 | Receive Data | RxD | Data from DCE |
| 3 | 2 | Transmit Data | TxD | Data from DTE |
| 4 | 20 | Data Terminal Ready | DTR | Handshake from DTE |
| 5 | 7 | Signal Ground | GND | Reference point for signals |
| 6 | 6 | Data Set Ready | DSR | Handshake from DCE |
| 7 | 4 | Request to Send | RTS | Handshake from DTE |
| 8 | 5 | Clear to Send | CTS | Handshake from DCE |
| 9 | 22 | Ring Indicator | RI | Handshake from DCE |

MAX214

Package Information

| D1 E A2 A3 D E1 A | DIM | INCHES |  | MILLIMETERS |  |
| --- | --- | --- | --- | --- | --- |
|  |  | MIN | MAX | MIN | MAX |
|  | A | – | 0.200 | – | 5.08 |
|  | A1 | 0.015 | – | 0.38 | – |
|  | A2 | 0.125 | 0.175 | 3.18 | 4.45 |
|  | A3 | 0.055 | 0.080 | 1.40 | 2.03 |
|  | B | 0.016 | 0.020 | 0.41 | 0.51 |
|  | B1 | 0.045 | 0.065 | 1.14 | 1.65 |
|  | C | 0.008 | 0.012 | 0.20 | 0.30 |
|  | D | 1.430 | 1.470 | 36.32 | 37.34 |
|  | D1 | 0.050 | 0.090 | 1.27 | 2.29 |
|  | E | 0.600 | 0.625 | 15.24 | 15.88 |
|  | E1 | 0.525 | 0.575 | 13.34 | 14.61 |
|  | e | 0.100 | BSC | 2.54 | BSC |
|  | eA | 0.600 | BSC | 15.24 | BSC |
|  | eB | – | 0.700 | – | 17.78 |
|  | L | 0.125 | 0.150 | 3.18 | 3.81 |
|  |  | 0˚ | 15˚ | 0˚ | 15˚ |
| A1 e C 21-342A L e 28-PIN PLASTIC B1 A DUAL-IN-LINE B eB PACKAGE |  |  |  |  |  |

| E H | DIM | INCHES |  | MILLIMETERS |  |
| --- | --- | --- | --- | --- | --- |
|  |  | MIN | MAX | MIN | MAX |
|  | A | 0.093 | 0.104 | 2.35 | 2.65 |
|  | A1 | 0.004 | 0.012 | 0.10 | 0.30 |
|  | B | 0.014 | 0.019 | 0.35 | 0.49 |
|  | C | 0.009 | 0.013 | 0.23 | 0.32 |
|  | D | 0.697 | 0.713 | 17.70 | 18.10 |
|  | E | 0.291 | 0.299 | 7.40 | 7.60 |
|  | e | 0.050 | BSC | 1.27 | BSC |
|  | H | 0.394 | 0.419 | 10.00 | 10.65 |
|  | h | 0.010 | 0.030 | 0.25 | 0.75 |
|  | L | 0.016 | 0.050 | 0.40 | 1.27 |
|  |  | 0˚ | 8˚ | 0˚ | 8˚ |
| 21-343A D h x 45˚ A 28-PIN PLASTIC 0.127mm 0.004in. SMALL-OUTLINE e B A1 C L PACKAGE |  |  |  |  |  |

Maxim cannot assume responsibility for use of any circuitry other than circuitry entirely embodied in a Maxim product.  No circuit patent licenses are implied. Maxim reserves the right to change the circuitry and specifications without notice at any time.

12    	Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA   94086 (408) 737-7600

©  1995 Maxim Integrated Products 	Printed USA	is a registered trademark of Maxim Integrated Products.