19-0352; Rev 2; 7/02

Microprocessor Programmable

Universal Active Filters

General Description

The MAX260/MAX261/MAX262 CMOS dual second- order universal switched-capacitor  active filters allow microprocessor  control of precise filter functions. No external components are required for a variety of band- pass, lowpass, highpass, notch, and allpass configura- tions. Each device  contains two second-order  filter sections that place center frequency, Q, and filter oper- ating mode under programmed control.

An input clock, along with a 6-bit f0 program  input, determine the filter's center or corner frequency without affecting other filter parameters. The filter Q is also pro- grammed independently.  Separate clock  inputs for each filter section operate with either a crystal, RC net- work, or external clock generator.

The MAX260 has offset and DC specifications superior to the MAX261 and MAX262 and a center frequency (f0) range of 7.5kHz. The MAX261 handles center fre- quencies to 57kHz, while the MAX262 extends the cen- ter frequency range to 140kHz by employing lower clock-to-f0 ratios. All devices are available in 24-pin DIP and small outline packages  in commercial, extended, and military temperature ranges.

Applications

µP-Tuned Filters Anti-Aliasing Filters Digital Signal Processing Adaptive Filters

Signal Analysis

Phase-Locked Loops

Functional Diagram

Features

♦ Filter Design Software Available

♦ Microprocessor Interface

♦ 64-Step Center Frequency Control

♦ 128-Step Q Control

♦ Independent Q and f0 Programming

♦ Guaranteed Clock to f0 Ratio-1% (A grade)

♦ 75kHz f0 Range (MAX262)

♦ Single +5V and ±5V Operation

Ordering Information

PART TEMP RANGE PACKAGE ACCURACY

MAX260ACNG 0°C to +70°C Plastic DIP 1%

MAX260BCNG 0°C to +70°C Plastic DIP 2%

MAX260AENG -40°C to +85°C Plastic DIP 1%

MAX260BENG -40°C to +85°C Plastic DIP 2%

MAX260ACWG 0°C to +70°C Wide SO 1%

MAX260BCWG 0°C to +70°C Wide SO 2%

MAX260AMRG -55°C to +125°C CERDIP 1%

MAX260BMRG -55°C to +125°C CERDIP 2%

*All devices—24-pin packages 0.3in-wide packages

Ordering Information continued at end of data sheet.

Pin  Configurations

TOP VIEW

+5V 	V

GND

IN     LP    HP     BP A

IN     LP   HP    BP B

BPA    1

N.C.    2

HPA    3

N.C.    4

INA     5

D1    6

A3    7

MAX260

24   LPA

23   INB

22   LPB

21    BPB

20   D0

19    OSC OUT

18    GND

BPA    1

OP OUT    2

HPA    3

OP IN    4

INA     5

D1    6

A3    7

MAX261

MAX262

24   LPA

23   INB

22   LPB

21    BPB

20   HPB

19   D0

18    OSC OUT

CLK OUT   8

17   V-

CLK OUT   8

17    GND

V+     9

16    WR

V+     9

16   V-

-5V	V-

MAX261

MAX262

A2   10

CLKA   11

15   A0

14   HPB

A2   10

CLKA   11

15    WR

14   A0

CLKA 	OSC  CLKOUT	CLKB

CRYSTAL

PROGRAM INPUTS

CLKB   12

13   A1

CLKB   12

13   A1

MAX260/MAX261/MAX262

FOURTH-ORDER BANDPASS FILTER

ABSOLUTE MAXIMUM RATINGS

Total Supply Voltage (V+ to V-) .............................................15V Input Voltage, any pin  ..........................(V-  - 0.3V) to (V+ + 0.3V) Input Current, any pin ......................................................±50mA Power Dissipation

Plastic DIP (derate 8.33mW/°C above 70°C) ...............660mW CERDIP (derate 12.5mW/°C above 70°C)  .................1000mW Wide SO (derate 11.8mW/°C above 70°C)  ..................944mW

Operating Temperature Ranges MAX260/MAX261/MAX262XCXG  .......................0°C  to +70°C MAX260/MAX261/MAX262XEXG .....................-40°C to +85°C MAX260/MAX261/MAX262XMXG  ..................-55°C  to +125°C

Storage Temperature Range .............................-65°C to +160°C Lead Temperature (Soldering, 10s) ................................+300°C

MAX260/MAX261/MAX262

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS

(V+  = +5V, V- = -5V, CLKA  = CLKB  = ±5V  350kHz  for the MAX260 and  1.5MHz  for the MAX261/MAX262,  fCLK/f0  = 199.49  for

MAX260/MAX261 and 139.80 for MAX262, Filter Mode 1, TA = +25°C, unless otherwise noted.)

| PARAMETER | CONDITIONS |  |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| f0 Center Frequency Range |  |  |  | See Table 1 |  |
| Maximum Clock Frequency |  |  |  | See Table 1 |  |
| fCLK/f0 Ratio Error (Note 1) | TA = TMIN to TMAX |  | MAX260A | ±0.2 ±1.0 | % |
|  |  |  | MAX260B | ±0.2 ±2.0 |  |
|  |  |  | MAX261/MAX262A | ±0.2 ±1.0 |  |
|  |  |  | MAX261/MAX262B | ±0.2 ±2.0 |  |
| f0 Temperature Coefficient |  |  |  | -5 | ppm/°C |
| Q Accuracy (deviation from ideal continuous filter) (Note 2) | TA = TMIN to TMAX | Q = 8 | MAX260A | ±1 ±6 | % |
|  |  | Q = 8 | MAX260B | ±1 ±10 |  |
|  |  | Q = 32 | MAX260A | ±2 ±10 |  |
|  |  | Q = 32 | MAX260B | ±2 ±15 |  |
|  |  | Q = 64 | MAX260A | ±4 ±20 |  |
|  |  | Q = 64 | MAX260B | ±4 ±25 |  |
|  |  | Q = 8 | MAX261/MAX262A | ±1 ±6 |  |
|  |  | Q = 8 | MAX261/MAX262B | ±1 ±10 |  |
|  |  | Q = 32 | MAX261/MAX262A | ±2 ±10 |  |
|  |  | Q = 32 | MAX261/MAX262B | ±2 ±15 |  |
|  |  | Q = 64 | MAX261/MAX262A | ±4 ±20 |  |
|  |  | Q = 64 | MAX261/MAX262B | ±4 ±25 |  |
| Q Temperature Coefficient |  |  |  | ±20 | ppm/°C |
| DC Lowpass Gain Accuracy |  |  | MAX260 | ±0.1 ±0.3 | dB |
|  |  |  | MAX261/MAX262 | ±0.1 ±0.5 |  |
| Gain Temperature Coefficient | Lowpass (at D.C.) Bandpass (at f0) |  | MAX260 | -5 | ppm/°C |
|  |  |  | MAX261/MAX262 | -5 |  |
|  |  |  | MAX260/MAX261/MAX262 | +20 |  |

MAX260/MAX261/MAX262

ELECTRICAL CHARACTERISTICS  (continued)

(V+  = +5V, V- = -5V, CLKA  = CLKB  = ±5V  350kHz  for the MAX260 and  1.5MHz  for the MAX261/MAX262,  fCLK/f0  = 199.49  for

MAX260/MAX261 and 139.80 for MAX262, Filter Mode 1, TA = +25°C, unless otherwise noted.)

| PARAMETER | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Offset Voltage At Filter Outputs—LP, BP, HP (Note 3) | TA = TMIN to TMAX, Q = 4 Mode 1 | MAX260A | ±0.05 ±0.25 | V |
|  |  | MAX260B | ±0.15 ±0.45 |  |
|  |  | MAX261A | ±0.40 ±1.00 |  |
|  |  | MAX261B | ±0.80 ±1.60 |  |
|  |  | MAX262A | ±0.40 ±1.20 |  |
|  |  | MAX262B | ±0.80 ±1.60 |  |
|  | Mode 3 | MAX260A | ±0.075 ±0.30 |  |
|  |  | MAX260B | ±0.075 ±0.50 |  |
|  |  | MAX261A | ±0.50 ±1.10 |  |
|  |  | MAX261B | ±0.90 ±1.60 |  |
|  |  | MAX262A | ±0.50 ±1.30 |  |
|  |  | MAX262B | ±0.90 ±1.60 |  |
| Offset Voltage Temperature Coefficient | fCLK/f0 = 100.53, Q = 4 TA = TMIN to TMAX |  | ±0.75 | mV/°C |
| Clock Feedthrough |  |  | ±4 | mV |
| Crosstalk |  |  | -70 | dB |
| Wideband Noise | Q = 1, 2nd-Order, LP/BP |  | See Typ. Oper. Char. | µVRMS |
|  | 4th-Order LP (Figure 26) |  | 90 |  |
|  | 4th-Order BP (Figure 24) (Note 4) |  | 100 |  |
| Harmonic Distortion at f0 | Q = 4, VIN = 1.5VP-P |  | -67 | dB |
| Supply Voltage Range | TA = TMIN to TMAX |  | ±2.37 ±5 ±6.3 | V |
| Power Supply Current (Note 5) | TA = TMIN to TMAX CMOS Level Logic Inputs | MAX260 | 15 20 | mA |
|  |  | MAX261 | 16 20 |  |
|  |  | MAX262 | 16 20 |  |
| Shutdown Supply Current | Q0A - Q6A = all 0, CMOS Level Logic Inputs (Note 5) |  | 1.5 | mA |
| INTERNAL AMPLIFIERS |  |  |  |  |
| Output Signal Swing | TA = TMIN to TMAX, 10kΩ load (Note 6) |  | ±4.75 | V |
| Output Signal Circuit Current | Source |  | 50 | mA |
|  | Sink |  | 2 |  |
| Power Supply Rejection Ratio | 0Hz to 10kHz |  | -70 | dB |
| Gain Bandwidth Product |  |  | 2.5 | MHz |
| Slew Rate |  |  | 6 | V/µs |

MAX260/MAX261/MAX262

ELECTRICAL CHARACTERISTICS  (for V± = ±2.5V ±5%)

(V+ = +2.37V, V- = -2.37V, CLKA = CLKB = ±2.5V 250kHz for the MAX260 and 1MHz for the MAX261/MAX262, fCLK/f0 = 199.49 for

MAX260/MAX261 and 139.80 for MAX262, Filter Mode 1, TA = +25°C, unless otherwise noted.)

| PARAMETER | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| f0 Center Frequency Range |  |  | (Note 7) |  |
| Maximum Clock Frequency |  |  | (Note 7) |  |
| fCLK/f0 Ratio Error (Notes 1, 8) | Q = 8 | MAX26XA | ±0.1 1 | % |
|  |  | MAX26XB | ±0.1 2 |  |
| Q Accuracy (deviation from ideal continuous filter) (Notes 2, 8) | Q = 8 fCLK/f0 = 199.49 | MAX260A | ±2 ±6 | % |
|  |  | MAX260B | ±2 ±10 |  |
|  | fCLK/f0 = 199.49 | MAX261A | ±2 ±6 |  |
|  |  | MAX261B | ±2 ±10 |  |
|  | fCLK/f0 = 139.80 | MAX262A | ±2 ±6 |  |
|  |  | MAX262B | ±2 ±10 |  |
| Output Signal Swing | All Outputs (Note 6) |  | ±2 | V |
| Power Supply Current | CMOS Level Logic Inputs (Note 5) |  | 7 | mA |
| Shutdown Current | CMOS Level Logic Inputs (Note 5) |  | 0.35 | mA |

Note 1: fCLK/f0 accuracy  is tested at 199.49 on the MAX260/MAX261, and at 139.8 on the MAX262.

Note 2: Q accuracy  tested at Q = 8, 32, and 64. Q of 32 and 64 tested at 1/2 stated clock frequency.

Note 3: The offset voltage is specified  for the entire filter. Offset is virtually independent of Q and fCLK/f0 ratio setting. The test clock frequency for mode 3 is 175kHz for the MAX260 and 750kHz for the MAX261/MAX262.

Note 4: Output noise is measured with an RC output smoothing filter at 4 ✕ f0 to remove clock feedthrough.

Note 5: TTL logic levels are: HIGH = 2.4V, LOW = 0.8V. CMOS logic levels are: HIGH = 5V, LOW = 0V. Power supply current is typi- cally 4mA higher with TTL logic and clock input levels.

Note 6: On the MAX260 only, the HP output signal swing is typically 0.75V less than the LP or BP outputs.

Note 7: At ±2.5V supplies,  the f0 range and maximum clock frequency are typically 75% of values listed in Table 1.

Note 8: fCLK/f0 and Q accuracy  are a function of the accuracy  of internal capacitor  ratios. No increase in error is expected  at ±2.5V

as compared to ±5V; however, these parameters are only tested to the extent indicated  by the MIN or MAX limits.

INTERFACE SPECIFICATIONS (Note 9)

(V+ = +5V, V+ = -5V, TA = +25°C, unless otherwise noted.)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| WR Pulse Width | tWR |  | 250 150 | ns |
| Address Setup | tAS |  | 25 | ns |
| Address Hold | tAH |  | 0 | ns |
| Data Setup | tDS |  | 100 50 | ns |
| Data Hold | tDH |  | 10 0 | ns |
| Logic Input High | VIH | WR, D0, D1, A0–A3, CLKA, CLKB TA =TMIN to TMAX | 2.4 | V |
| Logic Input Low | VIL | WR, D0, D1, A0–A3, CLKA, CLKB TA =TMIN to TMAX | 0.8 | V |
| Input Leakage Current | IIN | WR, D0, D1, A0–A3, CLKB CLKA TA =TMIN to TMAX | 10 6 60 | µA |
| Input Capacitance | CIN | WR, D0, D1, A0–A3, CLKA, CLKB | 15 | pF |

Note 9: Interface timing specifications are guaranteed  by design and are not subject to test.

MAX260/MAX261/MAX262

| Pin Description PIN |  | NAME | FUNCTION |
| --- | --- | --- | --- |
| MAX260 | MAX261/ MAX262 |  |  |
| 5, 23 | 5, 23 | INA, INB | Filter inputs |
| 1, 21 | 1, 21 | BPA, BPB | Bandpass outputs |
| 24, 22 | 24, 22 | LPA, LPB | Lowpass outputs |
| 3, 14 | 3, 20 | HPA, HPB | Highpass/notch/allpass outputs |
| 16 | 15 | WR | Write enable input |
| 15, 13, 10, 7 | 14, 13, 10, 7 | A0, A1, A2, A3 | Address inputs for f0 and Q input data locations |
| 20, 6 | 19, 6 | D0, D1 | Data inputs for f0 and Q programming |
|  | 2 | OP OUT | Output of uncommitted op amp on MAX261/ MAX262 only. Pin 2 is a no- connect on the MAX260. |
|  | 4 | OP IN | Inverting input of uncom- mitted op amp on MAX261/ MAX262 only (noninverting input is internally connected to ground). Pin 4 is a no- connect on the MAX260. |

| PIN |  | NAME | FUNCTION |
| --- | --- | --- | --- |
| MAX260 | MAX261/ MAX262 |  |  |
| 9 | 9 | V+ | Positive supply voltage |
| 17 | 16 | V- | Negative supply voltage |
| 18 | 17 | GND | Analog Ground. Connect to the system ground for dual supply operation or mid-supply for single sup- ply operation. GND should be well bypassed in single supply applications. |
| 11 | 11 | CLKA | Input to the oscillator and clock input to section A. This clock is internally divided by 2. |
| 12 | 12 | CLKB | Clock input to filter B. This clock is internally divided by 2. |
| 8 | 8 | CLK OUT | Clock output for crystal and R-C oscillator operation |
| 19 | 18 | OSC OUT | Connects to crystal or R-C for self-clocked operation |

Typical Operating Characteristics

(TA = +25°C, unless otherwise noted.)

Q ERROR vs. CLOCK FREQUENCY

MAX260

IDD vs. POWER SUPPLY VOLTAGE

IDD vs. CLOCK FREQUENCY

MAX260/61/62 toc01

5V

25C

fCLK/f0 → N = 0

MODE 4

CLK FREQ = 500KHz

25C

20

CLOCKS (5V, 0V)

19

MAX260/61/62 toc02

18

5V

CLOCK (2.4V, 0.8V)

MAX260/61/62 toc03

CLOCK (5V, 0V)

CONTROL PINS (5V, 0V)

IDD (mA)

15	25C

-10

10 	CLOCKS (5V, -5V)	15

14

IDD (mA)

CLOCK (5V, -5V)

-20

0.2

0.4

0.6

0.8	1.0      1.2     1.4

5

5	6	7

8	9	10	11	12

13

0.5	1.5	2.5	3.5

CLOCK FREQUENCY (MHz)

Q ERROR vs. CLOCK FREQUENCY MAX261/MAX262

5V

20 	Q = 8

V+ TO V- (V)

FCLK/F0 ERROR vs. CLOCK FREQUENCY MAX261/MAX262

MAX260/61/62 toc05

MODES 2, 3

0

CLOCK FREQUENCY (MHz)

OUTPUT SIGNAL SWING

vs. CLOCK FREQUENCY

MAX260/61/62 toc06

MAX261/MAX262 ALL MODES

PEAK TO PEAK, OUTPUT SWING (V)

7

TA = 25C

Q ERROR (%)

16        fCLK   → N = 0 f0

12

MODE 3

MAX260/61/62 toc04

FCLK/F0  ERROR (%)

-0.2

-0.4

Q = 8

TA = 25C 	5

4

0

MODES 1, 4

-4

-1.0

-1.2

5V

Q = 8

2	fCLK/f0  → N = 0

1 	MAX260 MODES 1, 2, 3

0

0.5

1.0

1.5

2.0	2.5	3.0	3.5

1.0	1.5	2.0	2.5	3.0	3.5

0.2

0.6

1.0

1.4      1.8      2.2      2.8      3.0

CLOCK FREQUENCY (MHz)

CLOCK FREQUENCY (MHz)

CLOCK FREQUENCY (MHz)

MAX260/MAX261/MAX262

Q ERROR (%)

| Wideband RMS Noise (db ref. to 2.47VRMS, 7VP-P) ±5V Supplies | MODE | Q = 1 |  |  | Q = 8 |  |  | Q = 64 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LP | BP | HP/AP/N | LP | BP | HP/AP/N | LP | BP | HP/AP/N |
| MAX261/ MAX262 | 1 | -84 | -90 | -84 | -80 | -82 | -85 | -72 | -73 | -85 |
|  | 2 | -88 | -90 | -88 | -84 | -82 | -84 | -77 | -73 | -76 |
|  | 3 | -84 | -90 | -88 | -80 | -82 | -82 | -73 | -73 | -74 |
|  | 4 | -83 | -89 | -84 | -79 | -81 | -85 | -71 | -73 | -85 |
| MAX260 | 1 | -87 | -89 | -86 | -81 | -81 | -86 | -73 | -73 | -86 |
|  | 2 | -89 | -88 | -85 | -83 | -80 | -82 | -75 | -72 | -74 |
|  | 3 | -87 | -88 | -85 | -80 | -82 | -80 | -71 | -72 | -72 |
|  | 4 | -87 | -88 | -86 | -81 | -81 | -86 | -71 | -72 | -86 |

Note 1: fCLK = 1MHz for MAX261/MAX262, fCLK = 350kHz for MAX260

Note 2: fCLK/f0 ratio programmed at N = 63 (see Table 2)

Note 3: Clock feedthrough  is removed with an RC lowpass ar 4f0, ie., R = 3.9k,

C = 2000pF for MAX261.

Noise Spectral Distribution

(MAX261, fCLK = 1MHz, dB ref. to 2.47VRMS,

| 7VP-P) MEASUREMENT BANDWIDTH | Q = 1 | Q = 8 | Q = 64 |
| --- | --- | --- | --- |
| Wideband | -84 | -80 | -72 |
| 3kHz | -87 | -87 | -86 |
| C Message Weighted | -93 | -93 | -93 |

SCN = SWITCH-CAPACITOR NETWORK

IN 	SCN

SCN

-

SAMPLE-HOLD MAX260 ONLY

S1

-

+

S-H

N/HP/AP

BP

	 	 	LP

+

-

S1	S2	S3	S2

MODE SELECT

SCN

M0	M1

S3

SCN

Q0–Q6 (TABLE 3)

F0–F5 (TABLE 2)

Figure 1. Filter Block Diagram (One Second-Order  Section)

MAX260/MAX261/MAX262

Introduction

Each MAX260/MAX261/MAX262 contains two second- order switched-capacitor  active filters. Figure 1 shows the filter's state variable topology, employed with two cascaded  integrators and one summing amplifier. The MAX261 and MAX262 also contain an uncommitted amplifier. On-chip switches and capacitors  provide feedback  to-control each filter section's f0  and Q. Internal capacitor  ratios are primarily responsible for the accuracy  of these parameters. Although these switched-capacitor  networks (SCN) are in fact sampled systems, their behavior very closely matches that of continuous filters, such as RC active filters. The ratio of the clock  frequency  to the filter center frequency (fCLK/f0) is kept large so that ideal second-order state- variable response is maintained.

The MAX262 uses a lower range of sampling (fCLK/f0) ratios than the MAX260 or MAX261 to allow higher operating f0 frequencies and signal bandwidths. These reduced sample rates result in somewhat more devia- tion from ideal continuous filter parameters than with the MAX260/MAX261. However, these differences can be compensated  using Figure 20 (see Application Hints) or Maxim's filter design software.

The MAX260 employs auto-zero circuitry not included in the MAX261 or MAX262. This provides improved DC characteristics,  and improved low-frequency perform-

ance at the expense of high-end f0 and signal band- width. The N/HP/AP outputs of the MAX260 are internal-

ly sample-and-held  as a result of its auto-zero operation. Signal swing at this output is somewhat reduced  as a result (MAX260 only). See Table 1 for bandwidth comparisons of the three filters.

Maxim also provides design programs that aid in con- verting filter response specifications  into the f0 and Q program codes used by the MAX260 series devices. This software also precompensates f0 and Q when low sample rates are used.

It is important to note that, in all MAX260 series filters, the filter's internal sample rate is one half the input clock rate (CLKA or CLKB) due to an internal division by two. All clock-related  data, tables, and other dis- cussions in this data sheet refer to the frequency at the CLKA or CLKB input, i.e., twice the internal sample rate, unless specifically stated otherwise.

Quick Look Design Procedure

The MAX260, MAX261, and MAX262, with Maxim's filter design software, greatly simplify the design procedures for many active filters. Most designs can be realized using a three-step process described  in this section. If the design software is not used, or if the filter complexi- ty is beyond the scope  of this section, refer to the remainder of this data sheet for more detailed applica- tions and design information.

100 	AB$ = "FILTER A" : GOSUB 150 : REM GET DATA FOR SECTION A

110 	ADD = 0 : GOSUB 220 : REM WRITE DATA TO THE PRINTER PORT

120 	AB$ = "FILTER B" : GOSUB 150 : REM GET DATA FOR B

130 	ADD = 32 : GOSUB 220 : REM WRITE DATA TO PRINTER PORT

140 	GOTO 100

150 	PRINT "MODE (1 to 4, see Table 5) "; AB$; : INPUT M

160 	IF M<1 OR M>4 THEN GOTO 150

170 	PRINT "CLOCK RATIO (0 to 63, N of Table 2) "; AB$; : INPUT F

180 	IF F<0 OR F>63 THEN GOTO 170

190 	PRINT "Q (0 to 127, N of Table 3) "; AB$; : INPUT Q

200 	IF Q<0 OR Q>127 THEN GOTO 190 ELSE : PRINT

210 	RETURN

220 	LPRINT CHR$(ADD+M-1); : ADD = ADD+4

230 	FOR I = 1 TO 3

240 	X = (ADD + (F - 4*INT(F/4))) : LPRINT CHR$(X);

250 	F=INT(F/4) : ADD = ADD + 4

260 	NEXT I

270 	FOR I = 1 TO 4

280 	X=(ADD + (Q - 4*INT(Q/4))) : LPRINT CHR$(X);

290 	Q=I(Q/4) :: ADD = ADD + 4

300 	NEXT I

310 	RETURN

IN

OUT

5

24

3

1

23

22 (20)14

21

INA LPA HPA BPA INB LPB HPB BPB

V+

MAX260

MAX261* MAX262*

GND

WR D0

D1

A0

A1

A2

A3

CLKA

CLKB

V-

16(15)

20(19)

6

15(14)

13

10

7

11

12

1

25

2	24

3	23

4	22

5

21

6

20

7

19

18

11

12

DB-25 MALE PLUG (BACK VIEW)

9 	18(17) 	17(16) 	CLK IN

+5V

0.1F 	0.1F

-5V

(SEE FIGURE 4)

TTL

*PIN  NUMBERS IN ( ) ARE FOR MAX261/MAX262

Figure 2. Basic Program and Hardware Connections  to Parallel Printer Port for “Quick Look” Using a PC

MAX260/MAX261/MAX262

Step 1—Filter Design Start with the program “PZ” to determine what type of filter is needed.  This helps  determine  the type (Butterworth, Chebyshev, etc.) and the number of poles for the optimum choice. The program also plots the fre-

quency response and calculates the pole/zero (f0) and Q values for each second-order section. Each MAX260/MAX261/MAX262 contains two second-order sections, and devices  can be cascaded  for higher order filters.

MAX260/MAX261/MAX262

Step 2—Generate Programming

Coefficients Starting with the f0 and Q values obtained in Step 1, use the program “MPP” to generate the digital coefficients that program each second-order section's f0 and Q. The program displays values for “N” (“N = _ for f0” and “N =

_ for Q”). N is the decimal equivalent of the binary code that sets the filter section’s f0 or Q. These are the same “N”s that are listed in Tables 2 and 3.

An input clock frequency and filter mode must also be selected in this step; however, if a specific-clock rate is not selected, “GEN” picks one. With regard to mode selection, mode 1 is the most convenient choice for most bandpass  and lowpass filters. Exceptions are elliptic  bandpass  and lowpass filters, which require mode 3. Highpass filters also use mode 3, while allpass filters use mode 4. For further information regarding these filter modes, see the Filter Operating Modes sec- tion.

Step 3—Loading the Filter When the N values for the f0 and Q of each second- order filter section are determined, the filter can then be programmed  and operated. What follows is a con- venient method of programming  the filter and evalu- ating a design if a PC is available.

A short BASIC program loads data into the MAX260/ MAX261/MAX262 through the PC's parallel printer port. The program asks for the filter mode, as well as the N values for the f0 and Q of each section. These coeffi- cients are then loaded into the filter in the form of ASCII characters. This program can be used with or without Maxim's other filter design software. The program and the appropriate hardware connections for a Centronics- type printer port are shown in Figure 2.

Filter Design Software

Maxim provides software programs to help speed the transition from frequency response design require- ments to working hardware. A series of programs are available, including:

Program PZ. Given the requirements, such as center frequency, Q, passband ripple, and stopband attenua- tion, PZ calculates the pole frequencies, Q's, zeros, and the number of stages needed.

Program MPP. For programmed filters, MPP computes the input codes to use and describes  the expected performance of the design.

Program FR. When a design of one or more stages is completed,  FR checks the final cascaded  assembly. The output frequency response can be compared with that expected from PZ.

Program PR.BAS Allows a MAX260/MAX261/MAX262 to be programmed  through a personal computer. The mode, f0, and Q of each section are typed in, and the proper codes are sent to the filter through the comput- er’s parallel printer port. This program is also provided in Figure 2.

Other design programs are also included  for use with other Maxim filter products.

Other Filter Products

Maxim has developed a number of other filter products in addition to the MAX260, MAX261, and MAX262.

PIN-PROGRAMMABLE ACTIVE FILTERS—A dual sec- ond-order universal filter that needs no external compo- nents. A microprocessor interface is not required.

MAX263  0.4Hz to 30kHz f0 range

MAX264  1Hz to 75kHz f0 range

RESISTOR AND PIN-PROGRAMMABLE FILTERS—A dual second-order  universal filter where f0 adjustment beyond pin-programmable resolution employs external resistors.

MAX265  0.4Hz to 30kHz f 0  range.  Includes  two uncommitted op amps.

MAX266  1Hz to 75kHz f0  range. Includes  two un- committed op amps.

MF10	Industry Standard, Resistor Programmed Only

PIN-PROGRAMMABLE BANDPASS FILTERS—A dual second-order  bandpass  that needs no external components.  A microprocessor  interface  is not required.

MAX267  0.4Hz to 30kHz f0 range

MAX268  1Hz to 75kHz f0 range

PROGRAMMABLE ANTI-ALIAS FILTER—A program- mable dual second-order  continuous (not switched) lowpass filter. No clock noise is generated. Designed for use as an anti-alias filter in front of, or as a smooth- ing filter following, any sampled filter or system.

MAX270  1kHz to 25kHz Cutoff Frequency Range

5th-ORDER LOW PASS FILTER—Features zero offset and drift errors for designs requiring high DC accuracy.

MAX280, LT1062	0.1Hz to 20kHz Cutoff Frequency

Range

| Table 1. Typical Clock and Center Frequency Limits PART | Q | MODE | fCLK | f0 |
| --- | --- | --- | --- | --- |
| MAX261 | 8 | 3 | 40Hz–1.7MHz | 0.4Hz–17kHz |
|  | 8 | 4 | 40Hz–2.7MHz | 0.4Hz–27kHz |
|  | 64 | 1 | 40Hz–2.0MHz | 0.4Hz–20kHz |
|  | 90 | 2 | 40Hz–1.2MHz | 0.4Hz–18kHz |
|  | 64 | 3 | 40Hz–1.2MHz | 0.4Hz–12kHz |
|  | 64 | 4 | 40Hz–2.0MHz | 0.4Hz–20kHz |
| MAX262 | 1 | 1 | 40Hz–4.0MHz | 1.0Hz–100kHz |
|  | 1 | 2 | 40Hz–4.0MHz | 1.4Hz–140kHz |
|  | 1 | 3 | 40Hz–4.0MHz | 1.0Hz–100kHz |
|  | 1 | 4 | 40Hz–4.0MHz | 1.0Hz–100kHz |
|  | 8 | 1 | 40Hz–2.5MHz | 1.0Hz–60kHz |
|  | 6 | 2 | 40Hz–1.4MHz | 1.4Hz–50kHz |
|  | 8 | 3 | 40Hz–1.4MHz | 1.0Hz–35kHz |
|  | 8 | 4 | 40Hz–2.5MHz | 1.0Hz–60kHz |
|  | 64 | 1 | 40Hz–1.5MHz | 1.0Hz–37kHz |
|  | 90 | 2 | 40Hz–0.9MHz | 1.4Hz–32kHz |
|  | 64 | 3 | 40Hz–0.9MHz | 1.0Hz–22kHz |
|  | 64 | 4 | 40Hz–1.5MHz | 1.0Hz–37kHz |

| PART | Q | MODE | fCLK | f0 |
| --- | --- | --- | --- | --- |
| MAX260 | 1 | 1 | 1Hz–400kHz | 0.01Hz–4.0kHz |
|  | 1 | 2 | 1Hz–425kHz | 0.01Hz–6.0kHz |
|  | 1 | 3 | 1Hz–500kHz | 0.01Hz–5.0kHz |
|  | 1 | 4 | 1Hz–400kHz | 0.01Hz–4.0kHz |
|  | 8 | 1 | 1Hz–500kHz | 0.01Hz–5.0kHz |
|  | 8 | 2 | 1Hz–700kHz | 0.01Hz–10.0kH |
|  | 8 | 3 | 1Hz–700kHz | 0.01Hz–5.0kHz |
|  | 8 | 4 | 1Hz–600kHz | 0.01Hz–4.0kHz |
|  | 64 | 1 | 1Hz–750kHz | 0.01Hz–7.5kHz |
|  | 90 | 2 | 1Hz–500kHz | 0.01Hz–7.0kHz |
|  | 64 | 3 | 1Hz–400kHz | 0.01Hz–4.0kHz |
|  | 64 | 4 | 1Hz–750kHz | 0.01Hz–7.5kHz |
| MAX261 | 1 | 1 | 40Hz–4.0MHz | 0.4Hz–40kHz |
|  | 1 | 2 | 40Hz–4.0MHz | 0.5Hz–57kHz |
|  | 1 | 3 | 40Hz–4.0MHz | 0.4Hz–40kHz |
|  | 1 | 4 | 40Hz–4.0MHz | 0.4Hz–40kHz |
|  | 8 | 1 | 40Hz–2.7MHz | 0.4Hz–27kHz |
|  | 8 | 2 | 40Hz–2.1MHz | 0.5Hz–30kHz |

INA

N/HP/APA 	BPA

LPA

INB

N/HP/APB 	BPB

LPB

V+

 	 	 		V-

MODE	f0

Q	CK

MODE	f0

Q	CK

GND

2	6	7

2	6	7

A PROGRAM MEMORY 	2

MODE, f0, Q

15

B PROGRAM MEMORY MODE, f0, Q

15

2

MAX261/MAX262 ONLY

-

+

INTERFACE LOGIC

2	4

D0, D1

A0–A3     WR	CLKA

OSC OUT	CLK OUT

CLKB

OP IN

OP OUT

MAX260/MAX261/MAX262

Figure 3. MAX260/MAX261/MAX262 Block Diagram

MAX260/MAX261/MAX262

Table 2. fCLK/f0 Program Selection Table

| fCLK/f0 RATIO |  |  |  | PROGRAM CODE |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MAX260/MAX261 |  | MAX262 |  |  |  |  |  |  |  |  |
| MODES 1,3,4 | MODE 2 | MODES 1,3,4 | MODE 2 | N | F5 | F4 | F3 | F2 | F1 | F0 |
| 100.53 | 71.09 | 40.84 | 28.88 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 102.10 | 72.20 | 42.41 | 29.99 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |
| 103.67 | 73.31 | 43.98 | 31.10 | 2 | 0 | 0 | 0 | 0 | 1 | 0 |
| 105.24 | 74.42 | 45.55 | 32.21 | 3 | 0 | 0 | 0 | 0 | 1 | 1 |
| 106.81 | 75.53 | 47.12 | 33.32 | 4 | 0 | 0 | 0 | 1 | 0 | 0 |
| 108.38 | 76.64 | 48.69 | 34.43 | 5 | 0 | 0 | 0 | 1 | 0 | 1 |
| 109.96 | 77.75 | 50.27 | 35.54 | 6 | 0 | 0 | 0 | 1 | 1 | 0 |
| 111.53 | 78.86 | 51.84 | 36.65 | 7 | 0 | 0 | 0 | 1 | 1 | 1 |
| 113.10 | 79.97 | 53.41 | 37.76 | 8 | 0 | 0 | 1 | 0 | 0 | 0 |
| 114.67 | 81.08 | 54.98 | 38.87 | 9 | 0 | 0 | 1 | 0 | 0 | 1 |
| 116.24 | 82.19 | 56.55 | 39.99 | 10 | 0 | 0 | 1 | 0 | 1 | 0 |
| 117.81 | 83.30 | 58.12 | 41.10 | 11 | 0 | 0 | 1 | 0 | 1 | 1 |
| 119.38 | 84.42 | 59.69 | 42.21 | 12 | 0 | 0 | 1 | 1 | 0 | 0 |
| 120.95 | 85.53 | 61.26 | 43.32 | 13 | 0 | 0 | 1 | 1 | 0 | 1 |
| 122.52 | 86.64 | 62.83 | 44.43 | 14 | 0 | 0 | 1 | 1 | 1 | 0 |
| 124.09 | 87.75 | 64.40 | 45.54 | 15 | 0 | 0 | 1 | 1 | 1 | 1 |
| 125.66 | 88.86 | 65.97 | 46.65 | 16 | 0 | 1 | 0 | 0 | 0 | 0 |
| 127.23 | 89.97 | 67.54 | 47.76 | 17 | 0 | 1 | 0 | 0 | 0 | 1 |
| 128.81 | 91.80 | 69.12 | 48.87 | 18 | 0 | 1 | 0 | 0 | 1 | 0 |
| 130.38 | 92.19 | 70.69 | 49.98 | 19 | 0 | 1 | 0 | 0 | 1 | 1 |
| 131.95 | 93.30 | 72.26 | 51.10 | 20 | 0 | 1 | 0 | 1 | 0 | 0 |
| 133.52 | 94.41 | 73.83 | 52.20 | 21 | 0 | 1 | 0 | 1 | 0 | 1 |
| 135.08 | 95.52 | 75.40 | 53.31 | 22 | 0 | 1 | 0 | 1 | 1 | 0 |
| 136.66 | 96.63 | 76.97 | 54.43 | 23 | 0 | 1 | 0 | 1 | 1 | 1 |
| 138.23 | 97.74 | 78.53 | 55.54 | 24 | 0 | 1 | 1 | 0 | 0 | 0 |
| 139.80 | 98.86 | 80.11 | 56.65 | 25 | 0 | 1 | 1 | 0 | 0 | 1 |
| 141.37 | 99.97 | 81.68 | 57.76 | 26 | 0 | 1 | 1 | 0 | 1 | 0 |
| 142.94 | 101.08 | 83.25 | 58.87 | 27 | 0 | 1 | 1 | 0 | 1 | 1 |
| 14.4.51 | 102.89 | 84.82 | 59.98 | 28 | 0 | 1 | 1 | 1 | 0 | 0 |
| 146.08 | 103.30 | 86.39 | 61.09 | 29 | 0 | 1 | 1 | 1 | 0 | 1 |
| 147.65 | 104.41 | 87.96 | 62.20 | 30 | 0 | 1 | 1 | 1 | 1 | 0 |
| 149.23 | 105.52 | 89.54 | 63.31 | 31 | 0 | 1 | 1 | 1 | 1 | 1 |
| 150.80 | 106.63 | 91.11 | 64.42 | 32 | 1 | 0 | 0 | 0 | 0 | 0 |
| 152.37 | 107.74 | 92.68 | 65.53 | 33 | 1 | 0 | 0 | 0 | 0 | 1 |
| 153.98 | 108.85 | 94.25 | 66.64 | 34 | 1 | 0 | 0 | 0 | 1 | 0 |
| 155.51 | 109.96 | 95.82 | 67.75 | 35 | 1 | 0 | 0 | 0 | 1 | 1 |
| 157.08 | 111.07 | 97.39 | 68.86 | 36 | 1 | 0 | 0 | 1 | 0 | 0 |
| 158.65 | 112.18 | 98.96 | 69.98 | 37 | 1 | 0 | 0 | 1 | 0 | 1 |
| 160.22 | 113.29 | 100.53 | 71.09 | 38 | 1 | 0 | 0 | 1 | 1 | 0 |
| 161.79 | 114.41 | 102.10 | 72.20 | 39 | 1 | 0 | 0 | 1 | 1 | 1 |
| 163.36 | 115.52 | 102.67 | 73.31 | 40 | 1 | 0 | 1 | 0 | 0 | 0 |
| 164.93 | 116.63 | 105.24 | 74.42 | 41 | 1 | 0 | 1 | 0 | 0 | 1 |
| 166.50 | 117.74 | 106.81 | 75.53 | 42 | 1 | 0 | 1 | 0 | 1 | 0 |
| 168.08 | 118.85 | 108.38 | 76.64 | 43 | 1 | 0 | 1 | 0 | 1 | 1 |
| 169.65 | 119.96 | 109.96 | 77.75 | 44 | 1 | 0 | 1 | 1 | 0 | 0 |
| 171.22 | 121.07 | 111.53 | 78.86 | 45 | 1 | 0 | 1 | 1 | 0 | 1 |

| fCLK/f0 RATIO |  |  |  | PROGRAM CODE |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MAX260/MAX261 |  | MAX262 |  |  |  |  |  |  |  |  |
| MODES 1,3,4 | MODE 2 | MODES 1,3,4 | MODE 2 | N | F5 | F4 | F3 | F2 | F1 | F0 |
| 172.79 | 122.18 | 113.10 | 79.97 | 46 | 1 | 0 | 1 | 1 | 1 | 0 |
| 174.36 | 123.29 | 114.66 | 81.08 | 47 | 1 | 0 | 1 | 1 | 1 | 1 |
| 175.93 | 124.40 | 11624 | 82.19 | 48 | 1 | 1 | 0 | 0 | 0 | 0 |
| 177.50 | 125.51 | 117.81 | 83.30 | 49 | 1 | 1 | 0 | 0 | 0 | 1 |
| 179.07 | 126.62 | 119.38 | 84.41 | 50 | 1 | 1 | 0 | 0 | 1 | 0 |
| 180.64 | 127.73 | 120.95 | 85.53 | 51 | 1 | 1 | 0 | 0 | 1 | 1 |
| 182.21 | 128.84 | 122.52 | 86.64 | 52 | 1 | 1 | 0 | 1 | 0 | 0 |
| 183.78 | 129.96 | 124.09 | 87.75 | 53 | 1 | 1 | 0 | 1 | 0 | 1 |
| 185.35 | 131.07 | 125.66 | 88.86 | 54 | 1 | 1 | 0 | 1 | 1 | 0 |
| 186.92 | 132.18 | 127.23 | 89.97 | 55 | 1 | 1 | 0 | 1 | 1 | 1 |
| 188.49 | 133.29 | 128.81 | 91.08 | 56 | 1 | 1 | 1 | 0 | 0 | 0 |
| 190.07 | 134.40 | 130.38 | 92.19 | 57 | 1 | 1 | 1 | 0 | 0 | 1 |
| 191.64 | 135.51 | 131.95 | 93.30 | 58 | 1 | 1 | 1 | 0 | 1 | 0 |
| 193.21 | 136.62 | 133.52 | 94.41 | 59 | 1 | 1 | 1 | 0 | 1 | 1 |
| 194.78 | 137.73 | 135.09 | 95.52 | 60 | 1 | 1 | 1 | 1 | 0 | 0 |
| 196.35 | 138.84 | 136.66 | 96.63 | 61 | 1 | 1 | 1 | 1 | 0 | 1 |
| 197.92 | 139.95 | 138.23 | 97.74 | 62 | 1 | 1 | 1 | 1 | 1 | 0 |
| 199.49 | 141.06 | 139.80 | 98.85 | 63 | 1 | 1 | 1 | 1 | 1 | 1 |

Note 1: For the MAX260/MAX261, fCLK/f0 = (64 + N) / 2 in modes 1, 3, and 4, where N varies from 0 to 63.

Note 2: For the MAX262, fCLK/f0 = (26 s N) / 2 in modes 1, 3, and 4, where N varies 0 to 63.

Note 3: In mode 2, all fCLK/f0 ratios are divided  by 2. The functions are then: MAX260/MAX261 fCLK/f0 = 1.11072 (64 + N), MAX262 fCLK/f0 = 1.11072 (26 + N)

MAX260/MAX261/MAX262

Detailed Description

f0 and Q Programming Figure 3 shows a block diagram of the MAX260. Each second-order filter section has its own clock input and independent  f0 and Q control. The actual center fre- quency is a function of the filter's clock rate, 6-bit f0 control word (see Table 2), and operating mode. The Q of each section is also set by a separate programmed input (see Table 3). This way, each half of a MAX260/ MAX261/MAX262 is tuned independently so that com- plex filter polynomials can be realized. Equations that convert program code numbers to fCLK/f0 and Q values are listed in the notes beneath Tables 2 and 3.

Oscillator and Clock Inputs The clock  circuitry of the MAX260/MAX261/MAX262 can operate with a crystal, resistor-capacitor (RC) net- work, or an external clock generator as shown in Figure

4. If an RC oscillator is used, the clock rate, fCLK, nomi- nally equals 0.45/RC.

The duty cycle of the clock at CLKA and CLKB is unim- portant because the input is internally divided  by 2 to generate the sampling clock for each filter section. It is important to note that this internal division also halves the sample rate when considering  aliasing and other sampled system phenomenon.

Microprocessor Interface f0, Q, and mode-selection data are stored in internal program memory. The memory contents are updated by writing to addresses selected by A0–A3. D0, and D1 are the data inputs. A map of the memory locations is shown in Table 4. Data is stored in the selected address on the rising edge of WR. Address and data inputs are TTL and CMOS compatible when the filter is powered from ±5V. With other power supply voltages, CMOS logic levels should be used. Interface timing is shown in Figure 5. Note: Clock inputs CLKA and CLKB have no relation to the digital interface. They control the switched-capacitor filter sample rate only.

Some noise may be generated on the filter outputs by transitions at the logic inputs. If this is objectionable,

MAX260/MAX261/MAX262

| Table 3. Q Program Selection Table PROGRAMMED Q |  | PROGRAM CODE |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MODES 1,3,4 | MODE 2 | N | Q6 | Q5 | Q4 | Q3 | Q2 | Q1 | Q0 |
| 0.615 | 0.870 | 24 | 0 | 0 | 1 | 1 | 0 | 0 | 0 |
| 0.621 | 0.879 | 25 | 0 | 0 | 1 | 1 | 0 | 0 | 1 |
| 0.627 | 0.887 | 26 | 0 | 0 | 1 | 1 | 0 | 1 | 0 |
| 0.634 | 0.896 | 27 | 0 | 0 | 1 | 1 | 0 | 1 | 1 |
| 0.640 | 0.905 | 28 | 0 | 0 | 1 | 1 | 1 | 0 | 0 |
| 0.646 | 0.914 | 29 | 0 | 0 | 1 | 1 | 1 | 0 | 1 |
| 0.653 | 0.924 | 30 | 0 | 0 | 1 | 1 | 1 | 1 | 0 |
| 0.660 | 0.933 | 31 | 0 | 0 | 1 | 1 | 1 | 1 | 1 |
| 0.667 | 0.943 | 32 | 0 | 1 | 0 | 0 | 0 | 0 | 0 |
| 0.674 | 0.953 | 33 | 0 | 1 | 0 | 0 | 0 | 0 | 1 |
| 0.681 | 0.963 | 34 | 0 | 1 | 0 | 0 | 0 | 1 | 0 |
| 0.688 | 0.973 | 35 | 0 | 1 | 0 | 0 | 0 | 1 | 1 |
| 0.696 | 0.984 | 36 | 0 | 1 | 0 | 0 | 1 | 0 | 0 |
| 0.703 | 0.995 | 37 | 0 | 1 | 0 | 0 | 1 | 0 | 1 |
| 0.711 | 1.01 | 38 | 0 | 1 | 0 | 0 | 1 | 1 | 0 |
| 0.719 | 1.02 | 39 | 0 | 1 | 0 | 0 | 1 | 1 | 1 |
| 0.727 | 1.03 | 40 | 0 | 1 | 0 | 1 | 0 | 0 | 0 |
| 0.736 | 1.04 | 41 | 0 | 1 | 0 | 1 | 0 | 0 | 1 |
| 0.744 | 1.05 | 42 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 0.753 | 1.06 | 43 | 0 | 1 | 0 | 1 | 0 | 1 | 1 |
| 0.762 | 1.08 | 44 | 0 | 1 | 0 | 1 | 1 | 0 | 0 |
| 0.771 | 1.09 | 45 | 0 | 1 | 0 | 1 | 1 | 0 | 1 |
| 0.780 | 1.10 | 46 | 0 | 1 | 0 | 1 | 1 | 1 | 0 |
| 0.790 | 1.12 | 47 | 0 | 1 | 0 | 1 | 1 | 1 | 1 |

| PROGRAMMED Q |  | PROGRAM CODE |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MODES 1,3,4 | MODE 2 | N | Q6 | Q5 | Q4 | Q3 | Q2 | Q1 | Q0 |
| 0.500* | 0.707* | 0* | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0.504 | 0.713 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| 0.508 | 0.718 | 2 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| 0.512 | 0.724 | 3 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |
| 0.516 | 0.730 | 4 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |
| 0.520 | 0.736 | 5 | 0 | 0 | 0 | 0 | 1 | 0 | 1 |
| 0.525 | 0.742 | 6 | 0 | 0 | 0 | 0 | 1 | 1 | 0 |
| 0.529 | 0.748 | 7 | 0 | 0 | 0 | 0 | 1 | 1 | 1 |
| 0.533 | 0.754 | 8 | 0 | 0 | 0 | 1 | 0 | 0 | 0 |
| 0.538 | 0.761 | 9 | 0 | 0 | 0 | 1 | 0 | 0 | 1 |
| 0.542 | 0.767 | 10 | 0 | 0 | 0 | 1 | 0 | 1 | 0 |
| 0.547 | 0.774 | 11 | 0 | 0 | 0 | 1 | 0 | 1 | 1 |
| 0.552 | 0.780 | 12 | 0 | 0 | 0 | 1 | 1 | 0 | 0 |
| 0.556 | 0.787 | 13 | 0 | 0 | 0 | 1 | 1 | 0 | 1 |
| 0.561 | 0.794 | 14 | 0 | 0 | 0 | 1 | 1 | 1 | 0 |
| 0.566 | 0.801 | 15 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| 0.571 | 0.808 | 16 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
| 0.577 | 0.815 | 17 | 0 | 0 | 1 | 0 | 0 | 0 | 1 |
| 0.582 | 0.823 | 18 | 0 | 0 | 1 | 0 | 0 | 1 | 0 |
| 0.587 | 0.830 | 19 | 0 | 0 | 1 | 0 | 0 | 1 | 1 |
| 0.593 | 0.838 | 20 | 0 | 0 | 1 | 0 | 1 | 0 | 0 |
| 0.598 | 0.646 | 21 | 0 | 0 | 1 | 0 | 1 | 0 | 1 |
| 0.604 | 0.854 | 22 | 0 | 0 | 1 | 0 | 1 | 1 | 0 |
| 0.609 | 0.862 | 23 | 0 | 0 | 1 | 0 | 1 | 1 | 1 |

Note 4: * Writing all 0s into Q0A–Q6A on Filter A activates a low-power shutdown mode. BOTH filter sections are deactivated.

Therefore, this Q value is only achievable  in filter B.

MAX260/MAX261/MAX262

| Table 3. Q Program Selection Table (continued) PROGRAMMED Q |  | PROGRAM CODE |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MODES 1,3,4 | MODE 2 | N | Q6 | Q5 | Q4 | Q3 | Q2 | Q1 | Q0 |
| 1.60 | 2.26 | 88 | 1 | 0 | 1 | 1 | 0 | 0 | 0 |
| 1.64 | 2.32 | 89 | 1 | 0 | 1 | 1 | 0 | 0 | 1 |
| 1.68 | 2.40 | 90 | 1 | 0 | 1 | 1 | 0 | 1 | 0 |
| 1.73 | 2.45 | 91 | 1 | 0 | 1 | 1 | 0 | 1 | 1 |
| 1.78 | 2.51 | 92 | 1 | 0 | 1 | 1 | 1 | 0 | 0 |
| 1.83 | 2.59 | 93 | 1 | 0 | 1 | 1 | 1 | 0 | 1 |
| 1.88 | 2.66 | 94 | 1 | 0 | 1 | 1 | 1 | 1 | 0 |
| 1.94 | 2.74 | 95 | 1 | 0 | 1 | 1 | 1 | 1 | 1 |
| 2.00 | 2.83 | 96 | 1 | 1 | 0 | 0 | 0 | 0 | 0 |
| 2.06 | 2.92 | 97 | 1 | 1 | 0 | 0 | 0 | 0 | 1 |
| 2.13 | 3.02 | 98 | 1 | 1 | 0 | 0 | 0 | 1 | 0 |
| 2.21 | 3.12 | 99 | 1 | 1 | 0 | 0 | 0 | 1 | 1 |
| 2.29 | 3.23 | 100 | 1 | 1 | 0 | 0 | 1 | 0 | 0 |
| 2.37 | 3.35 | 101 | 1 | 1 | 0 | 0 | 1 | 0 | 1 |
| 2.46 | 3.48 | 102 | 1 | 1 | 0 | 0 | 1 | 1 | 0 |
| 2.56 | 3.62 | 103 | 1 | 1 | 0 | 0 | 1 | 1 | 1 |
| 2.67 | 3.77 | 104 | 1 | 1 | 0 | 1 | 0 | 0 | 0 |
| 2.78 | 3.96 | 105 | 1 | 1 | 0 | 1 | 0 | 0 | 1 |
| 2.91 | 4.11 | 106 | 1 | 1 | 0 | 1 | 0 | 1 | 0 |
| 3.05 | 4.31 | 107 | 1 | 1 | 0 | 1 | 0 | 1 | 1 |
| 3.20 | 4.53 | 108 | 1 | 1 | 0 | 1 | 1 | 0 | 0 |
| 3.37 | 4.76 | 109 | 1 | 1 | 0 | 1 | 1 | 0 | 1 |
| 3.56 | 5.03 | 110 | 1 | 1 | 0 | 1 | 1 | 1 | 0 |
| 3.76 | 5.32 | 111 | 1 | 1 | 0 | 1 | 1 | 1 | 1 |
| 4.00 | 5.66 | 112 | 1 | 1 | 1 | 0 | 0 | 0 | 0 |
| 4.27 | 6.03 | 113 | 1 | 1 | 1 | 0 | 0 | 0 | 1 |
| 4.57 | 6.46 | 114 | 1 | 1 | 1 | 0 | 0 | 1 | 0 |
| 4.92 | 6.96 | 115 | 1 | 1 | 1 | 0 | 0 | 1 | 1 |
| 5.33 | 7.54 | 116 | 1 | 1 | 1 | 0 | 1 | 0 | 0 |
| 5.82 | 8.23 | 117 | 1 | 1 | 1 | 0 | 1 | 0 | 1 |
| 6.40 | 9.05 | 118 | 1 | 1 | 1 | 0 | 1 | 1 | 0 |
| 7.11 | 10.1 | 119 | 1 | 1 | 1 | 0 | 1 | 1 | 1 |
| 8.00 | 11.3 | 120 | 1 | 1 | 1 | 1 | 0 | 0 | 0 |
| 9.14 | 12.9 | 121 | 1 | 1 | 1 | 1 | 0 | 0 | 1 |
| 10.7 | 15.1 | 122 | 1 | 1 | 1 | 1 | 0 | 1 | 0 |
| 12.8 | 18.1 | 123 | 1 | 1 | 1 | 1 | 0 | 1 | 1 |
| 16.0 | 22.6 | 124 | 1 | 1 | 1 | 1 | 1 | 0 | 0 |
| 21.3 | 30.2 | 125 | 1 | 1 | 1 | 1 | 1 | 0 | 1 |
| 32.0 | 45.3 | 126 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |
| 64.0 | 90.5 | 127 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

| PROGRAMMED Q |  | PROGRAM CODE |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MODES 1,3,4 | MODE 2 | N | Q6 | Q5 | Q4 | Q3 | Q2 | Q1 | Q0 |
| 0.800 | 1.13 | 48 | 0 | 1 | 1 | 0 | 0 | 0 | 0 |
| 0.810 | 1.15 | 49 | 0 | 1 | 1 | 0 | 0 | 0 | 1 |
| 0.821 | 1.16 | 50 | 0 | 1 | 1 | 0 | 0 | 1 | 0 |
| 0.831 | 1.18 | 51 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
| 0.842 | 1.19 | 52 | 0 | 1 | 1 | 0 | 1 | 0 | 0 |
| 0.853 | 1.21 | 53 | 0 | 1 | 1 | 0 | 1 | 0 | 1 |
| 0.865 | 1.22 | 54 | 0 | 1 | 1 | 0 | 1 | 1 | 0 |
| 0.877 | 1.24 | 55 | 0 | 1 | 1 | 0 | 1 | 1 | 1 |
| 0.889 | 1.26 | 56 | 0 | 1 | 1 | 1 | 0 | 0 | 0 |
| 0.901 | 1.27 | 57 | 0 | 1 | 1 | 1 | 0 | 0 | 1 |
| 0.914 | 1.29 | 58 | 0 | 1 | 1 | 1 | 0 | 1 | 0 |
| 0.928 | 1.31 | 59 | 0 | 1 | 1 | 1 | 0 | 1 | 1 |
| 0.941 | 1.33 | 60 | 0 | 1 | 1 | 1 | 1 | 0 | 0 |
| 0.955 | 1.35 | 61 | 0 | 1 | 1 | 1 | 1 | 0 | 1 |
| 0.969 | 1.37 | 62 | 0 | 1 | 1 | 1 | 1 | 1 | 0 |
| 0.985 | 1.39 | 63 | 0 | 1 | 1 | 1 | 1 | 1 | 1 |
| 1.00 | 1.41 | 64 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1.02 | 1.44 | 65 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |
| 1.03 | 1.46 | 66 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |
| 1.05 | 1.48 | 67 | 1 | 0 | 0 | 0 | 0 | 1 | 1 |
| 1.07 | 1.51 | 68 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |
| 1.08 | 1.53 | 69 | 1 | 0 | 0 | 0 | 1 | 0 | 1 |
| 1.10 | 1.56 | 70 | 1 | 0 | 0 | 0 | 1 | 1 | 0 |
| 1.12 | 1.59 | 71 | 1 | 0 | 0 | 0 | 1 | 1 | 1 |
| 1.14 | 1.62 | 72 | 1 | 0 | 0 | 1 | 0 | 0 | 0 |
| 1.16 | 1.65 | 73 | 1 | 0 | 0 | 1 | 0 | 0 | 1 |
| 1.19 | 1.68 | 74 | 1 | 0 | 0 | 1 | 0 | 1 | 0 |
| 1.21 | 1.71 | 75 | 1 | 0 | 0 | 1 | 0 | 1 | 1 |
| 1.23 | 1.74 | 76 | 1 | 0 | 0 | 1 | 1 | 0 | 0 |
| 1.25 | 1.77 | 77 | 1 | 0 | 0 | 1 | 1 | 0 | 1 |
| 1.28 | 1.81 | 78 | 1 | 0 | 0 | 1 | 1 | 1 | 0 |
| 1.31 | 1.85 | 79 | 1 | 0 | 0 | 1 | 1 | 1 | 1 |
| 1.33 | 1.89 | 80 | 1 | 0 | 1 | 0 | 0 | 0 | 0 |
| 1.36 | 1.93 | 81 | 1 | 0 | 1 | 0 | 0 | 0 | 1 |
| 1.39 | 1.97 | 82 | 1 | 0 | 1 | 0 | 0 | 1 | 0 |
| 1.42 | 2.01 | 83 | 1 | 0 | 1 | 0 | 0 | 1 | 1 |
| 1.45 | 2.06 | 84 | 1 | 0 | 1 | 0 | 1 | 0 | 0 |
| 1.49 | 2.10 | 85 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| 1.52 | 2.16 | 86 | 1 | 0 | 1 | 0 | 1 | 1 | 0 |
| 1.56 | 2.21 | 87 | 1 | 0 | 1 | 0 | 1 | 1 | 1 |

Notes    5) In modes 1, 3, and 4: Q = 64 / (128 - N)

6) In mode 2, the listed Q values are those of mode 1 multiplied by 2. Then Q = 90.51 / (128 - N)

FILTER A

CLKA

11

FILTER B

OSC	CLK

OUT	OUT     CLKB

19(18)*	8	12

Table 4. Program Address Locations

CRYSTAL

*OSC OUT IS PIN 18 ON MAX261/MAX262

FILTER A

FILTER B

CLKA

11

OSC OUT

19(18)* R	C

CLK OUT

8	12

CLKB

| fCLK = 0.45 B B DATA BIT |  | ADDRESS |  |  |  | LOCATION |
| --- | --- | --- | --- | --- | --- | --- |
| D0 | D1 | A3 | A2 | A1 | A0 |  |
| FILTER A |  |  |  |  |  |  |
| M0A | M1A | 0 | 0 | 0 | 0 | 0 |
| F0A | F1A | 0 | 0 | 0 | 1 | 1 |
| F2A | F3A | 0 | 0 | 1 | 0 | 2 |
| F4A | F5A | 0 | 0 | 1 | 1 | 3 |
| Q0A | Q1A | 0 | 1 | 0 | 0 | 4 |
| Q2A | Q3A | 0 | 1 | 0 | 1 | 5 |
| Q4A | Q5A | 0 | 1 | 1 | 0 | 6 |
| Q6A |  | 0 | 1 | 1 | 1 | 7 |
| FILTER B |  |  |  |  |  |  |
| M0B | M1B | 1 | 0 | 0 | 0 | 8 |
| F0B | F1B | 1 | 0 | 0 | 1 | 9 |
| F2B | F3B | 1 | 0 | 1 | 0 | 10 |
| F4 | F5 | 1 | 0 | 1 | 1 | 11 |
| Q0B | Q1B | 1 | 1 | 0 | 0 | 12 |
| Q2B | Q3B | 1 | 1 | 0 | 1 | 13 |
| Q4B | Q5B | 1 | 1 | 1 | 0 | 14 |
| Q6B |  | 1 | 1 | 1 | 1 | 15 |

RC

FILTER A

CLKA

OSC

FILTER B

CLK

Note: Writing 0 into Q0A–Q6A (address  locations 4–7) on filter

A activates shutdown mode. BOTH filter sections deactivate.

OUT

11

N.C.

OUT

12

N.C.

CLKB

EXTERNAL CLOCK IN (ANY DUTY CYCLE)

A0–A3

tAS

VALID ADDRESS

tAH

Figure 4. Clock Input Connections

tWR

WR

the digital lines should be buffered from the device by logic gates as shown in Figure 6.

Shutdown Mode The MAX260/MAX261/MAX262 enters a shutdown/ standby mode when all zeroes are written to the Q addresses of filter A (Q0A–Q6A). When shut down,

D0, D1

tDS 	tDH

VALID DATA

SEE INTERFACE SPECIFICATIONS  FOR TIMING LIMITS

MAX260/MAX261/MAX262

power consumption with ±5V supplies typically drops to 10mW. When reactivating the filter after shutdown, allow 2ms to return to full operation.

Filter Operating Modes

There are several ways in which the summing amplifier and integrators in each MAX260/MAX261/MAX262 filter section can be configured.  The four most versatile interconnections (modes) are selected  by writing to

Figure 5. Interface Timing

inputs M0 and M1 (see Tables 4 and 5). These modes use no external components. A fifth mode, 3A, makes use of an additional op amp (included  in the MAX261 and MAX262) and external resistors, but uses the same internal configuration and is selected with the same programming code, as mode 3.

+5V

-5V

MODE 1

OCTAL D FLIP-FLOP

74HC374

3

20

VCC

V+	V- 	IN

2

SCN	N 	BP

-

+

A0	1D

4

A1	2D

1Q	A0

2Q     5 	A1

SCN

- 	LP

	 	

+

-

7

A2	3D

A3	  8    4D D1	13     5D D2	14     6D

OC

6

3Q

9

4Q

12

5Q

15

6Q

GND	CK

A2     MAX260

MAX261

A3     MAX262

D1

D2

SCN

SCN = SWITCHED-CAPACITOR NETWORK

Figure 7. Filter Mode 1: Second-Order  Bandpass,  Lowpass, and Notch

WR

-5V

1	10	11

WR

GND

different modes if desired. The f0, fN (notch), Q, and

various output gains in each case are shown in Table 5.

Filter Mode Selection MODE 1 (Figure 7) is useful when implementing allpole lowpass and bandpass  filters such as Butterworth,

Figure 6. Buffering/Latching Logic Inputs

Figures 7 through 11 show symbolic representations of the MAX260 filter modes. Only one second-order  sec- tion is shown in each case. The A and B sections of one MAX260/MAX261/MAX262 can be programmed for

Chebyshev, Basset, etc. It can also be used for notch filters, but only second-order notches because the rela- tive pole and zero locations are fixed. Higher order notch filters require more latitude in f0 and 1N, which is why they are more easily implemented with mode 3A.

Table 5. Filter Modes for Second-Order Functions

| f0   MODE | M1, M0 | FILTER FUNCTIONS | f0 | Q | fN | HOLP | HOBP | HON1 (f ➝ 0) | HON2 (f ➝ fCLK/4) | OTHER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0, 0 | LP, BP, N | SEE TABLE 2 | SEE TABLE 3 | f0 | -1 | -Q | -1 | -1 |  |
| 2 | 0, 1 | LP, BP, N |  |  | f0√2 | -0.5 | -Q/√2 | -0.5 | -1 |  |
| 3 | 1, 0 | LP, BP, HP |  |  |  | -1 | -Q |  |  | HOHP = -1 |
|  |  |  |  |  | RH L | -1 | -Q | RG L | RG H | HOHP = -1 |
| 4 | 1, 1 | LP, BP, AP |  |  |  | -2 | -2Q |  |  | HOAP = -1 fZ = f0, QZ = Q |

R	R	R

MAX260/MAX261/MAX262

Notes: 	f0 = Center Frequency fN = Notch Frequency

HOLP = Lowpass Gain at DC

HOBP = Bandpass  Gain at f0

HOHP = Highpass Gain as f approaches fCLK/4

HON1 = Notch Gain as f approaches DC HON2 = Notch Gain as f approaches fCLK/4

HOAP = Allpass Gain

fz, Qz = f and Q of Complex Pole Pair

MODE 2

SCN	N 		BP IN		-

MODE 3A

RG

RH

- 	N

RL

+

SCN

+

- 	LP

	 	

+

-

IN

SCN	HP

BP

-

SCN

SCN

SCN

- 	LP

	 	

+

+

-

Figure 8. Filter Mode 2: Second-Order  Bandpass,  Lowpass, and Notch

SCN

SCN

MODE 3

SCN 	HP 		BP IN		-

SCN = SWITCHED-CAPACITOR  NETWORK

Figure 10. Filter Mode 3a: Second-Order  Bandpass,  Lowpass, Highpass,  and Notch. For elliptic LP, BP, HP, and Notch, the N

SCN

+

-

+

SCN

LP

	 	

-

output is used.

MODE 3 (Figure 9) is the only mode that produces high-pass filters. The maximum clock  frequency is somewhat less than with mode 1 (see Table 1).

MODE 3A (Figure 10) uses a separate op amp to sum

MAX260/MAX261/MAX262

SCN

SCN = SWITCHED-CAPACITOR NETWORK

Figure 9. Filter Mode 3: Second-Order  Bandpass,  Lowpass, and Highpass

Mode 1, along with mode 4, supports the highest clock frequencies (see Table 1) because the input summing amplifier is outside the filter’s resonant loop (Figure 7). The gain of the lowpass and notch outputs is 1, while the bandpass  gain at the center frequency is Q. For bandpass  gains other than Q, the filter input or output can be scaled by a resistive divider or op amp.

MODE 2 (Figure 8) is used for all-pole lowpass and bandpass filters. Key advantages compared to mode 1 are higher available Qs (see Table 3) and lower output noise. Mode 2’s available fCLK/f0 ratios are 2 less than with mode 1 (see Table 2), so a wider overall range of f0s can be selected  from a single clock  when both modes are used together. This is demonstrated in the Wide Passband Chebyshev Bandpass design example.

the highpass and lowpass outputs of mode 3, creating a separate notch output. This output allows the notch to be set independently of f0 by adjusting the op amp’s feedback  resistor ratio (RH, RL). RH, RL, and RG are external resistors. Because the notch can be indepen- dently set, mode 3A is also useful when designing pole-zero filters such as elliptics.

MODE 4 (Figure 11) is the only mode that provides an allpass output. This is useful when implementing group delay equalization. In addition to this, mode 4 can also be used in all pole lowpass and bandpass filters. Along with mode 1, it is the fastest operating mode for the fil- ter, although the gains are different than in mode 1. When the allpass function is used, note that some amplitude peaking occurs (approximately 0.3dB when Q = 8) at f0. Also note that f0 and Q sampling errors are highest in mode 4 (see Figure 20).

MODE 4

BANDPASS OUTPUT

SCN 	AP	BP

IN	-

HOBP

GAIN (V/V)

0.707 HOBP

SCN	-

+

+

LP

	 	

-

SCN

SCN = SWITCHED-CAPACITOR NETWORK

Q  	fO

fL 	fO	fH

f(LOG SCALE)

, f

Figure 11. Filter Mode 4: Second-Order  Bandpass,  Lowpass,

fH    fL    O 



fLfH

2 	

and Allpass

fL   

fO   1  	  1 	

	     1

Description of Filter Functions

 2Q





 2Q 	



2 	

BANDPASS (Figure 12)

fH  

fO    1

	  1 

  1

	

For all pole bandpass and lowpass filters (Butterworth, Bessel, Chebyshev) use mode 1 if possible. If appropri-

 2Q



 2Q 	



ate fCLK/f0  or Q values are not available in mode 1, mode 2 provides  a selection that is closer to the required  values. Mode 1, however, has the highest bandwidth (see Table 1). For pole-zero filters, such as elliptics, see mode 3A.

Figure 12. Second-Order  Bandpass  Characteristics

2

G(s)  

HOBP   2

s(o  / Q)

G(s)  

HOHP

s2 

s

s(o  / Q) 

o 2

s	  s(o  / Q) 

o 2

HOHP = Highpass output gain as f approaches fCLK/4

HOBP = Bandpass output gain at  = o

f0 = 0 / 2 = The center frequency of the complex pole pair. Input-output phase shift is -180° at f0.

Q = The quality factor of the complex pole pair.

Also the ratio of f0 to -3dB bandwidth of the second-order bandpass response.

LOWPASS See bandpass text. (Figure 13)

o 2

f0 = 0 / 2

NOTCH (Figure 15)

Mode 3A is recommended for multi-pole notch filters. In second-order  filters, mode 1 can also be used. The advantages of mode 1 are higher bandwidth,  com- pared to mode 3 (higher fN can be implemented), and no need for external components as required in mode

3A.

G(s)  

HOLP

s	s(	/ Q)

2 	  2

2  	o

  o 2

G(s)  

HON2

2

s		n

HOLP = Lowpass output gain at DC

s	  s(o  / Q) 

o 2

MAX260/MAX261/MAX262

f0 = 0 / 2

HIGHPASS (Figure 14)

Mode 3 is the only mode with a highpass output. It works for all pole filter types such as Butterworth, Bessel and Chebyshev. Use mode 3A for filters employing both poles and zeros, such as elliptics.

HON2 = Notch output gain as f approaches fCLK/4

HON1 = Notch output gain as f approaches DC

fn = n / 2

ALLPASS

Mode 4 is the only configuration in which an allpass function can be realized.

LOWPASS OUTPUT

HIGHPASS OUTPUT

HOP

GAIN (V/V)

HOLP

0.707 HOLP

HOP

GAIN (V/V)

HOHP

0.707 HOHP

fP	fC

f(LOG SCALE)

fC 	fP

f(LOG SCALE)

f      f   X



1 

1    	1 





2

1       1





  

f      f   X 



1  	

2 	

1  	



2

C 	O 	

2Q2 

	2Q2 

1 

C 	O 	  



  

2Q  

2

1 



     1

2Q  	



fp	f



O

1 	1

2Q2

fp 	f



O

1 	1

2Q2

H 	  H 	X	1 	1

OP	OLP	1 	1

H 	  H 	X

1 

Q	4Q2

OP	OHP

1   1 

Q

1

4Q2

Figure 13. Second-Order  Lowpass Characteristics

Figure 14. Second-Order  Highpass Characteristics

G(s)  

HOAP  s     s(o   / Q) 

2

o 2

s2  

s(o  / Q) 

o 2

HON2

HON1

HOAP = Allpass output gain for DC < f < fCLK / 4

f0 = 0 / 2

Filter Design Procedure

GAIN (V/V)

The procedure for most filter designs is to first convert the required frequency response specifications  to f0s and Qs for the appropriate  number of second-order sections that implement the filter. This can be done by

HON

fN

f(LOG SCALE)

MAX260/MAX261/MAX262

using design equations or tables in available liter- ature, or can be conveniently calculated using Maxim's filter design software. Once the f0s and Qs have been found, the next step is to turn them into the digital pro- gram coefficients required by the MAX260/MAX261/ MAX262. An operating mode and clock frequency (or clock/center frequency ratio) must also be selected.

Next, if the sample rate (fCLK/2) is low enough to cause significant errors, the selected f0s and Qs should be corrected  to account for sampling effects by using Figure 20 or Maxim's design software. In most cases, the sampling errors are small enough to require no cor- rection, i.e., less than 1%. In any case, with or without correction, the required f0s and Qs can then be select- ed from Tables 2 and 3. Maxim's filter design software

Figure 15. Second-Order  Notch Characteristics

Table 6. Cascading Identical Bandpass

Filter Sections

| TOTAL SECTIONS | TOTAL B.W. | TOTAL Q |
| --- | --- | --- |
| 1 | 1.000 B | 1.00 Q |
| 2 | 0.644 B | 1.55 Q |
| 3 | 0.510 B | 1.96 Q |
| 4 | 0.435 B | 2.30 Q |
| 5 | 0.386 B | 2.60 Q |

Note: B = individual stage bandwidth, Q = individual stage Q.

can also perform this last step. The desired f0s and Qs are stated, and the appropriate  digital coefficients are supplied.

Cascading Filters In some designs, such as very narrow band filters, sev- eral second-order  sections with identical center fre- quency can be cascaded.  The total Q of the resultant filter is:

Q

Application Hints

Power Supplies The MAX260/MAX261/MAX262 can be operated with a variety of power supply configurations, including +5V to

+12V single supply  or ±2.5V to ±5V dual supplies. When a single supply is used, V- is connected to sys- tem ground and the filter's GND pin should be biased at V+/2. The input signal is then either capacitively cou- pled  to the filter input or biased to V+/2. Figure 16

Total QT   

21/ N

  1

shows circuit connections for single-supply operation.

When power supplies other than ±5V are used, CMOS

input logic levels (HIGH = V+, LOW = GND or V-) are

Q is the Q of each individual filter section, and N is the

number of sections. In Table 6, the total Q and band- width are listed for up to five identical second-order sections. B is the bandwidth of each section.

In high-order bandpass filters, stages with different f0s and Qs are also often cascaded.  When this happens, the overall filter gain at the bandpass center frequency

is not simply the product  of the individual  gains because f0, the frequency where each sections gain is specified,  is different for each second-order  section. The gain of each section at the cascaded filter's center frequency must be determined to obtain the total gain.

For all-pole filters the gain, H(f0), as each second-order section's f0 is divided  by an adjustment factor, G, to obtain that section's gain, H(f0BP), at the overall center frequency:

H1(f0BP) = H(f01) / G1 = Section 1’s Gain at f0BP

required  for WR, D0, D1, A0–A3, OLKA, and CLKB. With ±5V supplies, either TTL or CMOS levels can be used. Note, however, that power consumption at ±5V is reduced if CLKA and CLKB are driven with ±5V, rather than TTL or 0 to 5V levels. Operation with +5V or ±2.5V power lowers power consumption,  but also reduces bandwidth by approximately 25% compared to +12V or

±5V supplies.

Best performance is achieved if V+ and V- are bypassed to ground with 4.7µF electrolytic (Tantalum is preferred.) and 0.1µF ceramic capacitors. These should be located as close to the supply pins as possible. The lead length of the bypass capacitors should be shortest at the V+ and V- pins. When using a single supply, V+ and GND should be bypassed to V- as shown in Figure 16.

Output Swing and Clipping

MAX260/MAX261/MAX262 outputs are designed  to

    2 	2

2 1/ 2

drive 10k loads. For the MAX261 and MAX262, all fil-

G1  

Q  (F

    

1 	1

  1)

 (F1 / Q1)

F1

ter outputs swing to within 0.15V of each supply rail with a 10k load. In the MAX260 only, an internal sam- ple-hold circuit reduces voltage swing at the N/HP/AP

where F1 = f01 / fOBP

G1, Q1, and f01 are the gain adjustment factor, Q, and f0 for the first of the cascaded  second-order  sections. The gain of the other sections (2, 3, etc.) at f0BP is determined the same way. The overall gain is:

H(f0Bp) = H1(f0BP) x H2(f0BP) x etc.

For cascaded filters with zeros (fZ) such as elliptics, the gain adjustment factor for each stage is:

output compared  to LP and BR. N/HP/AP, therefore, swings to within 1V (10k load) of either rail on the MAX260.

To ensure that the outputs are not driven beyond their maximum range (output clipping),  the peak amplitude response, individual  section gains (HOBP, HOLP, HOHP), input signal level, and filter offset voltages must be carefully considered.  It is especially important to check unused outputs for clipping  (i.e., the lowpass output in a bandpass  hookup), because overload at

	2 	2     2 	2

2 1/ 2

Q1 FZ1

  F1

(F1

  1)

 (F1 / Q1)

any filter stage severely distorts the overall response.

G1  

    	 	

The maximum signal swing with ±4.75V supplies and a

2 	2 	

MAX260/MAX261/MAX262

F1  FZ1	1





where F1Z = fz1 / f0BP, and F1 is the same as above.

1.0V filter offset is approximately ±3.5V.

For example, lets assume a fourth-order lowpass filter is being implemented with a Q of 2 using mode 1. With a single 5V supply (i.e., ±2.5V with respect to chip GND) the maximum output signal is ±2V (w.r.t. GND). Since in

10k

10k

VIN 	0V

SEE

5V	NOTE

2.5k

0V	7.5k	

CMOS LOGIC LEVELS

WR A0–A3

D0, D1

INA OR INB

TO V+

VIN

ANY DC

MAX260

MAX261

MAX262	V+

TO GND PIN

+5V

VIN

5V

0V

4.7k

GND

4.7k

V-

4.7F

0.1F

4.7F

0.1F

NOTE: OP-AMP  LEVEL SHIFT CIRCUIT HAS A GAIN OF 0.5 FROM V*.

Figure 16. Power Supply and Input Connections  for Single Supply Operation

MAX260/MAX261/MAX262

mode 1 the maximum signal is 0 times the input signal, the input should not exceed ±(2/Q)V, or ±1V in this case.

Clock Feedthrough and Noise

Typical wideband  noise for MAX260 series devices is

0.5mVP-P from DC to 100kHz. The noise is virtually independent  of clock frequency. In multistage filters, the section with the highest Q should be placed first for lower output noise.

The output waveform of the MAX260 series and other switched capacitor filters appears as a sampled signal with stepping  or “staircasing”  of the output waveform occurring at the internal sample rate (fCLK/2). This step- ping, if objectionable, can be removed by adding a sin- gle-pole AC filter. With no input signal, clock-related feedthrough is approximately 8mVP-P. This can also be attenuated with an RC-smoothing filter as shown with the MAX261 in Figure 17.

Some noise also can be generated at the filter outputs by transitions at the logic inputs. If this is objectionable, the digital lines should be buffered from the device by logic gates as shown in Figure 6.

Input Impedance The input to each filter is the switched capacitor circuit shown in Figure 18. In the MAX260, the input capacitor charges to the input voltage VIN during the first half clock cycle. During the second half-cycle, its charge is transferred to the feedback  capacitor.  The resultant input impedance can be approximated by:

RIN = 1 / (CINfCLK / 2) = 2 / (CINfCLK).

CIN is around 12pF, hence, for a clock frequency of

500kHz, RIN = 333k. The input also has about 5pF of

fixed capacitance to ground.

The MAX261/MAX262 input structure is shown in Figure

19. Here CA = 12pF and CB = 0.016pF and only CB is switched, so the input resistance is 750 times larger compared  to  the  MAX260 (R IN  =  250M  ).  The MAX261/MAX262 have a fixed capacitance  of approxi- mately 5pF to ground.

f0 and Q at Low Sample Rates When low fCLK/f0  ratios and low Q settings are select- ed, deviation from ideal continuous filter response can be noticeable in some designs. This is due to interac- tion between Q and f0 at low fCLK/f0 ratios and Qs. The data in Figure 20 quantifies these differences. Since the

errors are predictable,  the graphs can be used to cor- rect the selected f0 and Q so that the actual realized parameters are on target. These predicted  errors are not unique to MAX260 series devices  and, in fact, occur with all types of sampled filters. Consequently, these corrections can be applied  to other switched capacitor filters. In the majority of cases, the errors are not significant, i.e., less than 1%, and correction is not needed. However, the MAX262 does employ a lower range of fCLK/f0 ratios than the MAX260 or MAX261 and is more prone to sampling errors, as the tables show.

Maxim's filter design software applies the previous cor- rections automatically as a function of desired fCLK/f0, and Q. Therefore, Figure 20 should not be used when Maxim's software determines f0 and Q. This results in overcompensation of the sampling errors since the cor- rection factors are then counted twice.

1s/div

INA 	BPA

R, 10k

TRACE B

TRACE C

A 1V/DIV OV

B 5mV/DIV OV

C mV/DIV OV

The data plotted in Figure 20 applies for modes 1 and

3. When using Figure 20 for mode 4, the f0  error obtained from the graph should be multiplied by 1.5 and the Q error should be multiplied by 3.0. In mode 2, the value of fCLK/f0 should be multiplied by 2 and the

TRACE A

500kHz TTL

MAX261

CLKA

C, 1000pF

programmed  Q should be divided  by 2 before using the graphs.

As with all sampled systems, frequency components of the input signal above one half the sampling rate are aliased. In particular, input signal components near the sampling rate generate difference frequencies  that often fall within the passband of the filter. Such aliased

Figure 17. MAX261 Bandpass  Output Clock Noise

fCLK

2

CFB

VIN

CIN

signals, when they appear at the output, are indistin- guishable from real input information. For example, the aliased output signal generated when a 99kHz wave- form is applied  to a filter sampling at 100kHz (fCLK  =

200kHz) is 1kHz. This waveform is an attenuated ver- sion of the output that would result from a true 1kHz input. Remember that, with the MAX260 series filters,

~5pF

RIN =

-

12pF 	+

2	 	 CIN fCLK

the nyquist rate (one half the sample rate) is in fact fCLK/4, because fCLK is internally divided by two.

A simple, passive RC lowpass input filter is usually suf- ficient to remove input frequencies  that can cause aliasing. In many cases, the input signal itself may be band limited and require no special anti-alias filtering.

Figure 18. MAX260 Input Model

fCLK

2

CB

CFB

The wideband  MAX262 uses lower fCLK/f0  ratios than the MAX260/MAX261 and, for this reason, is more likely to require input filtering than the MAX260 or MAX281.

Trimming DC Offset The DC offset voltage at the LP or notch output can be adjusted with the circuit in Figure 21. This circuit also uses the input op amp to implement a single-pole anti- alias filter. Note that the total offset is generally less in

VIN

~5pF

0.016pF

-

+

CA 12pF		 	 RIN =	2

750 CA fCLK

MAX260/MAX261/MAX262

multistage filters than when only one section is used,

Figure 19. MAX261/MAX262 Input Model

since each offset is typical negative and each section inverts. When the HP or BP outputs are used, the offset can be removed with capacitor coupling.

Design Examples

Fourth-Order Chebyshev Bandpass Filter Figure 22 shows both halves of a MAX260 cascaded to form a fourth-order Chebyshev bandpass  filter. The desired parameters are:

Center frequency (f0)	 = 1kHz Pass bandwidth 	= 200Hz Stop bandwidth 	= 600Hz Max passband ripple 	= 0.5dB

fO ERROR vs. fCLK/fO RATIO (MODE 1, 3)

fO ERROR (%)

20

18 	f0 ERROR IS PLOTTED FOR MODES  1 AND 3

MODE 2: MULTIPLY ICLKIO BY 2 and

16 	DIVIDE Q BY 2 BEFORE USING GRAPH

14 	MODE 4: MUTIPLY fO ERROR BY 1.5

12 	Q = 0.512

10 	Q = 0.512

8 	Q = 0.512

6 	Q = 0.512 Q = 0.512

Q = 0.512

4

2

0

40      60

80     100

120    140    160    180    200

Min stopband attenuation	= 15dB

Q ERROR (%)

From the previous parameters, the order (number of poles) and the f0 and Q of each section can be deter- mined. Such a derivation is beyond the scope of this data sheet; however, there are a number of sources that provide design data for this procedure.  These include look-up tables, design texts, and computer pro- grams. Design software is available from Maxim to pro- vide comprehensive solutions for most popular filter configurations. The A and B section parameters for the above filter are:

fCLK/fO RATIO

Q ERROR vs. fCLK/fO RATIO

-7

Q ERROR IS PLOTTED FOR MODES  1 AND 3

-6	MODE 2: MULTIPLY fCLK/fO BY 2 and

DIVIDE Q BY 2 BEFORE USING GRAPH

-5	MODE 4: MUTIPLY Q ERROR BY 1.5

Q = 0.5

-4	Q = 0.6

-3

Q = 0.83

Q = 1.21    Q = 3.05

QA = 7.05	QB = 7.05

To implement this filter, both halves operate in mode 1 and use the same clock. See Tables 2 and 3. The pro-

-1

0

40      60

80     100

Q = 7.11

120    140    160    180    200

grammed parameters are:

CLKA = CLKB = 150kHz

fCLK/f0A = 166.50 (Mode 1, N = 42), actual f0A = 902.4Hz fCLK/f0B  = 136.66 (Mode 1, N = 23), actual f0B  =

1099.7Hz

QA = QB = 7.11 (Mode 1, N = 119)

Sampling errors are very small at this fCLK/f0  ratio, so the actual realized Q is very close to 7.05 (see Figure

20 or program MPP in the Filter Design Software sec-

fCLK/fO RATIO

Figure 20. Sampling Errors in fCLK/f0 and Q at Low fCLK/f0 and

Q Settings

C1

R2 100k

VIN

tion). Often the realized Q is not exactly the target value at high Qs because programming  resolution lowers as

+5V

R3 270k

R1 100k

-

TO FILTER

Q increases. This does not affect most filter designs,

since three-digit  Q accuracy  is practically  never required, and a Q resolution of 1 is provided up to Qs

+

100k

OFFSET TRIM

GAIN = -R1/R2

1

fLP =  2R1C2

INPUT

of 10. The overall filter gain at f0 is 16.4V/V or 24.3dB

(see the Cascading  Filters section). If another gain is

required, amplification or attenuation must be added at the input, output, or between stages.

-5V

NOTE: OP AMP INCLUDED WITH MAX261/MAX262

MAX260/MAX261/MAX262

Figure 21. Circuit for DC Offset Adjustment

40	-180	30

GAIN

15

-90	15

GAIN (dB)

PHASE (DEGREES)

GAIN (dB)

-10	0	0

-35

PHASE

90

-15

-60

200

500 	1K	2K

5K	10K

180

20K

-30

1K	2K

5K	10K

20K

50K

100K

VIN

FREQUENCY (Hz)

5	1

INA 	BPA

MAX260

23	21

INB 	BPB

VOUT

FREQUENCY (Hz)

Figure 23. MAX261 Fourth-Order Chebyshev Bandpass  Using

Coefficients of Figure 22

From the previous parameters, we use either lookup tables, design texts, or Maxim's filter design programs to generate the order (number of poles), and the f0 and Q of each second-order section. The A and B parame-

CLKA

CLKB

WR, AX, DX

ters are:

11	12

CLK

PROGRAM

f0A = 639Hz	f0B = 1564Hz

QA = 2.01	QB = 2.01

To implement this filter, section A operates in mode 1 and section B uses mode 2 to provide a wider overall range of fCLK/f0  ratios. This way, one clock frequency can drive both sections A and B. See Tables 2 and 3.

| CLKA = CLKB = 120kHz CLKA,B | MODE | fOA | fOB | QA | QB |
| --- | --- | --- | --- | --- | --- |
| 150kHz | 1 | N = 42 | N = 23 | N = 119 | N = 119 |

MAX260/MAX261/MAX262

Figure 22. Fourth-Order Chebyshev Bandpass  Filter

In Figure 23, a series of response curves are shown for the previous configuration using a MAX261 with clock frequencies  ranging  from 750kHz to 4MHz (f0 from

500Hz to 30kHz). Note that the rightmost curve shows about 2dB of gain peaking compared to the lower fre- quency curves, indicating the upper limit of usable filter accuracy at this Q (see Table 1).

Wide Passband Chebyshev Bandpass In this example (Figure 24), the desired parameters are:

Center frequency (f0)	 = 1kHz Pass bandwidth 	= 1kHz Stop bandwidth 	= 3kHz Max passband ripple 	= 1dB Min stopband attenuation	= 20dB

fCLK/f0A = 188.49 (Mode 1, N = 56), actual f0A = 636.6Hz fCLK/f0B = 76.64 (Mode 2, N = 5), actual f0B = 156.5Hz QA = 2.000 (Mode 1, N = 96), QB = 2.01 (Mode 2, N =

83)

The  overall  passband  gain  at  f 0   is  0.64V/V  or

-3.9dB.

High-Frequency Chebyshev Bandpass

The same Chebyshev response shape shown in Figure

24 is implemented  at higher frequencies  with a MAX262 in Figure 25. The curves show plots for center frequencies of 15.6kHz, 31.3kHz, and 47kHz. Not only is this faster than the MAX260 implementation, but mode 1 can be used in both halves of the MAX262 for this filter because the range of available fCLK/f0 ratios is wider with the MAX262 than the MAX260.

10

-10

GAIN

-180

-90

0

-10

fO = 15.6kHz fCLK = 1MHz

GAIN (dB)

-30

-50

-70

PHASE

PHASE (DEGREES)

0

90

180

-20

GAIN (dB)

-30

-40

-50

fO = 31.3kHz fO = 47kHz

fCLK = 3MHz

100

200

500 	1K	2K

5K	10K

1K	2K

5K	10K

20K

50K

100K

VIN

FREQUENCY (Hz)

VOUT

VIN

FREQUENCY (Hz)

5	1

23	21

VOUT

5	1

INA 	BPA

23	21

INB 	BPB

INA 	BPA

INB 	BPB

MAX260

MAX262

CLKA

CLKB

WR, AX, DX

CLKA

CLKB

WR, AX, DX

11	12

11	12

CLK

PROGRAM

| CLK CLKA,B | MODE | fOA | fOB | QA | QB |
| --- | --- | --- | --- | --- | --- |
| 1 to 3MHz | 1 | N = 38 | N = 0 | N = 96 | N = 96 |

PROGRAM

| CLKA,B | MODEA | MODEB | fOA | fOB | QA | QB |
| --- | --- | --- | --- | --- | --- | --- |
| 120kHz | 1 | 2 | N = 56 | N = 5 | N = 96 | N = 83 |

Figure 24. Wide Passband Chebyshev Bandpass  Filter

Figure 25. High-Frequency Chebyshev Bandpass  Filter

MAX260/MAX261/MAX262

Fourth-Order Butterworth Lowpass Figure 26 shows a fourth-order Butterworth lowpass with a cutoff frequency of 3kHz. Sections A and B of a MAX260 are cascaded.  The f0 and Q parameters for each section are:

f0A = 3kHz	f0B = 3kHz

QA = 1.307	QB = 0.541

Mode 1 and a 400kHz clock are used. Because of low Q values, the sampling errors of Figure 20 begin to look significant in this case. From the graphs, using fCLK/f0 ratio near 133, f0A is about 4% high, f0B is 1.5% high, QA is -1.2% low, and QB is -0.5% low. If these errors are not a problem,  the corrections can be ignored. They are included here for best possible accuracy:

CLKA = CLKB = 400kHz

fCLK/f0A = 135.08 (N = 22), f0B = 2961Hz

(-1.3% correction)

fCLK/f0B = 139.80 (N = 25), f0A = 2861Hz

(-4.6% correction)

QA = 1.306 (N = 79, Q resolution prevents +0.5%

correction)

QB = 0.547 (N = 11 +1.1% correction)

Measured wideband noise for this filter is 123µV RMS. If mode 2 were used, the noise would be 87µV RMS. For lower noise with either mode, the first section should have the highest Q (section A in this example).

Ordering Information (continued) PART TEMP RANGE PACKAGE ACCURACY

MAX261ACNG 0°C to +70°C Plastic DIP 1%

MAX261BCNG 0°C to +70°C Plastic DIP 2%

MAX261AENG -40°C to +85°C Plastic DIP 1%

MAX261BENG -40°C to +85°C Plastic DIP 2%

MAX261ACWG 0°C to +70°C Wide SO 1%

MAX261BCWG 0°C to +70°C Wide SO 2%

MAX261AMRG -55°C to +125°C CERDIP 1%

MAX261BMRG -55°C to +125°C CERDIP 2%

MAX262ACNG 0°C to +70°C Plastic DIP 1%

MAX262BCNG 0°C to +70°C Plastic DIP 2%

MAX262AENG -40°C to +85°C Plastic DIP 1%

MAX2G2BENG -40°C to +85°C Plastic DIP 2%

MAX262ACWG 0°C to +70°C Wide SO 1%

MAX262BCWG 0°C to +70°C Wide SO 2%

MAX262AMRG -55°C to +125°C CERDIP 1%

MAX262BMRG -55°C to +125°C CERDIP 2%

Chip Topography

0.128in

3.251mm

HPA(OP OUT)

N.C.(HPA)

N.C.(HPB)

BPA LPA	INB   LPB  BPB

N.C.(OP IN) INA D1

D0

A3

0.199in

(5.055mm)

*All devices—24-pin packages 0.3in-wide packages

CLK OUT

V+

OSC OUT GND

A2	A1

A0 WR V-

CLKA    CLKB

HPB(N.C.)

MAX260/MAX261/MAX262

NOTE: LABELS IN PARENTHESES ( ) ARE FOR MAX261/MAX262 ONLY

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses are implied. Maxim reserves the right to change the circuitry and specifications without notice at any time.

26    	Maxim Integrated Products, 120 San Gabriel Drive, Sunnyvale, CA   94086 408-737-7600

©  2002 Maxim Integrated  Products 	Printed USA 	is a registered  trademark of Maxim Integrated  Products.