19-2737; Rev 0; 1/03

Quad, Rail-to-Rail, Fault-Protected, SPST  Analog Switches

General Description

The MAX312F/MAX313F/MAX314F are quad, single- pole/single-throw (SPST), fault-protected analog switch- es. They are pin compatible with the industry-standard nonprotected  MAX312/MAX313/MAX314. These switch- es feature fault-protected inputs and Rail-to-Rail signal- handling capability. All analog signal terminals are pro- tected from overvoltage faults up to ±36V with power on and up to ±40V with power off. During a fault condition, the COM_, NO_, or NC_ terminal becomes an open cir- cuit and only microamperes of leakage current flow from the source. On-resistance is 10 (max) and is matched between switches to 0.5 (max) at +25°C.

The MAX312F has four normally closed (NC) switches. The MAX313F has four normally open (NO) switches. The MAX314F has two NC and two NO switches. These CMOS switches operate with dual power supplies rang- ing from ±4.5V to ±20V or a single supply between +9V and +36V. All digital inputs have +0.8V and +2.4V logic thresholds, ensuring both TTL and CMOS logic com- patibility when using ±15V or a single +12V supply.

For supply voltages of ±5V, +5V, and +3V, refer to the

MAX4711/MAX4712/MAX4713 data sheet.

Applications

Communications Systems

Signal Routing Test Equipment Data Acquisition

Industrial and Process Control Systems

Features

♦ No Power-Supply Sequencing Required

♦ Rail-to-Rail Signal Handling

♦ All Switches Off with Power Off

♦ All Switches Off when V+ is Off and V- is On

♦ ±40V Fault Protection with Power Off

♦ ±36V Fault Protection with ±15V Supplies

♦ Control Line Fault Protection from

V- - 0.3V to V- + 40V

♦ Pin Compatible with Industry-Standard

DG411/DG412/DG413

♦ 600ns (typ) Fault Response Time

♦ 10 (max) RON with ±15V Supplies

♦ ±4.5V to ±20V Dual Supplies

♦ +9V to +36V Single Supply

♦ TTL- and CMOS-Compatible Logic Inputs with

±15V or Single +9V to +15V Supplies

Ordering Information

PART 	 TEMP RANGE 	PIN-PACKAGE MAX312FESE 	-40°C to +85°C 	16 SO MAX312FEPE	-40°C to +85°C 	16 Plastic DIP

Ordering Information continued at end of data sheet.

| Pin Configurations MAX312F |  |
| --- | --- |
| LOGIC | SWITCH |
| 0 1 | ON OFF |

| Avionics | TOP VIEW |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Redundant/Backup Systems |  | IN1 | 1 |  | 16 | IN2 |
| ATE Hot Swap |  | COM1 NC1 | 2 3 |  | 15 14 | COM2 NC2 |
|  |  | V- | 4 |  | 13 | V+ |
|  |  | GND | 5 | MAX312F | 12 | N.C. |
|  |  | NC4 | 6 |  | 11 | NC3 |
|  |  | COM4 | 7 |  | 10 | COM3 |
|  |  | IN4 | 8 |  | 9 | IN3 |
| Rail-to-Rail is a registered trademark of Nippon Motorola, Ltd. |  |  |  | DIP/SO |  |  |
| Functional Diagram appears at end of data sheet. |  |  |  |  |  |  |

MAX312F/MAX313F/MAX314F

Pin Configurations continued at end of data sheet.

N.C. = NOT CONNECTED. SWITCHES SHOWN FOR LOGIC 0 INPUT. ALL SWITCHES ARE OFF WITH POWER REMOVED.

ABSOLUTE MAXIMUM RATINGS

(Voltages Referenced to GND.)

V+ ...........................................................................-0.3V to +44V V- ............................................................................-44V to +0.3V V+ to V-...................................................................-0.3V to +44V IN_ ......................................................... (V- - 0.3V) to (V- + 40V) NO_, NC_ to COM_ (Note 1) .................................. -40V to +40V COM_, NO_, NC_ Voltage with

Power On (Note 1) .............................................. -36V to +36V COM_, NO_, NC_ Voltage with

Power Off (Note 1) .............................................. -40V to +40V Peak Current COM_, NO_, NC_

(pulsed at 1ms, 10% duty cycle) ................................300mA

Continuous Current (any other terminal) ..........................  30mA Continuous Current (COM_, NO_, NC_) .........................±100mA Continuous Power Dissipation (TA = +70°C)

16-Pin SO (derate 8.7mW/°C above +70°C)................696mW

16-Pin Plastic DIP (derate 10.53mW/°C

above +70°C)  ........................................................ 842mW Operating Temperature Range ..........................  -40°C to +85°C Junction Temperature  .................................................... +150°C Storage Temperature Range  ...........................  -65°C to +160°C Lead Temperature (soldering,  10s) ............................... +300°C

MAX312F/MAX313F/MAX314F

Note 1: COM_, NO_, and NC_ pins are fault protected. Signals on COM_, NO_, and NC_ exceeding  -36V to +36V may damage  the device during power-on conditions.  When the power is off, the maximum range is -40V to +40V.

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS—15V Dual Supplies

(V+ = +15V,  V- = -15V, VIH = +2.4V, VIL = +0.8V, GND = 0V, TA = TMIN to TMAX, unless otherwise  noted.  Typical  values are at

TA = +25C.) (Notes 2, 3)

| PARAMETER | SYMBOL | CONDITIONS | TA | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| ANALOG SWITCH |  |  |  |  |  |
| Fault-Free Analog Signal Range | VCOM_, VNO_, VNC_ |  | E | V- V+ | V |
| On-Resistance | RON | ICOM_ = 10mA; VNO_, VNC_ = 10V | +25°C | 8 10 | Ω |
|  |  |  | E | 13 |  |
| On-Resistance Match Between Channels (Note 4) | ∆RON | ICOM_ = 10mA; VNO_, VNC_ = 10V | +25°C | 0.05 0.5 | Ω |
|  |  |  | E | 0.75 |  |
| On-Resistance Flatness (Note 5) | RFLAT(ON) | ICOM_ = 10mA; VNO_, VNC_ = 5V, 0V | +25°C | 0.25 1 | Ω |
|  |  |  | E | 1.25 |  |
| NO_, NC_ Off-Leakage Current (Note 6) | INO_(OFF), INC_(OFF) | VCOM_ = 10V; VNO_, VNC_ = ±10V | +25°C | -1 +1 | nA |
|  |  |  | E | -60 +60 |  |
| COM_ Off-Leakage Current (Note 6) | ICOM_(OFF) | VCOM_ = 10V; VNO_, VNC_ = ±10V | +25°C | -1 +1 | nA |
|  |  |  | E | -60 +60 |  |
| COM_ On-Leakage Current (Note 6) | ICOM_(ON) | VCOM_ = 10V; VNO_, VNC_ = 10V or floating | +25°C | -2 +2 | nA |
|  |  |  | E | -60 +60 |  |
| FAULT |  |  |  |  |  |
| Fault-Protected Analog Signal Range | VCOM_, VNO_, VNC_ | V+ = +15V, V- = -15V | E | -36 +36 | V |
|  |  | V+ = 0V, V- = -15V | E | -36 +36 |  |
|  |  | V+ = V- = 0V | E | -40 +40 |  |
| NO_ or NC_ Off-Leakage Current (Note 6) | INO_(OFF), INC_(OFF) | VNO_, VNC_ = 36V; V+ = +15V, 0V; V- = -15V | +25°C | -1 +1 | µA |
|  |  |  | E | -10 +10 |  |
| COM_ Off-Leakage Current (Note 6) | ICOM_(OFF) | VCOM_ = 36V; V+ = +15V, 0V; V- = -15V | +25°C | -1 +1 | µA |
|  |  |  | E | -10 +10 |  |

MAX312F/MAX313F/MAX314F

ELECTRICAL CHARACTERISTICS—15V Dual Supplies (continued)

(V+ = +15V,  V- = -15V, VIH = +2.4V, VIL = +0.8V, GND = 0V, TA = TMIN to TMAX, unless otherwise  noted.  Typical  values are at

TA = +25C.) (Notes 2, 3)

| PARAMETER | SYMBOL | CONDITIONS | TA | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| NO_ or NC_ Leakage Current (Note 6) | INO_, INC_ | VNO_, VNC_ = 40V; V+ = V- = 0V | +25°C | -1 +1 | µA |
|  |  |  | E | -10 +10 |  |
| COM_ Leakage Current (Note 6) | ICOM_ | VCOM_ = 40V; V+ = V- = 0V | +25°C | -1 +1 | µA |
|  |  |  | E | -10 +10 |  |
| Fault-Trip Threshold |  |  | E | V- - 0.4 V+ + 0.4 | V |
|  Fault Response Time | tRES | VNO_, VNC_ = 36V; RL = 300Ω | E | 600 | ns |
|  Fault Recovery Time | tREC | VNO_, VNC_ = 36V; RL = 300Ω | E | 1 | µs |
| SWITCH DYNAMICS |  |  |  |  |  |
| Turn-On Time | tON | VNO_ or VNC_ = 10V, RL = 300Ω, CL = 35pF, Figure 2 | +25°C | 115 225 | ns |
|  |  |  | E | 275 |  |
| Turn-Off Time | tOFF | VNO_ or VNC_ = 10V, RL = 300Ω, CL = 35pF, Figure 2 | +25°C | 70 185 | ns |
|  |  |  | E | 235 |  |
| Break-Before-Make Time Delay (MAX314F Only) (Note 7) | tBBM | VNO_ or VNC_ = 10V, RL = 100Ω, CL = 10pF, Figure 3 | +25°C | 5 45 | ns |
|  |  |  | E | 2 |  |
| Charge Injection | Q | VGEN = 0V, RGEN = 0Ω, CL = 1nF, Figure 4 | +25°C | 70 | pC |
| NO_ or NC_ Off-Capacitance | CN_(OFF) | f = 1MHz, Figure 5 | +25°C | 20 | pF |
| COM_ Off-Capacitance | CCOM_(OFF) | f = 1MHz, Figure 5 | +25°C | 20 | pF |
| COM_ On-Capacitance | CCOM_(ON) | f = 1MHz, Figure 5 | +25°C | 43 | pF |
| Off-Isolation (Note 8) | VISO | f = 1MHz, RL = 50Ω, CL = 15pF, PIN = 0dBm, Figure 6 | +25°C | -55 | dB |
| Channel-to-Channel Crosstalk (Note 9) | VCT | f = 1MHz, RL = 50Ω, CL = 15pF, PIN = 0dBm, Figure 6 | +25°C | -104 | dB |
| LOGIC INPUT |  |  |  |  |  |
| Input Logic High | VIH |  | E | 2.4 | V |
| Input Logic Low | VIL |  | E | 0.8 | V |
| Input Leakage Current | IIN | VIN_ = 0V or V+ | E | -1 +1 | µA |
| POWER SUPPLY |  |  |  |  |  |
| Power-Supply Range | V+, V- |  | E | 4.5 20 | V |
| V+ Supply Current | I+ | All VIN_ = +5V, VCOM_ = 0V | +25°C | 340 500 | µA |
|  |  |  | E | 700 |  |
|  |  | All VIN_ = 0V or V+, VCOM_ = 0V | +25°C | 140 250 |  |
|  |  |  | E | 350 |  |

MAX312F/MAX313F/MAX314F

ELECTRICAL CHARACTERISTICS—15V Dual Supplies (continued)

(V+ = +15V,  V- = -15V, VIH = +2.4V, VIL = +0.8V, GND = 0V, TA = TMIN to TMAX, unless otherwise  noted.  Typical  values are at

TA = +25C.) (Notes 2, 3)

| PARAMETER | SYMBOL | CONDITIONS | TA | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| V- Supply Current | I- | All VIN_ = +5V, VCOM_ = 0V | +25°C | 140 200 | µA |
|  |  |  | E | 300 |  |
|  |  | All VIN_ = 0V or V+, VCOM_ = 0V | +25°C | 140 250 |  |
|  |  |  | E | 350 |  |
| GND Supply Current | IGND | All VIN_ = +5V, VCOM_ = 0V | +25°C | 200 300 | µA |
|  |  |  | E | 400 |  |
|  |  | All VIN_ = 0V or V+, VCOM_ = 0V | +25°C | 0 1 |  |
|  |  |  | E | 10 |  |

ELECTRICAL CHARACTERISTICS—Single +12V Supply

(V+ = +12V,  V- = 0V, VIH = +2.4V,  VIL = +0.8V,  GND = 0V, TA = TMIN to TMAX, unless  otherwise  noted.  Typical  values  are at

TA = +25C.) (Notes 2, 3)

| PARAMETER | SYMBOL | CONDITIONS | TA | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| ANALOG SWITCH |  |  |  |  |  |
| Fault-Free Analog Signal Range | VCOM_, VNO_, VNC_ |  | E | 0 V+ | V |
| On-Resistance | RON | ICOM_ = 10mA; VNO_, VNC_ = +10V | +25C | 16 25 | Ω |
|  |  |  | E | 30 |  |
| On-Resistance Match Between Channels (Note 4) | ∆RON | ICOM_ = 10mA; VNO_, VNC_ = +10V | +25C | 0.4 1.5 | Ω |
|  |  |  | E | 2 |  |
| On-Resistance Flatness | RFLAT(ON) | ICOM_ = 10mA; VNO_, VNC_ = +2V, +6V, +10V | +25C | 3 6 | Ω |
|  |  |  | E | 7 |  |
| NO_, NC_ Off-Leakage Current (Note 6) | INO_(OFF), INC_(OFF) | VCOM_ = +1V, +10V; VNO_, VNC_ = +10V, +1V | +25C | -1 +1 | nA |
|  |  |  | E | -60 +60 |  |
| COM_ Off-Leakage Current (Note 6) | ICOM_(OFF) | VCOM_ = +1V, +10V; VNO_, VNC_ = +10V, +1V | +25C | -1 +1 | nA |
|  |  |  | E | -60 +60 |  |
| COM_ On-Leakage Current (Note 6) | ICOM_(ON) | VCOM_ = +1V, +10V; VNO_, VNC_ = +1V, +10V, or floating | +25C | -2 +2 | nA |
|  |  |  | E | -60 +60 |  |
| FAULT |  |  |  |  |  |
| Fault-Protected Analog Signal Range | VCOM_, VNO_, VNC_ | V+ = +12V, V- = 0V | E | -36 +36 | V |
|  |  | V+ = V- = 0V | E | -40 +40 |  |
| NO_ or NC_ Off-Leakage Current (Note 6) | INO_(OFF), INC_(OFF) | VNO_, VNC_ = 36V; V+ = +12V; V- = 0V | +25C | -1 +1 | µA |
|  |  |  | E | -10 +10 |  |
| COM_ Off-Leakage Current (Note 6) | ICOM_(OFF) | VNO_, VNC_ = 36V; V+ = +12V; V- = 0V | +25C | -1 +1 | µA |
|  |  |  | E | -10 +10 |  |
| NO_ or NC_ Leakage Current (Note 6) | INO_, INC_ | V+ = V- = 0V; VNO_, VNC_ = 40V | +25C | -1 +1 | µA |
|  |  |  | E | -10 +10 |  |

MAX312F/MAX313F/MAX314F

ELECTRICAL CHARACTERISTICS—Single +12V Supply (continued)

(V+ = +12V,  V- = 0V, VIH = +2.4V,  VIL = +0.8V,  GND = 0V, TA = TMIN to TMAX, unless  otherwise  noted.  Typical  values  are at

TA = +25C.) (Notes 2, 3)

| PARAMETER | SYMBOL | CONDITIONS | TA | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| COM_ Leakage Current (Note 6) | ICOM_ | V+ = V- = 0V; VNO_, VNC_ = 40V | +25C | -1 +1 | µA |
|  |  |  | E | -10 +10 |  |
| Fault Response Time | tRES | VNO_, VNC_ = +36V; RL = 300Ω | E | 200 | ns |
| Fault Recovery Time | tREC | VNO_, VNC_ = +36V; RL = 300Ω | E | 1 | µs |
| SWITCH DYNAMICS |  |  |  |  |  |
| Turn-On Time | tON | VNO_ or VNC_ = +10V, RL = 300Ω, CL = 35pF, Figure 2 | +25C | 140 325 | ns |
|  |  |  | E | 425 |  |
| Turn-Off Time | tOFF | VNO_ or VNC_ = +10V, RL = 300Ω, CL = 35pF, Figure 2 | +25C | 75 175 | ns |
|  |  |  | E | 225 |  |
| Break-Before-Make Time Delay (MAX314F Only) (Note 6) | tBBM | VNO_ or VNC_ = +10V, RL = 100Ω, CL = 10pF, Figure 3 | +25C | 10 65 | ns |
|  |  |  | E | 5 |  |
| Charge Injection | Q | VGEN = 0V, RGEN = 0Ω, CL = 1nF, Figure 4 | +25C | -10 | pC |
| LOGIC INPUT |  |  |  |  |  |
| Input Logic High | VIH |  | E | 2.4 | V |
| Input Logic Low | VIL |  | E | 0.8 | V |
| Input Leakage Current (Note 6) | IIN | VIN_ = 0V or V+ | E | -1 +1 | µA |
| POWER SUPPLY |  |  |  |  |  |
| Power-Supply Range | V+ |  | E | +9 +36 | V |
| V+ Supply Current | I+ | All VIN_ = +5V, VCOM_ = +6V | +25C | 160 300 | µA |
|  |  |  | E | 400 |  |
|  |  | All VIN_ = 0V or V+, VCOM_ = +6V | +25C | 70 150 |  |
|  |  |  | E | 250 |  |

Note 2: The algebraic  convention is used in this data sheet; the most negative value is shown in the minimum column.

Note 3: Electrical specifications at -40C are guaranteed  by design and not production  tested.

Note 4: RON = RON(MAX) - RON(MIN).

Note 5: Flatness is defined as the difference between the maximum and minimum value of on-resistance over the specified  analog

signal range.

Note 6: Single-supply leakage parameters are guaranteed by testing with dual supplies at the maximum rated temperature.

Note 7: Guaranteed by design.

Note 8: Off-isolation = 20 log10 [VCOM/(VNC or VNO)], VNC or VNO = output, VCOM = input to off switch.

Note 9: Between any two switches.

Typical Operating Characteristics

(TA = +25°C, unless otherwise noted.)

ON-RESISTANCE vs. VCOM

(DUAL SUPPLIES)

25

V+ = +5V V- = -5V

20 	V+ = +10V

MAX312F/13F/14F toc01

V- = -10V

MAX312F/13F/14F toc02

ON-RESISTANCE vs. VCOM AND TEMPERATURE (DUAL SUPPLIES)

15

V+ = +15V

V- = -15V

12

TA = +85C

MAX312F/13F/14F toc03

ON-RESISTANCE vs. VCOM

(SINGLE SUPPLY)

RON ()

30

V+ = +9V

V- = 0V

25 	V+ = +12V

V- = 0V

20 	V+ = +15V

RON ()

15 	V+ = +15V

V- = -15V

10

5

V+ = +20V V- = -20V

0

9

TA = +25C

RON ()

6 	TA = -40C

3

0

V- = 0V

15

10

5 	V+ = +28V V- = 0V

0

V+ = +24V V- = 0V

V+ = +36V V- = 0V

-20

-15

-10    -5

0	5	10      15      20

VCOM (V)

-15

-10

-5	0	5	10	15

VCOM (V)

0	6	12	18

VCOM (V)

24	30	36

MAX312F/13F/14F toc04

ON-RESISTANCE vs. VCOM AND TEMPERATURE (SINGLE SUPPLY)

25

20

RON ()

15

10,000

1000

LEAKAGE CURRENT (pA)

100

ON/OFF-LEAKAGE  CURRENT

vs. TEMPERATURE

MAX312F/13F/14F toc05

ICOM(ON)

| 200 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DUAL SUPPLIES: 15V |  |  |  |  |  |
|  |  |  | SINGLE SUPPLY: +12V |  |  |  |  |

150

100

MAX312F/13F/14F toc06

CHARGE INJECTION vs. VCOM

10

10

1

5	0.1

0	0.01

ICOM(OFF)

Q (pC)

INO(OFF), INC(OFF)

DUAL SUPPLIES:

V+ = +15V, V- = -15V

50

0

-50

-100

| 0 2 4 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | TA = +85C TA = +25C TA = -40C |  |  |  |  |
| V+ = +12V V- = 0V |  |  |  |  |  |  |

6	8	10	12

-40

-15	10	35

60	85

-15

-10

-5	0

5	10	15

500

400

SUPPLY CURRENT (A)

300

200

100

0

-100

-200

-300

VCOM (V)

SUPPLY CURRENT vs. TEMPERATURE

DUAL SUPPLIES:

MAX312F/13F/14F toc07

V+ = +15V, V- = -15V, VIN_ = +5V I+

I- IGND

| 3.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DUAL SUPPLIES |  |  |  |  |  |  |  |  |
|  |  |  |  |  | SINGLE SUPPLY |  |  |  |

THRESHOLD VOLTAGE (V)

2.5

2.0

1.5

1.0

TEMPERATURE (C)

MAX312F/13F/14F toc08

LOGIC-LEVEL THRESHOLD VOLTAGE

vs. SUPPLY VOLTAGE

500

400

300

tON/tOFF (ns)

200

100

0

VCOM (V)

TURN-ON/TURN-OFF TIME

MAX312F/13F/14F toc09

| vs. SUPPLY VOLTAGE (DUAL SUPPLIES) |  |  |
| --- | --- | --- |
| tON |  |  |
| tOFF |  |  |

-40	-15	10	35	60	85

0	5	10     15

20     25

30     35      40

5	10	15	20

MAX312F/MAX313F/MAX314F

TEMPERATURE (C)

SUPPLY VOLTAGE (V)

SUPPLY VOLTAGE (V+, V-)

Typical Operating Characteristics (continued)

(TA = +25°C, unless otherwise noted.)

200

TURN-ON/TURN-OFF TIME

MAX312F/13F/14F toc10

| vs. SUPPLY VOLTAGE (SINGLE SUPPLY) |  |  |
| --- | --- | --- |
|  | tON |  |
|  | tOFF | V- = GND |

250

TURN-ON/TURN-OFF TIME

MAX312F/13F/14F toc11

| vs. TEMPERATURE (DUAL SUPPLIES) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | tON |  |  |
|  |  |  | tOFF |  |
|  |  | DUA | L SUPPLIE | S: 15V |

200

TURN-ON/TURN-OFF TIME

MAX312F/13F/14F toc12

vs. TEMPERATURE (SINGLE SUPPLY)

160

200

160

tON/tOFF (ns)

120

80

150

tON/tOFF (ns)

100

120

tON/tOFF (ns)

80

40	50	40

0

9	18	27	36

0

-40

-15	10

35	60	85

0

| -40 V+ = +12 | V, V- = 0V |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | tON |  |  |
|  |  |  | tOFF |  |

-15	10

MAX312F/13F/14F toc15

35	60	85

SUPPLY VOLTAGE (V+)

TEMPERATURE (C)

TEMPERATURE (C)

20

0

-20

-40

LOSS (dB)

-60

-80

-100

-120

FREQUENCY RESPONSE

V+ = +15V, V- = -15V ON-RESPONSE

OFF-ISOLATION

CROSSTALK

MAX312F/13F/14F toc13

THD + N  (%)

0.01

0.001

| 0.0001 +12V S |  | INGLE S | UPPLY |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | FOR 0V < VCOM < VSUPPLY, ICOM = VCOM/RL |  |  |  |

TOTAL HARMONIC DISTORTION PLUS NOISE vs. FREQUENCY

15V DUAL SUPPLIES:

IN = OUT = 600, VP-P = 5V

200

150

MAX312F/13F/14F toc14

100

ICOM (A)

50

0

-50

-100

-150

MAX312F/13F/14F toc16

-200

FAULT CURRENT vs. FAULT VOLTAGE (DUAL SUPPLIES)

DUAL SUPPLIES:

V+ = +15V, V- = -15V

FOR |VCOM| < VSUPPLY,

0.001

0.01

0.1	1

10	100

1000

10 	100 	1k

10k

-60

-40

-20

0	20	40	60

FREQUENCY (MHz)

FREQUENCY (Hz)

VCOM (V)

200

FAULT CURRENT vs. FAULT VOLTAGE (SINGLE SUPPLY)

150

100

ICOM (A)

50

0

-50

-100

-150

MAX312F/MAX313F/MAX314F

-200

-60

-40

-20

0	20	40	60

VCOM (V)

Typical Operating Characteristics (continued)

(TA = +25°C, unless otherwise noted.)

INPUT OVERVOLTAGE vs. OUTPUT

MAX312F/13F/14F toc17

2ms/div

+25V

VNO_, VNC_

INPUT

GND

-25V

+15V GND

-15V

FAULT RECOVERY TIME (POSITIVE INPUT)

MAX312F/13F/14F toc18

+25V

VNO_, VNC_

INPUT

+5V GND

+5V VCOM_ OUTPUT GND

1s/div

FAULT RECOVERY TIME (NEGATIVE INPUT)

MAX312F/13F/14F toc19

FAULT RESPONSE TIME (POSITIVE INPUT)

MAX312F/13F/14F toc20

GND

-5V

VNO_, VNC_

INPUT

-25V

VNO_, VNC_

INPUT

10V/div

GND

GND VCOM_ OUTPUT

-5V

VCOM_

OUTPUT

10V/div

GND

1s/div

400ns/div

FAULT RESPONSE TIME (NEGATIVE INPUT)

MAX312F/13F/14F toc21

GND

VNO_, VNC_

INPUT

10V/div

FAULT RESPONSE (V+ = 0V, V- = -15V)

MAX312F/13F/14F toc22

+25V

VNO_, VNC_

INPUT

GND

MAX312F/MAX313F/MAX314F

1s/div

GND VCOM_

OUTPUT

10V/div

200s/div

-25V

GND VCOM_ OUTPUT

500mV/div

Pin  Description

| PIN |  |  | NAME | FUNCTION |
| --- | --- | --- | --- | --- |
| MAX312F | MAX313F | MAX314F |  |  |
| 1, 16, 9, 8 | 1, 16, 9, 8 | 1, 16, 9, 8 | IN1, IN2, IN3, IN4 | Logic-Control Digital Inputs |
| 2, 15, 10, 7 | 2, 15, 10, 7 | 2, 15, 10, 7 | COM1, COM2, COM3, COM4 | Analog Switch Common Terminals |
| 3, 14, 11, 6 | — | — | NC1, NC2, NC3, NC4 | Analog Switch Normally Closed Terminals |
| — | 3, 14, 11, 6 | — | NO1, NO2, NO3, NO4 | Analog Switch Normally Open Terminals |
| — | — | 3, 6 | NO1, NO4 | Analog Switch Normally Open Terminals |
| — | — | 14, 11 | NC2, NC3 | Analog Switch Normally Closed Terminals |
| 4 | 4 | 4 | V- | Negative-Supply Voltage Input. Connect to GND for single- supply operation. Bypass with a 0.1µF capacitor to GND. |
| 5 | 5 | 5 | GND | Ground. Connect to digital ground. |
| 12 | 12 | 12 | N.C. | No Connection. Not internally connected. |
| 13 | 13 | 13 | V+ | Positive-Supply Voltage Input. Bypass with a 0.1µF capacitor to GND. |

MAX312F/MAX313F/MAX314F

Detailed Description

The MAX312F/MAX313F/MAX314F are fault-protected CMOS analog switches with unique operation and construction. These switches differ considerably  from traditional fault-protection switches, with several advan- tages. First, they are constructed  with two parallel FETs, allowing very low on-resistance when the switch is on. Second, they allow signals on the NO_ or NC_ pins that are within, or slightly beyond, the supply rails to be passed through the switch to the COM_ terminal (or vice versa), allowing true rail-to-rail signal operation. Third, the MAX312F/MAX313F/MAX314F have the same fault-protection performance on any of the NO_, NC_, or COM_ switch inputs. Operation is identical for both fault polarities. The fault protection extends to ±36V from GND with 15V supplies.

During a fault condition, the particular overvoltage input (COM_, NO_, NC_) pin becomes high impedance regardless of the switch state or load resistance. When power is removed, the fault protection is still in effect. In this case, the COM_, NO_, or NC_ terminals are a virtu- al open circuit. The fault can be up to ±40V with power off. The switches turn off when V+ is not powered, regardless of V-.

Pin Compatibility These switches have identical pinouts to common non- fault-protected CMOS switches. They allow for carefree

direct replacement in existing printed circuit boards since the NO_, NC_, and COM_ pins of each switch are fault protected.

Internal Construction Internal construction is shown in Figure 1, with the ana- log signal paths shown in bold. A single NO switch is shown. The NC configuration is identical except  the logic-level translator becomes an inverter. The analog switch is formed by the parallel combination of N-chan- nel FET (N1) and P-channel FET (P1), which are driven on and off simultaneously according  to the input fault condition and the logic-level state.

Normal Operation Two comparators continuously compare the voltage on the COM_, NO_, and NC_ pins with V+ and V-. When the signal on COM_, NO_, or NC_ is between V+ and V-, the switch acts normally, with FETs N1 and P1 turn- ing on and off in response to IN_ signals. The parallel combination of N1 and P1 forms a low-value resistor between NO_ (or NC_) and COM_ so that signals pass equally well in either direction.

Positive Fault Condition When the signal on NO_ (or NC_) and COM_ exceeds V+, the high-fault comparator output is high, turning off FETs N1 and P1. This makes the NO_ (or NC_) and COM_ pins high impedance  regardless of the switch

state. If the switch state is off, all FETs are turned off and both NO_ (or NC_) and COM_ are high impedance.

Negative Fault Condition When the signal on NO_ (or NC_) and COM_ exceeds V-, the low-fault comparator output is high, turning off FETs N1 and P1. This makes the NO_ (or NC_) and COM_ pins high impedance  regardless of the switch state. If the switch state is off, all FETs are turned off and both NO_ (or NC_) and COM_ are high impedance.

Transient Fault Response and Recovery When a fast rise-time and fall-time transient on NO_, NC_, or COM_ exceeds V+ or V-, the output follows the input to the supply rail with only a few nanoseconds delay. This delay is due to the switch on-resistance and circuit capacitance to ground. When the input transient returns to within the supply rails, however, there is a longer output recovery time delay. For positive faults, the recovery time is typically 1µs. For negative faults, the recovery time is typically  0.6µs. These values depend on the output resistance and capacitance, and are not production  tested or guaranteed. The delays are not dependent on the fault amplitude. Higher load resistance and capacitance increase recovery times.

Fault-Protection Voltage and Power Off The maximum fault voltage on the NO_ (or NC_) and COM_ pins is ±36V with power applied and ±40V with power off.

Failure Modes Exceeding the fault-protection voltage limits on NO_, NC_, or COM_, even for very short periods, can cause the device to fail (see the Absolute Maximum Ratings). The failure modes may not be obvious, and failure in one switch may or may not affect other switches in the same package.

Ground There is no galvanic connection between the analog signal paths and GND. The analog signal paths consist of an N-channel and P-channel MOSFET with their sources and drains paralleled and their gates driven out of phase to V+ and V- by the logic-level translators. However, the potential of the analog signals must be defined or at least limited with respect to GND.

V+ and GND power the internal logic and logic-level translators and set the input logic thresholds. The logic- level translators convert the logic levels to switched V+ and V- signals to drive the gates of the analog switch- es. This drive signal is the only connection between the power supplies and the analog signals.

Bipolar Supplies The MAX312F/MAX313F/MAX314F operate with bipolar supplies between ±4.5V and ±20V. The V+ and V- sup- plies need not be symmetrical, but their difference can- not exceed the absolute maximum rating of 44V.

Single Supply The MAX312F/MAX313F/MAX314F operate from a sin- gle supply between +9V and +36V when V- is connect- ed to GND.

Ordering Information (continued) PART TEMP RANGE PIN-PACKAGE

MAX313FESE -40°C to +85°C 16 SO

MAX313FEPE -40°C to +85°C 16 Plastic DIP

MAX314FESE -40°C to +85°C 16 SO

MAX314FEPE -40°C to +85°C 16 Plastic DIP

MAX312F/MAX313F/MAX314F

Chip Information

TRANSISTOR COUNT: 251

PROCESS: CMOS

SUBSTRATE CONNECTED TO: V+

NORMALLY OPEN SWITCH CONSTRUCTION V+

HIGH FAULT

P1

NO_

IN_

GND

LOW FAULT

ON

N1

MAX312F MAX313F MAX314F

COM_

V-

ESD DIODE

Figure 1. Functional Diagram

VIN_

V+

V+     NO_ OR NC_

MAX312F MAX313F

10V

VIN_

3V

0V

10V

50%

MAX314F COM_

VOUT 	90%

VOUT

90%

GND	V-

V-

RL 	CL

0V

tOFF

tON

MAX312F/MAX313F/MAX314F

V- IS CONNECTED TO GND (0V) FOR SINGLE-SUPPLY OPERATION.

Figure 2. Switch Turn-On/Turn-Off Times

Test Circuits/Timing Diagrams (continued)

VIN_

IN_ IN_

V+

V+	NO_ NC_

10V

3V VIN_

0V

50%

tF < 5ns tR < 5ns

MAX314F

COM_ COM_

VOUT

VNO_, VNC_

90%

GND	V- V-

RL 	CL

VOUT

0V

tBBM

V- IS CONNECTED TO GND (0V) FOR SINGLE-SUPPLY OPERATION.

Figure 3. MAX314F Break-Before-Make Interval

VIN_

V+

IN_  V+     NO_ OR NC_

MAX312F MAX313F MAX314F

RGEN

VGEN

3V VIN_

0V

GND

COM_

V-

V-

VOUT

CL

VOUT

VOUT

V- IS CONNECTED TO GND (0V) FOR SINGLE-SUPPLY OPERATION.

VOUT IS THE MEASURED VOLTAGE DUE TO CHARGE- TRANSFER ERROR Q WHEN THE CHANNEL  TURNS OFF.

Q = VOUT x CL

Figure 4. Charge Injection

V+

V+	NO_ V+

IN_

MAX312F MAX313F MAX314F

GND

NC_

COM_ V-

1MHz CAPACITANCE ANALYZER

MAX312F/MAX313F/MAX314F

V-

V- IS CONNECTED TO GND (0V) FOR SINGLE-SUPPLY OPERATION.

Figure 5. COM_, NO_, NC_ Capacitance

V+

0.1F

50 RESISTOR ONLY NEEDED FOR CROSSTALK AND ISOLATION

VIN

NETWORK ANALYZER

50	50

3.0V

V+	COM_

MAX312F MAX313F

50

IN_

MAX314F

NO_, NC_

VOUT

MEAS 	     	REF

GND	V-

50	50

MAX312F/MAX313F/MAX314F

0.1F

V-

MEASUREMENTS ARE STANDARDIZED AGAINST SHORT AND OPEN AT SOCKET TERMINALS. OFF-ISOLATION IS MEASURED BETWEEN COM_  AND OFF NO_ OR NC_ TERMINALS.

ON-RESPONSE IS MEASURED BETWEEN COM_  AND ON NO_ OR NC_ TERMINALS. CROSSTALK IS MEASURED BETWEEN COM_ TERMINALS WITH ALL SWITCHES ON. V- IS CONNECTED TO GND (0V) FOR SINGLE-SUPPLY OPERATION.

Figure 6. Frequency Response, Off-Isolation, and Crosstalk

Pin  Configurations (continued)

| TOP VIEW |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | IN1 | 1 |  | 16 | IN2 | IN1 | 1 | 16 | IN2 |
|  | COM1 | 2 |  | 15 | COM2 | COM1 | 2 | 15 | COM2 |
|  | NO1 | 3 |  | 14 | NO2 | NO1 | 3 | 14 | NC2 |
|  | V- | 4 |  | 13 | V+ | V- | 4 | 13 | V+ |
|  | GND | 5 | MAX313F | 12 | N.C. | GND | 5 MAX314F | 12 | N.C. |
|  | NO4 | 6 |  | 11 | NO3 | NO4 | 6 | 11 | NC3 |
|  | COM4 | 7 |  | 10 | COM3 | COM4 | 7 | 10 | COM3 |
|  | IN4 | 8 | DIP/SO | 9 | IN3 | IN4 | 8 DIP/SO | 9 | IN3 |

| MAX313F |  |
| --- | --- |
| LOGIC | SWITCH |
| 0 1 | OFF ON |

| N.C. = NOT CONNECTED. SWITCHES SHOWN FOR LOGIC 0 INPUT. ALL SWITCHES ARE OFF WITH POWER REMOVED. MAX314F |  |  |
| --- | --- | --- |
| LOGIC | SWITCHES 1, 4 | SWITCHES 2, 3 |
| 0 1 | OFF ON | ON OFF |

MAX312F/MAX313F/MAX314F

PDIPN.EPS

| Package Information (The package drawing(s) in this data sheet may not reflect the most current specifications. For the latest package outline information, go to www.maxim-ic.com/packages.) |  |  |
| --- | --- | --- |

MAX312F/MAX313F/MAX314F

16L SOIC.EPS

Package Information (continued) (The package  drawing(s)  in this data sheet may not reflect the most current specifications. For the latest package  outline information, go to  www.maxim-ic.com/packages.)

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses are implied. Maxim reserves the right to change the circuitry and specifications without notice at any time.

Maxim Integrated Products, 120 San Gabriel Drive, Sunnyvale, CA   94086 408-737-7600   	15

©  2003 Maxim Integrated  Products 	Printed USA 	is a registered  trademark of Maxim Integrated  Products.