19-0333; Rev 0; 12/94

3.3V-Powered, 10Mbps and  Slew-Rate-Limited

True RS-485/RS-422 Transceivers

General Description

The MAX3483, MAX3485, MAX3486, MAX3488, MAX3490, and MAX3491 are 3.3V, low-power trans- ceivers for RS-485 and RS-422 communication.  Each part  contains  one driver  and  one receiver.  The MAX3483 and MAX3488 feature slew-rate-limited dri- vers that minimize EMI and reduce reflections caused by improperly  terminated cables,  allowing error-free data transmission at data rates up to 250kbps. The par-

tially  slew-rate-limited  MAX3486  transmits  up  to

2.5Mbps.  The MAX3485, MAX3490, and MAX3491 transmit at up to 10Mbps.

Drivers are short-circuit current limited and are protect- ed against excessive power dissipation  by thermal shutdown circuitry that places the driver outputs into a high-impedance  state. The receiver input has a fail-safe feature that guarantees  a logic-high  output if both inputs are open circuit.

The MAX3488, MAX3490, and MAX3491 feature full- duplex communication, while the MAX3483, MAX3485, and MAX3486 are designed  for half-duplex  communi- cation.

Applications Low-Power RS-485/RS-422 Transceivers Telecommunications

Transceivers for EMI-Sensitive Applications

Industrial-Control Local Area Networks

Features

♦ Operate from a Single 3.3V Supply— No Charge Pump!

♦ Interoperable with +5V Logic

♦ 8ns Max Skew (MAX3485/MAX3490/MAX3491)

♦ Slew-Rate Limited for Errorless Data Transmission

(MAX3483/MAX3488)

♦ 2nA Low-Current Shutdown Mode

(MAX3483/MAX3485/MAX3486/MAX3491)

♦ -7V to +12V Common-Mode Input Voltage Range

♦ Allows up to 32 Transceivers on the Bus

♦ Full-Duplex and Half-Duplex Versions Available

♦ Industry Standard 75176 Pinout

(MAX3483/MAX3485/MAX3486)

♦ Current-Limiting and Thermal Shutdown for

Driver Overload Protection

Ordering Information PART TEMP. RANGE PIN-PACKAGE

MAX3483CPA 0°C to +70°C 8 Plastic DIP

MAX3483CSA 0°C to +70°C 8 SO

MAX3483C/D 0°C to +70°C Dice*

MAX3483EPA -40°C to +85°C 8 Plastic DIP

MAX3483ESA -40°C to +85°C 8 SO

MAX3485CPA 0°C to +70°C 8 Plastic DIP

MAX3485CSA 0°C to +70°C 8 SO

MAX3485C/D 0°C to +70°C Dice*

MAX3485EPA -40°C to +85°C 8 Plastic DIP

MAX3485ESA -40°C to +85°C 8 SO

MAX3483/MAX3485/MAX3486/MAX3488/MAX3490/MAX3491

Ordering Information continued at end of data sheet.

* Contact factory for for dice specifications.

Selection Table

| PART NUMBER | GUARANTEED DATA RATE (Mbps) | SUPPLY VOLTAGE (V) | HALF/FULL DUPLEX | SLEW-RATE LIMITED | DRIVER/ RECEIVER ENABLE | SHUTDOWN CURRENT (nA) | PIN COUNT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MAX3483 | 0.25 | 3.0 to 3.6 | Half | Yes | Yes | 2 | 8 |
| MAX3485 | 10 |  | Half | No | Yes | 2 | 8 |
| MAX3486 | 2.5 |  | Half | Yes | Yes | 2 | 8 |
| MAX3488 | 0.25 |  | Full | Yes | No | — | 8 |
| MAX3490 | 10 |  | Full | No | No | — | 8 |
| MAX3491 | 10 |  | Full | No | Yes | 2 | 14 |

ABSOLUTE MAXIMUM RATINGS

Supply Voltage (VCC) ...............................................................7V Control Input Voltage (RE, DE) ...................................-0.3V to 7V Driver Input Voltage (DI) .............................................-0.3V to 7V Driver Output Voltage (A, B, Y, Z)..........................-7.5V to 12.5V Receiver Input Voltage (A, B) ................................-7.5V to 12.5V Receiver Output Voltage (RO)....................-0.3V to (VCC + 0.3V) Continuous Power Dissipation  (TA = +70°C)

8-Pin Plastic DIP (derate 9.09mW/°C above +70°C) .....727mW

8-Pin SO (derate 5.88mW/°C above +70°C)..................471mW

14-Pin Plastic DIP (derate 10mW/°C above +70°C) ......800mW

14-Pin SO (derate 8.33mW/°C above +70°C)................667mW Operating  Temperature  Ranges

MAX34_ _C_ _.......................................................0°C to +70°C MAX34_ _E_ _ ....................................................-40°C to +85°C Storage Temperature  Range .............................-65°C to +160°C Lead Temperature  (soldering,  10sec) .............................+300°C

MAX3483/MAX3485/MAX3486/MAX3488/MAX3490/MAX3491

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions  beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

DC ELECTRICAL CHARACTERISTICS

(VCC = 3.3V ±0.3V, TA = TMIN to TMAX, unless otherwise noted.  Typical values are at TA = +25°C)

| OUT PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| Differential Driver Output | VOD | RL = 100Ω (RS-422), Figure 4 |  | 2.0 | V |
|  |  | RL = 54Ω (RS-485), Figure 4 |  | 1.5 |  |
|  |  | RL = 60Ω (RS-485), VCC = 3.3V, Figure 5 |  | 1.5 |  |
| Change in Magnitude of Driver Differential Output Voltage for Complementary Output States (Note 1) | ∆VOD | RL = 54Ω or 100Ω, Figure 4 |  | 0.2 | V |
| Driver Common-Mode Output Voltage | VOC | RL = 54Ω or 100Ω, Figure 4 |  | 3 | V |
| Change in Magnitude of Common-Mode Output Voltage (Note 1) | ∆VOC | RL = 54Ω or 100Ω, Figure 4 |  | 0.2 | V |
| Input High Voltage | VIH | DE, DI, RE |  | 2.0 | V |
| Input Low Voltage | VIL | DE, DI, RE |  | 0.8 | V |
| Logic Input Current | IIN1 | DE, DI, RE |  | ±2 | µA |
| Input Current (A, B) | IIN2 | DE = 0V, VCC = 0V or 3.6V | VIN = 12V | 1.0 | mA |
|  |  |  | VIN = -7V | -0.8 |  |
| Output Leakage (Y, Z) | IO | DE = 0V, RE = 0V, VCC = 0V or 3.6V, MAX3491 | VOUT = 12V | 20 | µA |
|  |  |  | VOUT = -7V | -20 |  |
| Output Leakage (Y, Z) in Shutdown Mode | IO | DE = 0V, RE = VCC, VCC = 0V or 3.6V, MAX3491 | V = 12V | 1 | µA |
|  |  |  | VOUT = -7V | -1 |  |
| Receiver Differential Threshold Voltage | VTH | -7V ≤ VCM ≤ 12V |  | -0.2 0.2 | V |
| Receiver Input Hysteresis | ∆VTH | VCM = 0V |  | 50 | mV |
| Receiver Output High Voltage | VOH | IOUT = -1.5mA, VID = 200mV, Figure 6 |  | VCC - 0.4 | V |
| Receiver Output Low Voltage | VOL | IOUT = 2.5mA, VID = 200mV, Figure 6 |  | 0.4 | V |
| Three-State (High Impedance) Output Current at Receiver | IOZR | VCC = 3.6V, 0V ≤ VOUT ≤ VCC |  | ±1 | µA |
| Receiver Input Resistance | RIN | -7V ≤ VCM ≤ 12V |  | 12 | kΩ |

MAX3483/MAX3485/MAX3486/MAX3488/MAX3490/MAX3491

DC ELECTRICAL CHARACTERISTICS (continued)

(VCC = 3.3V ±0.3V, TA = TMIN to TMAX, unless otherwise noted.  Typical values are at TA = +25°C)

| PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| Supply Current | ICC | No load, DI = 0V or VCC | D E = VCC, RE = 0V or VCC | 1.1 2.2 | mA |
|  |  |  | DE = 0V, RE = 0V | 0.95 1.9 |  |
| Supply Current in Shutdown Mode | ISHDN | DE = 0V, RE = VCC, DI = VCC or 0V |  | 0.002 1 | µA |
| Driver Short-Circuit Output Current | IOSD | VOUT = -7V |  | -250 | mA |
|  |  | VOUT = 12V |  | 250 |  |
| Receiver Short-Circuit Output Current | IOSR | 0V ≤ VRO ≤ VCC |  | ±8 ±60 | mA |

DRIVER SWITCHING CHARACTERISTICS—MAX3485, MAX3490, and MAX3491

(VCC = 3.3V, TA = +25°C)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Driver Differential Output Delay | tDD | RL = 60Ω, Figure 7 | 1 22 35 | ns |
| Driver Differential Output Transition Time | tTD | RL = 60Ω, Figure 7 | 3 8 25 | ns |
| Driver Propagation Delay, Low-to-High Level | tPLH | RL = 27Ω, Figure 8 | 7 22 35 | ns |
| Driver Propagation Delay, High-to-Low Level | tPHL | RL = 27Ω, Figure 8 | 7 22 35 | ns |
| ¦tPLH - tPHL¦ Driver Propagation Delay Skew (Note 2) | tPDS | RL = 27Ω, Figure 8 | 8 | ns |
| DRIVER OUTPUT ENABLE/DISABLE TIMES (MAX3485 | /MAX3491 o | nly) |  |  |
| Driver Output Enable Time to Low Level | tPZL | RL = 110Ω, Figure 10 | 45 90 | ns |
| Driver Output Enable Time to High Level | tPZH | RL = 110Ω, Figure 9 | 45 90 | ns |
| Driver Output Disable Time from High Level | tPHZ | RL = 110Ω, Figure 9 | 40 80 | ns |
| Driver Output Disable Time from Low Level | tPLZ | RL = 110Ω, Figure 10 | 40 80 | ns |
| Driver Output Enable Time from Shutdown to Low Level | tPSL | RL = 110Ω, Figure 10 | 650 900 | ns |
| Driver Output Enable Time from Shutdown to High Level | tPSH | RL = 110Ω, Figure 9 | 650 900 | ns |

DRIVER SWITCHING CHARACTERISTICS—MAX3486

(VCC = 3.3V, TA = +25°C)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Driver Differential Output Delay | tDD | RL = 60Ω, Figure 7 | 24 48 70 | ns |
| Driver Differential Output Transition Time | tTD | RL = 60Ω, Figure 7 | 15 35 60 | ns |
| Driver Propagation Delay, Low-to-High Level | tPLH | RL = 27Ω, Figure 8 | 20 48 70 | ns |
| Driver Propagation Delay, High-to-Low Level | tPHL | RL = 27Ω, Figure 8 | 20 48 70 | ns |
| ¦tPLH - tPHL¦ Driver Propagation Delay Skew (Note 2) | tPDS | RL = 27Ω, Figure 8 | 11 | ns |
| Driver Output Enable Time to Low Level | tPZL | RL = 110Ω, Figure 10 | 55 100 | ns |
| Driver Output Enable Time to High Level | tPZH | RL = 110Ω, Figure 9 | 55 100 | ns |
| Driver Output Disable Time from High Level | tPHZ | RL = 110Ω, Figure 9 | 45 80 | ns |
| Driver Output Disable Time from Low Level | tPLZ | RL = 110Ω, Figure 10 | 45 80 | ns |
| Driver Output Enable Time from Shutdown to Low Level | tPSL | RL = 110Ω, Figure 10 | 700 1000 | ns |
| Driver Output Enable Time from Shutdown to High Level | tPSH | RL = 110Ω, Figure 9 | 700 1000 | ns |

MAX3483/MAX3485/MAX3486/MAX3488/MAX3490/MAX3491

DRIVER SWITCHING CHARACTERISTICS—MAX3483 and MAX3488

(VCC = 3.3V, TA = +25°C)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Driver Differential Output Delay | tDD | RL = 60Ω, Figure 7 | 600 900 1400 | ns |
| Driver Differential Output Transition Time | tTD | RL = 60Ω, Figure 7 | 400 700 1200 | ns |
| Driver Propagation Delay, Low-to-High Level | tPLH | RL = 27Ω, Figure 8 | 700 1000 1500 | ns |
| Driver Propagation Delay, High-to-Low Level | tPHL | RL = 27Ω, Figure 8 | 700 1000 1500 | ns |
| ¦tPLH - tPHL¦ Driver Propagation Delay Skew (Note 2) | tPDS | RL = 27Ω, Figure 8 | 100 | ns |
| DRIVER OUTPUT ENABLE/DISABLE TIMES (MAX3483 | only) |  |  |  |
| Driver Output Enable Time to Low Level | tPZL | RL = 110Ω, Figure 10 | 900 1300 | ns |
| Driver Output Enable Time to High Level | tPZH | RL = 110Ω, Figure 9 | 600 800 | ns |
| Driver Output Disable Time from High Level | tPHZ | RL = 110Ω, Figure 9 | 50 80 | ns |
| Driver Output Disable Time from Low Level | tPLZ | RL = 110Ω, Figure 10 | 50 80 | ns |
| Driver Output Enable Time from Shutdown to Low Level | tPSL | RL = 110Ω, Figure 10 | 1.9 2.7 | µs |
| Driver Output Enable Time from Shutdown to High Level | tPSH | RL = 110Ω, Figure 9 | 2.2 3.0 | µs |

RECEIVER SWITCHING CHARACTERISTICS

(VCC = 3.3V, TA = +25°C)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Time to Shutdown | tSHDN | MAX3483/MAX3485/MAX3486/MAX3491 only (Note 3) | 80 190 300 | ns |
| Receiver Propagation Delay, Low-to-High Level | tRPLH | VID = 0V to 3.0V, CL = 15pF, Figure 11 | 25 65 90 | ns |
|  |  | MAX3483/MAX3488 | 25 75 120 |  |
| Receiver Propagation Delay, High-to-Low Level | tRPHL | VID = 0V to 3.0V, CL = 15pF, Figure 11 | 25 65 90 | ns |
|  |  | MAX3483/MAX3488 | 25 75 120 |  |
| ¦tPLH - tPHL¦ Receiver Propagation Delay Skew | tRPDS | VID = 0V to 3.0V, CL = 15pF, Figure 11 | 10 | ns |
|  |  | MAX3483/MAX3488 | 20 |  |
| Receiver Output Enable Time to Low Level | tPRZL | CL = 15pF, Figure 12, MAX3483/MAX3485/MAX3486/MAX3491 only | 25 50 | ns |
| Receiver Output Enable Time to High Level | tPRZH | CL = 15pF, Figure 12, MAX3483/MAX3485/MAX3486/MAX3491 only | 25 50 | ns |
| Receiver Output Disable Time from High Level | tPRHZ | CL = 15pF, Figure 12, MAX3483/MAX3485/MAX3486/MAX3491 only | 25 45 | ns |
| Receiver Output Disable Time from Low Level | tPRLZ | CL = 15pF, Figure 12, MAX3483/MAX3485/MAX3486/MAX3491 only | 25 45 | ns |
| Receiver Output Enable Time from Shutdown to Low Level | tPRSL | CL = 15pF, Figure 12, MAX3483/MAX3485/MAX3486/MAX3491 only | 720 1400 | ns |
| Receiver Output Enable Time from Shutdown to High Level | tPRSH | CL = 15pF, Figure 12, MAX3483/MAX3485/MAX3486/MAX3491 only | 720 1400 | ns |

Note 1: VOD and VOC are the changes in VOD and VOC, respectively, when the DI input changes state.

Note 2: Measured on |tPLH (Y) - tPHL (Y)| and |tPLH (Z) - tPHL (Z)|.

Note 3: The transceivers are put into shutdown by bringing RE high and DE low. If the inputs are in this state for less than 80ns, the parts are guaranteed not to enter shutdown. If the inputs are in this state for at least 300ns, the parts are guaranteed to have entered shutdown. See Low-Power Shutdown Mode section.

Typical Operating Characteristics

(VCC = 3.3V, TA = +25°C, unless otherwise noted.)

MAX3483-01

OUTPUT CURRENT vs. RECEIVER OUTPUT LOW VOLTAGE

25

OUTPUT CURRENT (mA)

| 20 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

15

10

5

0

MAX3483-02

| -20 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

-18

OUTPUT CURRENT (mA)

-16

-14

-12

-10

-8

-6

-4

-2

0

OUTPUT CURRENT vs. RECEIVER OUTPUT HIGH VOLTAGE

| 3.30 |  |  |  |  | IRO = 1 | .5mA |
| --- | --- | --- | --- | --- | --- | --- |

OUTPUT HIGH VOLTAGE (V)

3.25

3.20

3.15

3.10

3.05

3.00

RECEIVER OUTPUT HIGH VOLTAGE

MAX3483-03

vs. TEMPERATURE

0	0.5    1.0    1.5    2.0    2.5

OUTPUT LOW VOLTAGE (V)

3.0

3.5

0	0.5    1.0    1.5    2.0    2.5

OUTPUT HIGH VOLTAGE (V)

3.0

3.5

-40    -20	0	20	40	60

MAX3483-06

TEMPERATURE (°C)

80    100

0.8

0.7

0.6

0.5

0.4

0.3

OUTPUT LOW VOLTAGE (V)

0.2

0.1

0

RECEIVER OUTPUT LOW VOLTAGE

MAX3483-04

vs. TEMPERATURE

| 100 |  |  |  | IR | O = 2. | 5mA |
| --- | --- | --- | --- | --- | --- | --- |

90

OUTPUT CURRENT (mA)

80

70

60

50

40

30

20

10

MAX3483-07

0

DRIVER OUTPUT CURRENT vs. DIFFERENTIAL OUTPUT VOLTAGE

MAX3483-05

| 2.6 |  |  |  |  | R = 5 | 4 |
| --- | --- | --- | --- | --- | --- | --- |

DIFFERENTIAL OUTPUT VOLTAGE (V)

2.5

2.4

2.3

2.2

2.1

2.0

1.9

1.8

1.7

MAX3483-08

1.6

DRIVER DIFFERENTIAL OUTPUT VOLTAGE vs.TEMPERATURE

-40    -20	0	20	40	60

80    100

0	0.5    1.0    1.5    2.0    2.5

3.0

3.5

-40    -20	0	20	40	60

80    100

TEMPERATURE (°C)

DIFFERENTIAL OUTPUT VOLTAGE (V)

TEMPERATURE (°C)

| 175 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

150

125

100

75

OUTPUT CURRENT (mA)

50

25

OUTPUT CURRENT vs. DRIVER OUTPUT LOW VOLTAGE

-100

OUTPUT CURRENT (mA)

-80

-60

-40

-20

OUTPUT CURRENT vs. DRIVER OUTPUT HIGH VOLTAGE

0

0	2	4	6	8	10	12

0

-7  -6  -5  -4 -3  -2  -1

| 0 1 2 3 4 5 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

MAX3483/MAX3485/MAX3486/MAX3488/MAX3490/MAX3491

OUTPUT LOW VOLTAGE (V)

OUTPUT HIGH VOLTAGE (V)

Typical Operating Characteristics (continued)

1.2

SUPPLY CURRENT (mA)

1.1

1.0

0.9

0.8

0.7

SUPPLY CURRENT

vs. TEMPERATURE

MAX3483-09

X = DON’T CARE

DE = VCC, RE = X DE = 0, RE = 0

| 100 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

90

SHUTDOWN CURRENT (nA)

80

70

60

50

40

30

20

10

0

SHUTDOWN CURRENT

MAX3483-10

vs. TEMPERATURE

-40    -20	0	20	40	60

TEMPERATURE (°C)

NAME

80    100

-40    -20	0	20	40	60

TEMPERATURE (°C)

80    100

MAX3483/MAX3485/MAX3486/MAX3488/MAX3490/MAX3491

Pin  Description

| PIN |  |  | NAME | FUNCTION |
| --- | --- | --- | --- | --- |
| MAX3483/ MAX3485/ MAX3486 | MAX3488/ MAX3490 | MAX3491 |  |  |
| 1 | 2 | 2 | RO | Receiver Output. If A > B by 200mV, RO will be high; if A < B by 200mV, RO will be low. |
| 2 | — | 3 | – RE | Receiver Output Enable. RO is enabled when RE is low; RO is high imped- ance when RE is high. If RE is high and DE is low, the device will enter a low-power shutdown mode. |
| 3 | — | 4 | DE | Driver Output Enable. The driver outputs are enabled by bringing DE high. They are high impedance when DE is low. If RE is high and DE is low, the device will enter a low-power shutdown mode. If the driver outputs are enabled, the parts function as line drivers. While they are high impedance, they function as line receivers if RE is low. |
| 4 | 3 | 5 | DI | Driver Input. A low on DI forces output Y low and output Z high. Similarly, a high on DI forces output Y high and output Z low. |
| 5 | 4 | 6, 7 | GND | Ground |
| — | 5 | 9 | Y | Noninverting Driver Output |
| — | 6 | 10 | Z | Inverting Driver Output |
| 6 | — | — | A | Noninverting Receiver Input and Noninverting Driver Output |
| — | 8 | 12 | A | Noninverting Receiver Input |
| 7 | — | — | B | Inverting Receiver Input and Inverting Driver Output |
| — | 7 | 11 | B | Inverting Receiver Input |
| 8 | 1 | 13, 14 | VCC | Positive Supply: 3.0V ≤ VCC ≤ 3.6V |
| — | — | 1, 8 | N.C. | No Connect—not internally connected |

TOP VIEW

RO     1	R RE     2

DE    3

DI    4	D

8   VCC

7    B

6    A

5   GND

RO     1	R RE     2

DE    3

DI    4	D

8   VCC

7  B

Rt

6

A

5   GND

MAX3483 	DE

MAX3485

MAX3486	D	DI

B

Rt

A

R	RO

DIP/SO 	RE

NOTE: PIN LABELS Y AND Z ON TIMING, TEST, AND WAVEFORM DIAGRAMS REFER TO PINS A AND B WHEN DE IS HIGH.

Figure 1. MAX3483/MAX3485/MAX3486 Pin Configuration and Typical Operating Circuit

TOP VIEW

VCC

1

MAX3488

MAX3490

VCC

VCC     1	R	8   A RO    2		7   B

5   Y

3

DI 	D 	6   Z

Rt 	R	RO

DI   3

GND  4

6   Z D	5   Y

DIP/SO

8   A

2 	Rt

RO	R	7

B

4

GND

D	DI

GND

Figure 2. MAX3488/MAX3490 Pin Configuration and Typical Operating Circuit

TOP VIEW

DE	VCC

VCC    RE

4	13, 14

MAX3491

N.C.   1

RO     2	R RE     3

DE     4

DI   5

14   V CC

13   V CC

12  A

11  B

10  Z

5

DI 	D

2

9 	Y

10

Z

12     A

Rt

Rt 	R	RO

D GND    6

GND  7

9   Y

8    N.C.

RO

N.C.

1, 8

R	11

B

D	DI

DIP/SO

3	6, 7

RE  GND

GND 	DE

MAX3483/MAX3485/MAX3486/MAX3488/MAX3490/MAX3491

Figure 3. MAX3491 Pin Configuration and Typical Operating Circuit

VCC

D	VOD

RL

2

RL

2	VOC

VCC

D 	VOD	RL

375

375

VCM =

-7V to +12V

Figure 4. Driver VOD and VOC

Figure 5. Driver VOD with Varying Common-Mode Voltage

VID 	R

0V

VOL

IOL

(+)

VOH	IOH

(-)

Figure 6. Receiver VOH and VOL

3V

IN	1.5V CL

1.5V

0V

GENERATOR (NOTE 4)

50

VCC

60

CL

CL = 15pF (NOTE 5)

OUT

50%

10%

tTD

90%

90%

50%

10%

tTD

2.0V

-2.0V

MAX3483/MAX3485/MAX3486/MAX3488/MAX3490/MAX3491

Figure 7. Driver Differential Output Delay and Transition Times

GENERATOR

VOM

| S1 |  |  |
| --- | --- | --- |

RL = 27

IN	1.5V

tPLH

1.5V

tPHL

3V

0V VOH

(NOTE 4)

50

VCC

VOM =

VOH + VOL

2

1.5V

CL = 15pF (NOTE 5)

Y OUT

Z OUT

VOM

tPHL 	tPLH

VOM

VOM

VOM

VOL

VOH

VOL

Figure 8. Driver Propagation Times

0V OR 3V

GENERATOR (NOTE 4)

S1

D

CL = 50pF (NOTE 5)

50

VOM =

VOH + VOL

2

OUT RL = 110

1.5V

IN	1.5V

tPZH

OUT

VOM

1.5V

tPHZ

0.25V

3V

0V VOH

0V

Figure 9. Driver Enable and Disable Times (tPZH, tPSH, tPHZ)

0V OR 3V

GENERATOR (NOTE 4)

S1

D

CL = 50pF

(NOTE 5)

50

VCC

RL = 110

OUT

IN	1.5V

tPSL

OUT

VOM

1.5V

tPLZ

0.25V

3V

0V VCC

VOL

MAX3483/MAX3485/MAX3486/MAX3488/MAX3490/MAX3491

Figure 10. Driver Enable and Disable Times (tPZL, tPSL, tPLZ)

GENERATOR

VID 	R

OUT

IN	1.5V

1.5V

3.0V

(NOTE 4)

50 	CL = 15pF (NOTE 5)

tRPLH

0V

tRPHL

1.5V

0V

VOM =

VCC

2

OUT

VOM 	VOM

VCC

0V

Figure 11. Receiver Propagation Delay

1.5V

-1.5V

S3

VID 	R

S1

1k 	VCC

S2

GENERATOR

(NOTE 4)	50

CL

(NOTE 5)

IN	1.5V

tPRZH

tPRSH

3V

0V VOH

S1 OPEN

S2 CLOSED S3 = 1.5V

IN	1.5V

tPRZL

tPRSL

3V

0V VCC

S1 CLOSED S2 OPEN

S3 = -1.5V

OUT

1.5V

0V

OUT

1.5V

VOL

IN

OUT

0.25V

1.5V

tPRHZ

3V

0V VOH

S1 OPEN

S2 CLOSED S3 = 1.5V

IN	1.5V

tPRLZ

3V

0V VCC

S1 CLOSED S2 OPEN

S3 = -1.5V

OUT

0.25V

MAX3483/MAX3485/MAX3486/MAX3488/MAX3490/MAX3491

0V	VOL

Figure 12. Receiver Enable and Disable Times

Note 4: The input pulse is supplied by a generator with the following characteristics:  PRR = 250kHz, 50% duty cycle, tr  6.0ns, ZO = 50.

Note 5: CL includes probe and stray capacitance.

Function Tables

Devices with Receiver/Driver Enable

(MAX3483/MAX3485/MAX3486/MAX3491)

Table 1. Transmitting

| INPUTS |  |  | OUTPUTS |  | MODE |
| --- | --- | --- | --- | --- | --- |
| RE | DE | DI | B* | A* |  |
| X | 1 | 1 | 0 | 1 | Normal |
| X | 1 | 0 | 1 | 0 | Normal |
| 0 | 0 | X | High-Z | High-Z | Normal |
| 1 | 0 | X | High-Z | High-Z | Shutdown |

* B and A outputs are Z and Y, respectively, for full-duplex part

(MAX3491).

X = Don’t care; High-Z = High impedance

Table 2. Receiving

| INPUTS |  |  | OUTPUTS | MODE |
| --- | --- | --- | --- | --- |
| RE | DE | A, B | RO |  |
| 0 | 0* | ≥ +0.2V | 1 | Normal |
| 0 | 0* | ≤ -0.2V | 0 | Normal |
| 0 | 0* | Inputs Open | 1 | Normal |
| 1 | 0 | X | High-Z | Shutdown |

* DE is a “don’t care” (x) for the full-duplex part (MAX3491). X = Don’t care; High-Z = High impedance

Devices without Receiver/Driver Enable

(MAX3488/MAX3490)

| Table 3. Transmitting Table 4. Receiving INPUT | OUTPUTS |  |
| --- | --- | --- |
| DI | Z | Y |
| 1 | 0 | 1 |
| 0 | 1 | 0 |

| INPUTS | OUTPUT |
| --- | --- |
| A, B | RO |
| ≥ +0.2V | 1 |
| ≤ -0.2V | 0 |
| Inputs Open | 1 |

Applications Information

The MAX3483/MAX3485/MAX3486/MAX3488/MAX3490/ MAX3491 are low-power transceivers for RS-485 and RS-422 communications.   The MAX3483 and MAX3488 can transmit and receive at data rates up to 250kbps, the MAX3486 at up to 2.5Mbps,  and the MAX3485/ MAX3490/MAX3491 at up to 10Mbps. The MAX3488/ MAX3490/MAX3491 are full-duplex transceivers, while

the MAX3483/MAX3485/MAX3486 are half-duplex. Driver Enable (DE) and Receiver Enable (RE) pins are

included on the MAX3483/MAX3485/MAX3486/ MAX3491.  When disabled,  the driver and receiver out- puts are high impedance.

Reduced EMI  and Reflections

(MAX3483/MAX3486/MAX3488) The MAX3483/MAX3488 are slew-rate limited, minimiz- ing EMI and reducing reflections caused by improperly terminated cables.   Figure 13 shows both the driver output waveform of a MAX3485/MAX3490/MAX3491 transmitting a 125kHz signal and the Fourier analysis of that waveform.   High-frequency  harmonics with large amplitudes are evident.  Figure 14 shows the same infor- mation, but for the slew-rate-limited MAX3483/MAX3488 transmitting the same signal.   The high-frequency  har- monics have much lower amplitudes, and the potential for EMI is significantly reduced.

Low-Power Shutdown Mode

(MAX3483/MAX3485/MAX3486/MAX3491) A low-power shutdown mode is initiated by bringing both RE high and DE low.   The devices will not shut down unless both the driver and receiver are disabled  (high impedance).   In shutdown, the devices typically  draw only 2nA of supply current.

For these devices,  the tPSH and tPSL enable times assume the part was in the low-power shutdown mode; the tPZH and tPZL enable times assume the receiver or driver was disabled, but the part was not shut down.

10dB/div

10dB/div

0Hz

500kHz/div

5MHz

0Hz

500kHz/div

5MHz

MAX3483/MAX3485/MAX3486/MAX3488/MAX3490/MAX3491

Figure 13. Driver Output Waveform and FFT Plot of MAX3485/ MAX3490/MAX3491 Transmitting a 125kHz Signal

Figure 14. Driver Output Waveform and FFT Plot of MAX3483/ MAX3488 Transmitting a 125kHz Signal

DI

2V/div

Z

1V/div

Y

1V/div

B

1V/div

A

1V/div

RO

2V/div

20ns/div

20ns/div

Figure 15. MAX3485/MAX3490/MAX3491 Driver Propagation

Delay

Figure 16. MAX3485/MAX3490/MAX3491 Receiver

Propagation Delay Driven by External RS-485 Device

DI

2V/div

Z

1V/div

B

1V/div

A

1V/div

Y

1V/div

1 s/div

RO

2V/div

1 s/div

Figure 17. MAX3483/MAX3488 Driver Propagation Delay

Figure 18. MAX3483/MAX3488 Receiver Propagation Delay

DI

5V/div

VY - VZ

2V/div

RO

5V/div

DI

5V/div

VY - VZ

2V/div

RO

5V/div

2 s/div

2 s/div

MAX3483/MAX3485/MAX3486/MAX3488/MAX3490/MAX3491

Figure 19. MAX3483/MAX3488 System Differential Voltage at

125kHz Driving 4000 ft of Cable

Figure 20. MAX3485/MAX3490/MAX3491 System Differential

Voltage at 125kHz Driving 4000 ft of Cable

DI 	B

D

120	120

DE B

D

DI

DE	A	B	A

B	A	A

RO	R

RE

MAX3483

MAX3485

MAX3486

R	RO RE

R		R D	D

DI	DE

RO  RE

DI 	DE

RO  RE

Figure 21. MAX3483/MAX3485/MAX3486 Typical RS-485 Network

MAX3483/MAX3485/MAX3486/MAX3488/MAX3490/MAX3491

Driver Output Protection Excessive output current and power dissipation caused by faults or by bus contention are prevented  by two mechanisms. A foldback current limit on the output stage provides immediate protection against short circuits over the whole common-mode  voltage range (see Typical Operating Characteristics). In addition,  a thermal shut- down circuit forces the driver outputs into a high-imped- ance state if the die temperature rises excessively.

Propagation Delay Figures 15–18 show the typical propagation delays. Skew time is simply the difference between the low-to-high and high-to-low propagation  delay. Small driver/receiver skew times help maintain a symmetrical mark-space ratio (50% duty cycle).

The receiver skew time, |tPRLH - tPRHL|, is under 10ns

(20ns for the MAX3483/MAX3488). The driver skew

times are 8ns for the MAX3485/MAX3490/MAX3491,

11ns for the MAX3486, and typically  under 100ns for the MAX3483/MAX3488.

Line Length vs.  Data Rate

The RS-485/RS-422 standard covers line lengths up to

4000 feet.  For line lengths greater than 4000 feet, see

Figure 23.

Figures 19 and 20 show the system differential voltage for parts driving 4000 feet of 26AWG twisted-pair  wire at 125kHz into 120 loads.

Typical Applications The MAX3483, MAX3485, MAX3486, MAX3488, MAX3490, and MAX3491 transceivers are designed for bidirectional  data communications  on multipoint bus transmission lines.  Figures 21 and 22 show typical net- work applications  circuits.   These parts can also be used as line repeaters, with cable lengths longer than

4000 feet, as shown in Figure 23.

To minimize reflections, the line should be terminated at both ends in its characteristic  impedance,  and stub lengths off the main line should be kept as short as possible.   The slew-rate-limited MAX3483/MAX3488 and the partially slew-rate-limited MAX3486 are more tolerant of imperfect termination.

RO	R

RE DE

DI	D

A	Y

120 	120 		D	DI B 		Z

DE Z	B

RE

120 	120 	R	RO

Y	A

Y	Z	B	A

Y	Z	B	A

R	R

D	D	MAX3488

MAX3490

MAX3491

DI	DE

RE RO

DI 	DE  RE RO

NOTE:  RE AND DE ON MAX3491 ONLY.

Figure 22. MAX3488/MAX3490/MAX3491 Full-Duplex RS-485 Network

RO	R

RE

DE

MAX3488

MAX3490

A	MAX3491

120 	DATA IN

B

Z

DI	D

120

Y

DATA OUT

MAX3483/MAX3485/MAX3486/MAX3488/MAX3490/MAX3491

NOTE:  RE AND DE ON MAX3491 ONLY.

Figure 23. Line Repeater for MAX3488/MAX3490/MAX3491

_Ordering Information (continued) PART TEMP. RANGE PIN-PACKAGE

MAX3486CPA 0°C to +70°C 8 Plastic DIP

MAX3486CSA 0°C to +70°C 8 SO

MAX3486C/D 0°C to +70°C Dice*

MAX3486EPA -40°C to +85°C 8 Plastic DIP

MAX3486ESA -40°C to +85°C 8 SO

MAX3488CPA 0°C to +70°C 8 Plastic DIP

MAX3488CSA 0°C to +70°C 8 SO

MAX3488C/D 0°C to +70°C Dice*

MAX3488EPA -40°C to +85°C 8 Plastic DIP

MAX3488ESA -40°C to +85°C 8 SO

MAX3490CPA 0°C to +70°C 8 Plastic DIP

MAX3490CSA 0°C to +70°C 8 SO

MAX3490C/D 0°C to +70°C Dice*

MAX3490EPA -40°C to +85°C 8 Plastic DIP

MAX3490ESA -40°C to +85°C 8 SO

MAX3491CPD 0°C to +70°C 14 Plastic DIP

MAX3491CSD 0°C to +70°C 14 SO

MAX3491C/D 0°C to +70°C Dice*

MAX3491EPD -40°C to +85°C 14 Plastic DIP

MAX3491ESD -40°C to +85°C 14 SO

Chip Topography

V CC

RO

A RE

B

Z/B

0.146"

(3.71mm)

* Contact factory for for dice specifications.

DE DI

GND

0.086"

(2.18mm)

GND

Y/A

MAX3483/MAX3485/MAX3486/MAX3488/MAX3490/MAX3491

TRANSISTOR COUNT: 810

SUBSTRATE CONNECTED TO GROUND

Package Information

DIM

INCHES	MILLIMETERS

D

0°-8° A

MIN

A	0.053

A1	0.004

B	0.014

C	0.007

E	0.150

MAX

0.069

0.010

0.019

0.010

0.157

MIN

1.35

0.10

0.35

0.19

3.80

MAX

1.75

0.25

0.49

0.25

4.00

0.101mm

0.004in.

e	0.050

1.27

e 	A1

B

H

C 	L 	L

0.228

0.016

0.244

0.050

5.80

0.40

6.20

1.27

INCHES	MILLIMETERS

SO

E 	H 	SMALL OUTLINE PACKAGE

(0.150 in.)

DIM

D D D

PINS

8

14

16

MIN

0.189

0.337

0.386

MAX

0.197

0.344

0.394

MIN

4.80

8.55

9.80

MAX

5.00

8.75

10.00

21-0041A

MAX3483/MAX3485/MAX3486/MAX3488/MAX3490/MAX3491

| 0° - 15° E D E1 A3 A A2 L A1 e B1 eA B eB D1 | DIM | INCHES |  |  |  | MILLIMETERS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MIN |  | MAX |  | MIN |  | MAX |  |
|  | A | – |  | 0.200 |  | – |  | 5.08 |  |
|  | A1 | 0.015 |  | – |  | 0.38 |  | – |  |
|  | A2 | 0.125 |  | 0.175 |  | 3.18 |  | 4.45 |  |
|  | A3 | 0.055 |  | 0.080 |  | 1.40 |  | 2.03 |  |
|  | B | 0.016 |  | 0.022 |  | 0.41 |  | 0.56 |  |
|  | B1 | 0.045 |  | 0.065 |  | 1.14 |  | 1.65 |  |
|  | C | 0.008 |  | 0.012 |  | 0.20 |  | 0.30 |  |
|  | D1 | 0.005 |  | 0.080 |  | 0.13 |  | 2.03 |  |
|  | E | 0.300 |  | 0.325 |  | 7.62 |  | 8.26 |  |
|  | E1 | 0.240 |  | 0.310 |  | 6.10 |  | 7.87 |  |
|  | e | 0.100 |  | – |  | 2.54 |  | – |  |
|  | eA | 0.300 |  | – |  | 7.62 |  | – |  |
|  | eB | – |  | 0.400 |  | – |  | 10.16 |  |
|  | L | 0.115 |  | 0.150 |  | 2.92 |  | 3.81 |  |
| Plastic DIP PLASTIC DUAL-IN-LINE PACKAGE (0.300 in.) | DIM | PINS | INCHES |  |  |  | MILLIMETERS |  |  |
|  |  |  | MIN |  | MAX |  | MIN |  | MAX |
|  | D | 8 | 0.348 |  | 0.390 |  | 8.84 |  | 9.91 |
|  | D | 14 | 0.735 |  | 0.765 |  | 18.67 |  | 19.43 |
|  | D | 16 | 0.745 |  | 0.765 |  | 18.92 |  | 19.43 |
|  | D | 18 | 0.885 |  | 0.915 |  | 22.48 |  | 23.24 |
|  | D | 20 | 1.015 |  | 1.045 |  | 25.78 |  | 26.54 |
|  | D | 24 | 1.14 |  | 1.265 |  | 28.96 |  | 32.13 |

| C |  |  |  |
| --- | --- | --- | --- |

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses  are implied. Maxim reserves the right to change the circuitry and specifications  without notice at any time.

16   	Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA  94086 (408) 737-7600

©  1994 Maxim Integrated Products	Printed USA	  is a registered trademark of Maxim Integrated Products.