19-0389; Rev. 2; 9/96

Fault-Protected Analog Multiplexers

General Description

The MAX354/MAX355 fault-protected  multiplexers (muxes) use a series N-channel, P-channel, N-channel structure that protects the devices from overvoltage up to 40V beyond the supply rails during power-up, power- down, and fault conditions. The MAX354/MAX355 also protect  sensitive circuit  components  against voltages near or beyond the normal supplies.

The MAX354 single 8-channel mux and the MAX355 dual 4-channel mux protect analog signals while oper- ating from a single 4.5V to 36V supply or ±4.5V to ±18V dual supplies. These muxes have 350 on-resistance and can be used for demultiplexing  as well as multi- plexing.  Input leakage current is less than 0.5nA at

+25°C and less than 5nA at +85°C.

All digital inputs have 0.8V and 2.4V logic thresholds, ensuring both TTL and CMOS logic compatibility  with- out pull-up  resistors. Break-before-make operation is guaranteed  and  power  consumption  is less than

1.5mW.

Applications

Data-Acquisition Systems Industrial and Process Control Avionics

ATE Equipment Signal Routing Redundant/Backup Systems

Features

♦ 350 Max On-Resistance

♦ Improved 2nd Source for MAX358/MAX359 and

DG458/DG459

♦ Pin Compatible with ADG508F/ADG509F

♦ All Switches Off with Supplies Off

♦ On Switch Turns Off with Overvoltage

♦ Output Clamps at 1.5V Below Supply Rails

♦ 0.5nA Max Input Leakage at +25°C (5nA at +85°C)

♦ No Power-Up Sequencing Required

♦ TTL and CMOS-Logic Compatibility

Ordering Information

PART TEMP. RANGE PIN-PACKAGE

MAX354CPE 0°C to +70°C 16 Plastic DIP

MAX354CWE 0°C to +70°C 16 Wide SO

MAX354C/D 0°C to +70°C Dice*

MAX354EPE -40°C to +85°C 16 Plastic DIP

MAX354EWE -40°C to +85°C 16 Wide SO

MAX354MJE -55°C to +125°C 16 CERDIP**

MAX355CPE 0°C to +70°C 16 Plastic DIP

MAX355CWE 0°C to +70°C 16 Wide SO

MAX355C/D 0°C to +70°C Dice*

MAX355EPE -40°C to +85°C 16 Plastic DIP

MAX355EWE -40°C to +85°C 16 Wide SO

MAX355MJE -55°C to +125°C 16 CERDIP**

*  Dice are tested at TA = +25°C only.

** Contact factory for availability.

MAX354/MAX355

Pin  Configurations

|  |  |  |  |  |  | TOP VIEW |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MAX354 |  |  |  |  |  |  | MAX355 |  |  |
| A0 | 1 |  |  | 16 | A1 |  | A0 | 1 |  | 16 | A1 |
| EN | 2 |  |  | 15 | A2 |  | EN | 2 |  | 15 | GND |
| V- | 3 | LOGIC |  | 14 | GND |  | V- | 3 | LOGIC | 14 | V+ |
| NO1 | 4 |  |  | 13 | V+ |  | NO1A | 4 |  | 13 | NO1B |
| NO2 | 5 |  |  | 12 | NO5 |  | NO2A | 5 |  | 12 | NO2B |
| NO3 | 6 |  |  | 11 | NO6 |  | NO3A | 6 |  | 11 | NO3B |
| NO4 | 7 |  |  | 10 | NO7 |  | NO4A | 7 |  | 10 | NO4B |
| COM | 8 |  |  | 9 | NO8 |  | COMA | 8 |  | 9 | COMB |
|  |  | DIP/SO |  |  |  |  |  |  | DIP/SO |  |  |

ABSOLUTE MAXIMUM RATINGS

(Voltages referenced to GND, unless otherwise noted.)

V+ ...........................................................................-0.3V to +44V V- ............................................................................+0.3V to -44V V+ to V-...................................................................-0.3V to +44V Digital Inputs.........................................(V+ + 0.3V) to (V- - 0.3V) Input Overvoltage  with Mux Power On

V+ = +15V ....................................................................... +25V V- = -15V ............................................................................-25V

Input Overvoltage  with Mux Power Off

V+ = 0V .............................................................................+40V V- = 0V ...............................................................................-40V Continuous Current into Any Terminal .............................±30mA

Peak Current into Any Terminal ........................................±50mA Continuous Power Dissipation  (TA = +70°C)

Plastic DIP (derate 10.53mW/°C above +70°C) ...........842mW

Wide SO (derate 9.52mW/°C above +70°C)................ 762mW CERDIP (derate 10.00mW/°C above +70°C)  ...............800mW

Operating  Temperature  Ranges

MAX35_C_ _ ........................................................0°C to +70°C MAX35_E_ _......................................................-40°C to +85°C MAX35_M_ _ ...................................................-55°C to +125°C Storage Temperature  Range .............................-65°C to +150°C Lead Temperature  (soldering,  10sec) ............................ +300°C

MAX354/MAX355

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions  beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS

±

±

±

(V+ = +15V, V- = -15V, GND = 0V, VAH = VENH = 2.4V, VAL = VENL = 0.8V, TA = TMIN to TMAX, unless otherwise noted.)

| PARAMETER | SYMBOL | CONDITIONS |  |  |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SWITCH |  |  |  |  |  |  |  |
| Analog Signal Range | VCOM, VNO | (Note 1) |  |  |  | (V+ - 40) (V- + 40) | V |
| Fault-Free Analog Signal Range |  | V+ = +15V, V- = -15V (Note 1) |  |  |  | -12 12 | V |
| On-Resistance (Note 2) | RON | INO = 1.0mA, VCOM = ±10V |  | TA = +25°C |  | 285 350 | Ω |
|  |  |  |  | TA = TMIN to TMAX | C, E | 450 |  |
|  |  |  |  |  | M | 500 |  |
| On-Resistance Matching Between Channels | ∆RON | INO = 1.0mA, VCOM = ±10V (Note 3) |  | TA = +25°C |  | 7 12 | Ω |
|  |  |  |  | TA = TMIN to TMAX |  | 15 |  |
| NO-Off Leakage Current (Note 4) | INO(OFF) | VCOM = 10V, VNO = ±10V, VEN = 0V |  | TA = +25°C |  | -0.5 0.01 0.5 | nA |
|  |  |  |  | TA = TMIN to TMAX | C, E | -5.0 5.0 |  |
|  |  |  |  |  | M | -50 50 |  |
| COM-Off Leakage Current (Note 4) | ICOM(OFF) | VCOM = 10V, VNO = ±10V, VEN = 0V | MAX354 | TA = +25°C |  | -0.5 0.02 0.5 | nA |
|  |  |  |  | TA = TMIN to TMAX | C, E | -25 25 |  |
|  |  |  |  |  | M | -100 100 |  |
|  |  | VCOM = ±10V, VNO = 10V, VEN = 0V | MAX355 | TA = +25°C |  | -0.5 0.02 0.5 |  |
|  |  |  |  | TA = TMIN to TMAX | C, E | -15 15 |  |
|  |  |  |  |  | M | -50 50 |  |
| COM-On Leakage Current (Note 4) | ICOM(ON) | VCOM = ±10V, VNO = ±10V, sequence each switch on | MAX354 | TA = +25°C |  | -0.5 0.02 0.5 | nA |
|  |  |  |  | TA = TMIN to TMAX | C, E | -30 30 |  |
|  |  |  |  |  | M | -200 200 |  |
|  |  |  | MAX355 | TA = +25°C |  | -0.5 0.02 0.5 |  |
|  |  |  |  | TA = TMIN to TMAX | C, E | -15 15 |  |
|  |  |  |  |  | M | -100 100 |  |

MAX354/MAX355

ELECTRICAL CHARACTERISTICS (continued)

(V+ = +15V, V- = -15V, GND = 0V, VAH = VENH = 2.4V, VAL = VENL = 0.8V, TA = TMIN to TMAX, unless otherwise noted.)

pF

| PARAMETER | SYMBOL | CONDITIONS |  |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- | --- |
| FAULT |  |  |  |  |  |  |
| Output Leakage Current (with Overvoltage) |  | VD = 0V, analog overvoltage = ±33V |  | TA = +25°C | -5 0.01 5 | nA |
|  |  |  |  | TA = TMIN to TMAX | -2 2 | µA |
| Input Leakage Current (with Overvoltage) |  | VIN = ±25V, VO = ±10V |  | TA = +25°C | -0.1 0.001 0.1 | µA |
|  |  |  |  | TA = TMIN to TMAX | -2 2 |  |
| Input Leakage Current (with Power Supplies Off) |  | VIN = ±25V, VEN = VO = 0V, VA0 = VA1 = VA2 = 0V or 5V |  | TA = +25°C | -0.1 0.001 0.1 | µA |
|  |  |  |  | TA = TMIN to TMAX | -2 2 |  |
| DIGITAL LOGIC INPUT |  |  |  |  |  |  |
| Logic High Input Voltage | VA_H, VENH |  |  | TA = TMIN to TMAX | 2.4 | V |
| Logic Low Input Voltage | VA_L, VENL |  |  | TA = TMIN to TMAX | 0.8 | V |
| Input Current with Input Voltage High | IA_H, IENH | VA = VEN = 2.4V |  | TA = +25°C | -1 1 | µA |
|  |  |  |  | TA = TMIN to TMAX | -5 5 |  |
| Input Current with Input Voltage Low | IA_L, IENL | VA = VEN = 0.8V |  | TA = +25°C | -1 1 | µA |
|  |  |  |  | TA = TMIN to TMAX | -5 5 |  |
| SUPPLY |  |  |  |  |  |  |
| Power-Supply Range |  |  |  |  | ±4.5 ±18 | V |
| Positive Supply Current | I+ | VEN = VA = 5V |  | TA = +25°C | -300 300 | µA |
|  |  |  |  | TA = TMIN to TMAX | -500 500 |  |
| Negative Supply Current | I- | VEN = VA = 0V |  | TA = +25°C | -1 1 | µA |
|  |  |  |  | TA = TMIN to TMAX | -100 100 |  |
| DYNAMIC |  |  |  |  |  |  |
| Transition Time | tTRANS | Figure1 |  | TA = +25°C | 180 250 | ns |
|  |  |  |  | TA = TMIN to TMAX | 400 |  |
| Enable Turn-On Time | tON(EN) | Figure 2 |  | TA = +25°C | 160 250 | ns |
|  |  |  |  | TA = TMIN to TMAX | 400 |  |
| Enable Turn-Off Time | tOFF(EN) | Figure 2 |  | TA = +25°C | 80 200 | ns |
|  |  |  |  | TA = TMIN to TMAX | 300 |  |
| Break-Before-Make Interval | tOPEN | Figure 3 |  | TA = +25°C | 50 100 | ns |
| Charge Injection | VCTE | CL = 10nF, VS = 0V, RS = 0Ω, Figure 4 |  | TA = +25°C | 80 | pC |
| Off Isolation | VISO | VEN = 0V, RL = 1kΩ, f = 100kHz, Figure 5 |  | TA = +25°C | 100 | dB |
| Crosstalk Between Channels | VCT | VEN = 2.4V, f = 100kHz, VGEN = 1Vp-p, RL = 1kΩ, Figure 6 |  | TA = +25°C | 92 | dB |
| Logic Input Capacitance | CIN | f = 1MHz, Figure 7 |  | TA = +25°C | 2.5 | pF |
| NO-Off Capacitance | CNO(OFF) | f = 1MHz, VEN = VD = 0V |  | TA = +25°C | 1.6 | pF |

ELECTRICAL CHARACTERISTICS (continued)

(V+ = +15V, V- = -15V, GND = 0V, VAH = VENH = 2.4V, VAL = VENL = 0.8V, TA = TMIN to TMAX, unless otherwise noted.)

| PARAMETER | SYMBOL | CONDITIONS |  |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- | --- |
| DYNAMIC (cont’d) |  |  |  |  |  |  |
| COM-Off Capacitance | CCOM(OFF) | f = 1MHz, Figure 7, VEN = VD = 0V | MAX354 | TA = +25°C | 11 | pF |
|  |  |  | MAX355 |  | 5 |  |
| COM-On Capacitance | CCOM(ON) | f = 1MHz, Figure 7, VEN = VD = 0V | MAX354 | TA = +25°C | 28 | pF |
|  |  |  | MAX355 |  | 14 |  |
| Setting Time (Note 5) | tSETT | 0.1% |  | TA = +25°C | 1 | µs |
|  |  | 0.01% |  |  | 2.5 |  |

Note 1: When the analog signal exceeds  +13.5V or -13.5V, the  blocking  action of Maxim’s gate structure goes into operation.   Only leakage currents flow, and the channel on-resistance rises to infinity (see Typical Operating Characteristics).

Note 2: Electrical characteristics such as on-resistance  will change when power supplies  other than ±15V are used.

Note 3: RON = RON(MAX) - RON(MIN)

Note 4: Leakage parameters  are 100% tested at maximum rated hot operating  temperature, and guaranteed by correlation  at +25°C.

Note 5: Guaranteed  by design.

Typical Operating Characteristics

(TA = +25°C, unless otherwise noted.)

100

MAX354/5-1a

ON-RESISTANCE vs. ANALOG VOLTAGE

2000

ON-RESISTANCE vs. ANALOG VOLTAGE

MAX354/5-1b

700

ON-RESISTANCE vs. VCOM  AND TEMPERATURE

10

RON (M  )

1

0.1

0.01

0.001

0.0001

)

V+ = +5V V- = -5V

V+ = +15V V- = -15V

1800

1600

)

1400

1200

RON (

1000

800

600

400

200

0

V+ = +15V V- = -15V

V+ = +5V V- = -5V

V+ = +10V V- = -10V

600

500

RON (

400

300

200

100

V+ = +15V

V- = -15V

A: +125°C

MAX354/5-2

B: +85°C C: +70°C D: +25°C

A B

C D

-15

-10	-5	0

5	10	15

-15

-10	-5	0

5	10	15

-10	-5	0

5	10

100

OFF LEAKAGE (nA)

10

ON LEAKAGE (nA)

1

ANALOG VOLTAGE (V)

OFF LEAKAGE vs. TEMPERATURE

V+ = +15V V- = - 15V VNO_ = ±10V

VCOM_ =  10V	ICOM(OFF)

MAX354-3

100

10

1

ANALOG VOLTAGE (V)

MAX354-4

ON LEAKAGE vs. TEMPERATURE

200

150

100

Qj (pC)

50

0

-50

VCOM (V)

MAX354-5

CHARGE INJECTION vs. VCOM

V+ = +15V V- = -15V

0.1

0.01

-75  -50   -25    0

25    50

INO(OFF)

75   100  125

0.1

0.01

| -75 -50 -25 0 V+ = +15V V- = -15V VCOM_ = ±10V |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

25    50

75   100  125

-100

-150

-200

-10	-5	0

5	10

MAX354/MAX355

TEMPERATURE (°C)

TEMPERATURE (°C)

VCOM (V)

Typical Operating Characteristics (continued)

| (TA = +25°C, unless otherwise noted.) V+ = +15V V- = -15V VEN = +5V |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | I+ (VA_ = 5V) |  |  |
|  |  |  |  |  | I+ (VA_ = 0V) |  |  |
|  |  |  |  |  | I- |  |  |

1000

100

I+, I- (  A)

10

1

0.1

SUPPLY CURRENT vs. TEMPERATURE

MAX354-6

10m

1m

100

10

1

100m

10n

FAULT CURRENT (A)

1n

100p

10p

FAULT CURRENT vs. FAULT VOLTAGE

V+ = V- = 0V

V+ = +15V V- = -15V

-75  -50   -25    0

25    50

75   100  125

-70

-50

-30

-10	10	30	50	70

TEMPERATURE (°C)

FAULT VOLTAGE (V)

MAX354/MAX355

Pin  Description

| PIN |  | NAME | FUNCTION |
| --- | --- | --- | --- |
| MAX354 | MAX355 |  |  |
| 1, 15, 16 | — | A0, A2, A1 | Address Logic Inputs |
| — | 1, 16 | A0, A1 | Address Logic Inputs |
| 2 | 2 | EN | Enable Logic Input. See truth tables. |
| 3 | 3 | V- | Negative Supply Voltage Input. Connect to GND for single-supply operation. |
| 4–7 | — | NO1–NO4 | Analog Inputs—bidirectional |
| — | 4–7 | NO1A–NO4A | Analog Inputs—bidirectional “A” switch |
| 8 | — | COM | Analog Output—bidirectional |
| — | 8, 9 | COMA, COMB | Analog Outputs—bidirectional |
| 9–12 | — | NO8–NO5 | Analog Inputs—bidirectional |
| — | 10–13 | NO4B–NO1B | Analog Inputs—bidirectional “B” switch |
| 13 | 14 | V+ | Positive Supply Voltage Input |
| 14 | 15 | GND | Ground |

Note:  Analog inputs and outputs are electrically  identical  and completely interchangeable.

Test Circuits/Timing Diagrams

A2

A1

A0

+15V V+

NO1

NO2-NO7

+10V

VEN    EN

50

MAX354

GND	V-

NO8

COM

300

-10V

35pF

VOUT

LOGIC INPUT VEN

+3V

0V

50%

tR < 20ns tF < 20ns

+15V V+

A1

A0

-15V

NO1B

+10V

SWITCH OUTPUT VOUT

VNO1

0V

90%

90%

NO1A-NO4A

VNO8

VEN   EN

50

MAX355

GND

NO4B COMB

V-

-10V

300 	35pF

VOUT

tTRANS ON

tTRANS

-15V

Figure 1. Transition Time

+15V

VEN

V+

EN	NO1

NO2–NO8

+10V

A0

A1	MAX354

A2

50 	GND

COM V-

1k

VOUT

35pF

LOGIC INPUT VEN

+3V

0V

50%

tR < 20ns tF < 20ns

VEN    EN

+15V V+

-15V

NO1B

+10V

SWITCH OUTPUT

VOUT

tON(EN)

0V

10%

tOFF(EN)

NO1A–NO4A

NO2B–NO4B,	 	 A0	COMA

MAX355

A1

90%

50 	GND

COMB

V-

1k

VOUT

35pF

MAX354/MAX355

-15V

Figure 2. Enable Switching Time

Test Circuits/Timing Diagrams (continued)

+15V

+2.4V

VEN    EN

A0

V+

NO1–NO8

+10V

LOGIC INPUT VA

+3V

0V

50%

tR < 20ns tF < 20ns

MAX354

A1

VA

A2

GND

50

COM V-

300

VOUT

35pF

SWITCH OUTPUT VOUT

+5V

0V

80%

tOPEN

-15V

Figure 3. Break-Before-Make Interval

RS

VS

CHANNEL SELECT

VEN

+15V

NO	V+ EN

A0 	MAX354

A1

A2

GND

COM

V-

VOUT

CL = 1000nF

LOGIC INPUT VEN

VOUT

+3V

0V

OFF	ON

OFF

VOUT

-15V

VOUT IS THE MEASURED VOLTAGE DUE TO CHARGE TRANSFER ERROR VCTE WHEN THE CHANNEL TURNS OFF.

VCTE =    VOUT x CL

MAX354/MAX355

Figure 4. Charge Injection

Test Circuits/Timing Diagrams (continued)

+15V

10nF

+15V

10nF

VIN

RS = 50

NO1

NO8

A0

A1

A2  GND

V+

MAX354

EN

COM V-

VOUT

RL

1k

R = 1k

RG = 50

VIN

NO1

NO2

NO8

A0

A1

A2  GND

V+

MAX354

EN

COM V-

VOUT

RL

1k

10nF

-15V

OFF ISOLATION = 20log

VOUT

VIN

10nF

-15V

CROSSTALK = 20log

VOUT

VIN

Figure 5. Off Isolation	Figure 6. Crosstalk

CHANNEL SELECT

A2

A1

A0

GND

+15V V+

MAX354

EN

NO1

NO8

COM V-

1MHz CAPACITANCE ANALYZER

f = 1MHz

-15V

Figure 7. NO/COM Capacitance

MAX354/MAX355

Detailed Description

Fault-Protection Circuitry Maxim’s MAX354/MAX355 are fully fault protected  for continuous input voltages up to ±40V, whether or not the V+ and V- power supplies  are present. These devices use a “series FET” protection scheme that not only protects  the multiplexer output from overvoltage, but also limits the input current to sub-microamp levels. When signal voltages exceed  or are within approxi- mately 1.5V of the supply rails, on-resistance increas- es. This greater on-resistance limits fault currents and output voltage, protecting  sensitive circuits and com- ponents. The protected output clamps at approximately

1.5V below the supply rails and maintains the correct polarity. There are no glitches  or polarity reversals going into or coming out of a fault condition.

Figures 8 and 9 show how the series FET circuit protects against overvoltage conditions. When power is off, the gates of all three FETs are at ground. With a -25V input, N-channel FET Q1 is turned on by the +25V gate-to- source voltage. The P-channel device (Q2), however, has +25V VGS and is turned off, thereby preventing the input signal from reaching the output. If the input volt- age is +25V, Q1 has a negative VGS, which turns it off. Similarly, only sub-microamp leakage currents can flow from the output back to the input, since any voltage will turn off either Q1 or Q2.

Figure 10 shows the condition of an off channel with V+ and V- present. As with Figures 8 and 9, either an N- channel or a P-channel device will be off for any input voltage from -40V to +40V. The leakage current with negative overvoltages will immediately drop to a few nanoamps at +25°C. For positive overvoltages,  that fault current will initially be 10µA or 20µA, decaying over a few seconds  to the nanoamp level. The time constant of this decay  is caused  by the discharge  of stored charge from internal nodes and does not com- promise the fault-protection scheme.

Figure 11 shows the condition of the on channel with V+ and V- present. With input voltages less than ±10V, all three FETs are on and the input signal appears  at the output. If the input voltage exceeds  V+ minus the N-channel threshold voltage (VTN), the N-channel FET will turn off. For voltages more negative than V- minus the P-channel threshold (VTP), the P-channel device will turn off. Since VTN is typically 1.5V and VTP is typically

3V, the multiplexer’s output swing is limited to about -12V

to +13.5V with ±15V supplies.

Switching Characteristics and Charge Injection

Table 1 shows typical  charge  injection levels versus

power-supply  voltages and analog input voltage. The charge injection that occurs during switching creates a voltage transient whose magnitude is inversely propor- tional to the capacitance on the multiplexer output.

Table 1. MAX354 Charge Injection

| Supply Voltage | Analog Input Level | Injected Charge |
| --- | --- | --- |
| ±5V | +2V 0V -2V | 52pC 35pC 16pC |
| ±10V | +5V 0V -5V | 105pC 65pC 25pC |
| ±15V | +10V 0V -10V | 180pC 80pC 15pC |

Test Conditions:  CL, = 1000pF  on mux output;  the tabulated analog input level is applied  to channel 1; channels  2–8 inputs are open circuited. EN = +5V, VA1 = VA2 = 0V, VO is toggled  at a 2kHz rate between  0V and 3V. +100pC  of charge  creates  a

+100mV step when injected  into a 1000pF load capacitance.

-25V OVERVOLTAGE  S

N-CHANNEL MOSFET IS TURNED ON BECAUSE VGS = +25V

Q1	-25V     Q2	Q3

D	S		D	S		D G		G			G

+25V OVERVOLTAGE  S

N-CHANNEL MOSFET IS TURNED OFF

Q1	Q2	Q3

D	S	D	S	D

G	G	G

P-CHANNEL MOSFET IS OFF

BECAUSE VGS = -25V

Figure 8. -25V Overvoltage with Multiplexer Power Off

Figure 9. +25V Overvoltage with Multiplexer Power Off

-15V

+15V

-15V

-15V

+15V

-15V

Q1

-25V

OVERVOLTAGE

| N-CHANNEL MOSFET IS TURNED ON BECAUSE VGS = +10V |  |  |  |
| --- | --- | --- | --- |

Q2	Q3

+25V FORCED

ON COMMON OUTPUT LINE BY EXTERNAL CIRCUITRY

N-CHANNEL

+25V OVERVOLTAGE

N-CHANNEL MOSFET IS TURNED OFF

Q1	13.5V

Q2	Q3

13.5V

| OUTPUT | VTN = 1.5V |  |  |
| --- | --- | --- | --- |

N-CHANNEL

-15V FROM

DRIVERS

+15V FROM

DRIVERS

MOSFET IS OFF

BECAUSE VGS = -10V

+15V FROM DRIVERS

-15V FROM

DRIVERS

MOSFET IS ON

MAX354/MAX355

P-CHANNEL

MOSFET IS OFF

Figure 10. -25V Overvoltage on an Off Channel with

Multiplexer Power Supply On

Figure 11. +25V Overvoltage Input to the On Channel

MAX354/MAX355

The channel-to-channel  switching  time is typically

180ns, with about 100ns of break-before-make  delay. This 100ns break-before-make  delay prevents the input-to-input  short that would occur if two input chan- nels were simultaneously connected to the output. In a typical  data acquisition  system, the dominant delay is not the switching time of the multiplexer, but is the set- tling time of the amplifiers and S/H. Another limiting fac- tor is the RC time constant of the multiplexer RON plus the signal source impedance  multiplied  by the load capacitance  on the output of the multiplexer. Even with low signal source impedances,  100pF of capacitance on the multiplexer output will approximately double the settling time to 0.01% accuracy.

Operation with Supply Voltages

Other than ±15V The main effect of supply voltages other than ±15V is the reduction in output signal range. The MAX354 limits the output voltage to about 1.5V below V+ and about

3V above V-. In other words, the output swing is limited to +3.5V to -2V when operating from ±5V. The Typical Operating Characteristics show RON for +15V and ±5V power  supplies.  Maxim tests and guarantees  the MAX354/MAX355 for operation from ±4.5V to ±18V supplies. The switching delays are increased by about a factor of 2 at ±5V, but break-before-make  action is preserved.

The MAX354/MAX355 can operate with a single +4.5V to +30V supply,  as well as asymmetrical power sup- plies such as +15V and -5V. The digital  threshold remains approximately  1.6V above the GND pin, and the analog characteristics,  such as RON, are deter- mined by the total voltage difference between V+ and V-. Connect V- to 0V when operating  with a +4.5V to

+30V single supply.

The MAX354 digital threshold is relatively independent of the power-supply  voltages, going from 1.6V typical when V+ is 15V to 1.5V typical  when V+ is 5V. This means that the MAX354/MAX355 operate with standard TTL-logic levels, even with ±5V power supplies. In all cases, the threshold of the enable (EN) pin is the same as the other logic inputs.

Digital Interface Levels The typical  digital  threshold of both the address  lines and the enable pin is 1.6V, with a temperature coeffi- cient of about -3mV/°C. This ensures compatibility  with

0.8V to 2.4V TTL-logic swings over the entire tempera- ture range. The digital  threshold is relatively indepen- dent of the supply voltages, moving from 1.6V typical to

1.5V typical  as the power supplies  are reduced  from

±15V to ±5V. In all cases, the digital threshold is refer- enced to the GND pin.

The digital inputs can also be driven with CMOS-logic levels swinging  from either V+ to V- or from V+ to ground. The digital input current is just a few nanoamps of leakage at all input voltage levels, with a guaranteed maximum of 1µA.

Operation as  a Demultiplexer The MAX354/MAX355 function as demultiplexers where the input is applied to the output pin, and the input pins are used as outputs. The MAX354/MAX355 provide both break-before-make action and full fault protection when operated as demultiplexers, unlike earlier genera- tions of fault-protected muxes.

Channel-to-Channel Crosstalk, Off-Isolation, and Digital Feedthrough

At DC and low frequencies  the channel-to-channel

crosstalk is caused by variations in output leakage cur- rents as the off-channel input voltages are varied. The MAX354 output leakage varies only a few picoamps as all seven off inputs are toggled from -10V to +10V. The output voltage change  depends  on the impedance level at the MAX354 output, which is RON plus the input signal source resistance in most cases, since the load driven by the MAX354 is usually high impedance. For a signal source impedance  of 10k or lower, the DC crosstalk exceeds 120dB.

Tables 2a and 2b show typical  AC crosstalk and off- isolation performance. Digital feedthrough  is masked by the analog charge  injection when the output is enabled.  When the output  is disabled,  the digital feedthrough  is virtually unmeasureable, since the digi- tal pins are physically isolated from the analog section by the GND and V- pins. The ground plane formed by these lines is continued onto the MAX354/MAX355 die to provide over 100dB isolation between the digital and analog sections.

Table 2a. Typical Off-Isolation Rejection

Ratio

Table 2b. Typical Crosstalk Rejection Ratio

| Frequency | 100kHz | 1MHz |
| --- | --- | --- |
| One Channel Driven | 100dB | 80dB |

Test Conditions:  VIN =  20Vp-p  at the tabulated  frequency, RL = 1.5k between OUT and ground,  EN = 0V.

| Test Conditions: Specified RL connected from OUT to ground, EN = +5V, A0 = A1 = A2 = +5V (Channel 1 selected). 20Vp-p at the tabulated frequency is applied to Channel 2. All other Frequency | 100kHz | 1MHz |
| --- | --- | --- |
| RL = 1.5kΩ | 92dB | 72dB |
| RL = 10kΩ | 76dB | 56dB |

VISO

= 20log

20Vp-p

—————

VOUT (p-p)

channels  are open circuited. Similar crosstalk  rejection  can be observed  between any two channels.

Functional Diagrams/Truth Tables

MAX354

V+	V-	GND

NO1

NO2

NO3

NO4

NO5

NO6

NO7

NO8

COM

DECODERS / DRIVERS

A0 	A1 	A2 	EN

| LOGIC "O" VAL +0.8V, LOGIC "1" VAH +2.4V MAX354 |  |  |  |  |
| --- | --- | --- | --- | --- |
| A2 | A1 | A0 | EN | ON SWITCH |
| X 0 0 0 0 1 1 1 1 | X 0 0 1 1 0 0 1 1 | X 0 1 0 1 0 1 0 1 | 0 1 1 1 1 1 1 1 1 | NONE 1 2 3 4 5 6 7 8 |

| MAX355 |  |  |  |
| --- | --- | --- | --- |
| A1 | A0 | EN | ON SWITCH |
| X 0 0 1 1 | X 0 1 0 1 | 0 1 1 1 1 | NONE 1 2 3 4 |

MAX355

V+	V-	GND

NO1A NO2A NO3A

NO4A

COMA

NO1B NO2B NO3B

NO4B

COMB

DECODERS / DRIVERS

LOGIC "O" VAL     +0.8V,  LOGIC "1" VAH     +2.4V

MAX354/MAX355

A0 	A1 	EN

Chip Topographies

MAX354

MAX355

EN	A0	A1	A2	GND

EN	A0	A1	GND

V-

N01

V+

N05

V-

N01A

V+

N.C.

N01B

0.130"

(3.30mm)

0.130"

(3.30mm)

N02

N06

N02A

N02B

N03

N.C.

N03A

N03B

N04	COM    N08	N07

N04A

COMA   COMB     N04B

0.115"

(2.92mm)

0.115"

(2.92mm)

MAX354/MAX355

TRANSISTOR COUNT: 256

SUBSTRATE CONNECTED TO V+

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses  are implied. Maxim reserves the right to change the circuitry and specifications  without notice at any time.

12   	Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA  94086 (408) 737-7600

©  1996 Maxim Integrated Products	Printed USA	is a registered trademark of Maxim Integrated Products.