19-0326; Rev 0; 12/94

Signal-Line Circuit Protectors

General Description

The MAX366 and MAX367 are multiple, two-terminal circuit protectors.  Placed in series with signal lines, each two-ter- minal device guards sensitive circuit components against voltages near and beyond the normal supply voltages. These devices are used at interfaces where sensitive cir- cuits are connected  to the external world and could encounter damaging voltages (up to 35V beyond the sup- ply rails) during power-up, power-down, or fault conditions.

The MAX366 contains three independent protectors and the MAX367 contains eight.  They can protect analog sig- nals using either unipolar (4.5V to 36V) or bipolar (±2.25V to ±18V) power supplies.  Each protector is symmetrical. Input and output terminals may be freely interchanged.

These devices are voltage-sensitive MOSFET transistor arrays that are normally on when power is applied and normally open circuit when power is off.  With ±10V sup- plies, on-resistance is 100 max and leakage is less than

1nA at +25°C.

When signal voltages exceed or are within approximately

1.5V of either power-supply voltage (including when power is off), the two-terminal resistance increases dra- matically, limiting fault current as well as output voltage to sensitive circuits.  The protected side of the switch main- tains the correct polarity and clamps approximately 1.5V below the supply rail.  There are no “glitches”  or polarity reversals going into or coming out of a fault condition.

Applications Process Control Systems	Redundant/Backup Systems Hot-Insertion Boards/Systems ATE Equipment

Data-Acquisition Systems	Sensitive Instruments

Features

♦ ±40V Overvoltage Protection

♦ Open Signal Paths with Power Off

100 Signal Paths with Power On

♦ 1nA Max Path Leakage at +25°C

♦ 44V Maximum Supply Voltage Rating

♦ Automatic Protection; No Programming or

Controls

Ordering Information

PART† TEMP. RANGE PIN-PACKAGE

MAX366CPA 0°C to +70°C 8 Plastic DIP

MAX366CSA 0°C to +70°C 8 SO

MAX366C/D 0°C to +70°C Dice*

MAX366EPA -40°C to +85°C 8 Plastic DIP

MAX366ESA -40°C to +85°C 8 SO

MAX366MJA -55°C to +125°C 8 CERDIP**

MAX367CPN 0°C to +70°C 18 Plastic DIP

MAX367CWN 0°C to +70°C 18 Wide SO

MAX367C/D 0°C to +70°C Dice*

MAX367EPN -40°C to +85°C 18 Plastic DIP

MAX367EWN -40°C to +85°C 18 Wide SO

MAX367MJN -55°C to +125°C 18 CERDIP**

† MAX367 available after January 1, 1995.

* Dice are tested at TA = +25°C only.

* Contact factory for availability.

Pin Configurations appear at end of data sheet.

Typical Operating Circuit

FAULT!

ELECTRONICS

PROTECTOR

REMOTE SENSOR

+10V REG.

+28V

MAX366	V+    8

+12V

1     IN1

OUT1    7

(SHORT)

2     IN2

OUT2    6

SENSITIVE AMPLIFIER

3     IN3

OUT3    5

(OPEN)

4     V-

MAX366/MAX367

FAULT!

Maxim Integrated Products    1

Call toll free 1-800-998-8800 for free samples or literature.

ABSOLUTE MAXIMUM RATINGS

V+ to V-......................................................................-0.3V, +44V IN_, OUT_ ..................................................(V- + 44V), (V+ - 44V) Continuous Current into Any Terminal ..............................±30mA Peak Current into Any Terminal

(pulsed at 1ms, 10% duty cycle) ...................................±70mA Continuous Power Dissipation (TA = +70°C)

8-Pin Plastic DIP (derate 9.09mW/°C above +70°C) ....727mW

8-Pin SO (derate 5.88mW/°C above +70°C).................471mW

8-Pin CERDIP (derate 8.00mW/°C above +70°C).........640mW

18-Pin Plastic DIP (derate 11.11mW/°C above +70°C) ...889mW

18-Pin Wide SO (derate 9.52mW/°C above +70°C) .....762mW

18-Pin CERDIP (derate 10.53mW/°C above +70°C).....842mW Operating Temperature Ranges

MAX36_C_ _ ........................................................0°C to +70°C MAX36_E_ _......................................................-40°C to +85°C MAX36_M_ _ ...................................................-55°C to +125°C Storage Temperature Range .............................-65°C to +150°C Lead Temperature (soldering, 10sec) .............................+300°C

MAX366/MAX367

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions beyond those indicated in the operational sections of the specifications is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS

(V+ = +15V, V- = -15V, TA = TMIN to TMAX, unless otherwise noted.)

| PARAMETER | SYMBOL | CONDITIONS | TEMP. RANGE | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| Analog Signal Range | VIN, VOUT | (Note 1) | All | (V+ - 40) (V- + 40) | V |
| Fault-Free Analog Signal Range | VIN, VOUT | V+ = 15V, V- = -15V (Note 2) | All | -11 11 | V |
| Analog-Signal Output Range (Fault) | VOUT | VIN = V+ or V-, 100kΩ < ROUT < 1000MΩ (Note 1) | All | (V- + 3) (V+ - 1.5) | V |
| Analog-Signal-Path Resistance | R(IN-OUT) | V+ = 15V, V- = -15V, VIN = ±10V, IOUT = 1mA | +25°C | 62 85 | Ω |
|  |  |  | C, E | 100 |  |
|  |  |  | M | 125 |  |
|  |  | V+ = 10V, V- = -10V, VIN = ±5V, IOUT = 1mA | +25°C | 62 100 |  |
|  |  |  | C, E | 125 |  |
|  |  |  | M | 150 |  |
|  |  | V+ = 5V, V- = -5V, VIN = ±2V, IOUT = 1mA | +25°C | 140 350 |  |
|  |  |  | C, E, M | 400 |  |
| Signal-Path Resistance Match | ∆R(IN-OUT) | VIN = ±10V, IOUT = 1mA | +25°C | 7 | Ω |
|  |  |  | C, E, M | 10 |  |
| Signal-Path Leakage (Power Off) | IIN(OFF) | V+ = V- = 0V, VIN = ±35V, VOUT = open circuit | +25°C | -10 10 | nA |
|  |  |  | C, E, M | -1000 1000 |  |
| Signal-Path Leakage (without Fault Condition) | IOUT(ON) | VIN = VOUT = ±10V | +25°C | -1 1 | nA |
|  |  |  | C, E, M | -100 100 |  |
| Signal-Path Leakage (with Fault Condition) | IIN(ON) | VIN = ±25V, VOUT = open circuit | +25°C | -10 10 | nA |
|  |  |  | C, E, M | -1000 1000 |  |
| Signal-Path Leakage (with Overvoltage) | IIN(OFF) | V+ = V- = 0V, VOUT = 0V, VIN = ±35V | +25°C | -10 10 | nA |
|  |  |  | C, E, M | -1000 1000 |  |
| POWER SUPPLY |  |  |  |  |  |
| Power-Supply Range | V+, V- |  | +25°C, C, E, M | 0 ±18 | V |
| Power-Supply Range (without Fault Condition) | V+, V- | R(IN-OUT) < 1000Ω (Note 2) | +25°C, C, E, M | ±2.25 ±18 | V |
| Power-Supply Current | I+, I- |  | +25°C | -1 1 | µA |
|  |  |  | C, E, M | -10 10 |  |

Note 1: Guaranteed, but not tested.

Note 2: See Typical Operating Characteristics curves for fault-free analog signal range at various supply voltages.

Typical Operating Characteristics

(V+ = +15V, V- = -15V, TA = +25°C, unless otherwise noted.)

TRANSFER CHARACTERISTICS (BIPOLAR SUPPLIES)

15

MAX366/7-02

MAX366/7-03

V+ = +15V, V- = -15V

TRANSFER CHARACTERISTICS (SINGLE SUPPLY)

25

OUTPUT LOAD = 1M

10

5

0

-5

OUTPUT VOLTAGE (V), INPUT & OUTPUT CURRENT (  A)

-10

V+ = +10V, V- = -10V

V+ = +3V, V- = -3V

V+ = +5V, V- = -5V

V+ = +10V, V- = -10V OUTPUT

V- = 0V

20

15

10

OUTPUT VOLTAGE (V), INPUT & OUTPUT CURRENT (  A)

5

V+ = 25V

V+ = 15V V+ = 10V

PATH RESISTANCE (  )

MAX366/7-05

V+ = 5V

-15

LOAD = 1M

V+ = +15V, V- = -15V

0

-35

-25

-15	-5  0  5

15     25     35

0	5	10

15    20

25     30     35

INPUT VOLTAGE (V)

INPUT VOLTAGE (V) VIN > (V+ - 35V)

1E+08

1E+07

1E+06

1E+05

1E+04

PATH RESISTANCE (  )

1E+03

1E+02

1E+01

PATH RESISTANCE vs. INPUT VOLTAGE (BIPOLAR SUPPLIES)

| 500 |  | V± | = ±3V |  |  |
| --- | --- | --- | --- | --- | --- |
| V± = ±15V |  |  |  |  |  |
| V | ± = ±10V |  |  |  |  |
|  | V± = ±5V |  |  |  |  |
| Circuit o | f Fig. 6 |  |  |  |  |

450

MAX366/7-04

400

350

300

250

200

150

100

50

0

PATH RESISTANCE vs. INPUT VOLTAGE (BIPOLAR SUPPLIES)

| -15 |  | V± = ±3V |  |  |  |
| --- | --- | --- | --- | --- | --- |
| V± = ±15V |  |  |  |  |  |
| V | ± = ±10V |  |  |  |  |
|  | V± = ±5V |  |  |  |  |
| Circuit | of Fig. | 6 |  |  |  |

-10	-5	0

5	10	15

-15

-10	-5	0

5	10	15

MAX366/MAX367

INPUT VOLTAGE (V)

INPUT VOLTAGE (V)

Typical Operating Characteristics (continued)

(V+ = +15V, V- = -15V, TA = +25°C, unless otherwise noted.)

1G

100M

PATH RESISTANCE (  )

10M

1M

100k

10k

1k

100

PATH RESISTANCE vs. INPUT VOLTAGE (SINGLE SUPPLY)

500

450

MAX366/7-06

PATH RESISTANCE (  )

400

350

300

250

200

150

100

50

PATH RESISTANCE vs. INPUT VOLTAGE (SINGLE SUPPLY)

V+ = 10V

V+ = 15V

MAX366/7-07

V+ = 25V V+ = 5V

V+ = 35V

10

| 1 10 |  | V+ = 25V |
| --- | --- | --- |
| V+ = 10V |  |  |
|  |  | V+ = 35V |
| V+ = 15V |  |  |
| V+ = 5V Circuit of |  |  |
|  | Fig. 6 | V- = 0V |

100

Circuit of Fig. 6

0

1	10

V- = 0V

MAX366-TOC9

100

INPUT VOLTAGE (V)

INPUT VOLTAGE (V)

OVERVOLTAGE RAMP

0

-2

-4

LOSS (dB)

-6

-8

-10

MAX366 FREQUENCY RESPONSE

MAX366/MAX367

V+ = 5V, V- = -5V

CHAN 1: INPUT OVERVOLTAGE RAMP ±7V, 2V/div

CHAN 2: OUTPUT; OUTPUT LOAD = 1000 , 2V/div

-12

| 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | SOURCE = 50 LOAD = 50 V+ = 5V V- = -5V |  |  |  |  |  |  |

100

1k    10k  100k  1M   10M 100M FREQUENCY (Hz)

Pin  Description

| PIN |  | NAME* | FUNCTION |
| --- | --- | --- | --- |
| MAX366 | MAX367 |  |  |
| 1, 2, 3 | 1, 2, 3 | IN1, IN2, IN3 | Signal Inputs 1, 2, 3 |
| – | 4–8 | IN4–IN8 | Signal Inputs 4–8 |
| 4 | 9 | V- | Negative Supply Voltage Input |
| – | 10–14 | OUT8–OUT4 | Signal Outputs 4–8 |
| 5, 6, 7 | 15, 16, 17 | OUT3, OUT2, OUT1 | Signal Outputs 1, 2, 3 |
| 8 | 18 | V+ | Positive Supply Voltage Input |

* Inputs and outputs are names for convenience only; inputs and outputs are identical and interchangeable.

Background Information

When a voltage outside the supply range is applied to most integrated circuits, there is a strong possibility they will be damaged or “latch up” (that is, fail to operate prop- erly even after the offending voltage is removed).  If an IC’s input or output pin is supplied with a voltage when the IC’s power is off, and power is subsequently applied, the device may act as an SCR and destroy itself and/or other circuitry.   Such “faults” are commonly encountered in modular control systems where power and signals to inter- connected modules may be interrupted and re-estab- lished at random.  They can happen during production testing, maintenance, start-up, or a power “brownout.”

The MAX366/MAX367 are designed to protect delicate input and output circuitry from overvoltage faults up to

±40V (with or without power applied), in devices such as op amps, analog-to-digital/digital-to-analog  converters, and voltage references.  These circuit protectors automati- cally limit signal voltages and currents to safe levels with- out degrading  normal signal performance, even in very high-impedance circuits.  They are powered by the power supply of the protected circuit and inserted into the signal lines.  There are no control lines, programming pins, or adjustments.

Detailed Description

Internal Construction Figure 1 shows the simplified internal construction of each protector inside the MAX366/MAX367. Each circuit consists of two N-channel FETs and one P-channel FET. All the FETs are enhancement types; that is, the N chan- nels must have approximately 1.3V of positive gate volt- age in order to conduct, and the P channel must have approximately 2V of negative gate voltage in order to conduct.

During normal operation, V+ is connected to a positive potential and V- is connected to a negative potential. Since their gates are tied to V+, transistors Q1 and Q3 conduct as long as their sources are at least 1.3V below V+ (the N-channel gate threshold.)  Transistor Q2’s gate is tied to V-, so it conducts as long as its source is 2V or more above V- (the P-channel gate threshold.)

V-

P

Unlike shunt diode networks, these devices are low- impedance FETs that become high impedance during a fault condition, so fault current and power dissipation are extremely low.  Equally important, leakage current during normal and fault conditions is extremely low.  In addition, unlike most discrete networks, these parts protect circuits both when power is off and during power transitions.

IN

Q2

N	N

Q1	Q3

V+

OUT

MAX366/MAX367

Figure 1.  Simplified Internal Structure

As long as the signal is within these limits, all three tran- sistors conduct and a low-resistance path is maintained from the IN to OUT pin.  (Note that, since the device is symmetrical, IN and OUT pins can be interchanged.) When the signal is beyond the gate threshold of either Q2 or Q1/Q3, the path resistance rises dramatically. When power is off, none of the transistors have gate bias, so the circuit from IN to OUT is open.

Normal Operation In normal operation, the protector is placed  in series with the signal line and the power supplies are con- nected to V+ and V- (see Figure 2).  V- is ground when operating with a single supply.  When power is applied, each protector acts as a resistor in the signal path. Any voltage source on the “input” side of the switch will be conducted through the protector to the output. (Note that, since the protector is symmetrical, IN and OUT pins can be interchanged.)

If the output load is resistive, it will draw current, and a voltage divider will be formed with the internal resistance so the output voltage will be lower than the input voltage. Since the internal resistance is typically less than 100, high-impedance loads will be relatively unaffected by the presence of the protector.   The protector’s path resis- tance is a function of the supply voltage and the signal voltage (see Typical Operating Characteristics).

MAX366

Power Off When power is off (i.e., V+ = V- = 0V), the protector is a virtual open circuit, and all voltages on each side are isolated from each other up to ±40V.  With ±40V applied to the input pin, the output pin will be 0V, regardless of its resistance to ground.

Fault Conditions A fault condition exists when the voltage on either sig- nal pin is within about 1.5V of either supply rail or exceeds either supply rail.  This definition is valid when power is applied and when it is off, as well as during all the states as power ramps up or down.

During a fault, the protector acts as a variable resistor, conducting only enough to sustain the other side of the switch within about 1.5V of the supply rail.  This voltage is known as the “fault knee voltage,” and is not symmet- rical.   It is approximately 1.3V down from the positive supply (V+ pin) or approximately 2.0V up from the neg- ative supply (V- pin).   Each fault knee voltage varies slightly with supply voltage, with output current, and from device to device.

During a fault condition, all the fault current flows from one signal pin through the protector and out the other signal pin.  No fault current flows through either supply pin.  (There will be a few pico-amps of leakage current from each signal pin to each supply pin, but this is independent of fault current.)

During the fault condition, enough current will flow to maintain the output voltage at the fault knee voltage, so the fault current is a function of the output resistance and the supply voltage.  The output voltage and cur- rent have the same polarity as the fault.

The maximum input fault voltage is 40V from the “oppo- site-polarity supply rail.”  This means the input can go

V-

VIN

4

V-

1     IN1

8

V+

OUT1    7

VOUT

V+

ROUT

VLOW

to ±35V with ±5V supplies or to ±25V with ±15V sup-

plies.  The fault voltage is highest (±40V) when the sup- plies are off (V+ = V- = 0V).

Using the circuit of Figure 2, the approximate fault cur- rents are as follows:

1) For positive faults:

I(F)  (V+ - 1.3V - VLOW) ÷ ROUT

2) For negative faults:

I(F)  (V- + 2V + VLOW) ÷ ROUT

where VLOW is the terminating voltage at the far end of

ROUT. VLOW = 0V when ROUT is grounded.

MAX366/MAX367

Figure 2.  Application Circuit

The current through each protector should never exceed

OUTPUT VOLTAGE (V)

30mA.  Always calculate the power dissipated by all the protectors in worst-case conditions (maximum voltage and current through each protector) to ensure the pack- age dissipation limit is not reached.

With single-supply operation, grounded loads will have zero voltage (and current) whenever the input voltage is below approximately 2V.  In effect, both the IN and OUT pins are in fault condition.

A special case arises when power is off:  The part is in a perpetual fault condition but no fault current flows because all the internal FETs are off.

Single-Supply Output Operation

Single-supply operation is a special case.  Signals can-

5

4

3

2

1

0

-1

-2

-3

-4

-30

MAX366/7-fig03

| V- = -5V |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| V+ = + | 5V |  |  |  |  |
| ROUT | = 100M |  |  |  |  |

-20     -10	0

10	20	30

MAX366/MAX367

not go to ground, since from 0V to approximately +2V is a fault condition.

Extremely Low-Current Operation Figure 3 shows the typical high-impedance  transfer characteristics with a 100M load.   Compared to the transfer characteristic at 1M (see Typical Operating Characteristics), the two knees are closer to the supply voltages and the slopes of the flat portions of the curve (fault conditions) are steeper.  As the load resistance is increased even further, the positive and negative knees increase, and the slopes in fault conditions increase even more.  Eventually, at some extremely high output resistance (e.g., Tera ohms), the output voltage can exceed the supply voltage during fault conditions.  This is due to extremely low leakage currents from the input to output.

When the protector’s output side is connected to very high-resistance, very low-current loads (such as op- amp inputs), a small leakage current flows from the input to the output during fault conditions.  This current is typically below a nano-ampere (<10-9A) but, if the output resistance is high enough, it can cause the out- put voltage to exceed the supply voltages during fault conditions.

This condition can be self-correcting,  however, if the high-resistance load has protection diodes to the sup- ply rails (either external or internal to the op amp). These diodes conduct the leakage current to the supply rails and safely limit the output voltage.  An alternative is to add a high-value resistor to ground in parallel with the load.   This resistor may be as low as 1000M; its value must be determined experimentally at the highest anticipated operational temperature.

The fault protectors will not normally be used with high- impedance FET-input amplifiers that lack input protection diodes.   Such amplifiers are fragile and are normally

INPUT VOLTAGE (V)

Figure 3.  High-Impedance Transfer Characteristic

reserved for use when ultra-low leakage (pA) is needed. The MAX366/MAX367 have nano-amperes of leakage, which would negate the low leakage of the unprotected amplifier.

Low-Voltage Operation The MAX366/MAX367 “operate”  with supply voltages all the way down to 0V, but what they do to the signal is not obvious.   With a total supply voltage of 3.5V, the protector is in a fault condition with nearly any input that is not close to 2.0V.  Below 3.5V (including power off), the protector is perpetually in a fault condition (i.e., high impedance).

When the supply voltage(s) ramps up (and/or down) from zero, the signal path is initially in a fault condition (open), until the supply voltage passes the input volt- age.   The output starts at zero and is delayed from reaching the input voltage as the part comes out of the fault condition.   If the supply voltage exceeds about

3.5V, but never exceeds the input voltage, the output will follow the supply, always remaining about 1.3V below the positive supply voltage or 2V above the neg- ative supply voltage.  If the input voltage subsequently comes out of the fault condition, the output returns to the input value.   This set of conditions  is exactly reversed when power ramps down to zero.

Since the input and output pins are identical and inter- changeable,  predicting  whether or not the part is in a fault condition is easy:  If either IN or OUT exceeds V+ or V-, a fault condition exists and the current that flows will be just enough to cause the other signal pin (OUT or IN) to approach the appropriate supply rail.

Bipolar Faults The MAX366/MAX367 V+ and V- pins are normally con- nected to a circuit’s most positive and most negative power supplies.   When a circuit has multiple power supplies (such as ±5V and ±12V) and the MAX366/ MAX367 V+ and V- pins are connected  to the lower supply, it is possible to have fault conditions on both sides of the signal path at once, if both sides of the switch have paths to higher voltages.  If the polarity of these faults is the same, the signal path will be open

1     IN1

MAX366

V+ OUT1

8

10   F

7

100k

+5V

and there is no conflict.

If the IN and OUT pins are driven in opposite polarities from low-impedance  sources, the lower of the two impedances  will overcome the higher impedance,  just as if the protector were not present.   (Make sure the current does not exceed the 30mA absolute maximum rating.)   As the lower impedance  source approaches and exceeds the fault knee voltage, the protector will conduct enough current to maintain the other signal pin near the fault knee voltage.   This means when the fault knee voltage is reached, the current through the pro- tector shifts from the higher current capability  of the lower impedance source to the lower current capability of the higher impedance source.

2     IN2

3     IN3

4     V-

Figure 4.  Turn-On Delay

OUT2

OUT3

6

5

10   F

100k

OP AMP

-5V

MAX366/MAX367

Typical Applications

Driven Switches The MAX366/MAX367 have low supply  currents (<1µA), which allows the supply  pins to be driven directly by other active circuitry, instead of connected directly to the power sources.   In this configuration, the parts can be used as driven  fault-protected switches with V+ or V- pins used as the control pins. For example, if the V- pin is grounded,  you can turn the V+ pin on and off by driving it with the output of a CMOS gate.   This effectively connects and discon- nects three or eight separate signal lines at once.  (If bipolar signals or signals that go to ground are being switched, the V- pin must be driven simultaneously to a negative potential.)  Always ensure that the driving source(s) does not drive the V+ pin more negative than the V- pin.

Figure 4 shows a simple turn-on delay that takes advantage of the MAX366’s low power consumption. The two RC networks cause gradual  application  of power to the MAX366, which in turn applies the input signals smoothly after the amplifier has stabilized. The two diodes discharge the two capacitors rapidly when power is turned off.

This circuit can be tailored to nearly any rate of turn- on by selecting the RC time constants in the V+ and V- pins, without affecting  the time constant of the measuring circuit.

Protectors as  Circuit Elements Any of the individual protectors in a MAX366 or MAX367 may be used as a switched resistor, independent of the functions of other elements in the same package.   For example, Figure 5 shows a MAX366 with two of the pro- tectors used to protect the input of an op amp, and the third element used to sequence a power supply. Combining the circuits of Figures 4 and 5 produces  a delayed action on the switched +5V, as well as smooth application of signals to the amplifier input.

Testing Circuit Protectors

Measuring Path Resistance Measuring path resistance requires special techniques, since path resistance varies dramatically with the IN and OUT voltages relative to the supply voltages. Conventional ohmmeters should not be used, for two reasons:  1) the applied voltage and currents are usual- ly not predictable,  and 2) the true resistance is a func- tion of the applied voltage, which is dramatically altered by the ohmmeter itself.   Autoranging ohmmeters are particularly unreliable.

SWITCHED +5V

P

1     IN1

MAX366

8

V+

OUT1    7

100k

+5V

100mV

VIN

IN_

MAX366

OUT_

A

VOUT

2     IN2

OUT2    6

3     IN3

OUT3    5

OP AMP

V-    4

V-

8

V+	V+

4     V-

ADJUSTABLE ANALOG VOLTAGE

PATH RESISTANCE = 100mV/A

-5V

MAX366/MAX367

Figure 5.  Power-Supply Sequencing

Figure 6 shows a circuit that can give reliable results. This circuit uses a 100mV voltage source and a low- voltage-drop ammeter as the measuring circuit, and an adjustable supply to sweep the analog voltage across its whole range.   The ammeter must have a voltage drop of less than one millivolt (at any current) for accu- rate results.  (A Keithley Model 617 Electrometer has a suitable ammeter circuit, appropriate  ranges, and a built-in voltage source designed  for this type of mea- surement.)   Measurements are made by setting the analog voltage, measuring the current, and calculating the path resistance.   The procedure  is repeated at each analog voltage and supply voltage.

It is important to use a voltage source of 100mV or less. As shown in Figure 4, this voltage is added to the VIN voltage to form the VOUT voltage.  Using a higher volt- age could cause the OUT pin to go into a fault condi- tion prematurely.

Figure 6.  Path-Resistance Measuring Circuit

High-Frequency Performance In 50 systems, signal response is reasonably flat up to several megahertz (see Typical Operating Characteristics).   Above 5MHz, the response has sev- eral minor peaks, which are highly layout dependent. Because the path resistance is dependent on the sup- ply voltage and signal amplitude, the impedance is not controlled.  Adjacent channel attenuation up to 5MHz is about 3dB above that of a bare IC socket, and is due entirely to capacitive coupling.

Pulse response is reasonable, but because the imped- ance changes rapidly, fast rise times may induce ringing as the signal approaches the fault voltage.  At very high amplitudes (such as noise spikes), the capacitive cou- pling across the signal pins will transfer considerable energy, despite the fact that the DC path is a virtual open circuit.

High-Voltage Surge Suppression

These devices are not high-voltage arresters, nor are they substitutes for surge suppressers.   In systems that use these forms of protection, however, the MAX366/MAX367 can fill a vital gap.   Figure 7 shows a typical circuit. Although the surge suppressers are extremely fast shunt elements, they have very soft current knees.  Their clamp voltage must be chosen well above the normal signal levels, because they have excessive leakage currents as the knee is approached.   This current can interfere with normal operation when signal levels are low or imped- ances are high.  If the clamp voltage is too high, however, the input can be damaged.

Using a MAX366/MAX367 after the surge suppresser allows the surge-suppresser  voltage to be set above the supply voltage (but within the overvoltage limits), dramatically reducing the effects of leakage (Figure 7). During a surge, the surge suppresser clamps the input voltage   to  roughly   ±10V. 	This  protects   the MAX366/MAX367, but the MAX366/MAX367 still dis-

SURGE SUPPRESSERS (+10V)

1     IN1

2     IN2

3     IN3

4     V-

MAX366

8

V+

OUT1    7

OUT2    6

OUT3    5

+5V

OP AMP

-5V

connect the signal from the op amp well within the ±5V

supply.

Figure 7.  Surge-Suppression Circuit

Pin  Configurations

Chip Topography

TOP VIEW

IN1 1

18   V+

IN1

V+

IN1 1

8   V+

IN2    2

IN3    3

17   OUT1

16   OUT2

IN2

OUT1

IN2    2

IN3    3

V-    4

MAX366

DIP/SO

7   OUT1

6   OUT2

5   OUT3

IN4    4

IN5    5

IN6    6

IN7    7

IN8    8

V-    9

MAX367

DIP/SO

15   OUT3

14   OUT4

13   OUT5

12   OUT6

11   OUT7

10   OUT8

IN3

V-

0.112"

(2.84mm) OUT2

OUT3

MAX366/MAX367

0.085"

(2.16mm)

TRANSISTOR COUNT: 21

SUBSTRATE CONNECTED TO V+

Package Information

| 0° - 15° E D E1 A3 A A2 L A1 C e B1 eA B eB D1 | DIM | INCHES |  |  |  | MILLIMETERS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MIN |  | MAX |  | MIN |  | MAX |  |
|  | A | – |  | 0.200 |  | – |  | 5.08 |  |
|  | A1 | 0.015 |  | – |  | 0.38 |  | – |  |
|  | A2 | 0.125 |  | 0.175 |  | 3.18 |  | 4.45 |  |
|  | A3 | 0.055 |  | 0.080 |  | 1.40 |  | 2.03 |  |
|  | B | 0.016 |  | 0.022 |  | 0.41 |  | 0.56 |  |
|  | B1 | 0.045 |  | 0.065 |  | 1.14 |  | 1.65 |  |
|  | C | 0.008 |  | 0.012 |  | 0.20 |  | 0.30 |  |
|  | D1 | 0.005 |  | 0.080 |  | 0.13 |  | 2.03 |  |
|  | E | 0.300 |  | 0.325 |  | 7.62 |  | 8.26 |  |
|  | E1 | 0.240 |  | 0.310 |  | 6.10 |  | 7.87 |  |
|  | e | 0.100 |  | – |  | 2.54 |  | – |  |
|  | eA | 0.300 |  | – |  | 7.62 |  | – |  |
|  | eB | – |  | 0.400 |  | – |  | 10.16 |  |
|  | L | 0.115 |  | 0.150 |  | 2.92 |  | 3.81 |  |
| Plastic DIP PLASTIC DUAL-IN-LINE PACKAGE (0.300 in.) | DIM | PINS | INCHES |  |  |  | MILLIMETERS |  |  |
|  |  |  | MIN |  | MAX |  | MIN |  | MAX |
|  | D | 8 | 0.348 |  | 0.390 |  | 8.84 |  | 9.91 |
|  | D | 14 | 0.735 |  | 0.765 |  | 18.67 |  | 19.43 |
|  | D | 16 | 0.745 |  | 0.765 |  | 18.92 |  | 19.43 |
|  | D | 18 | 0.885 |  | 0.915 |  | 22.48 |  | 23.24 |
|  | D | 20 | 1.015 |  | 1.045 |  | 25.78 |  | 26.54 |
|  | D | 24 | 1.14 |  | 1.265 |  | 28.96 |  | 32.13 |

DIM

INCHES 	MILLIMETERS

D

0°-8° A

MIN

A 	0.053

A1     0.004

B 	0.014

C 	0.007

E 	0.150

MAX

0.069

0.010

0.019

0.010

0.157

MIN

1.35

0.10

0.35

0.19

3.80

MAX

1.75

0.25

0.49

0.25

4.00

0.101mm

0.004in.

e 	0.050

1.27

e	A1

B

H

C	L	L

0.228

0.016

0.244

0.050

5.80

0.40

6.20

1.27

INCHES 	MILLIMETERS

SO	DIM   PINS

MIN

MAX

MIN

MAX

MAX366/MAX367

E	H	SMALL OUTLINE PACKAGE

(0.150 in.)

D 	8

D 	14

D 	16

0.189

0.337

0.386

0.197

0.344

0.394

4.80

8.55

9.80

5.00

8.75

10.00

21-0041A

MAX366/MAX367

Package Information (continued)

| D 0°- 8° A 0.101mm e B 0.004in. A1 C L | DIM | INCHES |  |  |  | MILLIMETERS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MIN |  | MAX |  | MIN |  | MAX |  |
|  | A | 0.093 |  | 0.104 |  | 2.35 |  | 2.65 |  |
|  | A1 | 0.004 |  | 0.012 |  | 0.10 |  | 0.30 |  |
|  | B | 0.014 |  | 0.019 |  | 0.35 |  | 0.49 |  |
|  | C | 0.009 |  | 0.013 |  | 0.23 |  | 0.32 |  |
|  | E | 0.291 |  | 0.299 |  | 7.40 |  | 7.60 |  |
|  | e | 0.050 |  |  |  | 1.27 |  |  |  |
|  | H | 0.394 |  | 0.419 |  | 10.00 |  | 10.65 |  |
|  | L | 0.016 |  | 0.050 |  | 0.40 |  | 1.27 |  |
| E H Wide SO SMALL OUTLINE PACKAGE (0.300 in.) | DIM | PINS | INCHES |  |  |  | MILLIMETERS |  |  |
|  |  |  | MIN |  | MAX |  | MIN |  | MAX |
|  | D | 16 | 0.398 |  | 0.413 |  | 10.10 |  | 10.50 |
|  | D | 18 | 0.447 |  | 0.463 |  | 11.35 |  | 11.75 |
|  | D | 20 | 0.496 |  | 0.512 |  | 12.60 |  | 13.00 |
|  | D | 24 | 0.598 |  | 0.614 |  | 15.20 |  | 15.60 |
|  | D | 28 | 0.697 |  | 0.713 |  | 17.70 |  | 18.10 |
| 21-0042A |  |  |  |  |  |  |  |  |  |

e

| B1 E1 E A D Q 0°-15° L L1 C B | DIM | INCHES |  |  |  | MILLIMETERS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MIN |  | MAX |  | MIN |  | MAX |  |
|  | A | – |  | 0.200 |  | – |  | 5.08 |  |
|  | B | 0.014 |  | 0.023 |  | 0.36 |  | 0.58 |  |
|  | B1 | 0.038 |  | 0.065 |  | 0.97 |  | 1.65 |  |
|  | C | 0.008 |  | 0.015 |  | 0.20 |  | 0.38 |  |
|  | E | 0.220 |  | 0.310 |  | 5.59 |  | 7.87 |  |
|  | E1 | 0.290 |  | 0.320 |  | 7.37 |  | 8.13 |  |
|  | e | 0.100 |  |  |  | 2.54 |  |  |  |
|  | L | 0.125 |  | 0.200 |  | 3.18 |  | 5.08 |  |
|  | L1 | 0.150 |  | – |  | 3.81 |  | – |  |
|  | Q | 0.015 |  | 0.070 |  | 0.38 |  | 1.78 |  |
|  | S | – |  | 0.098 |  | – |  | 2.49 |  |
|  | S1 | 0.005 |  | – |  | 0.13 |  | – |  |
| S1 S CERDIP CERAMIC DUAL-IN-LINE PACKAGE (0.300 in.) | DIM | PINS | INCHES |  |  |  | MILLIMETERS |  |  |
|  |  |  | MIN |  | MAX |  | MIN |  | MAX |
|  | D | 8 | – |  | 0.405 |  | – |  | 10.29 |
|  | D | 14 | – |  | 0.785 |  | – |  | 19.94 |
|  | D | 16 | – |  | 0.840 |  | – |  | 21.34 |
|  | D | 18 | – |  | 0.960 |  | – |  | 24.38 |
|  | D | 20 | – |  | 1.060 |  | – |  | 26.92 |
|  | D | 24 | – |  | 1.280 |  | – |  | 32.51 |

Maxim cannot assume responsibility for use of any circuitry other than circuitry entirely embodied in a Maxim product.  No circuit patent licenses are implied. Maxim reserves the right to change the circuitry and specifications without notice at any time.

12    	Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA   94086 (408) 737-7600

©  1994 Maxim Integrated Products 	Printed USA	is a registered trademark of Maxim Integrated Products.