19-1902; Rev 1; 8/94

High-Voltage, Fault-Protected

Analog Multiplexers

General Description

The MAX378 8-channel single-ended (1-of-8) multiplexer and the MAX379 4-channel differential (2-of-8) multiplexer use a series N-channel/P-channel/N-channel structure to provide significant fault protection.  If the power supplies to the MAX378/MAX379 are inadvertently turned off while input voltages are still applied, all channels in the muxes are turned off, and only a few nanoamperes of leakage cur- rent will flow into the inputs.  This protects not only the MAX378/MAX379 and the circuitry they drive, but also the sensors or signal sources that drive the muxes.

The series N-channel/P-channel/N-channel protection structure has two significant advantages over the simple current-limiting protection scheme of the industry’s first- generation fault-protected muxes. First, the Maxim protec- tion scheme limits fault currents to nanoamp leakage values rather than many milliamperes. This prevents dam- age to sensors or other sensitive signal sources.  Second, the MAX378/MAX379 fault-protected muxes can withstand a continuous ±60V input, unlike the first generation, which had a continuous ±35V input limitation imposed by power dissipation considerations.

All digital inputs have logic thresholds of 0.8V and 2.4V, ensuring both TTL and CMOS compatibility without requir- ing pull-up resistors.  Break-before-make operation is guaranteed.  Power dissipation is less than 2mW.

Applications

Data Acquisition Systems

Industrial and Process Control Systems

Avionics Test Equipment

Signal Routing Between Systems

Features

♦ Fault Input Voltage ±75V with Power Supplies Off

♦ Fault Input Voltage ±60V with ±15V Power Supplies

♦ All Switches Off with Power Supplies Off

♦ On Channel Turns OFF if Overvoltage Occurs on

Input or Output

♦ Only Nanoamperes of Input Current Under All

Fault Conditions

♦ No Increase in Supply Currents Due to Fault

Conditions

♦ Latchup-Proof Construction

♦ Operates from ±4.5V to ±18V Supplies

♦ All Digital Inputs are TTL and CMOS Compatible

♦ Low-Power Monolithic CMOS Design

Ordering Information

PART TEMP. RANGE PIN-PACKAGE

MAX378CPE 0°C to +70°C 16 Plastic DIP

MAX378CWG 0°C to +70°C 24 Wide SO

MAX378CJE 0°C to +70°C 16 CERDIP

MAX378C/D 0°C to +70°C Dice**

MAX378EPE -40°C to +85°C 16 Plastic DIP

MAX378EWG -40°C to +85°C 24 Wide SO

MAX378EJE -40°C to +85°C 16 CERDIP

MAX378MJE -55°C to +125°C 16 CERDIP

MAX378MLP -55°C to +125°C 20 LCC*

Ordering Information continued at end of data sheet.

* Contact factory for availability.

**The substrate may be allowed to float or be tied to V+ (JI CMOS).

Pin  Configurations

A0    1

EN   2

V-    3

16   A1

15   A2

14   GND

TOP VIEW

A0    1

EN   2

V-    3

16   A1

15   GND

14   V+

IN1    4

IN2    5

MAX378

13    V+

12   IN5

IN1A    4

IN2A    5

MAX379

13   IN1B

12   IN2B

IN3    6

IN4    7

OUT   8

11   IN6

10   IN7

9    IN8

IN3A    6

IN4A    7

OUTA   8

11   IN3B

10   IN4B

9    OUTB

DIP

Pin Configurations continued at end of data sheet.

DIP

MAX378/MAX379

Maxim Integrated Products    1

ABSOLUTE MAXIMUM RATINGS

Voltage between Supply Pins ..............................................+44V V+ to Ground ...................................................................+22V V- to Ground......................................................................-22V

Digital Input Overvoltage:

{

VEN, VA	V+ ......................................................................+4V V- ........................................................................-4V

Analog Input with Multiplexer Power On..............................±65V

Continuous Current, IN or OUT...........................................20mA Peak Current, IN or OUT

(Pulsed at 1ms, 10% duty cycle max) ............................40mA Power Dissipation (Note 1) (CERDIP) ................................1.28W Operating Temperature Range:

MAX378/379C .....................................................0°C to +70°C MAX378/379E ..................................................-40°C to +85°C

{Recommended }

V+ .....................................+15V

MAX378/379M ...............................................-55°C to +125°C

Power Supplies 	V- .......................................-15V

Analog Input with Multiplexer Power Off..............................±80V

Storage Temperature Range .............................-65°C to +150°C

Note 1:  Derate 12.8mW/°C above TA = +75°C

MAX378/MAX379

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions beyond those indicated in the operational sections of the specifications is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS

(V+ = +15V, V- = -15V; VAH (Logic Level High) = +2.4V, VAL (Logic Level Low) = +0.8V, unless otherwise noted.)

DS(ON)

IN(OFF)

| OUT(OFF) PARAMETER | SYMBOL | CONDITIONS | TEMP | -55°C to +125°C | 0°C to +70°C and -40°C to +85°C | UNITS |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | MIN TYP MAX | MIN TYP MAX |  |
| STATIC |  |  |  |  |  |  |
| ON Resistance | r | VOUT = ±10V, IIN = 100µA VAL = 0.8V, VAH = 2.4V | +25°C | 2.0 3.0 | 2.0 3.5 | kΩ |
|  |  |  | Full | 3.0 4.0 | 3.0 4.0 |  |
| OFF Input Leakage Current | I | VIN = ±10V, VOUT = 10V VEN = 0.8V (Note 6) | +25°C | -0.5 0.03 0.5 | -1.0 0.03 1.0 | nA |
|  |  |  | Full | -50 50 | -50 50 |  |
| OFF Output Leakage Current | IOUT(OFF) | VOUT = ±10V, VIN = 10V VEN = 0.8V MAX378 (Note 6) MAX379 | +25°C | -1.0 0.1 1.0 | -2.0 0.1 2.0 | nA |
|  |  |  | Full | -200 200 | -200 200 |  |
|  |  |  | Full | -100 100 | -100 100 |  |
| ON Channel Leakage Current | IOUT(ON) | VIN(ALL) = VOUT = ±10V VAH = VEN = 2.4V MAX378 VAL = 0.8V (Note 5) MAX379 | +25°C | -10 0.1 10 | -20 0.1 20 | nA |
|  |  |  | Full | -600 600 | -600 600 |  |
|  |  |  | Full | -300 300 | -300 300 |  |
| Analog Signal Range | VAN | (Note 2) | Full | -15 +15 | -15 +15 | V |
| Differential OFF Output Leakage Current | IDIFF | MAX379 only (Note 6) | Full | -50 50 | -50 50 | nA |
| FAULT |  |  |  |  |  |  |
| Output Leakage Current (with Input Overvoltage) | I | VOUT = 0V, VIN = ±60V (Notes 3, 4) | +25°C | 20 | 20 | nA |
|  |  |  | Full | 10 | 20 | µA |
| Input Leakage Current (with Overvoltage) | IIN(OFF) | VIN = ±60V, VOUT = ±10V (Notes 3, 4) | +25°C | 25 | 40 | µA |
| Input Leakage Current (with Power Supplies Off) | IIN(OFF) | VIN = ±75V, VEN = VOUT = 0V A0 = A1 = A2 = 0V or 5V | +25°C | 10 | 20 | µA |
| CONTROL |  |  |  |  |  |  |
| Input Low Threshold | VAL | (Note 4) | Full | 0.8 | 0.8 | V |
| Input High Threshold | VAH | (Note 4) | Full | 2.4 | 2.4 | V |
| Input Leakage Current (High or Low) | IA | VA = 5V or 0V (Note 5) | Full | -1.0 1.0 | -1.0 1.0 | µA |

MAX378/MAX379

ELECTRICAL CHARACTERISTICS (continued)

(V+ = +15V, V- = -15V; VAH (Logic Level High) = +2.4V, VAL (Logic Level Low) = +0.8V, unless otherwise noted.)

| OUT(OFF) PARAMETER | SYMBOL | CONDITIONS | TEMP | -55°C to +125°C | 0°C to +70°C and -40°C to +85°C | UNITS |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | MIN TYP MAX | MIN TYP MAX |  |
| DYNAMIC |  |  |  |  |  |  |
| Access Time | tA | Figure 1 | +25°C | 0.5 1.0 | 0.5 1.0 | µs |
| Break-Before-Make Delay (Figure 2) | tON-tOFF | VEN = +5V, VIN = ±10V A0, A1, A2 strobed | +25°C | 25 200 | 25 200 | ns |
| Enable Delay (ON) | tON(EN) | Figure 3 | +25°C | 400 750 | 400 1000 | ns |
|  |  |  | Full | 1000 | 1500 |  |
| Enable Delay (OFF) | tOFF(EN) | Figure 3 | +25°C | 300 500 | 300 | ns |
|  |  |  | Full | 1000 | 1000 |  |
| Settling Time (0.1%) (0.01%) | tSETT |  | +25°C | 1.2 | 1.2 | µs |
|  |  |  |  | 3.5 | 3.5 |  |
| “OFF Isolation” | OFF(ISO) | VEN = 0.8V, RL = 1kΩ, CL = 15pF V = 7VRMS, f = 100kHz | +25°C | 50 68 | 50 68 | dB |
| Channel Input Capacitance | CIN(OFF) |  | +25°C | 5 | 5 | pF |
| Channel Output Capacitance | C | MAX378 MAX379 | +25°C | 25 | 25 | pF |
|  |  |  |  | 12 | 12 |  |
| Digital Input Capacitance | CA |  | +25°C | 5 | 5 | pF |
| Input to Output Capacitance | CDS(OFF) |  | +25°C | 0.1 | 0.1 | pF |
| SUPPLY |  |  |  |  |  |  |
| Positive Supply Current | I+ | VEN = 0.8V or 2.4V All VA = 0V or 5V | +25°C | 0.1 0.6 | 0.2 1.0 | mA |
|  |  |  | Full | 0.3 0.7 | 0.5 1.0 |  |
| Negative Supply Current | I- | VEN = 0.8V or 2.4V All VA = 0V or 5V | +25°C | 0.01 0.1 | 0.01 0.1 | mA |
|  |  |  | Full | 0.02 0.2 | 0.02 0.1 |  |
| Power-Supply Range for Continuous Operation | VOP | (Note 7) | +25°C | ±4.5 ±18 | ±4.5 ±18 | V |

Note 2: When the analog signal exceeds +13.5V or -12V, the blocking action of Maxim’s gate structure goes into operation.  Only leakage currents flow and the channel ON resistance rises to infinity.

Note 3: The value shown is the steady-state value. The transient leakage is typically 50µA. See Detailed Description.

Note 4: Guaranteed by other static parameters.

Note 5: Digital input leakage is primarily due to the clamp diodes.  Typical leakage is less than 1nA at +25°C.

Note 6: Leakage currents not tested at TA = cold temp.

Note 7: Electrical characteristics, such as ON Resistance, will change when power supplies other than ±15V are used.

Typical Operating Characteristics

INPUT LEAKAGE vs.

INPUT VOLTAGE WITH V+ = V- = 0V

| 1m |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | OPERATING RANGE |  |  |  |  |  |
| -80V |  |  |  |  |  |  |  | +80V |

100

+60V

MAX378-2

OFF CHANNEL LEAKAGE CURRENT vs. INPUT VOLTAGE WITH ±15V  SUPPLIES

OUTPUT LEAKAGE CURRENT vs. OFF CHANNEL OVERVOLTAGE WITH ±15V  SUPPLIES

MAX378-3

10n

MAX378-1

100	10

10	1	1n

100n

IIN(OFF) (A)

10n

-60V

10n

1n

OPERATING RANGE

IOUT(OFF) (A)

-60V

100p

OPERATING RANGE

1n	100p

10p

100p

10p

10p

-100	-50

0

VIN (V)

50	100

1p

-120	-60

0

VIN (V)

60	120

1p

-120	-60

0

VIN(OFF) (V)

+60V

60	120

MAX3784

DRAIN-SOURCE ON-RESISTANCE vs.

ANALOG INPUT VOLTAGE

7

| 6 | +3.5V |  | +4V |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| +13V |  |  |  |  |  |  |
|  |  |  | ±5V SUPPLIES |  |  |  |
|  |  |  |  |  | +13V |  |
|  |  |  |  | ±15V SUPPLIES |  |  |

5

NOTE:  Typical RDS(ON) match @ +10V Analog in (±15V supplies) = 2% for lowest to highest R DS(ON) channel;   @  -10V  Analog  in,

4

RDS(ON) (k  )

3

2

1

0

-15

-10     -5	0

5	10     15      20

match = 3%.

ANALOG INPUT (V)

50%

+10V

MAX378:  VAH = 3.0V

90%

ADDRESS DRIVE (VA)

0V

OUTPUT A

-10V

VA

+VAH

50

A2

IN1

IN2

A1      MAX378 IN2-IN7

A0	IN8

EN		OUT GND

±10V

10V

10M

14pF

PROBE

MAX378/MAX379

INPUT CURRENT (A)

tA

Figure 1.  Access Time vs. Logic Level (High)

MAX358:  VAH = 3.0V

0V

ADDRESS DRIVE (VA)

VA

2.4V

A2

IN1

IN2

A1     MAX378* IN2-IN7

A0	IN8

+5V

VOUT

50%

50%

OUTPUT

50 	EN

GND

OUT

1k

12.5pF

tOPEN

*SIMILAR  CONNECTION FOR MAX379

Figure 2.  Break-Before-Make Delay (tOPEN)

50%

MAX378:  VAH = 3.0V

ENABLE DRIVE

0V

A2	IN1

A1     MAX378* IN2-IN7

A0

+10V

90%

tON(EN)

tOFF(EN)

OUTPUT

90%

VA 	EN

50

GND

OUT

1k

12.5pF

*SIMILAR  CONNECTION FOR MAX379

Figure 3.  Enable Delay (tON(EN), tOFF(EN))

+5V

+15V	0V

±60V

A0

A1

A2

EN

I 	IN1

IN8

V

±10V ANALOG SIGNAL

V-

MAX378

V-

-15V

OUT GND

10k

±75V

+5V

or

0V I

A0

A1

A2

EN IN1

V-

MAX378

V-

0V

OUT GND

10k

MAX378/MAX379

Figure 4.  Input Leakage Current (Overvoltage)

Figure 5.  Input Leakage Current (with Power Supplies OFF)

| Truth Table—MAX378 A2 | A1 | A0 | EN | ON SWITCH |
| --- | --- | --- | --- | --- |
| X 0 0 0 0 1 1 1 1 | X 0 0 1 1 0 0 1 1 | X 0 1 0 1 0 1 0 1 | 0 1 1 1 1 1 1 1 1 | NONE 1 2 3 4 5 6 7 8 |

Truth Table—MAX379

| A1 | A0 | EN | ON SWITCH |
| --- | --- | --- | --- |
| X 0 0 1 1 | X 0 1 0 1 | 0 1 1 1 1 | NONE 1 2 3 4 |

Note:  Logic “0” = VAL  0.8V, Logic “1” = VAH  2.4V

+15V

THERMOCOUPLE

STRAIN GUAGE

4-20mA LOOP TRANSMITTER

IN1

IN2

IN3

V+

OUT

MAX420

+15V

-15V

IN4

MAX378

+15V	1M

V+

+10V

GAIN REFERENCE ZERO REFERENCE

IN5

IN6

IN7

IN8

V- 	GND

-15V

OUT

DG508A MAX358

OR MAX378

V- 	GND

IN1

IN2

IN3

IN4

IN5

100k

10k

1k

111

-15V

Figure 6.  Typical Data Acquisition Front End

MAX378/MAX379

Typical Applications

Figure 6 shows a typical  data acquisition  system using the MAX378 multiplexer.   Since the multiplexer is driving a high-impedance  input, its error is a func- tion of its own resistance (RDS(ON)) times the multi- plexer leakage current (IOUT(ON)) and the amplifier bias current (IBIAS):

VERR = RDS(ON) x (IOUT(ON) + IBIAS (MAX420))

= 2.0k x (2nA + 30pA)

= 18.0µV maximum error

In most cases, this error is low enough that preamplifi- cation of input signals is not needed, even with very low-level signals such as 40µV/°C from type J thermo- couples.

In systems with fewer than eight inputs, an unused chan- nel can be connected to the system ground reference point for software zero correction.   A second channel connected to the system voltage reference allows gain correction of the entire data acquisition system as well.

A MAX420 precision op amp is connected  as a pro- grammable-gain amplifier, with gains ranging from 1 to

10,000.  The guaranteed 5µV unadjusted offset of the MAX420 maintains high signal accuracy, while program- mable gain allows the output signal level to be scaled to the optimum range for the remainder of the data acqui- sition system, normally a Sample/Hold and A/D.  Since the gain-changing  multiplexer is not connected  to the external sensors, it can be either a DG508A multiplexer or the fault-protected MAX358 or MAX378.

Input switching, however, must be done with a fault- protected  MAX378 multiplexer, to provide the level of protection and isolation required with most data acqui- sition inputs.  Since external signal sources may contin- ue to supply voltage when the multiplexer and system power are turned off, non-fault-protected  multiplexers, or even first-generation fault-protected  devices,  will allow many milliamps of fault current to flow from out- side sources into the multiplexer.   This could result in damage to either the sensors or the multiplexer.  A non- fault-protected multiplexer will also allow input overvolt- ages to appear  at its output, perhaps  damaging Sample/Holds or A/Ds.  Such input overdrives may also cause input-to-input shorts, allowing the high current output of one sensor to possibly damage another.

The MAX378 eliminates all of the above problems.   It not only limits its output voltage to safe levels, with or without power applied  (V+ and V-), but also turns all channels off when power is removed.  This allows it to draw only sub-microamp fault currents from the inputs, and maintain isolation between inputs for continuous input levels up to ±75V with power supplies off.

Detailed Description

+60V 	Q1	Q2	Q3

OVERVOLTAGE    S	D	S	D	S	D

N-CHANNEL  MOSFET 	G	G	G IS TURNED OFF

BECAUSE VGS = -60V

Figure 8.  +60V Overvoltage with Multiplexer Power OFF

-15V 	+15V 	-15V   +60V FORCED ON COMMON

OUTPUT

LINE BY EXTERNAL CIRCUITRY

Fault Protection Circuitry

The MAX378/MAX379 are fully fault protected for contin- uous input voltages up to ±60V, whether or not the V+ and V- power supplies are present.  These devices use

-60V	Q1

OVERVOLTAGE

N-CHANNEL MOSFET IS TURNED OFF

-60V   Q2	Q3

a “series FET” switching scheme which not only pro-

tects the multiplexer output from overvoltage, but also limits the input current to sub-microamp levels.

Figures 7 and 8 show how the series FET circuit pro- tects against overvoltage conditions.   When power is

BECAUSE VGS = +45V

-15V FROM

DRIVERS

P-CHANNEL MOSFET IS OFF

+15V FROM

DRIVERS

N-CHANNEL

MOSFET IS OFF

off, the gates of all three FETs are at ground.  With a -60V

input, N-channel FET Q1 is turned on by the +60V gate-

Figure 9.  -60V Overvoltage on an OFF Channel with

Multiplexer Power Supply ON

-15V	+15V	-15V

-60V	Q1

OVERVOLTAGE    S	D

-60V   Q2	Q3

S	D	S	D

+60V OVERVOLTAGE

Q1	+13.5V    Q2	Q3

+13.5V OUTPUT

N-CHANNEL MOSFET

IS TURNED ON 	G	G	G

N-CHANNEL MOSFET

BECAUSE VGS = -45V

VTN = +1.5V

+15V FROM

-15V FROM

N-CHANNEL

P-CHANNEL MOSFET IS OFF

DRIVERS

DRIVERS

MOSFET IS ON

MAX378/MAX379

Figure 7.  -60V Overvoltage with Multiplexer Power OFF

Figure 10.  +60V Overvoltage Input to the ON Channel

MAX378/MAX379

to-source voltage.  The P-channel device (Q2), howev- er, has +60V VGS and is turned off, thereby preventing the input signal from reaching the output.   If the input voltage is +60V, Q1 has a negative VGS, which turns it off.  Similarly, only sub-microamp leakage currents can flow from the output back to the input, since any volt- age will turn off either Q1 or Q2.

Figure 9 shows the condition of an OFF channel with V+ and V- present.  As with Figures 7 and 8, either an N-channel or a P-channel device will be off for any input voltage from -60V to +60V.  The leakage current with negative overvoltages will immediately drop to a few nanoamps at +25°C.   For positive overvoltages, that fault current will initially be 40µA or 50µA, decaying over a few seconds to the nanoamp level.   The time constant of this decay is caused by the discharge of stored charge from internal nodes, and does not com- promise the fault-protection scheme.

Figure 10 shows the condition of the ON channel with V+ and V- present.  With input voltages less than ±10V, all three FETs are on and the input signal appears at the output.   If the input voltage exceeds V+ minus the N- channel threshold voltage (VTN), then the N-channel FET will turn off.  For voltages more negative than V- minus the P-channel threshold (VTP), the P-channel device will turn off.  Since VTN is typically 1.5V and VTP is typically 3V, the multiplexer’s output swing is limited to about -12V to +13.5V with ±15V supplies.

The Typical Operating Characteristics graphs show typi- cal leakage vs. input voltage curves.  Although the max- imum rated  input  of these devices  is ±65V,  the MAX378/MAX379 typically have excellent performance up to ±75V, providing additional margin for the unknown transients that exist in the real world.   In summary, the MAX378/MAX379 provide superior protection from all fault conditions while using a standard, readily pro- duced junction-isolated CMOS process.

Switching Characteristics and  Charge Injection

Table 1 shows typical  charge-injection  levels vs.

power-supply voltages and analog input voltage.  Note that since the channels are well matched, the differen- tial charge injection for the MAX379 is typically less than 5pC.   The charge injection that occurs during switching creates a voltage transient whose magnitude is inversely proportional to the capacitance on the mul- tiplexer output.

The channel-to-channel switching time is typically 600ns, with about 200ns of break-before-make delay.  This 200ns break-before-make delay prevents the input-to-input short that would occur if two input channels were simultaneous-

ly connected to the output.  In a typical data acquisition system, such as in Figure 6, the dominant delay is not the switching time of the MAX378 multiplexer, but is the set- tling time of the following amplifiers and S/H. Another limit- ing factor is the RC time constant of the multiplexer RDS(ON) plus the signal source impedance multiplied by the load capacitance  on the output of the multiplexer. Even with low signal source impedances, 100pF of capac- itance on the multiplexer output will approximately double the settling time to 0.01% accuracy.

Operation with Supply Voltage

Other than ±15V The main effect of supply voltages other than ±15V is the reduction in output signal range.  The MAX378 limits the output voltage to about 1.5V below V+ and about 3V above V-.  In other words, the output swing is limited to

+3.5V to -2V when operating from ±5V.   The Typical

Operating Characteristics graphs show typical RDS(ON), for ±15V, ±10V, and ±5V power supplies.  Maxim tests and guarantees the MAX378/MAX379 for operation from

±4.5V to ±18V supplies.   The switching delays are increased by about a factor of 2 at ±5V, but break- before-make action is preserved.

The MAX378/MAX379 can be operated with a single +9V to +22V supply, as well as asymmetrical power supplies such as +15V and -5V.  The digital threshold will remain approximately 1.6V above GND and the analog character- istics such as RDS(ON) are determined by the total voltage difference between V+ and V-.  Connect V- to 0V when operating with a +9V to +22V single supply.

This means that the MAX378/MAX379 will operate with standard TTL-logic levels, even with ±5V power sup- plies.   In all cases, the threshold of the EN pin is the same as the other logic inputs.

Table 1a.  MAX378 Charge Injection

| Supply Voltage | Analog Input Level | Injected Charge |
| --- | --- | --- |
| ±5V | +1.7V 0V -1.7V | +100pC +70pC +45pC |
| ±10V | +5V 0V -5V | +200pC +130pC +60pC |
| ±15V | +10V 0V -10V | +500pC +180pC +50pC |

Test Conditions:  CL = 1000pF on multiplexer output; the tabu- lated analog input level is applied  to channel 1; channels 2 through 8 are open circuited.   EN = +5V, A1 = A2 = 0V, A0 is toggled at 2kHz rate between 0V and 3V.  +100pC of charge creates a +100mV step when injected into a 1000pF load capacitance.

Table 1b.  MAX379 Charge Injection

| Out B Supply Voltage | Analog Input Level | Injected Ch |  | rge |
| --- | --- | --- | --- | --- |
|  |  | Out A | a | Differential A-B |
| ±5V | +1.7V 0V -1.7V | +105pC +73pC +48pC | +107pC +74pC +50pC | -2pC -1pC -2pC |
| ±10V | +5V 0V -5V | +215pC +135pC +62pC | +220pC +139pC +63pC | -5pC -4pC -1pC |
| ±15V | +10V 0V -10V | +525pC +180pC +55pC | +530pC +185pC +55pC | -5pC -5pC 0pC |

Test Conditions:  CL = 1000pF on Out A and Out B; the tabulat- ed analog input level is applied to inputs 1A and 1B; channels

2 through 4 are open circuited.  EN = +5V, A1 = 0V, A0 is tog- gled from 0V to 3V at a 2kHz rate.

rents as the off-channel input voltages are varied.  The MAX378 output leakage varies only a few picoamps as all seven off inputs are toggled from -10V to +10V.  The output voltage change depends on the impedance level at the MAX378 output, which is RDS(ON) plus the input signal source resistance in most cases, since the load driven by the MAX378 is usually a high impedance.  For a signal source impedance  of 10k or lower, the DC crosstalk exceeds 120dB.

Table 2 shows typical AC crosstalk and off-isolation per- formance.  Digital feedthrough is masked by the analog charge injection when the output is enabled.  When the output is disabled,  the digital feedthrough is virtually unmeasurable, since the digital pins are physically iso- lated from the analog section by the GND and V- pins. The ground plane formed by these lines is continued onto the MAX378/MAX379 die to provide over 100dB isolation between the digital and analog sections.

Digital Interface Levels The typical digital threshold of both the address lines and the EN pin is 1.6V, with a temperature coefficient of about -3mV/°C.  This ensures compatibility with 0.8V to

2.4V TTL-logic swings over the entire temperature range.  The digital threshold is relatively independent of the supply voltages, moving from 1.6V typical to 1.5V typical as the power supplies are reduced from ±15V to

±5V.  In all cases, the digital threshold is referenced to

GND.

The digital inputs can also be driven with CMOS-logic levels swinging from either V+ to V- or from V+ to GND.

Table 2a.  Typical Off-Isolation

Rejection Ratio

| Frequency | 100kHz | 500kHz | 1MHz |
| --- | --- | --- | --- |
| One Channel Driven | 74dB | 72dB | 66dB |
| All Channels Driven | 64dB | 48dB | 44dB |

Test Conditions:   VIN = 20VP-P at the tabulated frequency, RL = 1.5k between OUT and GND, EN = 0V.

The digital input current is just a few nanoamps of leak- age at all input voltage levels, with a guaranteed maxi- mum of 1µA.  The digital inputs are protected from ESD by a 30V zener diode between the input and V+, and can be driven ±4V beyond the supplies without drawing

OIRR = 20 Log

20VP-P

VOUT (P-P)

excessive current.

Operation as a Demultiplexer

Table 2b.  Typical Crosstalk

Rejection Ratio

MAX378/MAX379

The MAX378/MAX379 will function as a demultiplexer,

where the input is applied to the OUT pin, and the input pins are used as outputs.   The MAX378/MAX379 pro- vide both break-before-make action and full fault protec- tion when operated as a demultiplexer, unlike earlier generations of fault-protected multiplexers.

Channel-to-Channel Crosstalk, Off  Isolation, and  Digital Feedthrough

At  DC  and  low  frequencies,  channel-to-channel

crosstalk is caused by variations in output leakage cur-

| Frequency | 100kHz | 500kHz | 1MHz |
| --- | --- | --- | --- |
| FL = 1.5k | 70dB | 68dB | 64dB |
| RL = 10k | 62dB | 46dB | 42dB |

Test Conditions:   Specified RL connected  from OUT to GND, EN = +5V, A0 = A1 = A2 = +5V (Channel 1 selected).  20VP-P at the tabulated frequency is applied  to Channel 2.  All other channels are open circuited.  Similar crosstalk rejection can be observed between any two channels.

Pin  Configurations (continued)

TOP VIEW

A0    1

EN    2

N.C.    3

N.C.    4

24   A1

23   A2

22    GND

21   N.C.

A0    1

EN    2

N.C.    3

N.C.    4

24   A1

23   N.C.

22    GND

21   N.C.

V-    5

IN1    6

MAX378

20   V+

19   IN5

V-    5

IN1A    6

MAX379

20   V+

19   IN1B

IN2    7

IN3    8

18    IN6

17   N.C.

IN2A    7

IN3A    8

18    IN2B

17   IN3B

IN4    9

16   IN7

IN4A    9

16   IN4B

N.C.   10

N.C.  11

15   N.C.

14   N.C.

N.C.   10

N.C.  11

15   N.C.

14   N.C.

OUT  12

13   IN8

OUTA  12

13    OUTB

A0

N.C.

20 A1

19 A2

3

EN

2

A0

1

N.C.

20 A1

SO	SO

3

EN

2

V- 4

18  GND

V-  4

18  V+

IN1 5

N.C. 6

IN2 7

1

MAX378

17  V+

16  N.C.

15  IN5

IN1A 5

N.C. 6

IN2A 7

N.C.  11

MAX379

19 GND

17  IN1B

16  N.C.

15  IN2B

IN4     9

OUT  10

N.C.  11

IN7   13

IN3 8

IN8   12

OUTB   12

14  IN6

IN4A

9

OUTA  10

IN4B   13

IN3A 8

14  IN3B

MAX378/MAX379

LCC	LCC

_Ordering Information (continued) PART TEMP. RANGE PIN-PACKAGE

MAX379CPE 0°C to +70°C 16 Plastic DIP

MAX379CWG 0°C to +70°C 24 Wide SO

MAX379CJE 0°C to +70°C 16 CERDIP

MAX379C/D 0°C to +70°C Dice**

MAX379EPE -40°C to +85°C 16 Plastic DIP

MAX379EWG -40°C to +85°C 24 Wide SO

MAX379EJE -40°C to +85°C 16 CERDIP

MAX379MJE -55°C to +125°C 16 CERDIP

MAX379MLP -55°C to +125°C 20 LCC*

Chip Topographies

MAX378

IN8  OUT

IN4

IN7

* Contact factory for availability.

**The substrate may be allowed to float or be tied to V+ (JI CMOS).

IN7

IN6

IN3

IN2

0.229"

(5.816mm)

IN5

V+

IN1

V-

GND

A2	A1	A0	EN

0.151"

(3.835mm)

NOTE:   Connect substrate to V+ or leave it floating.

MAX379

OUTB   OUTA

IN4A

IN4B

IN3B

IN3A

0.229"

IN2B

(5.816mm)

IN2A

IN1B V+

IN1A V-

MAX378/MAX379

GND

A1	A0	EN

0.151"

(3.835mm)

NOTE:   Connect substrate to V+ or leave it floating.

MAX378/MAX379

| D 0°- 8° A 0.101mm e B 0.004in. A1 C L | DIM | INCHES |  |  |  | MILLIMETERS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MIN |  | MAX |  | MIN |  | MAX |  |
|  | A | 0.093 |  | 0.104 |  | 2.35 |  | 2.65 |  |
|  | A1 | 0.004 |  | 0.012 |  | 0.10 |  | 0.30 |  |
|  | B | 0.014 |  | 0.019 |  | 0.35 |  | 0.49 |  |
|  | C | 0.009 |  | 0.013 |  | 0.23 |  | 0.32 |  |
|  | E | 0.291 |  | 0.299 |  | 7.40 |  | 7.60 |  |
|  | e | 0.050 |  |  |  | 1.27 |  |  |  |
|  | H | 0.394 |  | 0.419 |  | 10.00 |  | 10.65 |  |
|  | L | 0.016 |  | 0.050 |  | 0.40 |  | 1.27 |  |
| E H Wide SO SMALL-OUTLINE PACKAGE (0.300 in.) | DIM | PINS | INCHES |  |  |  | MILLIMETERS |  |  |
|  |  |  | MIN |  | MAX |  | MIN |  | MAX |
|  | D | 16 | 0.398 |  | 0.413 |  | 10.10 |  | 10.50 |
|  | D | 18 | 0.447 |  | 0.463 |  | 11.35 |  | 11.75 |
|  | D | 20 | 0.496 |  | 0.512 |  | 12.60 |  | 13.00 |
|  | D | 24 | 0.598 |  | 0.614 |  | 15.20 |  | 15.60 |
|  | D | 28 | 0.697 |  | 0.713 |  | 17.70 |  | 18.10 |
| 21-0042A |  |  |  |  |  |  |  |  |  |

| D1 E D E1 A3 A A2 | DIM | INCHES |  | MILLIMETERS |  |
| --- | --- | --- | --- | --- | --- |
|  |  | MIN | MAX | MIN | MAX |
|  | A | – | 0.200 | – | 5.08 |
|  | A1 | 0.015 | – | 0.38 | – |
|  | A2 | 0.125 | 0.150 | 3.18 | 3.81 |
|  | A3 | 0.055 | 0.080 | 1.40 | 2.03 |
|  | B | 0.016 | 0.022 | 0.41 | 0.56 |
|  | B1 | 0.050 | 0.065 | 1.27 | 1.65 |
|  | C | 0.008 | 0.012 | 0.20 | 0.30 |
|  | D | 0.745 | 0.765 | 18.92 | 19.43 |
|  | D1 | 0.005 | 0.030 | 0.13 | 0.76 |
|  | E | 0.300 | 0.325 | 7.62 | 8.26 |
|  | E1 | 0.240 | 0.280 | 6.10 | 7.11 |
|  | e | 0.100 | BSC | 2.54 | BSC |
|  | eA | 0.300 | BSC | 7.62 | BSC |
|  | eB | – | 0.400 | – | 10.16 |
|  | L | 0.115 | 0.150 | 2.92 | 3.81 |
|  |  | 0˚ | 15˚ | 0˚ | 15˚ |
| 21-587A L A1 16-PIN PLASTIC e C DUAL-IN-LINE B1 eA PACKAGE B eB |  |  |  |  |  |

Maxim cannot assume responsibility for use of any circuitry other than circuitry entirely embodied in a Maxim product.  No circuit patent licenses are implied. Maxim reserves the right to change the circuitry and specifications without notice at any time.

12    	Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA   94086 (408) 737-7600

©  1994 Maxim Integrated Products 	Printed USA	  is a registered trademark of Maxim Integrated Products.