19-0198; Rev 0; 10/93

+3.3V-Powered, EIA/TIA-562 Dual Transceiver with Receivers Active in Shutdown

General Description

The MAX563 is a +3.3V-powered EIA/TIA-562 transceiver with two transmitters and two receivers. Because it imple- ments the EIA/TIA-562 standard, the MAX563 communi- cates with RS-232 transceivers, yet consumes far less power; this makes it ideal for battery-powered,  hand-held computers. And, the MAX563 guarantees a 116kbps data rate while maintaining ±3.7V EIA/TIA-562 signal levels, which makes it compatible with LapLink™ software.

An on-board charge pump converts the +3.3V supply to the ±6.6V needed to produce the EIA/TIA-562 output volt- age levels. Four 0.1µF charge-pump  capacitors  and a bypass capacitor of similar size are the only external com- ponents required.

When the MAX563’s charge pumps and transmitters are shut down to save power, the receivers remain active to continuously monitor signals from external devices (for example, ring indicator from modems). The two receivers’ outputs can be enabled and disabled independently of the shutdown function to allow two ports—generally of different types—to be wire-OR connected at the UART.

Applications

Handiterminals

Battery-Powered Equipment

Bar-Code Readers

Notebook and Palmtop Computers

Typical Operating Circuit

+3.3V	C3

Features

♦ Guaranteed Interoperability with RS-232

♦ Operates from a Single +3.0V to +3.6V Supply

♦ 2 Drivers, 2 Receivers

♦ Receivers Active in Shutdown Mode

♦ Low-Power Shutdown: 10µA Max

♦ Small Package—18-Pin Wide SO

♦ Three-State TTL/CMOS Receiver Outputs

♦ 116kbps Guaranteed Data Rate

Ordering Information

PART TEMP. RANGE PIN-PACKAGE

MAX563CPN 0°C to +70°C 18 Plastic DIP

MAX563CWN 0°C to +70°C 18 Wide SO

MAX563C/D 0°C to +70°C Dice *

* Dice are specified at TA = +25°C.

Pin  Configuration

2  C1+ C1

4   C1-

5   C2+

17

VCC VOLTAGE DOUBLER

ALL CAPACITORS 0.1µF

+6.6V V+   3

-6.6V

TOP VIEW

EN  1

18   SHDN

C2	6

C2-

7

VOLTAGE INVERTER    V-	C4

C1+  2

V+    3

17  VCC

16   GND

CMOS INPUTS

12   T1IN

400k

400k

VCC

VCC

T1OUT  15

RS-562

OUTPUTS

C1-  4

C2+    5

C2-  6

MAX563

15  T1OUT

14  R1IN

13  R1OUT

11   T2IN

T2OUT  8

V-  7

12  T1IN

CMOS

13   R1OUT

R1IN  14

5k

RS-562/RS-232

T2OUT   8

R2IN  9

11  T2IN

10  R2OUT

OUTPUTS

10   R2OUT

R2IN   9

INPUTS

1   EN

5k

GND

SHDN 18

ON/OFF

DIP/SO

MAX563

™ LapLink is a registered trademark of Traveling Software, Inc.

ABSOLUTE MAXIMUM RATINGS

Power Supply Ranges

Supply Voltage (VCC) ...........................................-0.3V to +6V Input Voltages

TIN.........................................................-0.3V to (VCC -0.3V) RIN ...............................................................................±25V TOUT (Note 1) ...................................................................±15V

Output Voltages TOUT.............................................................................±15V ROUT ....................................................-0.3V to (VCC +0.3V)

Driver/Receiver Output Short Circuit to GND .........Continuous

Continuous Power Dissipation  (TA = +70°C)

Plastic DIP ....................................................................889mW

Wide SO .......................................................................762mW Operating  Temperature  Range...............................0°C to +70°C Storage Temperature  Range .............................-65°C to +160°C Lead Temperature  (soldering,  10sec) .............................+300°C

MAX563

Note 1: Input voltage measured  with TOUT in high-impedance state.  SHDN  or VCC = 0V.

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions  beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS

(VCC = 3.0V to  3.6V, C1-C4 = 0.1µF, TA = TMIN to TMAX, unless otherwise noted.)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| RS-562 TRANSMITTERS |  |  |  |  |
| Output Voltage Swing |  | All transmitter outputs loaded with 3kΩ to GND | ±3.7 ±4.5 | V |
| Data Rate |  | RL = 3kΩ, CL = 1000pF, TA = +25°C | 200 116 | kbps |
| Input Logic Threshold Low | VIL |  | 0.6 | V |
| Input Logic Threshold High | VIH |  | 2.4 | V |
| Logic Pull-Up/Input Current |  | SHDN = VCC | 2 20 | µA |
|  |  | SHDN = 0V | ±0.01 ±1 |  |
| Output Leakage Current |  | VCC = 3.6V, SHDN = 0V, VOUT = ±15V | ±0.01 ±10 | µA |
|  |  | VCC = SHDN = 0V, VOUT = ±15V | ±0.01 ±10 |  |
| Transmitter Output Resistance |  | VCC = V+ = V- = 0V, VOUT = ±2V | 300 10M | Ω |
| Output Short-Circuit Current |  | VOUT = 0V | ±15 ±60 | mA |
| RS-232/RS-562 RECEIVERS |  |  |  |  |
| Input Voltage Operating Range |  |  | ±25 | V |
| Input Threshold Low | VIL | VCC = 3.3V | 0.4 | V |
| Input Threshold High | VIH | VCC = 3.3V | 2.4 | V |
| Input Hysteresis |  | SHDN = VCC = 3.3V (no hysteresis when SHDN = 0V) | 0.1 0.5 1.0 | V |
| Input Resistance | RIN |  | 3 5 7 | kΩ |
| Output Voltage Low | VOL | IOUT = 3.2mA | 0.2 0.4 | V |
| Output Voltage High | VOH | IOUT = -0.5mA | VCC-0.6 VCC-0.2 | V |
| Output Leakage Current |  | EN = VCC, 0V ≤ VOUT ≤ VCC | ±0.05 ±10 | µA |
| EN Input Threshold Low | VIL |  | 0.6 | V |
| EN Input Threshold High | VIH |  | 2.4 | V |

MAX563

ELECTRICAL CHARACTERISTICS (continued)

(VCC = 3.0V to  3.6V, C1-C4 = 0.1µF, TA = TMIN to  TMAX, unless otherwise noted.)

| PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| POWER SUPPLY |  |  |  |  |  |
| Operating Supply Voltage | VCC |  |  | 3.0 3.6 | V |
| VCC Supply Current | ICC | No load |  | 3 8 | mA |
|  |  | RL = 3kΩ on both outputs, inputs static |  | 8 |  |
| Shutdown Supply Current | ICC | Figure 1 | TA = +25°C | 0.1 10 | µA |
|  |  |  | TA = TMIN to TMAX | 2 50 |  |
| SHDN Input Leakage Current |  |  |  | 0.1 ±1 | µA |
| SHDN Threshold Low | VIL |  |  | 0.6 | V |
| SHDN Threshold High | VIH |  |  | 2.4 | V |
| AC CHARACTERISTICS |  |  |  |  |  |
| Transition Slew Rate |  | CL = 50pF to 2500pF, RL = 3kΩ to 7kΩ, VCC = 3.3V, TA = +25°C, measured from +3V to -3V or -3V to +3V (Note 2) |  | 4 6 30 | V/µs |
| Transmitter Propagation Delay | tPHLT | Figure 2 |  | 1.3 3.5 | µs |
|  | tPLHT |  |  | 1.5 3.5 |  |
| Receiver Propagation Delay (Normal Operation) | tPHLR | Figure 3 |  | 0.5 1.0 | µs |
|  | tPLHR |  |  | 0.6 1.0 |  |
| Receiver Propagation Delay (Shutdown) | tPHLS | Figure 3 |  | 0.5 10.0 | µs |
|  | tPLHS |  |  | 2.5 10.0 |  |
| Receiver-Output Enable Time | tER | Figure 4 |  | 125 500 | ns |
| Receiver-Output Disable Time | tDR | Figure 4 |  | 160 500 | ns |
| Transmitter-Output Enable Time | tET | Includes charge pump start-up |  | 300 | µs |
| Transmitter-Output Disable Time | tDT | Figure 5 |  | 600 | ns |
| Transmitter Propagation Delay Skew | tPHLT – tPLHT |  |  | 300 | ns |
| Receiver Propagation Delay Skew (Normal Operation) | tPHLR – tPLHR |  |  | 100 | ns |

Note 2: Minimum slew rate is specified with CL = 1000pF for data rates above 20kbps, corresponding with EIA/TIA-562.

Pin  Description

| PIN | NAME | FUNCTION |
| --- | --- | --- |
| 1 | EN | Receiver enable. Connect EN to GND to enable receivers, and take EN high to disable receivers. |
| 2 | C1+ | Positive terminal of positive charge-pump capacitor. |
| 3 | V+ | +2VCC voltage generated by the positive charge pump. The voltage on V+ collapses to VCC when SHDN is low. |
| 4 | C1- | Negative terminal of positive charge-pump capacitor. |
| 5 | C2+ | Positive terminal of inverting charge-pump capacitor. |
| 6 | C2- | Negative terminal of inverting charge-pump capacitor. |
| 7 | V- | -2VCC voltage generated by the inverting charge pump. The voltage on V- collapses to GND when SHDN is low. |
| 8, 15 | T2OUT, T1OUT | EIA/TIA-562 voltage-level transmitter outputs. These outputs are disabled (Hi-Z) when SHDN is low. |
| 9, 14 | R2IN, R1IN | EIA/TIA-562 and EIA/TIA-232 voltage-level receiver inputs. |
| 10, 13 | R2OUT, R1OUT | CMOS receiver outputs. These outputs are active regardless of the state of SHDN. They are enabled when EN is low, and disabled (Hi-Z) when EN is high. |
| 11, 12 | T2IN, T1IN | CMOS driver inputs. |
| 16 | GND | Ground. |
| 17 | VCC | +3.0V to +3.6V supply voltage. |
| 18 | SHDN | Shutdown control. Connect to GND to shut down the charge pumps and the transmitters. Take high to turn on the charge pumps and to enable the transmitters. |

MAX563

Detailed Description

The MAX563 consists of three sections: charge-pump voltage converters, transmitters (drivers), and receivers. Both the transmitters and the receivers are inverting.

+3V  to ±6V, Dual Charge-Pump Voltage

Converter Two charge pumps either invert or double the incoming VCC to generate the voltages required by the transmit- ters. The first charge  pump  uses capacitor  C1 to double VCC; the resulting voltage is stored on the V+ reservoir capacitor.  The second  charge  pump  uses capacitor  C2 to invert V+; this negative voltage is stored on the V- capacitor.

When SHDN  is low, the charge pumps are turned off, V+ is pulled  down to VCC by a 1k resistor, and V- rises to GND.

It is possible to draw some power from the V+ and V- pins for external use. However, doing so diminishes the charge-pump  output voltages and reduces noise mar- gins, so it is not recommended.

EIA/TIA-562 Transmitters The MAX563’s drivers are inverting level translators that convert +3V logic inputs to EIA/TIA-562 voltage levels. With a VCC supply  of only 3.0V, the driver outputs deliver the EIA/TIA-562 ±3.7V minimum specification under worst-case conditions—when  both transmitters are loaded  with 3k receivers (either EIA/TIA-232 or EIA/TIA-562).

The transmitters are fast: the guaranteed data rate with standard loads is 116kbps,  which is the highest rate commonly used by PC-to-PC communication software, such as LapLink. The highest practical  data rate may be reduced if the other communicating device (RS-562 or RS-232) is not as fast, or if the cables present an excessive capacitive  load (>1000pF). Each transmitter is designed  to drive a single receiver; transmitters can be paralleled to drive multiple receivers.

When SHDN  is low, the driver outputs are turned off. Their output leakage currents are less than 10µA when pulled  to GND or when driven to ±15V. This enables two transmitters to be connected to the same line, pro- vided  that one of them is always disabled  by taking SHDN low.

+3.6V

0.1µF

0.1µF

0.1µF

ISHDN

C1+ C1- C2+

VCC

VCC

MAX563

0.1µF

V+ V-

0.1µF

INPUT

OUTPUT

+3V

OV

V+ OV V-

+3.6V

C2-

TIN

400k

T1 & T2

TOUT

tPLHT

tPHLT

0V +3.6V DRIVE

ROUT

EN

SHDN

R1 & R2

GND

RIN

5k

3k

+3.6V

Figure 2. Transmitter Propagation Delay Timing

+3V

INPUT

50%

50%

0V

Figure 1. Shutdown Current Test Circuit

The inputs of unused drivers may be left unconnected because  they have internal 400k pull-ups  to VCC. Unused inputs may also be connected to GND or VCC, but VCC provides lower power consumption because of the internal pull-ups.

EIA/TIA-562 and EIA/TIA-232 Receivers

OUTPUT

tPHLR

tPHLS

50%

50% GND

tPLHR

tPLHS

VCC

MAX563

The MAX563’s receivers  convert ±3.7V to ±13.2V EIA/TIA-562 signal levels into +3V logic levels; they are rated to receive signals up to ±25V to accommodate EIA/TIA-232 signals as well. Both receivers invert.  Their inputs are each equipped  with an internal 5k (nomi- nal) terminating resistor connected  to ground, and the input logic thresholds are 0.4V and 2.4V. The positive logic-low threshold (VIL) ensures the receiver outputs remain high whenever their inputs are left open.

The receivers are active when EN  is low, and have high-impedance  outputs when EN is high.

When SHDN  is high, the receivers have hysteresis. This produces  clean output transitions, even with slow- moving input signals that exhibit moderate amounts of noise and ringing. When shut down, the receivers have no hysteresis, and the propagation delay increases.

Shutdown and Enable Control The SHDN  and EN  controls are independent.  Both receivers are always active when EN is low. With EN low and SHDN high, the receivers operate at full speed

Figure 3. Receiver Propagation Delay Timing

and have hysteresis. When active in shutdown mode (EN = SHDN  = low), the receivers operate at reduced power and speed, and without hysteresis.

The charge pumps and transmitters operate only when SHDN  is high; they are unaffected  by EN. When shut down (SHDN  = low) or unpowered  (VCC = 0V), the transmitter outputs are high impedance  if they are backdriven with voltages not exceeding ±15V. The pull- up resistors at the driver inputs are disconnected  in shutdown mode to save power. During shutdown, V+ is pulled down to VCC, and V- rises to GND.

EN INPUT

VCC

RECEIVER OUTPUT

CL = 150pF

EN	0V INPUT

0V

tER

VCC - 0.6V

+0.4V VCC

tDR

+3V SHDN

V+

V-

0V

tDT

+3.7V

0V

-3.7V

RECEIVER OUTPUTS

VOH - 0.1V

RL = 1k

VOL + 0.1V

0.5VCC

A. TIMING DIAGRAM

1 OR 0 	TX

Figure 4. Receiver Output Enable and Disable Timing

B. TEST CIRCUIT

3k 	50pF

Applications Information

Charge-Pump Capacitor Selection Normally, 0.1µF capacitors  can be used for all loca- tions. To reduce output ripple,  increase the values of the V+ and V- capacitors.  If your capacitors  have a very wide tolerance, consider using a nominal value a little larger than 0.1µF to ensure that the actual capaci- tance does not fall below about 80nF. For circuits designed  to operate over a wide range of tempera- tures, consider using slightly larger capacitors to com- pensate for any loss of capacitance  at temperature extremes. No advantage  is gained  by using values larger than 10µF.

Figure 5. Transmitter Output Disable Timing

MAX563

Power-Supply Decoupling Use a capacitor  with the same value as the charge- pump capacitors.

MAX563

Table 1.  Summary of EIA/TIA-232E/V.28 and EIA/TIA-562 Specifications

| PARAMETER |  | CONDITION | EIA/TIA-232E/V.28 SPECIFICATION | EIA/TIA-562 SPECIFICATION |
| --- | --- | --- | --- | --- |
| Dr | iver Output Voltage 0 Level | 3kΩ to 7kΩ load | 5.0V to 15.0V | 3.7V to 13.2V |
|  | 1 Level |  | -5.0V to -15.0V | -3.7V to -13.2V |
| Maximum Output Level |  | No load | ±25V | ±13.2V |
| Signal Rate (3kΩ ≤ RL ≤ 7kΩ) |  | CL = 2500pF | Up to 20kbps | Up to 20kbps |
|  |  | CL = 1000pF | Not defined | Up to 64kbps |
| Re | ceiver Input Thresholds 0 Level |  | 3.0V to 15.0V | 3.0V to 15.0V |
|  | 1 Level |  | -3.0V to -15.0V | -3.0V to -15.0V |
| Maximum Input Level |  |  | ±25V | ±25V |
| Maximum Instantaneous Slew Rate |  |  | 30V/µs | 30V/µs |
| Maximum Driver Output Short-Circuit Current |  |  | 100mA | 60mA |
| Transition Rate on Driver Output |  |  | V.28 1ms or 3% of the period | 4V/µs |
|  |  |  | RS-232 4% of the period |  |
| Driver Output Resistance with Power Off |  | -2V < VOUT < 2V | 300Ω | 300Ω |

Table 2.  DB9/DB25 Cable Connections Commonly Used for EIA/TIA-232E, EIA/TIA-562 and V.24

Asynchronous Interfaces

| DB9 PIN | DB25 PIN | NAME | SYMBOL | FUNCTION |
| --- | --- | --- | --- | --- |
| 1 | 8 | Received Line Signal Detector (some- times called Data Carrier Detect) | DCD | Handshake from DCE |
| 2 | 3 | Receiver Data | RxD | Data from DCE |
| 3 | 2 | Transmit Data | TxD | Data from DTE |
| 4 | 20 | Data Terminal Ready | DTR | Handshake from DTE |
| 5 | 7 | Signal Ground | GND | Reference point for signals |
| 6 | 6 | Data Set Ready | DSR | Handshake from DCE |
| 7 | 4 | Request to Send | RTS | Handshake from DTE |
| 8 | 5 | Clear to Send | CTS | Handshake from DCE |
| 9 | 22 | Ring Indicator | RI | Handshake from DCE |

Chip Topography

V+   C1+

SHDN

GND

EN	V CC

C1-

C2+

T1 OUT

C2- V-

0.105"

(2.667  mm)

R1 IN

R1 OUT

T2 OUT

T1 IN

R2 IN

R2 OUT

0.070"

T2 IN

MAX563

(1.778  mm)

TRANSISTOR COUNT: 189;

SUBSTRATE CONNECTED TO V+.

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses  are implied. Maxim reserves the right to change the circuitry and specifications  without notice at any time.

8   	Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA  94086 (408) 737-7600

©  1993 Maxim Integrated Products	Printed USA	is a registered trademark of Maxim Integrated Products.