19-1146; Rev 3; 3/00

1.2A, Current-Limited, High-Side

P-Channel Switch with Thermal Shutdown

General Description

The MAX890L smart, low-voltage, P-channel, MOSFET power switch is intended for high-side load-switching applications.  This switch operates with inputs from

+2.7V to +5.5V, making it ideal for both +3V and +5V systems. Internal current-limiting circuitry protects the input supply against overload. Thermal-overload pro- tection limits power dissipation and junction tempera- tures.

The MAX890L’s maximum current limit is 1.2A. The cur- rent limit through the switch is programmed  with a resistor from SET to ground. The quiescent supply cur- rent is a low 10µA. When the switch is off, the supply current decreases to 0.1µA.

The MAX890L is available in an 8-pin SO package.

Features

♦ +2.7V to +5.5V Input Range

♦ Programmable Current Limit

♦ Low Quiescent Current

10µA (typ) at VIN = +3.3V

0.1µA (typ) with Switch Off

♦ Thermal Shutdown

♦ FAULT Indicator Output

♦ 0.09Ω (typ) On-Resistance

Applications

Ordering Information

PCMCIA Slots

Access Bus Slots

PART*

TEMP. RANGE

PIN- PACKAGE

CURRENT LIMIT

Portable Equipment

MAX890LC/D 	0°C to +70°C

Dice**

1.2A

MAX890LESA 	-40°C to +85°C 	8 SO

1.2A

* To order this unit in tape and reel, add (-T) to the end of the part number.

** Dice are tested at TA = +25°C.

Typical Operating Circuit

Pin  Configuration

TOP VIEW

INPUT

IN

IN

100kΩ

MAX890L

OUT OUT

OUTPUT

0.1µF

IN   1

IN   2

MAX890L

8    FAULT

7    OUT

1µF

FAULT	ON	ON/OFF

GND 	SET

ON   3

GND   4

6    OUT

5    SET

MAX890L

RSET

SO

ABSOLUTE MAXIMUM RATINGS

IN to GND ................................................................-0.3V to +6V ON, FAULT to GND  .................................................-0.3V to +6V SET, OUT to GND ...................................... -0.3V to (VIN + 0.3V) Maximum Continuous Switch Current ..................................1.5A Continuous Power Dissipation (TA = +70°C)

SO (derate 5.88mW/°C above +70°C)  ..........................471mW

Operating Temperature Range

MAX890LESA ....................................................-40°C to +85°C Storage Temperature Range  ...........................  -65°C to +150°C Lead Temperature (soldering,  10s) . ...............................+300°C

MAX890L

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS

(VIN = +3V, TA = 0°C to +85°C, unless otherwise noted. Typical values are at TA = +25°C.)

| µs PARAMETER | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- |
| Operating Voltage |  | 2.7 5.5 | V |
| Quiescent Current | VIN = 5V, ON = GND, IOUT = 0 | 13 20 | µA |
| Off-Supply Current | ON = IN, VIN = VOUT = 5.5V | 0.03 1 | µA |
| Off-Switch Current | ON = IN, VIN = 5.5V, VOUT = 0 | 0.04 15 | µA |
| Undervoltage Lockout | Rising edge, 1% hysteresis | 2.0 2.4 2.6 | V |
| On-Resistance | VIN = 4.5V | 75 130 | mΩ |
|  | VIN = 3.0V | 90 150 |  |
| Current-Limit-Amplifier Threshold | VSET required to turn the switch off (Note 1) | 1.178 1.240 1.302 | V |
| Maximum Output Current Limit |  | 1.2 | A |
| IOUT to ISET Current Ratio | IOUT = 500mA, VOUT > 1.6V | 970 1110 1300 | A/A |
| ON Input Low Voltage | VIN = 2.7V to 5.5V | 0.8 | V |
| ON Input High Voltage | VIN = 2.7V to 3.6V | 2.0 | V |
|  | VIN = 4.5V to 5.5V | 2.4 |  |
| ON Input Leakage Current | V ON = 5.5V | 0.01 1 | µA |
| ISET Bias Current | VSET = 1.24V, IOUT = 0; VIN = VOUT | 0.5 3 | µA |
| FAULT Logic Output Low Voltage | ISINK = 1mA, VSET = 1.4V | 0.4 | V |
| FAULT Logic Output High Leakage Current | V FAULT = 5.5V, VSET = 1V | 0.05 1 | µA |
| Slow-Current-Loop Response Time | 20% current overdrive, VCC = 5V | 5 | µs |
| Fast-Current-Loop Response Time |  | 2 | µs |
| Turn-On Time | VIN = 5V, IOUT = 500mA | 120 200 |  |
|  | VIN = 3V, IOUT = 500mA | 185 |  |
| Turn-Off Time | VIN = 5V | 2 5 | µs |

MAX890L

ELECTRICAL CHARACTERISTICS

(VIN = +3V, TA = -40°C to +85°C, unless otherwise noted.) (Note 2)

| PARAMETER | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- |
| Operating Voltage |  | 3.0 5.5 | V |
| Quiescent Current | VIN = 5V, ON = GND, IOUT = 0 | 50 | µA |
| Off-Supply Current | ON = IN, VIN = VOUT = 5.5V | 2.2 | µA |
| Off-Switch Current | ON = IN, VIN = 5.5V, VOUT = 0 | 15 | µA |
| Undervoltage Lockout | Rising edge, 1% hysteresis | 2.0 2.9 | V |
| On-Resistance | VIN = 4.5V | 130 | mΩ |
|  | VIN = 3.0V | 150 |  |
| Current-Limit-Amplifier Threshold | VSET required to turn the switch off (Note 1) | 1.14 1.34 | V |
| IOUT to ISET Current Ratio | IOUT = 500mA, VOUT > 1.6V | 925 1390 | A/A |
| FAULT Logic Output Low Voltage | ISINK = 1mA, VSET = 1.4V | 0.4 | V |
| Turn-On Time | VIN = 5V | 200 | µs |
| Turn-Off Time | VIN = 5V | 1 20 | µs |

Note 1: Tested with IOUT = 100mA and VSET raised until VIN - VOUT ≥ 0.8V.

Note 2: Specifications to -40°C are guaranteed  by design, not production  tested.

Typical Operating Characteristics

(TA = +25°C, unless otherwise noted.)

QUIESCENT CURRENT

vs. INPUT VOLTAGE

MAX890 toc01

14

12 	ON = GND IOUT = 0

10

QUIESCENT CURRENT

vs. TEMPERATURE

14

QUIESCENT CURRENT (µA)

13

12

MAX890 toc02

100

10

OFF-SUPPLY  CURRENT

MAX890 toc03

vs. TEMPERATURE

8	11

1

6	10

4	9

OFF-SUPPLY CURRENT (nA)

0.1

2	8

0

0	1	2	3	4	5	6

7

| -40 -20 0 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | VIN = 5V |  |
|  |  |  |  |  | VIN = 3V |  |

20	40

60	80

100

0.01

-40    -20	0

20	40	60

80      100

1000

OFF-SWITCH CURRENT (nA)

100

10

INPUT VOLTAGE (V)

OFF-SWITCH  CURRENT

vs. TEMPERATURE

MAX890 toc04

1.3

1.2

NORMALIZED RON

1.1

1.0

TEMPERATURE (°C)

MAX890 toc05

NORMALIZED ON-RESISTANCE

vs. TEMPERATURE

IOUT = 1/2ILIMIT

1400

1200

IOUT/ISET RATIO

| 1000 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

800

TEMPERATURE (°C)

MAX890 toc06

IOUT/ISET RATIO

vs. ILIMIT

1	0.9

600

0.1

0.8

0.7

400

200

0.01

-40

-20

0	20	40

60	80

100

0.6

-40    -20	0

20	40

60	80

100

0

0	20	40

60	80

100     120

140

TEMPERATURE (°C)

TEMPERATURE (°C)

ILIMIT (%)

1.2

NORMALIZED OUTPUT CURRENT

1.0

0.8

0.6

0.4

0.2

| 0 |  |  |  |  | ILIMIT |
| --- | --- | --- | --- | --- | --- |
| VIN = 5V |  |  |  |  |  |

NORMALIZED OUTPUT CURRENT

vs. OUTPUT VOLTAGE

MAX890 toc07

260

240

220

200

tON (µs)

180

160

140

120

100

80

60

TURN-ON TIME

MAX890 toc08

vs. TEMPERATURE

ILOAD = ILIMIT

VIN = 3V

VIN = 5V

TURN-OFF TIME

MAX890 toc09

vs. TEMPERATURE

8

ILOAD = ILIMIT

7

6

tOFF (µs)

5

4

3

2

1

0

VIN = 5V

VIN = 3V

0	1	2	3	4	5	6

-40    -20	0

20	40

60	80

100

-40    -20	0

20	40

60	80

100

MAX890L

QUIESCENT CURRENT (µA)

OUTPUT VOLTAGE (V)

TEMPERATURE (°C)

TEMPERATURE (°C)

Typical Operating Characteristics (continued)

(TA = +25°C, unless otherwise noted.)

OUTPUT OVER-LOAD (SLOW LOOP RESPONSE)

MAX890 toc10

OUTPUT SHORT-CIRCUIT (FAST LOOP RESPONSE)

MAX890 toc11

A	5V

B

A	5V

B

C

0V, 0A

2µs/div

CIN = 100µF,  COUT = 0.1µF A: VIN, 1V/div, AC-COUPLED

B: IOUT, 1A/div

C: VOUT, 2V/div

0A

C

0V

2µs/div

CIN = 100µF,  COUT = 0.1µF A: VIN, 1V/div, AC-COUPLED

B: IOUT, 5A/div

C: VOUT, 2V/div

LOAD-TRANSIENT RESPONSE

MAX890 toc12

SWITCH TURN-ON TIME

MAX890 toc13

A	A

0A

0V

B	B

VIN = 5V

A: IOUT = 0mA TO 750mA,

0.5A/div

5ms/div

B: VOUT RIPPLE,

200mV/div, AC COUPLED

SWITCH TURN-OFF TIME

MAX890 toc14

VIN = 5V,  IOUT = ILIMIT

A: VON, 2V/div

B: VOUT, 2V/div

20µs/div

A

0V

B

MAX890L

0V

VIN = 5V,  IOUT = ILIMIT

A: VON, 2V/div

B: VOUT, 2V/div

2µs/div

Pin  Description

| PIN | NAME | FUNCTION |
| --- | --- | --- |
| 1, 2 | IN | Input. P-channel MOSFET source. Bypass IN with a 1µF capacitor to ground. |
| 3 | ON | Active-Low Switch On Input. A logic low turns the switch on. |
| 4 | GND | Ground |
| 5 | SET | Set Current-Limit Input. A resistor from SET to ground sets the current limit for the switch. RSET = 1.38 x 103 / ILIMIT, where ILIMIT is the desired current limit in amperes. |
| 6, 7 | OUT | Switch Output. P-channel MOSFET drain. Bypass OUT with a 0.1µF capacitor to ground. |
| 8 | FAULT | Fault-Indicator Output. This open-drain output goes low when in current limit or when the die temperature exceeds +135°C. |

Detailed Description

The MAX890L P-channel MOSFET power switch limits output current to a programmed level. When the output current is increased beyond the programmed  current limit, or 1.2A (IMAX), the current also increases through the replica  switch (IOUT/1110) and through RSET (Figure 1). The current-limit error amplifier compares the voltage across RSET to the internal +1.24V refer- ence, and regulates the current back to the lesser of the programmed limit (ILIMIT) or 1.2A.

This switch is not bidirectional; therefore, the input volt- age must be higher than the output voltage.

Setting the Current Limit

IN FAULT

N

P

P

REPLICA AMPLIFIER

MAX890L

ON

P

SET

OUT

The MAX890L features internal current-limiting circuitry with a maximum programmable  value (IMAX) of 1.2A. For best performance,  set the current limit (ILIMIT) between 0.2 IMAX ≤ ILIMIT  ≤ IMAX. This current limit remains in effect throughout the input supply-voltage range.

Program the current limit with a resistor (RSET) from	ON

CONTROL

ON	CURRENT-LIMIT

AMPLIFIER

1.24V HOT

RSET

GND

MAX890L

SET to ground (Figure 2) as follows:

ISET = ILIMIT / 1110

RSET = 1.24V / I8 = 1.38 x 103 / ILIMIT

where ILIMIT is the desired current limit.

Short-Circuit Protection The MAX890L is a short-circuit-protected  switch. In the event of an output short circuit or current-overload con- dition, the current through the switch is limited by the internal current-limiting error amplifier to 1.5 x ILIMIT. When the fault condition is removed, the replica error amplifier will set the current limit back to ILIMIT.

CIRCUITRY

Figure 1.  Functional Diagram

For a high dVDS/dt during an output short-circuit condi- tion, the switch turns off and disconnects the input supply from the output. The current-limiting amplifier then slow- ly turns the switch on with the output current limited to

1.5 x ILIMIT. When the fault condition is removed, the current limit is set back to ILIMIT. Refer to Output Short- Circuit Fast-Loop Response and Output Over-Load

MAX890L

Figure 2.  Setting the Current Limit

SET

GND

RSET

most applications;  however, higher capacitor  values will further reduce the voltage drop at the input.

Output Capacitor Connect a 0.1µF capacitor  from OUT to GND. One function of this capacitor  is to prevent inductive para- sitics from pulling OUT negative during turn-off.

Layout and Thermal-Dissipation

Consideration To take full advantage of the switch-response time to output short-circuit conditions, it is very important to keep all traces as short as possible to reduce the effect of undesirable parasitic inductance.  Place input and output capacitors  as close as possible to the device (no more than 5mm).

Under normal operating conditions, the package  can dissipate and channel heat away. Calculate the maxi- mum power as follows:

MAX890L

Slow-Loop  Response  in  the  Typical  Operating

Characteristics.

Thermal Shutdown The MAX890L features thermal shutdown. The switch turns off when the junction temperature exceeds

+135°C. Once the device cools by 10°C, the switch turns back on. If the fault short-circuit condition is not removed, the switch will cycle on and off, resulting in a pulsed output.

Fault Indicator The MAX890L provides  a fault output (FAULT). This open-drain output goes low when in current limit or when the die temperature exceeds +135°C. A 100kΩ pull-up resistor from FAULT to IN provides  a logic- control signal.

Applications Information

Input Capacitor To limit the input voltage drop during momentary output short-circuit conditions, connect a capacitor from IN to GND. A 1µF ceramic capacitor  will be adequate for

P = I2LIMIT x RON

where RON is the on-resistance of the switch.

When the output is short circuited,  the voltage drop across the switch equals the input supply. Hence, the power dissipated across the switch increases, as does the die temperature. If the fault condition  is not removed, the thermal-overload-protection circuitry turns the switch off until the die temperature falls by 10°C. A ground plane in contact with the device will help dissi- pate additional heat.

Chip Information

TRANSISTOR COUNT: 396

MAX890L

SOICN.EPS

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses are implied. Maxim reserves the right to change the circuitry and specifications without notice at any time.

8    	Maxim Integrated Products, 120 San Gabriel Drive, Sunnyvale, CA   94086 408-737-7600

©  2000 Maxim Integrated  Products 	Printed USA 	is a registered  trademark of Maxim Integrated  Products.