19-1450; Rev 0; 7/99

1.2A, Current-Limited, High-Side

P-Channel Switch with Thermal Shutdown

General Description

The MAX893L smart, low-voltage, P-channel, MOSFET power switch is intended for high-side load-switching applications.  This switch operates with inputs from

+2.7V to +5.5V, making it ideal for both +3V and +5V systems. Internal current-limiting circuitry protects the input supply against overload. Thermal overload pro- tection limits power dissipation and junction tempera- ture.

The MAX893L’s maximum programmed  current limit is

1.2A. The typical short-circuit current is 1.5 times the programmed  current; therefore, a 1.2A programmed limit will result in a 1.8A short-circuit current limit. The current limit through the switch is programmed  with a resistor from SET to ground. The quiescent supply cur- rent is a low 13µA. When the switch is off, the supply current decreases to 0.1µA.

The MAX893L is available in an 8-pin SO package.

Features

♦ +2.7V to +5.5V Input Range

♦ Programmable Current Limit

0.2A to 1.2A Range

±20% Accuracy

♦ 1.2A Continuous Load Current

♦ 1.8A Short-Circuit Current

♦ Low Quiescent Current

13µA at VIN = 3.3V

0.1µA with Switch Off

♦ Thermal Shutdown

♦ FAULT Indicator Output

♦ 0.07 On-Resistance

Applications

USB Ports USB Hubs PCMCIA Slots Access Bus Slots

Portable Equipment

Ordering Information

PART 	TEMP. RANGE 	PIN-PACKAGE

MAX893LESA 	-40°C to +85°C 	8 SO

Typical Operating Circuit

Pin  Configuration

TOP VIEW

INPUT

IN

IN

100k

MAX893L

OUT

OUT

OUTPUT

0.1  F*

IN   1

8    FAULT

FAULT	ON	ON/OFF

1  F	GND 	SET

IN   2

ON   3

GND   4

MAX893L

7    OUT

6    OUT

5    SET

MAX893L

RSET

SO

*USB SPECIFICATIONS  REQUIRE HIGHER CAPACITANCE.

ABSOLUTE MAXIMUM RATINGS

IN to GND ................................................................-0.3V to +6V ON, FAULT to GND  .................................................-0.3V to +6V SET, OUT to GND ...................................... -0.3V to (VIN + 0.3V) Maximum Continuous Short-Circuit Switch Current .............2.0A Continuous Power Dissipation (TA = +70°C)

SO (derate 5.88mW/°C above +70°C)  .........................471mW

Operating Temperature Range

MAX893LESA ...................................................-40°C to +85°C Storage Temperature Range  ...........................  -65°C to +150°C Lead Temperature (soldering,  10sec) .............................+300°C

MAX893L

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS

(VIN = 3V, TA = 0°C to +85°C, unless otherwise noted. Typical values are at TA = +25°C.)

| PARAMETER | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- |
| Operating Voltage |  | 2.7 5.5 | V |
| Quiescent Current | VIN = 5V, ON = GND, IOUT = 0 | 13 20 | µA |
| Off-Supply Current | ON = IN, VIN = VOUT = 5.5V | 0.03 1 | µA |
| Off-Switch Current | ON = IN, VIN = 5.5V, VOUT = 0 | 0.04 15 | µA |
| Undervoltage Lockout | Rising edge, 1% hysteresis | 2.0 2.4 2.6 | V |
| On-Resistance | VIN = 4.5V | 70 125 | mΩ |
|  | VIN = 3.0V | 88 160 |  |
| Current-Limit-Amplifier Threshold | VSET required to turn the switch off (Note 1) | 1.178 1.240 1.302 | V |
| Maximum Programmable Continuous Output Current Limit |  | 1.2 | A |
| Short-Circuit Current |  | 1.8 | A |
| IOUT to ISET Current Ratio | IOUT = 500mA, VOUT > 1.6V | 920 1080 1250 | A/A |
| ON Input Voltage Low | VIN = 2.7V to 5.5V | 0.8 | V |
| ON Input Voltage High | VIN = 2.7V to 3.6V | 2.0 | V |
|  | VIN = 4.5V to 5.5V | 2.4 |  |
| ON Input Leakage Current | V ON = 5.5V | 0.01 1 | µA |
| ISET Bias Current | VSET = 1.24V, IOUT = 0; VIN = VOUT | 0.5 3 | µA |
| FAULT Logic Output Voltage Low | ISINK = 1mA, VSET = 1.4V | 0.4 | V |
| FAULT Logic Output High Leakage Current | V FAULT = 5.5V, VSET = 1V | 0.05 1 | µA |
| Slow Current-Loop Response Time | 20% current overdrive, VIN = 5V | 5 | µs |
| Fast Current-Loop Response Time |  | 2 | µs |
| Turn-On Time | VIN = 5V, IOUT = 500mA | 80 200 | µs |
|  | VIN = 3V, IOUT = 500mA | 115 |  |
| Turn-Off Time | VIN = 5V, IOUT = 500mA | 2 4 | µs |

MAX893L

ELECTRICAL CHARACTERISTICS

(VIN = 3V, TA = -40°C to +85°C, unless otherwise noted.) (Note 2)

| PARAMETER | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- |
| Operating Voltage |  | 3.0 5.5 | V |
| Quiescent Current | VIN = 5V, ON = GND, IOUT = 0 | 30 | µA |
| Off-Supply Current | ON = IN, VIN = VOUT = 5.5V | 2.2 | µA |
| Off-Switch Current | ON = IN, VIN = 5.5V, VOUT = 0 | 15 | µA |
| Undervoltage Lockout | Rising edge, 1% hysteresis | 2.0 2.9 | V |
| On-Resistance | VIN = 4.5V | 125 | mΩ |
|  | VIN = 3.0V | 160 |  |
| Current-Limit-Amplifier Threshold | VSET required to turn the switch off (Note 1) | 1.14 1.34 | V |
| IOUT to ISET Current Ratio | IOUT = 500mA, VOUT > 1.6V | 865 1300 | A/A |
| FAULT Logic Output Voltage Low | ISINK = 1mA, VSET = 1.4V | 0.4 | V |
| Turn-On Time | VIN = 5V, IOUT = 500mA | 200 | µs |
| Turn-Off Time | VIN = 5V, IOUT = 500mA | 1 20 | µs |

Note 1:  Tested with IOUT = 100mA and VSET raised until VIN - VOUT  0.8V.

Note 2:  Specifications to -40°C are guaranteed  by design, not production  tested.

Typical Operating Characteristics

(TA = +25°C, unless otherwise noted.)

QUIESCENT CURRENT

vs. INPUT VOLTAGE

MAX89L toc01

14

12 	ON = GND IOUT = 0

10

8

6

4

2

QUIESCENT CURRENT

vs. TEMPERATURE

14

QUIESCENT CURRENT (  A)

13

12

11

10

9

MAX89L toc02

VIN = 5V VIN = 3V

10

1

0.1

OFF-SUPPLY CURRENT (nA)

0.01

OFF-SUPPLY  CURRENT

MAX893L toc03

vs. TEMPERATURE

0

0	1	2	3	4	5	6

8

-40    -20	0

20	40

60	80

100

0.001

-40    -20	0

20	40	60

80      100

INPUT VOLTAGE (V)

TEMPERATURE (°C)

TEMPERATURE (°C)

1000

OFF-SWITCH CURRENT (nA)

100

10

1

OFF-SWITCH  CURRENT

vs. TEMPERATURE

MAX893L toc04

1.3

1.2

NORMALIZED RON

1.1

1.0

0.9

NORMALIZED ON-RESISTANCE

vs. TEMPERATURE

IOUT = 1/2 ILIMIT

VIN = 5V

MAX893L toc05

1400

1200

IOUT/ISET RATIO

1000

800

600

MAX89L toc06

IOUT/ISET RATIO

vs. ILIMIT

0.1

0.8

0.7

400

200

0.01

-40

-20

0	20

40	60	80

100

0.6

-40    -20	0

20	40

60	80

100

0

| 0 20 40 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

60	80

100     120

140

TEMPERATURE (°C)

TEMPERATURE (°C)

ILIMIT (%)

1.4

NORMALIZED OUTPUT CURRENT

1.2

1.0

NORMALIZED OUTPUT CURRENT

vs. OUTPUT VOLTAGE

MAX893L toc07

150

140

130

TURN-ON TIME

vs. TEMPERATURE

| ILOAD = ILIMIT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | ILIMIT = | 1.2A |
| VIN = 5V |  |  |  |  |  |

MAX893L toc08

VIN = 3V

TURN-OFF TIME

MAX893L toc09

vs. TEMPERATURE

6

ILOAD = ILIMIT

5

0.8

0.6

0.4

0.2

0

120

110

tON (  s)

100

90

80

70

60

4

tOFF (  s)

3

2

VIN = 5V

1

0

VIN = 5V

VIN = 3V

0	1	2	3	4	5	6

-40    -20	0

20	40

60	80

100

-40    -20	0

20	40

60	80

100

MAX893L

QUIESCENT CURRENT (  A)

OUTPUT VOLTAGE (V)

TEMPERATURE (C)

TEMPERATURE (°C)

MAX893L toc11

Typical Operating Characteristics (continued)

(TA = +25°C, unless otherwise noted.)

OUTPUT OVERLOAD (SLOW-LOOP RESPONSE)

OUTPUT SHORT CIRCUIT (FAST-LOOP RESPONSE)

MAX893L toc10

A	A

C	C

0V 	VOUT 0V

B

0A

2µs/div

CIN = 100µF,  COUT = 0.1µF

A: VIN, 1V/div, AC-COUPLED

B: IOUT, 1A/div

C: VOUT, 5V/div

B IOUT 0V

2µs/div

CIN = 100µF,  COUT = 0.1µF

A: VIN, 1V/div, AC-COUPLED

B: IOUT, 2A/div

MAX893L toc14

C: VOUT, 5V/div

MAX893L toc13

SWITCH TURN-ON TIME

SWITCH TURN-OFF TIME

A

A

0V	0V

B	B

0V	0V

MAX893L

20µs/div

VIN = 5V,  IOUT = ILIMIT/2

A: VON, 2V/div

B: VOUT, 2V/div

1µs/div

VIN = 5V,  IOUT = ILIMIT/2

A: VON, 2V/div

B: VOUT, 2V/div

Pin  Description

| PIN | NAME | FUNCTION |
| --- | --- | --- |
| 1, 2 | IN | Input. P-channel MOSFET source. Bypass IN with a 1µF capacitor to ground. |
| 3 | ON | Active-Low Switch On Input. A logic low turns the switch on. |
| 4 | GND | Ground |
| 5 | SET | Set Current-Limit Input. A resistor from SET to ground sets the current limit for the switch. RSET = 1.34 · 103 / ILIMIT, where ILIMIT is the desired current limit in amperes. |
| 6, 7 | OUT | Switch Output. P-channel MOSFET drain. Bypass OUT with a 0.1µF capacitor to ground. |
| 8 | FAULT | Fault-Indicator Output. This open-drain output goes low when in current limit or when the die temperature exceeds +165°C. |

Detailed Description

The MAX893L P-channel MOSFET power switch limits output current to a programmed level. When the output current is increased beyond the programmed  current limit, or 1.2A (IMAX), the current also increases through the replica switch (IOUT / 1080) and through RSET (Figure 1). The current-limit error amplifier compares the voltage across RSET to the internal 1.24V reference, and regulates the current to the programmed  current limit (ILIMIT).

This switch is not bidirectional; therefore, the input volt- age must be higher than the output voltage.

Setting the Current Limit

IN FAULT

N

P

P

REPLICA AMPLIFIER

MAX893L

ON

P

SET

OUT

The MAX893L features internal current-limiting circuitry with a maximum programmable  value (IMAX) of 1.2A. For best performance,  set the current limit (ILIMIT) between 0.2A  ILIMIT  1.2A. This current limit remains in effect throughout the input supply-voltage range.

Program the current limit with a resistor (RSET) from

SET to ground (Figure 2) as follows:	ON

CONTROL

ON	CURRENT-LIMIT

AMPLIFIER

1.24V HOT

RSET

GND

MAX893L

ISET = ILIMIT / 1080

RSET = 1.24V / ISET = 1.34 · 103 / ILIMIT

where ILIMIT is the desired current limit.

Short-Circuit Protection The MAX893L is a short-circuit protected switch. In the event of an output short circuit or a current overload condition, the current through the switch is limited by the internal current-limiting error amplifier to 1.5 · ILIMIT. The short-circuit current is typically  1.8A for a pro- grammed current limit of 1.2A. When the short-circuit condition is removed, the replica error amplifier will set the current limit back to ILIMIT.

CIRCUITRY

Figure 1. Functional Diagram

For a high VDS/t during an output short-circuit condi- tion, the switch turns off and disconnects the input supply from the output. The current-limiting amplifier then slow- ly turns the switch on with the output current limited to

1.5 · ILIMIT. When the short-circuit condition is removed, the current limit is set back to ILIMIT. See Output Short- Circuit (Fast-Loop Response) and Output Overload (Slow-Loop Response) in the Typical  Operating Characteristics.

MAX893L

SET

INPUT VOLTAGE

+5V

100k

IN

MAX893L

OUT

VOUT

RSET

ON/OFF	GND R

GND

BLANKED

SIGNAL TO µP

100k

FAULT 	SET

C

0.1   F

RSET

Figure 2. Setting the Current Limit

Figure 3. Fault-Blanking Circuit

MAX893L

Thermal Shutdown The MAX893L features thermal shutdown. The switch turns off when the junction temperature exceeds

+165°C. Once the device cools by 10°C, the switch turns back on. If the fault condition is not removed, the switch will cycle on and off, resulting in a pulsed out- put.

Fault Indicator The MAX893L provides  a fault output (FAULT). This open-drain output goes low when in current limit or when the die temperature exceeds +165°C. A 100k pull-up resistor from FAULT to IN provides  a logic- control signal.

Fault Blanking During start-up in USB applications,  the MAX893L charges the relatively large USB capacitance. This may activate an unwanted fault signal if the charging current exceeds the programmed  current limit. To “blank out” this start-up fault signal, add  a simple lowpass RC delay circuit as shown in Figure 3. This circuit provides a 10ms delay.

Applications Information

Input Capacitor To limit the input voltage drop during momentary output short-circuit conditions, connect a capacitor  no more than 5mm from IN to GND. A 1µF ceramic capacitor will be adequate for most applications;  however, higher capacitor values will further reduce the voltage drop at the input.

Output Capacitor Connect a 0.1µF capacitor from OUT to GND to prevent inductive parasitics from pulling OUT negative during turn-off.

Layout and Thermal-Dissipation

Considerations To take full advantage of the switch-response time to output short-circuit conditions, it is very important to keep all traces as short as possible to reduce the effect of undesirable parasitic inductance.  Place input and output capacitors  as close to the device as possible (no more than 5mm).

Under normal operating conditions, the package  can dissipate and channel heat away. Calculate the maxi- mum power as follows:

P = (ILIMIT)2 · RON

where RON is the on-resistance of the switch.

When the output is short circuited,  the voltage drop across the switch equals the input supply. Hence, the power dissipated across the switch increases, as does the die temperature. If the fault condition  is not removed, the thermal-shutdown protection circuitry turns the switch off until the die temperature falls by

10°C. A ground plane in contact with the device will help dissipate additional heat.

Chip Information

TRANSISTOR COUNT: 340

MAX893L

SOICN.EPS

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses are implied. Maxim reserves the right to change the circuitry and specifications without notice at any time.

8    	Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA  94086 408-737-7600

©  1999 Maxim Integrated  Products 	Printed USA 	is a registered  trademark of Maxim Integrated  Products.