Not Recommended for New Designs

The MAX900 was manufactured for Maxim by an outside wafer foundry using a process that is no longer available.  It is not recommended for new designs. A Maxim replacement or an industry second-source may be available. The data sheet remains available for existing users. The other parts on the following data sheet are not affected.

For further information, please see the QuickView data sheet for this part or contact technical support for assistance.

19-2887; Rev. 5; 2/05

High-Speed, Low-Power Voltage Comparators

General Description

The MAX900–MAX903 high-speed,  low-power, single/ dual/quad  voltage comparators  feature differential ana- log inputs and TTL-logic outputs with active internal pull- ups. Fast propagation  delay (8ns typ at 5mV overdrive) makes the MAX900–MAX903 ideal for fast A/D convert- ers and sampling circuits, line receivers, V/F converters, and many other data-discrimination applications.

All comparators  can be powered from separate analog and digital power supplies or from a single combined sup- ply voltage. The analog input common-mode  range includes the negative rail, allowing ground sensing when powered from a single supply. The MAX900–MAX903 consume 18mW per comparator when powered from +5V.

The MAX900–MAX903 are equipped  with independent TTL-compatible latch inputs. The comparator  output states are held when the latch inputs are driven low. The MAX901 provides  the same performance  as the MAX900/MAX902/MAX903 with the exception  of the latches.

For newer, pin-for-pin compatible  parts with the same speed  and only half the power dissipation,  see the MAX9201/MAX9202/MAX9203 data sheet.

Applications

Features

♦ 8ns (typ) Propagation Delay

♦ 18mW/Comparator Power Consumption (+5V, typ)

♦ Separate Analog and Digital Supplies

♦ Flexible Analog Supply: +5V to +10V or ±5V

♦ Input Range Includes Negative Supply Rail

♦ TTL-Compatible Outputs

♦ TTL-Compatible Latch Inputs (Except MAX901)

Ordering Information PART TEMP RANGE PIN-PACKAGE

MAX900ACPP 0°C to +70°C 20 Plastic DIP

MAX900BCPP 0°C to +70°C 20 Plastic DIP MAX900ACWP 0°C to +70°C 20 Wide SO

MAX900BCWP 0°C to +70°C 20 Wide SO

MAX900AEPP -40°C to +85°C 20 Plastic DIP

MAX900BEPP -40°C to +85°C 20 Plastic DIP

MAX900AEWP -40°C to +85°C 20 Wide SO

MAX900BEWP -40°C to +85°C 20 Wide SO

MAX901ACPE 0°C to +70°C 16 Plastic DIP

MAX901BCPE 0°C to +70°C 16 Plastic DIP

High-Speed A/D Converters

High-Speed V/F Converters

Line Receivers

Threshold Detectors

Input Trigger Circuitry

High-Speed Data

Sampling

PWM Circuits

Ordering Information continued at end of data sheet.

Pin  Configurations

TOP VIEW

MAX901

MAX902

MAX903

IN- (A)   1

IN+ (A)    2

GND   3

OUT (A)    4

16  IN- (D)

15  IN+ (D)

14  VCC** A	D

13  OUT (D)

IN- (A)    1

IN+ (A)    2

GND    3

LATCH (A)    4	A

14  VCC**

13  N.C.

12  OUT (B) B

11  LATCH (B)

VCC**    1

IN+    2	 	   IN-    3

VEE*    4

8   VDD***

7   OUT

6    GND

5   LATCH

OUT (B)    5

B	C     12

OUT (C)

OUT (A)    5

10

VDD***

VEE*   6      	 IN+ (B)    7

IN- (B)    8

11

10

9

DIP/SO

VDD** IN+ (C)

IN- (C)

N.C.    6

VEE*    7

DIP/SO

9   IN+ (B)

8   IN- (B)

DIP/SO

*ANALOG V- AND SUBSTRATE

**ANALOG V+

***DIGITAL V+

MAX900–MAX903

Pin Configurations continued at end of data sheet.

ABSOLUTE MAXIMUM RATINGS

Analog Supply Voltage (VCC to VEE) ...................................+12V Digital Supply Voltage (VDD to GND) ....................................+7V Differential Input Voltage..................(VEE - 0.2V) to (VCC + 0.2V) Common-Mode Input Voltage ..........(VEE - 0.2V) to (VCC + 0.2V) Latch-Input Voltage (MAX900/MAX902/

MAX903 only)  .........................................-0.2V to (VDD + 0.2V) Output Short-Circuit  Duration

To GND .......................................................................Indefinite

To VDD ...............................................................................1min

Internal Power Dissipation................................................500mW Derate above +100°C ................................................10mW/°C

Operating  Temperature  Ranges:

MAX900–MAX903_C_ _ .......................................0°C to +70°C MAX900–MAX903_E_ _ ....................................-40°C to +85°C Junction Temperature........................................-65°C to +160°C Storage Temperature  Range .............................-65°C to +150°C Lead Temperature  (soldering,  10s) .................................+300°C

MAX900–MAX903

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions  beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS

(VCC = +5V, VEE = -5V, VDD = +5V, LE1–LE4 = logic high, TA = +25°C, unless otherwise noted.)

| PARAMETER | SYMBOL | CONDITIONS | MAX900A/MAX901A MIN TYP MAX | MAX900B/MAX901B/ MAX902/MAX903 MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| Input Offset Voltage | VOS | VCM = 0 VO = 1.4V | 0.5 2.0 | 1.0 4.0 | mV |
| Input Bias Current | IB | IIN+ or IIN- | 3 6 | 4 10 | µA |
| Input Offset Current | IOS | VCM = 0; VO = 1.4V | 50 250 | 100 500 | nA |
| Input Voltage Range | VCM | (Note 1) | VCC - VEE - 0.1 2.25 | VCC - VEE - 0.1 2.25 | V |
| Common-Mode Rejection Ratio | CMRR | -5V < VCM < +2.75V, VO = 1.4V (Note 2) | 50 150 | 75 250 | µV/V |
| Power-Supply Rejection Ratio | PSRR | (Note 2) | 50 150 | 100 250 | µV/V |
| Output High Voltage | VOH | VIN > 250mV, ISRC = 1mA | 2.4 3.5 | 2.4 3.5 | V |
| Output Low Voltage | VOL | VIN > 250mV, ISINK = 8mA | 0.3 0.4 | 0.3 0.4 | V |
| Latch-Input Voltage High | VLH | (Note 3) | 1.4 2.0 | 1.4 2.0 | V |
| Latch-Input Voltage Low | VLL | (Note 3) | 0.8 1.4 | 0.8 1.4 | V |
| Latch-Input Current High | ILH | VLH = 3.0V (Note 3) | 1 20 | 1 20 | µA |
| Latch-Input Current Low | ILL | VLL = 0.3V (Note 3) | 1 20 | 1 20 | µA |

MAX900–MAX903

ELECTRICAL CHARACTERISTICS (continued)

(VCC = +5V, VEE = -5V, VDD = +5V, LE1–LE4 = logic high, TA = +25°C, unless otherwise noted.)

| PARAMETER | SYMBOL | CONDITIONS | MAX900A/MAX901A MAX900B/MAX901B MIN TYP MAX | MAX902 MIN TYP MAX | MAX903 MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- | --- |
| Positive Analog Supply Current | ICC | (Note 7) | 10 15 | 5 8 | 2.5 4 | mA |
| Negative Analog Supply Current | IEE | (Note 7) | 7 12 | 3.5 6 | 2 3 | mA |
| Digital Supply Current | IDD | (Note 7) | 4 6 | 2 3 | 1 1.5 | mA |
| Power Dissipation | PD | VCC = VDD = +5V, VEE = 0 | 70 105 | 35 55 | 18 28 | mW |

TIMING CHARACTERISTICS

(VCC = +5V, VEE = -5V, VDD = +5V, LE1–LE4 = logic high, TA = +25°C, unless otherwise noted.)

| PARAMETER | SYMBOL | CONDITIONS | MAX900A/MAX901A MAX900B/MAX901B MIN TYP MAX | MAX902 MIN TYP MAX | MAX903 MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- | --- |
| Input-to-Output High Response Time | tpd+ | VOD = 5mV, CL = 15pF, IO = 2mA (Note 4) | 8 10 | 8 10 | 8 10 | ns |
| Input-to-Output Low Response Time | tpd- | VOD = 5mV, CL = 15pF, IO = 2mA (Note 4) | 8 10 | 8 10 | 8 10 | ns |
| Difference in Response Time Between Outputs | ∆tpd | (Notes 4, 5) | 0.5 2.0 | 0.5 2.0 | 0.5 2.0 | ns |
| Latch Disable to Output High Delay | tpd+ (D) | (Notes 3, 6) | 10 | 10 | 10 | ns |
| Latch Disable to Output Low Delay | tpd- (D) | (Notes 3, 6) | 12 | 12 | 12 | ns |
| Minimum Setup Time | ts | (Notes 3, 6) | 2 | 2 | 2 | ns |
| Minimum Hold Time | th | (Notes 3, 6) | 1 | 1 | 1 | ns |
| Minimum Latch Disable Pulse Width | tpw (D) | (Notes 3, 6) | 10 | 10 | 10 | ns |

MAX900–MAX903

ELECTRICAL CHARACTERISTICS

(VCC = +5V, VEE = -5V, VDD = +5V, LE1–LE4 = logic high, TA = full operating temperature,  unless otherwise noted.)

| PARAMETER | SYMBOL | CONDITIONS | MAX900A/MAX901A MIN TYP MAX | MAX900B/MAX901B/ MAX902/MAX903 MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| Input Offset Voltage | VOS | VCM = 0, VO = 1.4V | 1 3 | 2 6 | mV |
| Input Bias Current | IB | IIN+ or IIN- | 4 10 | 6 15 | µA |
| Input Offset Current | IOS | VCM = 0, VO = 1.4V | 100 500 | 200 800 | nA |
| Input Voltage Range | VCM | (Note 1) | VEE - VCC - 0.1 2.25 | VEE - VCC - 0.1 2.25 | V |
| Common-Mode Rejection Ratio | CMRR | -5V < VCM < +2.75V, VO = 1.4V (Note 2) | 80 250 | 120 500 | µV/V |
| Power-Supply Rejection Ratio | PSRR | (Note 2) | 100 250 | 150 500 | µV/V |
| Output High Voltage | VOH | VIN > 250mV, ISRC = 1mA | 2.4 3.5 | 2.4 3.5 | V |
| Output Low Voltage | VOL | VIN > 250mV, ISINK = 8mA | 0.3 0.4 | 0.3 0.4 | V |
| Latch Input Voltage High | VLH | (Note 7) | 1.4 2.0 | 1.4 2.0 | V |
| Latch Input Voltage Low | VLL | (Note 7) | 0.8 1.4 | 0.8 1.4 | V |
| Latch Input Current High | ILH | VLH = 3.0V (Note 7) | 2 20 | 1 20 | µA |
| Latch Input Current Low | ILL | VLL = 0.3V (Note 7) | 2 20 | 1 20 | µA |

MAX900–MAX903

ELECTRICAL CHARACTERISTICS (continued)

(VCC = +5V, VEE = -5V, VDD = +5V, LE1–LE4 = logic high, TA = full operating temperature,  unless otherwise noted.)

| PARAMETER | SYMBOL | CONDITIONS | MAX900A/MAX901A/ MAX900B/MAX901B MIN TYP MAX | MAX902 MIN TYP MAX | MAX903 MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- | --- |
| Positive Analog Supply Current | ICC | (Note 7) | 10 25 | 5 12 | 2.5 6 | mA |
| Negative Analog Supply Current | IEE | (Note 7) | 7 20 | 3.5 10 | 2 5 | mA |
| Digital Supply Current | IDD | (Note 7) | 4 10 | 2 5 | 1 2.5 | mA |
| Power Dissipation | PD | VCC = VDD = +5V, VEE = 0 | 70 105 | 35 55 | 18 28 | mW |

TIMING CHARACTERISTICS

(VCC = +5V, VEE = -5V, VDD = +5V, LE1–LE4 = logic high, TA = full operating temperature,  unless otherwise noted.)

| PARAMETER | SYMBOL | CONDITIONS | MAX900A/MAX901A MIN TYP MAX | MAX900B/MAX901B/ MAX902/MAX903 MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| Input-to-Output High Response Time | tpd+ | VOD = 5mV, CL = 15pF, IO = 2mA (Note 4) | 10 15 | 10 15 | ns |
| Input-to-Output Low Response Time | tpd- | VOD = 5mV, CL = 15pF, IO = 2mA (Note 4) | 10 15 | 10 15 | ns |
| Difference in Response Time Between Outputs | ∆tpd | (Notes 4, 5) | 1 3 | 1 3 | ns |

Note 1: The input common-mode voltage and input signal voltages should not be allowed to go negative by more than 0.2V below VEE. The upper-end of the common-mode voltage range is typically VCC - 2V, but either or both inputs can go to a maximum of VCC + 0.2V without damage.

Note 2: Tested for +4.75V < VCC < +5.25V, and -5.25V < VEE < -4.75V with VDD = +5V, although permissible analog power-supply

range is +4.75V < VCC < +10.5V for single-supply operation with VEE grounded.

Note 3: Specification does not apply to MAX901.

Note 4: Guaranteed by design. Times are for 100mV step inputs (see Propagation Delay Characteristics in Figures 2 and 3).

Note 5: Maximum difference in propagation delay between any of the four comparators in the MAX900–MAX903.

Note 6: See Timing Diagram (Figure 2). Owing to the difficult and critical nature of switching measurements involving the latch, these parameters cannot be tested in a production environment. Typical specifications listed are taken from measurements using a high-speed test-jig.

Note 7: ICC tested for +4.75V < VCC < +10.5V with VEE grounded. IEE tested for -5.25V < VEE < -4.75V with VCC = +5V. IDD tested for +4.75V < VDD < +5.25V with the worst-case condition of all four comparator outputs at logic low.

Typical Operating Characteristics

(TA = +25°C, unless otherwise noted.)

INPUT OFFSET VOLTAGE

vs. TEMPERATURE

INPUT BIAS CURRENT

MAX900-03 toc02

vs. TEMPERATURE

OUTPUT LOW VOLTAGE (VOL) vs. LOAD CURRENT

MAX900-03 toc01

INPUT BIAS VOLTAGE (µV)

| 2 4.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

0.5

1	3.5

| 0 3.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

0.4

OUTPUT LOW VOLTAGE (V)

0.3

TA = -55°C

MAX900-03 toc06

MAX900-03 toc03

TA = +25°C

TA = +125°C

-40 -20   0

20  40  60  80 100 120

-40 -20   0

20  40  60  80 100 120

2	4	6

8	10

TEMPERATURE (°C)

TEMPERATURE (°C)

LOAD CURRENT (mA)

3.0

ICC SUPPLY CURRENT (mA)

2.8

2.6

2.4

2.2

2.0

1.8

1.6

ICC SUPPLY CURRENT (PER COMPARATOR)

MAX900-03 toc04

vs. VCC SUPPLY VOLTAGE

VDD = +5V

TA = +125°C

TA = +25°C

TA = -55°C

4

3

2

OUTPUT VOLTAGE

1

0

100

0

INPUT VOLTAGE

-100

IINPUT OVERDRIVE vs. tpd+ RESPONSE TIME

25mV

2mV

5mV INPUT OVERDRIVE (VOD)

INPUT VOLTAGE OUTPUT VOLTAGE

4

3

2

MAX900-03 toc05

1

0

100

0

-100

INPUT OVERDRIVE vs. tpd- RESPONSE TIME

2mV

25mV

5mV

INPUT OVERDRIVE (VOD)

4	5	6	7

8	9	10

0	2	4	6

8    10   12   14

0	2	4	6

8    10   12   14

VCC SUPPLY VOLTAGE (V)

tpd+ RESPONSE TIME (ns)

tpd-  RESPONSE TIME (ns)

MAX900-03 toc07

RESPONSE TIME  vs. TEMPERATURE (5mV OVERDRIVE)

14

13

12

RESPONSE TIME (ns)

11

tpd+

10

9

8

MAX900-03 toc08

RESPONSE TIME  vs. LOAD CAPACITANCE (5mV OVERDRIVE)

14

13

12

11

10

| 9 tpd+ |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | t |  |  |  |  |

MAX900–MAX903

INPUT OFFSET VOLTAGE (mV)

7	tpd-	8

6	7

5

-40 -20   0   20  40  60  80 100 120

TEMPERATURE (°C)

10   20   30    40   50   60   70   80

LOAD CAPACITANCE (pF)

MAX900–MAX903

| Pin Descriptions PIN | NAME | FUNCTION |
| --- | --- | --- |
| 1, 8, 9, 16 | IN- (A, B, C, D) | Negative Input (Channels A, B, C, D) |
| 2, 7, 10, 15 | IN+ (A, B, C, D) | Positive Input (Channels A, B, C, D) |
| 3 | GND | Ground Terminal |
| 4, 5, 12, 13 | OUT (A, B, C, D) | Output (Channels A, B, C, D) |
| 6 | VEE | Negative Analog Supply and Substrate |
| 11 | VDD | Positive Digital Supply |
| 14 | VCC | Positive Analog Supply |

MAX900	MAX901

| PIN | NAME | FUNCTION |
| --- | --- | --- |
| 1, 10, 11, 20 | IN- (A, B, C, D) | Negative Input (Channels A, B, C, D) |
| 2, 9, 12, 19 | IN+ (A, B, C, D) | Positive Input (Channels A, B, C, D) |
| 3 | GND | Ground Terminal |
| 4, 7, 14, 17 | LATCH (A, B, C, D) | Latch Input (Channels A, B, C, D) |
| 5, 6, 15, 16 | OUT (A, B, C, D) | Output (Channels A, B, C, D) |
| 8 | VEE | Negative Analog Supply and Substrate |
| 13 | VDD | Positive Digital Supply |
| 18 | VCC | Positive Analog Supply |

| MAX902 MAX903 PIN | NAME | FUNCTION |
| --- | --- | --- |
| 1 | VCC | Positive Analog Supply |
| 2 | IN+ | Positive Input |
| 3 | IN- | Negative Input |
| 4 | VEE | Negative Analog Supply and Substrate |
| 5 | LATCH | Latch Input |
| 6 | GND | Ground Terminal |
| 7 | OUT | Output |
| 8 | VDD | Positive Digital Supply |

| PIN | NAME | FUNCTION |
| --- | --- | --- |
| 1, 8 | IN- (A, B) | Negative Input (Channels A, B) |
| 2, 9 | IN+ (A, B) | Positive Input (Channels A, B) |
| 3 | GND | Ground Terminal |
| 4, 11 | LATCH (A, B) | Latch Input (Channels A, B) |
| 5, 12 | OUT (A, B) | Output (Channels A, B) |
| 6, 13 | N.C. | No Connection. Not internally connected. |
| 7 | VEE | Negative Analog Supply and Substrate |
| 10 | VDD | Positive Digital Supply |
| 14 | VCC | Positive Analog Supply |

Applications Information

Circuit Layout Because of the large gain-bandwidth  transfer function of the MAX900–MAX903, special  precautions  must be taken to realize their full high-speed capability. A printed circuit board with a good, low-inductance ground plane is mandatory.  All decoupling  capacitors  (the small

100nF ceramic type is a good choice) should be mount- ed as close as possible  to the power-supply  pins. Separate decoupling  capacitors for analog VCC and for digital  VDD are also recommended.  Close attention should be paid to the bandwidth of the decoupling  and terminating  components.  Short lead lengths on the inputs and outputs are essential to avoid unwanted par- asitic feedback  around the comparators.  Solder the device directly  to the printed  circuit  board  instead of using a socket.

Input Slew-Rate Requirements As with all high-speed comparators, the high gain-band- width product of the MAX900–MAX903 can create oscil- lation problems  when the input traverses the linear region. For clean output switching without oscillation or steps in the output waveform, the input must meet mini- mum slew-rate requirements. Oscillation is largely a function of board layout and of coupled  source imped- ance and stray input capacitance.  Both poor layout and large-source  impedance  will cause the part to oscillate and increase the minimum slew-rate requirement.  In some applications, it may be helpful to apply some posi- tive feedback  between the output and + input. This

pushes the output through the transition region cleanly, but applies a hysteresis in threshold seen at the input terminals.

TTL Output and Latch Inputs The comparator TTL-output stages are optimized for dri- ving low-power Schottky TTL with a fan-out of four.

When the latch is connected  to a logic high level, the comparator  is transparent and immediately responds to changes  at the input terminals. When the latch is con- nected to a TTL low level, the comparator output latches in the same state as at the instant that the latch command is applied, and will not respond to subsequent changes at the input. No latch is provided on the MAX901.

Power Supplies The MAX900–MAX903 can be powered from separate analog and digital supplies or from a single +5V supply. The analog supply can range from +5V to +10V with VEE grounded  for single-supply  operation (Figures 1A and 1B) or from a split ±5V supply (Figure 1C). The VDD digital supply always requires +5V.

In high-speed,  mixed-signal applications  where a com- mon ground is shared, a noisy digital environment can adversely affect the analog input signal. When set up with 	separate 	supplies 	(Figure 	1C), 	the MAX900–MAX903 isolate analog and digital signals by providing a separate AGND (VEE) and DGND.

Typical Power-Supply Alternatives

+10V

VCC

VEE

+5V

VDD

GND

OUT

+5V

VCC

VEE

VDD

GND

OUT

+5V

VCC

VEE

+5V

VDD

GND

OUT

-5V

MAX900–MAX903

Figure 1A. Separate Analog Supply, Common Ground

Figure 1B. Single +5V Supply, Common

Ground

Figure 1C. Split ±5V Supply, Separate

Ground

Definitions of Terms

VOS 	Input Offset Voltage: Voltage applied between the two input terminals to obtain TTL-logic threshold (+1.4V) at the

output.

tpd+ (D) 	Latch Disable-to-Output High Delay: The propagation delay measured from the latch-signal crossing the TTL threshold in a low-to-high transition to the point of the output crossing TTL threshold in a low-to-high transition.

VIN 	Input Voltage Pulse Amplitude: Usually set to 100mV for comparator specifications.

tpd- (D) 	Latch Disable-to-Output Low Delay: The propagation delay measured from the latch-signal crossing the TTL threshold in a low-to-high transition to the point of the output crossing TTL threshold in a high-to-low transition.

VOD 	Input Voltage Overdrive: Usually set to

5mV and in opposite polarity to VIN for

comparator specifications.

ts	Minimum Setup Time: The minimum time before the negative transition of the latch signal that an input signal change must be present in order to be acquired and held at the outputs.

tpd+ 	Input-to-Output High Delay: The propagation delay measured from the time the input signal crosses the input offset voltage to the TTL-logic threshold of an output low-to-high transition

th	Minimum Hold Time: The minimum time after the negative transition of the latch signal that an input signal must remain unchanged in order to be acquired and held at the output.

MAX900–MAX903

tpd- 	Input-to-Output Low Delay: The propagation delay measured from the time the input signal crosses the input offset voltage to the TTL-logic threshold of an output high-to-low transition.

tpw (D)	Minimum Latch-Disable Pulse Width: The minimum time that the latch signal must remain high in order to acquire and hold an input-signal change.

LATCH ENABLE INPUT

COMPARE	COMPARE

1.4V

+5V

LATCH	LATCH	LATCH

DIFFERENTAL INPUT VOLTAGE

VIN

ts 	th

VDD

tpw(D)

VOS

OUTPUT

0

VOS

+5mV

INPUT

100mV

tpd-

tpd+ (D)

5ns/div

Figure 3. tpd+  Response Time to 5mV Overdrive

COMPARATOR OUTPUT

1.4V

Figure 2. MAX900/MAX902/MAX903 Timing Diagram

OUTPUT

+5V

INPUT TO 10X

SCOPE PROBE	 	 (10MΩ, 14pF)

10Ω

VCC +5V

100nF

VDD +5V RL

INPUT

5ns/div

0

100mV

VOS

+5mV

PRECISION

STEP GENERATOR

VDC OFFSET ADJUST

10kΩ

100nF

1kΩ

100nF

10Ω

D.U.T.

VEE -5V

2.43kΩ

OUTPUT TO 10X SCOPE PROBE (10MΩ, 14pF)

100nF

MAX900–MAX903

Figure 4. tpd- Response Time to 5mV Overdrive

Figure 5. Response-Time Setup

OUTPUT

1V/div

OUTPUT

1V/div

INPUT

10mV/div

5ns/div

Figure 6. Response to 50MHz Sine Wave

INPUT

10mV/div

5ns/div

Figure 7. Response to 100MHz Sine Wave Photo

MX7228

OCTAL

8-BIT DAC

+1.25V VREF

VDAC1

IN1

VOUT1

IN2

IN3

IN4

UNDER LIMIT

OVER LIMIT

UNDER LIMIT

UNDER LIMIT

Typical Application

Programmed, Variable-Alarm Limits By combining  two quad  analog comparators  with an octal 8-bit D/A converter (the MX7228), several alarm and limit-defect functions can be performed simultane- ously without external adjustments

The MX7228’s internal latches  allow the system processor  to set the limit points for each comparator independently  and update  them at any time. Set the upper and lower thresholds for a single transducer by pairing the D/A converter and comparator sections.

MSB D7

MAX901

8-BIT DATA INPUT

LSB D1

8 x 8

DATA LATCH

IN5

IN6

UNDER LIMIT

UNDER LIMIT

IN7

OVER LIMIT

A0

A1 	CONTROL LOGIC

A2

VDAC8

VOUT8

IN8

OVER LIMIT

MAX900–MAX903

MAX901

Figure 8. Alarm Circuit Level Monitors Eight Separate Inputs

Ordering Information (continued) 	 	Pin  Configurations (continued)

TOP VIEW PART TEMP RANGE PIN-PACKAGE

MAX901ACSE 0°C to +70°C 16 Narrow SO

MAX901BCSE 0°C to +70°C 16 Narrow SO

MAX901AEPE -40°C to +85°C 16 Plastic DIP

MAX901BEPE -40°C to +85°C 16 Plastic DIP

MAX901AESE -40°C to +85°C 16 Narrow SO

MAX901BESE -40°C to +85°C 16 Narrow SO

MAX902CPD 0°C to +70°C 14 Plastic DIP

MAX902CSD 0°C to +70°C 14 Narrow SO

MAX902EPD -40°C to +85°C 14 Plastic DIP

MAX902ESD -40°C to +85°C 14 Narrow SO

MAX903CPA 0°C to +70°C 8 Plastic DIP

MAX903CSA 0°C to +70°C 8 SO

MAX903EPA -40°C to +85°C 8 Plastic DIP

MAX903ESA -40°C to +85°C 8 SO

IN- (A)    1

IN+ (A)    2

GND   3

LATCH (A)    4

OUT (A)    5

OUT (B)    6

LATCH (B)    7

A	D

MAX900

B	C

20   IN- (D)

19   IN+ (D)

18   VCC**

17   LATCH (D)

16    OUT (D)

15    OUT (C)

14   LATCH (C)

VEE*    8      	 IN+ (B)    9

IN- (B)   10

13

12

11

VDD*** IN+ (C)

IN- (C)

DIP/SO

*ANALOG V- AND SUBSTRATE

**ANALOG V+

***DIGITAL V+

MAX900–MAX903

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses  are implied. Maxim reserves the right to change the circuitry and specifications  without notice at any time.

12  	Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA  94086 408-737-7600

©  2005 Maxim Integrated  Products 	Printed USA 	is a registered  trademark  of Maxim Integrated  Products, Inc.