Not Recommended for New Designs

The MAX909 was manufactured for Maxim by an outside wafer foundry using a process that is no longer available.  It is not recommended for new designs. A Maxim replacement or an industry second-source may be available. The data sheet remains available for existing users. The other parts on the following data sheet are not affected.

For further information, please see the QuickView data sheet for this part or contact technical support for assistance.

19-0129; Rev 7; 1/06

Dual/Quad/Single, High-Speed, Ultra-Low-Power, Single-Supply TTL Comparators

General Description

The MAX907/MAX908/MAX909 are dual/quad/single, high-speed,  ultra-low-power voltage comparators designed  for use in systems powered  from a single

+5V supply; the MAX909 also accepts  dual ±5V sup- plies. Their 40ns propagation  delay (with 5mV input overdrive) is achieved  with a power consumption  of only 3.5mW per comparator.  The wide input common- mode range extends from 200mV below ground (below the negative supply rail for the MAX909) to within 1.5V of the positive supply rail.

Because they are micropower,  high-speed  compara- tors that operate from a single +5V supply and include built-in hysteresis, these devices  replace  a variety of older comparators in a wide range of applications.

MAX907/MAX908/MAX909 outputs are TTL-compatible, requiring no external pullup circuitry. All inputs and out- puts can be continuously shorted to either supply rail without damage. These easy-to-use comparators incor- porate internal hysteresis to ensure clean output switch- ing even when the devices are driven by a slow-moving input signal.

The MAX909 features complementary  outputs and an output latch.   A separate supply pin for extending the analog input range down to -5V is also provided.

The dual MAX907 and single MAX909 are available in

8-pin DIP and SO packages, and the quad MAX908 is available in 14-pin DIP and SO packages. These com- parators are ideal for single +5V-supply  applications that require the combination  of high speed, precision, and ultra-low power dissipation.

Features

♦ 40ns Propagation Delay

♦ 700µA (3.5mW) Supply Current per Comparator

♦ Single 4.5V to 5.5V Supply Operation

(or ±5V, MAX909 only)

♦ Wide Input Range Includes Ground

(or -5V, MAX909 only)

♦ Low, 500µV Offset Voltage

♦ Internal Hysteresis Provides Clean Switching

♦ TTL-Compatible Outputs

(Complementary on MAX909)

♦ Input and Output Short-Circuit Protection

♦ Internal Latch (MAX909 only)

Ordering Information

PART TEMP RANGE PIN-PACKAGE

MAX907CPA 0°C to +70°C 8 Plastic DIP

MAX907CSA 0°C to +70°C 8 SO

MAX907EPA -40°C to +85°C 8 Plastic DIP

MAX907ESA -40°C to +85°C 8 SO

MAX907MSA/PR* -55°C to +125°C 8 SO

Ordering Information continued at end of data sheet.

*Go to www.maxim-ic.com/PR-1 for details  on high-reliability plastic   processing.

Pin  Configurations

TOP VIEW

Applications

Battery-Powered Systems High-Speed A/D Converters High-Speed V/F Converters Line Receivers

Threshold Detectors/Discriminators

High-Speed Sampling Circuits

Zero-Crossing Detectors

OUTA    1

INA-    2

INA+    3

GND    4

MAX907

DIP/SO

8   V+

7   OUTB

6   INB-

| 5 INB+ OUTA | 1 |  |  |  |  | 14 | OUTD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| INA- | 2 |  |  |  |  | 13 | IND- |
| INA+ | 3 |  |  |  |  | 12 | IND+ |
| V+ | 4 |  | MAX9 |  | 08 | 11 | GND |
| INB+ | 5 |  |  |  |  | 10 | INC+ |
| INB- | 6 |  |  |  |  | 9 | INC- |
| OUTB | 7 |  |  |  |  | 8 | OUTC |

MAX907/MAX908/MAX909

DIP/SO

Pin Configurations continued at end of data sheet.

ABSOLUTE MAXIMUM RATINGS

Positive Supply Voltage (V+ to GND) ........................ +6V Negative Supply Voltage (V- to GND, MAX909 only) ......... -7V Differential Input Voltage

MAX907/MAX908 .......................... -0.3V to (V+ + 0.3V) MAX909 .............................. (V- - 0.3V) to (V+ + 0.3V)

Common-Mode Input Voltage

MAX907/MAX908 .......................... -0.3V to (V+ + 0.3V) MAX909 .............................. (V- - 0.3V) to (V+ + 0.3V) Latch Input Voltage (MAX909 only) ......... -0.3V to (V+ + 0.3V) Input/Output Short-Circuit  Duration to V+ or GND . . . Continuous

Continuous Power Dissipation  (TA = +70°C)

8-Pin Plastic DIP (derate 9.09mW/°C above +70°C) . . . 727mW

8-Pin SO (derate 5.88mW/°C above +70°C) ........... 471mW

14-Pin Plastic DIP (derate 10.00mW/°C above +70°C)  . . . 800mW

14-Pin SO (derate 8.33mW/°C above +70°C) .......... 667mW Operating  Temperature  Ranges:

MAX90_C_ _ ...................................... 0°C to +70°C MAX90_E_ _  ................................... -40°C to +85°C MAX907MSA/PR .............................. -55°C to +125°C Storage Temperature  Range ................... -65°C to +150°C

Lead Temperature  (soldering,  10s) ...................... +300°C

MAX907/MAX908/MAX909

Stresses beyond those listed under “Absolute Maximum Ratings" may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions  beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS

(V+ = 5V, TA = +25°C; MAX909 only: V- = 0V, VLATCH = 0V; unless otherwise noted.)

| PARAMETER | SYMBOL | CONDITIONS |  |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- | --- |
| Positive Trip Point | VTRIP+ | (Note 1) |  |  | 2 4 | mV |
| Negative Trip Point | VTRIP- | (Note 1) |  |  | -2 -4 | mV |
| Input Offset Voltage | VOS | (Note 2) |  |  | 0.5 2.0 | mV |
| Input Bias Current | IB | VCM = 0V, VIN = VOS |  |  | 100 300 | nA |
| Input Offset Current | IOS | VCM = 0V, VIN = VOS |  |  | 25 50 | nA |
| Input Voltage Range | VCMR | (Notes 3, 4) | MAX907/MAX908/MAX909 |  | -0.2 V+ - 1.5 | V |
|  |  |  | MAX909 only: V- = -5V |  | -5.2 V+ - 1.5 |  |
| Common-Mode Rejection Ratio | CMRR | (Notes 4, 5) |  |  | 50 150 | µV/V |
| Power-Supply Rejection Ratio | PSRR | (Notes 4, 6) |  |  | 50 200 | µV/V |
| Output High Voltage | VOH | ISOURCE = 100µA |  |  | 3.0 3.5 | V |
| Output Low Voltage | VOL | ISINK = 3.2mA |  |  | 0.36 0.475 | V |
|  |  | ISINK = 8mA |  |  | 0.4 |  |
| Positive Supply Current per Comparator | I+ | (Note 7) |  | MAX907/MAX908 | 0.7 1.0 | mA |
|  |  |  |  | MAX909 | 1.2 1.8 |  |
| Negative Supply Current | I- | MAX909 only: V- = -5V |  |  | 60 100 | µA |
| Power Dissipation per Comparator | PD | (Note 8) |  | MAX907/MAX908 | 3.5 5.5 | mW |
|  |  |  |  | MAX909 | 6 10 |  |
| Output Rise Time | tr | VOUT = 0.4V to 2.4V, CL = 10pF |  |  | 12 | ns |
| Output Fall Time | tf | VOUT = 2.4V to 0.4V, CL = 10pF |  |  | 6 | ns |

MAX907/MAX908/MAX909

ELECTRICAL CHARACTERISTICS (continued)

(V+ = 5V, TA = +25°C; MAX909 only: V- = 0V, VLATCH = 0V; unless otherwise noted.)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Propagation Delay | tPD+, tPD- | VIN = 100mV, VOD = 5mV (Note 9) | 40 50 | ns |
| Differential Propagation Delay | ∆tPD | VIN = 100mV, VOD = 5mV (Note 10) | 1 | ns |
| Propagation Delay Skew | tPDskew | MAX909 only: VIN = 100mV, VOD = 5mV (Note 11) | 2 | ns |
| Latch Input Voltage High | VIH | (Note 12) | 2.0 | V |
| Latch Input Voltage Low | VIL | (Note 12) | 0.8 | V |
| Latch Input Current | IIH, IIL | (Note 12) | 20 | µA |
| Latch Setup Time | ts | (Note 12) | 2 | ns |
| Latch Hold Time | th | (Note 12) | 2 | ns |

ELECTRICAL CHARACTERISTICS

(V+ = 5V, TA = TMIN to TMAX; MAX909 only: V- = 0V, VLATCH = 0V; unless otherwise noted.)

| PARAMETER | SYMBOL | CONDITIONS |  |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- | --- |
| Positive Trip Point | VTRIP+ | (Note 1) |  |  | 2 5 | mV |
| Negative Trip Point | VTRIP- | (Note 1) |  |  | -2 -5 | mV |
| Input Offset Voltage | VOS | (Note 2) |  |  | 1 3.5 | mV |
| Input Bias Current | IB | VCM = 0V, VIN = VOS |  |  | 200 500 | nA |
| Input Offset Current | IOS | VCM = 0V, VIN = VOS |  |  | 50 100 | nA |
| Input Voltage Range | VCMR | C/E temp. ranges (Notes 3, 4) | MAX907/MAX908/MAX909 |  | -0.2 V+ - 1.5 | V |
|  |  |  | MAX909 only, V- = -5V |  | -5.2 V+ - 1.5 |  |
|  |  | M temp. range (Notes 3, 4) | MAX907MSA/PR |  | -0.1 V+ - 1.5 |  |
| Common-Mode Rejection Ratio | CMRR | (Notes 4, 5) |  |  | 75 300 | µV/V |
| Power-Supply Rejection Ratio | PSRR | (Notes 4, 6) |  |  | 75 400 | µV/V |
| Output High Voltage | VOH | ISOURCE = 100µA |  |  | 2.8 3.5 | V |
| Output Low Voltage | VOL | ISINK = 3.2mA |  | TA = TMAX | 0.45 | V |
|  |  |  |  | TMIN = 0°C | 0.525 |  |
|  |  |  |  | TMIN = -40°C | 0.550 |  |
| Positive Supply Current per Comparator | I+ | (Note 7) |  | MAX907/MAX908 | 0.8 1.2 | mA |
|  |  |  |  | MAX909 | 1.2 2.0 |  |
| Negative Supply Current | I- | MAX909 only; V- = -5V |  |  | 100 200 | µV |
| Power Dissipation per Comparator | PD | (Note 8) |  | MAX907/MAX908 | 4 7 | mW |
|  |  |  |  | MAX909 | 6 11 |  |

MAX907/MAX908/MAX909

ELECTRICAL CHARACTERISTICS (continued)

(V+ = 5V, TA = TMIN to TMAX; MAX909 only: V- = 0V, VLATCH = 0V; unless otherwise noted.)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Propagation Delay | tPD+, tPD- | VIN = 100mV, VOD = 5mV (Note 9) | 45 70 | ns |
| Differential Propagation Delay | ∆tPD | VIN = 100mV, VOD = 5mV (Note 10) | 2 | ns |
| Propagation Delay Skew | tPDskew | MAX909 only: VIN = 100mV, VOD = 5mV (Note 11) | 4 | ns |
| Latch Input Voltage High | VIH | (Note 12) | 2.0 | V |
| Latch Input Voltage Low | VIL | (Note 12) | 0.8 | V |
| Latch Input Current | IIH, IIL | (Note 12) | 20 | µA |
| Latch Setup Time | ts | (Note 12) | 4 | ns |
| Latch Hold Time | th | (Note 12) | 4 | ns |

Note 1: 	Trip Point is defined as the input voltage required  to  make the comparator output change state.  The difference between upper (VTRIP+) and lower (VTRIP-) trip points is equal to the width of the input-referred hysteresis zone (VHYST). Specified  for an input common-mode voltage (VCM) of 0V (see Figure 1).

Note 2: 	Input Offset Voltage is defined as the center of the input-referred hysteresis zone. Specified  for VCM = 0V (see Figure 1).

Note 3: 	Inferred from the CMRR test. Note that a correct logic result is obtained  at the output, provided that at least one input is within the VCMR limits. Note also that either or both inputs can be driven to the upper or lower absolute maximum limit with- out damage  to the part.

Note 4: 	Tested with V+ = 5.5V (and V- = 0V for MAX909). MAX909 also tested over the full analog input range (i.e., with

V- = -5.5V).

Note 5: 	Tested over the full input voltage range (VCMR).

Note 6: 	Specified  over the full tolerance of operating  supply voltage:  MAX907/MAX908 tested with 4.5V < V+ < 5.5V.  MAX909 tested with 4.5V < V+ < 5.5V and with -5.5V < V- < 0V.

Note 7: 	Positive Supply Current specified with the worst-case  condition  of all outputs at logic low (MAX907/MAX908),  and with V+ = 5.5V.

Note 8: 	Typical power specified with V+ = 5V; maximum with V+ = 5.5V (and with V- = -5.5V for MAX909).

Note 9: 	Due to difficulties  in measuring  propagation delay with 5mV of overdrive in automatic test equipment, this parameter  is guaranteed by design for the MAX907 and MAX908. Correlation tests show that the specification can be guaranteed if all other DC parameters  are within the specified limits. VOS must be added  to the overdrive voltage for low values of overdrive. For the MAX909, propagation delay is typical only and there is no guaranteed maximum limit.

Note 10:  Differential Propagation  Delay is specified as the difference  between any two channels in the MAX907/MAX908 (both out- puts making either a low-to-high or a high-to-low transition).

Note 11:  Propagation Delay Skew is specified as the difference between any single channel’s output low-to-high transition (tPD+)

and high-to-low transition (tPD-), and also between the QOUT and  QOUT transition on the MAX909.

Note 12:  Latch specifications apply to MAX909 only (see Figure 2).

Typical Operating Characteristics

(V+ = 5V, TA = +25°C, unless otherwise noted.)

PROPAGATION DELAY

MAX907 TOC01

vs. INPUT OVERDRIVE

50

PROPAGATION DELAY

MAX907 TOC2

vs. SOURCE IMPEDANCE

80

PROPAGATION DELAY

MAX907 TOC03

vs. CAPACITIVE LOAD

70

40	60	60

PD

| PD VOD = 5mV CLOAD = 15pF |  |  |
| --- | --- | --- |
|  | tPD+ t - |  |

PROPAGATION DELAY (ns)

PROPAGATION DELAY (ns)

30	40	50

PROPAGATION DELAY (ns)

20	20	40

10

| 1 10 100 RS = 10Ω CLOAD = 15pF |  |
| --- | --- |
|  | t + tPD- |

0

10	100	1k

10k

30

0	20	40

60	80

100

| 120 VOD = 5 RS = 10 | mV Ω |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | tPD+ |  |
|  |  |  |  | tPD- |  |

INPUT OVERDRIVE (mV)

SOURCE IMPEDANCE (Ω)

CAPACITIVE LOAD (pF)

MAX907 TOC04

PROPAGATION DELAY

vs. TEMPERATURE

70

5.0

OUTPUT HIGH VOLTAGE

MAX907 TOC05

| vs. SOURCE CURRENT VIN = 100mV |  |  |
| --- | --- | --- |
| TA = +125°C |  |  |
| TA = +25°C TA = -55°C |  |  |

VOL (V)

0.5

OUTPUT LOW VOLTAGE

vs. SINK CURRENT

MAX907 TOC06

VIN = 100mV

60

| 50 tPD+ VOD = 5mV RS = 10Ω CLOAD = 15pF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | tPD- |  |  |

4.5

VOH (V)

4.0

0.4

0.3

TA = +25°C

TA = -55°C

PROPAGATION DELAY (ns)

40	3.5

0.2

TA = +125°C

30

-60 -40 -20  0

20  40  60  80 100 120 140

3.0

1	10

100

1000

0.1

0	2	4

6	8	10	12

2.0

TEMPERATURE (°C)

MAX907

MAX907 TOC07

| TOTAL POSITIVE SUPPLY CURRENT vs. POSITIVE SUPPLY VOLTAGE (OUTPUTS AT VOL) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TA = +125°C |  |  |  |
|  |  |  | TA = +25°C |  |  |
|  |  |  | TA = - | 55°C |  |

2.0

ISOURCE (µA)

MAX907

MAX907 TOC08

TOTAL POSITIVE  SUPPLY CURRENT vs. POSITIVE SUPPLY VOLTAGE (OUTPUTS AT VOH)

4.0

ISINK (mA)

MAX908

MAX907 TOC09

TOTAL POSITIVE  SUPPLY CURRENT vs. POSITIVE SUPPLY VOLTAGE (OUTPUTS AT VOL)

1.5

TOTAL ICC (mA)

1.0

TOTAL ICC (mA)

1.5

1.0

TA = +125°C

TOTAL ICC (mA)

TA = +25°C TA = -55°C

3.0

2.0

TA = +25°C TA = -55°C

0.5

0.5

1.0

MAX907/MAX908/MAX909

0

2	3	4

5	6	7	8

VCC (V)

0

2	3	4

5	6	7	8

VCC (V)

0

2	3	4

5	6	7	8

VCC (V)

Typical Operating Characteristics (continued)

(V+ = 5V, TA = +25°C, unless otherwise noted.)

3.0

2.0

MAX908

TOTAL POSITIVE  SUPPLY CURRENT vs. POSITIVE SUPPLY VOLTAGE (OUTPUTS AT VOH)

MAX907 TOC10

TOTAL I+ (mA)

TA = +125°C TA = +25°C TA = -55°C

1.0

MAX909

POSITIVE  SUPPLY CURRENT

MAX907 TOC11

| vs. POSITIVE SUPPLY VOLTAGE V- = -5V |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | TA = +125°C |  |  |
|  |  |  | TA = +25°C |  |  |
|  |  |  | TA = -55°C |  |  |

TOTAL I- (µA)

100

MAX909

NEGATIVE SUPPLY CURRENT

vs. NEGATIVE SUPPLY VOLTAGE

V+ = +5V

MAX907 TOC12

TA = +125°C

TA = +25°C

1.0

TA = -55°C

0

2	3	4

5	6	7	8

0

2	3	4	5	6	7	8

0

0	-1	-2

-3	-4	-5	-6	-7

VCC (V)

INPUT VOLTAGE RANGE

vs. TEMPERATURE

V+ (V)

MAX909

INPUT VOLTAGE RANGE

vs. TEMPERATURE

V- (V)

SHORT-CIRCUIT  OUTPUT CURRENT

MAX907 TOC15

vs. TEMPERATURE

VCMR+

4

MAX907 TOC14

VCMR+

MAX907 TOC13

INPUT VOLTAGE RANGE (V)

4

V+ = +5V V- = -5V

SHORT-CIRCUIT OUTPUT CURRENT (mA)

OUTPUT SHORTED TO V+ (SINKING)

30

INPUT VOLTAGE RANGE (V)

0

-1

-60 -40 -20  0

VCMR-

20  40  60  80 100 120 140

-5

-6

-60 -40 -20  0

VCMR-

20  40  60

80 100 120 140

0

-60 -40 -20  0

OUTPUT SHORTED TO

GND (SOURCING)

20  40  60  80 100 120 140

TEMPERATURE (°C)

TEMPERATURE (°C)

TEMPERATURE (°C)

0.5

INPUT OFFSET VOLTAGE

vs. TEMPERATURE

VOUT = 1.4V VCM = 0V

MAX907 TOC16

INPUT CURRENT (nA)

300

INPUT BIAS CURRENT

MAX907 TOC17

vs. TEMPERATURE

4

VCM = 0V

2

TRIP POINT

MAX907 TOC18

vs. TEMPERATURE

VTRIP+

VOS (mV)

VOS (mV)

0	200	0

VTRIP-

-2

-0.5

-60 -40 -20  0

20  40  60  80 100 120 140

100

| -60 -40 -20 0 VCM = 0V VIN = VOS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

20  40  60  80 100 120 140

-4

-60 -40 -20  0

20  40  60  80 100 120 140

MAX907/MAX908/MAX909

TOTAL ICC (mA)

TEMPERATURE (°C)

TEMPERATURE (°C)

TEMPERATURE (°C)

Typical Operating Characteristics (continued)

(V+ = 5V, TA = +25°C, unless otherwise noted.)

INPUT

20mV/div

MAX907/MAX908

PROPAGATION DELAY (tPD+) (5mV  OVERDRIVE)

MAX907  TOC19

tPD+

10ns/div

5mV OVERDRIVE INPUT GND

OUTPUT

500mV/div

TTL

THRESHOLD (1.4V)

OUTPUT GND

INPUT

20mV/div

MAX907/MAX908

PROPAGATION DELAY (tPD-) (5mV  OVERDRIVE)

MAX907  TOC20

tPD-

10ns/div

INPUT GND

-5mV OVERDRIVE

OUTPUT

500mV/div

TTL THRESHOLD (1.4V)

OUTPUT GND

INPUT

20mV/div

MAX909

PROPAGATION DELAY (tPD+) (5mV  OVERDRIVE)

MAX907  TOC21

5mV OVERDRIVE INPUT GND

QOUT

1V/div

INPUT

20mV/div

MAX909

PROPAGATION DELAY (tPD-) (5mV  OVERDRIVE)

MAX907  TOC22

INPUT GND

-5mV OVERDRIVE

QOUT

1V/div

OUTPUT  	 GND

1.4V QOUT

OUTPUT GND

1.4V QOUT

tPD+	tPD SKEW

10ns/div

tPD-	tPD SKEW

10ns/div

MAX907 TOC23

RESPONSE TO 10MHz SINE WAVE

20mVP-P

10MHz SINE WAVE

INPUT

COMPARATOR OUTPUT

1V/div

GND

MAX907/MAX908/MAX909

50ns/div

Pin  Description

| PIN |  |  | NAME | FUNCTION |
| --- | --- | --- | --- | --- |
| MAX907 | MAX908 | MAX909 |  |  |
| 1 | 1 | — | OUTA | Comparator A Output |
| 2 | 2 | — | INA- | Comparator A Inverting Input |
| 3 | 3 | — | INA+ | Comparator A Noninverting Input |
| 4 | 11 | 6 | GND | Ground |
| 5 | 5 | — | INB+ | Comparator B Noninverting Input |
| 6 | 6 | — | INB- | Comparator B Inverting Input |
| 7 | 7 | — | OUTB | Comparator B Output |
| 8 | 4 | 1 | V+ | Positive Supply |
| — | 8 | — | OUTC | Comparator C Output |
| — | 9 | — | INC- | Comparator C Inverting Input |
| — | 10 | — | INC+ | Comparator C Noninverting Input |
| — | 12 | — | IND+ | Comparator D Noninverting Input |
| — | 13 | — | IND- | Comparator D Inverting Input |
| — | 14 | — | OUTD | Comparator D Output |
| — | — | 2 | IN+ | Noninverting Input |
| — | — | 3 | IN- | Inverting Input |
| — | — | 4 | V- | Negative Supply or Ground |
| — | — | 5 | LE | The latch is transparent when LE is low. The comparator output is stored when LE is high. |
| — | — | 7 | QOUT | Comparator Output |
| — | — | 8 | QOUT | Inverted Comparator Output |

MAX907/MAX908/MAX909

Detailed Description

Timing Noise or undesired parasitic AC feedback  cause most high-speed comparators to oscillate in the linear region (i.e., when the voltage on one input is at or near the

voltage on the other input). The MAX907/MAX908/

MAX909 eliminate this problem  by incorporating  inter- nal hysteresis. When the two comparator input voltages are equal, hysteresis effectively causes one comparator input voltage to move quickly past the other, thus taking the input out of the region where oscillation occurs. Standard comparators require that hysteresis be added through the use of external resistors. The MAX907/MAX908/MAX909’s fixed internal hysteresis eliminates these resistors (and the equations required to determine appropriate values).

Adding  hysteresis to a comparator  creates two trip points: one for the input voltage rising and one for the input voltage falling (Figure 1). The difference between these two input-referred trip points is the hysteresis.

Figure 1 illustrates the case where IN- is fixed and IN+ is varied. If the inputs were reversed, the figure would look the same, except the output would be inverted.

The MAX909 includes  an internal latch, allowing the result of a comparison  to be stored. If LE is low, the latch is transparent (i.e., the comparator  operates as though the latch is not present). The state of the com- parator output is stored when LE is high (Figure 2).

Note that the MAX909 can be operated  with V- con- nected to ground or to a negative supply voltage. The MAX909’s input range extends from (V- - 0.2V) to (V+ - 1.5V).

VTRIP+

VHYST

VIN+

VOS

=

VTRIP+ + VTRIP-

2

3V LE	1.4V

COMPARE

LATCH

VTRIP-

VIN- = 0V

0V	th

ts

COMPARATOR OUTPUT

VOH

DIFFERENTIAL INPUT VOLTAGE

VOS

VOD

VIN

VOL

Figure 1. Input and Output Waveforms, Noninverting Input

Varied

Applications Information

Circuit Layout

OUTPUT (QOUT)

OUTPUT (QOUT)

VOH

1.4V VOL

VOH

1.4V VOL

tPD+

tSKEW

MAX907/MAX908/MAX909

Because of the MAX907/MAX908/MAX909’s high gain bandwidth,  special precautions  must be taken to real- ize the full high-speed  capability.  A printed  circuit board  with a good,  low-inductance  ground  plane is mandatory. Place the decoupling  capacitor  (a 0.1µF ceramic capacitor  is a good choice) as close to V+ as possible. Pay close attention to the decoupling  capaci-

tor’s bandwidth,  keeping  leads  short. Short lead lengths on the inputs and outputs are also essential to avoid unwanted parasitic  feedback  around the com- parators. Solder the device directly to the printed circuit board instead of using a socket.

Overdriving the Inputs The inputs to the MAX907/MAX908/MAX909 may be driven beyond the voltage limits given in the Absolute Maximum Ratings, as long as the current flowing into

the device is limited to 25mA. However, if the inputs are

overdriven, the output may be inverted. The addition of an external diode prevents this inversion by limiting the

input  voltage  to  200mV  to  300mV  below  ground

(Figure 3).

Figure 2. MAX909 Timing Diagram

Battery-Operated Infrared Data Link Figure 4's circuit allows reception of infrared data. The MAX403 converts the photodiode  current to a voltage, and the MAX907 determines whether the amplifier output

is high enough to be called a “1”. The current consump-

tion of this circuit is minimal:  The MAX403 and MAX907 require typically 250µA and 700µA, respectively.

10pF

1MΩ

VCLAMP = -200mV  TO -300mV

1/2 MAX907

MAX403

2

+5V

7

0.1µF

+5V

0.1µF

SIEMENS BP-104	6	3	8

ISRC

PHOTODIODE     3

4

1

2	DATA

4

100kΩ

V-	1000pF

47kΩ

100kΩ

+5V

MAX907

1000pF PART TEMP RANGE PIN-PACKAGE

MAX908CPD 0°C to +70°C 14 Plastic DIP

MAX908EPD -40°C to +85°C 14 Plastic DIP

MAX908ESD -40°C to +85°C 14 SO

MAX909CPA 0°C to +70°C 8 Plastic DIP

MAX909CSA 0°C to +70°C 8 SO

MAX909EPA -40°C to +85°C 8 Plastic DIP

MAX909ESA -40°C to +85°C 8 SO

Figure 3. Schottky Clamp for Input Driven Below Ground

Figure 4. Battery-Operated Infrared Data Link Consumes Only

1mA

Pin  Configurations (continued)	   Ordering Information (continued)

TOP VIEW

V+    1

IN+    2

IN-  3

V-   4

MAX909

DIP/SO

8   QOUT

7   QOUT

6   GND

5    LE

MAX907/MAX908/MAX909

Chip Information

MAX907 TRANSISTOR COUNT: 262

MAX908 TRANSISTOR COUNT: 536

MAX909 TRANSISTOR COUNT: 140

PROCESS: Bipolar

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses  are implied. Maxim reserves the right to change the circuitry and specifications  without notice at any time.

10  	Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA  94086 408-737-7600

©  2006 Maxim Integrated  Products 	Printed USA 	is a registered  trademark  of Maxim Integrated  Products, Inc.