19-0194; Rev 1; 2/97

Ultra Low-Power, Low-Cost

Comparators with 2% Reference

General Description The MAX931-MAX934 single, dual, and quad micropower, low-voltage  comparators  plus an on-board  2% accurate

reference feature the lowest power consumption available.

These comparators  draw less than 4µA supply  current over temperature  (MAX931), and include  an internal

1.182V ±2% voltage reference, programmable  hysteresis, and TTL/CMOS outputs that sink and source current.

Ideal  for 3V or 5V single-supply  applications,  the MAX931-MAX934 operate  from a single +2.5V to +11V supply  (or a ±1.25V  to ±5V  dual  supply),  and  each comparator’s  input  voltage  range  extends  from the negative supply rail to within 1.3V of the positive supply.

The MAX931-MAX934’s unique output stage continuously sources  as much as 40mA. And by eliminating  power- supply  glitches  that commonly occur  when comparators change  logic  states, the MAX931-MAX934 minimize parasitic feedback, which makes them easier to use.

The single MAX931 and dual MAX932/MAX933 provide  a unique and simple method for adding  hysteresis without feedback  and complicated  equations, using the HYST pin and two resistors.

| For applications that require increased precision with similar power requirements, see the MAX921-MAX924 data sheet. These devices include a 1% precision reference. PART | INTERNAL 2% REFERENCE | COMPARATORS PER PACKAGE | INTERNAL HYSTERESIS | PACK- AGE |
| --- | --- | --- | --- | --- |
| MAX931 | Yes | 1 | Yes | 8-Pin DIP/SO/ µMAX |
| MAX932 | Yes | 2 | Yes | 8-Pin DIP/SO/ µMAX |
| MAX933 | Yes | 2 | Yes | 8-Pin DIP/SO/ µMAX |
| MAX934 | Yes | 4 | No | 16-Pin DIP/SO |

Features

♦ Ultra-Low 4µA Max Quiescent Current

Over Extended Temp. Range (MAX931)

♦ Power Supplies:

Single +2.5V to +11V Dual ±1.25V to ±5.5V

♦ Input Voltage Range Includes Negative Supply

♦ Internal 1.182V ±2% Bandgap Reference

♦ Adjustable Hysteresis

♦ TTL-/CMOS-Compatible Outputs

♦ 12µs Propagation Delay (10mV Overdrive)

♦ No Switching Crowbar Current

♦ 40mA Continuous Source Current

♦ Available in Space-Saving µMAX Package

Ordering Information

PART TEMP. RANGE PIN-PACKAGE

MAX931CPA 0°C to +70°C 8 Plastic DIP

MAX931CSA 0°C to +70°C 8 SO

MAX931CUA 0°C to +70°C 8 µMAX

MAX931EPA -40°C to +85°C 8 Plastic DIP

MAX931ESA -40°C to +85°C 8 SO

Ordering Information continued on last page.

For similar devices guaranteed over the military temp. range, see the MAX921-MAX924 data sheet. The MAX931, MAX933, and MAX934 are pin-compatible with the 1% accurate MAX921, MAX923, and MAX924, respectively. The MAX932 and MAX922 are not pin-compatible.

Typical Operating Circuit

Applications

Battery-Powered Systems

VIN

3   IN+

4   IN-

5    HYST

7

V+

OUT   8

Threshold Detectors Window Comparators Oscillator Circuits

6    REF

MAX931

V-	GND

2	1

MAX931-MAX934

Alarm Circuits

THRESHOLD DETECTOR

ABSOLUTE MAXIMUM RATINGS

V+ to V-, V+ to GND, GND to V-................................-0.3V,  +12V Inputs

Current, IN_+, IN_-, HYST ...............................................20mA Voltage, IN_+, IN_-, HYST................(V+ + 0.3V) to (V- – 0.3V)

Outputs

Current, REF....................................................................20mA Current, OUT_ .................................................................50mA Voltage, REF ....................................(V+ + 0.3V) to (V- – 0.3V) Voltage, OUT_ (MAX931/934) .....(V+ + 0.3V) to (GND – 0.3V) Voltage, OUT_ (MAX932/933) ..........(V+ + 0.3V) to (V- – 0.3V) OUT_ Short-Circuit Duration (V+  5.5V) ...............Continuous

Continuous Power Dissipation (TA = +70°C)

8-Pin Plastic DIP (derate 9.09mW/°C above +70°C) ...727mW

8-Pin SO (derate 5.88mW/°C above +70°C)................471mW

8-Pin µMAX (derate 4.1mW/°C above +70°C) .............330mW

16-Pin Plastic DIP (derate 10.53mW/°C above +70°C)..842mW

16-Pin SO (derate 8.70mW/°C above +70°C) ................696mW Operating Temperature Ranges:

MAX93_C_ _ .......................................................0°C to +70°C MAX93_E_ _.....................................................-40°C to +85°C Storage Temperature Range .............................-65°C to +150°C Lead Temperature (soldering,  10sec) .............................+300°C

MAX931-MAX934

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS—5V Operation

(V+ = 5V, V- = GND = 0V, TA = TMIN to TMAX, unless otherwise noted.)

| PARAMETER | CONDITIONS |  |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| POWER REQUIREMENTS |  |  |  |  |  |
| Supply Voltage Range | (Note 1) |  |  | 2.5 11 | V |
| Supply Current | IN+ = IN- + 100mV | MAX931, HYST = REF | TA = +25°C | 2.5 3.2 | µA |
|  |  |  | C/E temp. ranges | 4 |  |
|  |  | MAX932, HYST = REF | TA = +25°C | 3.1 4.5 |  |
|  |  |  | C/E temp. ranges | 6 |  |
|  |  | MAX933, HYST = REF | TA = +25°C | 3.1 4.5 |  |
|  |  |  | C/E temp. ranges | 6 |  |
|  |  | MAX934 | TA = +25°C | 5.5 6.5 |  |
|  |  |  | C/E temp. ranges | 8.5 |  |
| COMPARATOR |  |  |  |  |  |
| Input Offset Voltage | VCM = 2.5V |  |  | ±10 | mV |
| Input Leakage Current (IN-, IN+) | IN+ = IN- = 2.5V, C/E temp. ranges |  |  | ±0.01 ±5 | nA |
| Input Leakage Current (HYST) | MAX931, MAX932, MAX933 |  |  | ±0.02 | nA |
| Input Common-Mode Voltage Range |  |  |  | V- V+ – 1.3 | V |
| Common-Mode Rejection Ratio | V- to (V+ – 1.3V) |  |  | 0.1 1.0 | mV/V |
| Power-Supply Rejection Ratio | V+ = 2.5V to 11V |  |  | 0.1 1.0 | mV/V |
| Voltage Noise | 100Hz to 100kHz |  |  | 20 | µVRMS |
| Hysteresis Input Voltage Range | MAX931, MAX932, MAX933 |  |  | REF – 0.05 REF | V |
| Response Time | TA = +25°C, 100pF load |  | Overdrive = 10mV | 12 | µs |
|  |  |  | Overdrive = 100mV | 4 |  |

MAX931-MAX934

ELECTRICAL CHARACTERISTICS—5V Operation (continued)

(V+ = 5V, V- = GND = 0V, TA = TMIN to TMAX, unless otherwise noted.)

X

| PARAMETER | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Output High Voltage | C/E temp. ranges, IOUT = 17mA |  | V+ – 0.4 | V |
| Output Low Voltage | C/E temp. ranges, IOUT = 1.8mA | MAX932, MAX933 | V- + 0.4 | V |
|  |  | MAX931, MAX934 | GND + 0.4 |  |
| REFERENCE |  |  |  |  |
| Reference Voltage | C temp. range |  | 1.158 1.182 1.206 | V |
|  | E temp. range |  | 1.147 1.217 |  |
| Source Current | TA = +25°C |  | 15 25 | µA |
|  | C/E temp. ranges |  | 6 |  |
| Sink Current | TA = +25°C |  | 8 15 | µA |
|  | C/E temp. ranges |  | 4 |  |
| Voltage Noise | 100Hz to 100kHz |  | 100 | µVRMS |

Note 1: MAX934 comparators work below 2.5V, see Low-Voltage Operation section for more details.

ELECTRICAL CHARACTERISTICS—3V Operation

(V+ = 3V, V- = GND = 0V, TA = TMIN to TMAX, unless otherwise noted.)

| PARAMETER | CONDITIONS |  |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| POWER REQUIREMENTS |  |  |  |  |  |
| Supply Current | IN+ = (IN- + 100mV) | MAX931, HYST=REF | TA = +25°C | 2.4 3.0 | µA |
|  |  |  | C/E temp. ranges | 3.8 |  |
|  |  | MAX932, HYST=REF | TA = +25°C | 3.4 4.3 |  |
|  |  |  | C/E temp. ranges | 5.8 |  |
|  |  | MAX933, HYST=REF | TA = +25°C | 3.4 4.3 |  |
|  |  |  | C/E temp. ranges | 5.8 |  |
|  |  | MAX934 | TA = +25°C | 5.2 6.2 |  |
|  |  |  | C/E temp. ranges | 8.0 |  |
| COMPARATOR |  |  |  |  |  |
| Input Offset Voltage | VCM = 1.5V |  |  | ±10 | mV |
| Input Leakage Current (IN-, IN+) | IN+ = IN- = 1.5V, C/E temp. ranges |  |  | ±0.01 ±1 | nA |
| Input Leakage Current (HYST) | MAX931, MAX932, MAX933 |  |  | ±0.02 | nA |

MAX931-MAX934

ELECTRICAL CHARACTERISTICS—3V Operation (continued)

(V+ = 3V, V- = GND = 0V, TA = TMIN to TMAX, unless otherwise noted.)

| PARAMETER | CONDITIONS |  |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| Input Common-Mode Voltage Range |  |  |  | V- V+ – 1.3 | V |
| Common-Mode Rejection Ratio | V- to (V+ – 1.3V) |  |  | 0.2 1 | mV/V |
| Power-Supply Rejection Ratio | V+ = 2.5V to 11V |  |  | 0.1 1 | mV/V |
| Voltage Noise | 100Hz to 100kHz |  |  | 20 | µVRMS |
| Hysteresis Input Voltage Range | MAX931, MAX932, MAX933 |  |  | REF – 0.05 REF | V |
| Response Time | TA = +25°C, 100pF load | Overdrive = 10mV |  | 14 | µs |
|  |  | Overdrive = 100mV |  | 5 |  |
| Output High Voltage | C/E temp. ranges, IOUT = 10mA |  |  | V+ – 0.4 | V |
| Output Low Voltage | C/E temp. ranges, IOUT = 0.8mA |  | MAX932, MAX933 | V- + 0.4 | V |
|  |  |  | MAX931 | GND + 0.4 |  |
| REFERENCE |  |  |  |  |  |
| Reference Voltage | C temp. range |  |  | 1.158 1.182 1.206 | V |
|  | E temp. range |  |  | 1.147 1.217 |  |
| Source Current | TA = +25°C |  |  | 15 25 | µA |
|  | C/E temp. ranges |  |  | 6 |  |
| Sink Current | TA = +25°C |  |  | 8 15 | µA |
|  | C/E temp. ranges |  |  | 4 |  |
| Voltage Noise | 100Hz to 100kHz |  |  | 100 | µVRMS |

Typical Operating Characteristics

(V+ = 5V, V- = GND, TA = +25°C, unless otherwise noted.)

2.5

2.0

OUTPUT VOLTAGE LOW

vs. LOAD CURRENT

MAX921/4-TOC1

5.0

4.5

4.0

OUTPUT VOLTAGE HIGH vs.

LOAD CURRENT

1.190

1.185

1.180

REFERENCE OUTPUT VOLTAGE vs.

OUTPUT LOAD CURRENT

MAX921/924-TOC3

SINK

VOL (V)

1.5

1.0

VOH (V)

3.5

3.0

MAX921/924-TOC2

1.175

1.170

SOURCE

0.5

0.0

| 0 |  |  |  |  | V+ = 5V |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | V+ = 3V |  |  |  |  |  |  |  |  |

4	8	12	16

2.5

2.0

1.5

20

| 0 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | V+ = 5V |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | V+ = 3V |  |  |

20	30	40

1.165

REFERENCE OUTPUT VOLTAGE (V)

1.160

1.155

50

V+ = 5V OR

V+ = 3V

0	5	10

15	20

25	30

1.22

1.21

1.20

1.19

1.18

1.17

REFERENCE VOLTAGE (V)

1.16

1.15

1.14

LOAD CURRENT (mA)

REFERENCE VOLTAGE

vs. TEMPERATURE

EXTENDED TEMP. RANGE COMMERCIAL

TEMP. RANGE

MAX921/924-TOC4

SUPPLY CURRENT ( A)

4.5

4.0

3.5

3.0

2.5

2.0

LOAD CURRENT (mA)

MAX931

MAX921/924-TOC5

| SUPPLY CURRENT vs. TEMPERATURE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V+ = 5V, V- = -5V |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | V+ = 3V, V- = 0V |  |  |  |
| V+ = 5V, V- = 0V |  |  |  |  |  |  |  |  |  |

5.0

4.5

3.5

SUPPLY CURRENT ( A)

3.0

2.5

2.0

OUTPUT LOAD CURRENT ( A)

MAX932

MAX921/924-TOC6

SUPPLY CURRENT vs. TEMPERATURE

IN+ = IN- +100mV

V+ = 5V, V- = 0V

V+ = 3V, V- = 0V

-60 -40  -20   0

20 40

60 80 100 120 140

-60

-20	20

60	100

140

-60

-20	20

60	100

140

5.0

4.5

4.0

3.5

SUPPLY CURRENT ( A)

3.0

2.5

TEMPERATURE ( C)

MAX933

MAX921/924-TOC7

| SUPPLY CURRENT vs. TEMPERATURE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IN+ = IN- +100mV |  |  |  |  |  |  |  |  |  |
|  |  | V+ = 5V, V- = 0V |  |  |  |  |  |  |  |
|  |  | V+ = 3V, V- = 0V |  |  |  |  |  |  |  |

TEMPERATURE ( C)

MAX934

MAX921/924-TOC8

SUPPLY CURRENT vs. TEMPERATURE

10

IN+ = (IN- + 100mV)

9

SUPPLY CURRENT ( A)

8

7

V+ = 5V, V- = -5V

6

SUPPLY CURRENT ( A)

5	V+ = 5V, V- = 0V

4	V+ = 3V, V- = 0V

10

1

0.1

TEMPERATURE ( C)

MAX934

SUPPLY CURRENT vs. LOW SUPPLY VOLTAGES

MAX921/924-TOC9

IN+ = IN- +100mV

2.0

-60

-20	20

60	100

140

3

-60

-20	20

60	100

140

0.01

1.0

1.5

2.0	2.5

MAX931-MAX934

TEMPERATURE ( C)

TEMPERATURE ( C)

SINGLE-SUPPLY VOLTAGE (V)

MAX921/924 TOC12

Typical Operating Characteristics (continued)

(V+ = 5V, V- = GND, TA = +25°C, unless otherwise noted.)

80

| 60 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OUTPUT HIGH |  |  |  |  |  |  |  |  |  |
|  |  |  |  | NO CHANGE |  |  |  |  |  |
| OUTPUT LOW |  |  |  |  |  |  |  |  |  |

40

20

0

-20

-40

-60

-80

HYSTERESIS CONTROL

5.0

4.5

MAX921/924 TOC10

OUTPUT VOLTAGE (V)

4.0

RESPONSE TIME ( s)

3.5

3.0

2.5

2.0

1.5

1.0

0.5

0

TRANSFER FUNCTION

100k

V0

10 F

RESPONSE TIME  vs. LOAD CAPACITANCE

MAX921/924-TOC11

| 18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | VOHL |  |  |  |  |  |
|  |  |  |  | VOLH |  |  |  |  |  |

16

14

12

10

8

6

4

2

0	10

20	30	40	50

-0.3

-0.2

-0.1

0	0.1

0.2

0.3

0	20

40	60

80	100

OUTPUT VOLTAGE (V)

| 5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 100mV |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 20mV |  |  |  |  |
| 50mV |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 10mV |  |  |

4

3

2

1

0

INPUT VOLTAGE (mV)

0

100

VREF -VHYST (mV)

MAX921/924-TOC13

RESPONSE TIME  FOR VARIOUS INPUT OVERDRIVES

OUTPUT VOLTAGE (V)

| 5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 10mV |  |  |  |  |
| 100mV |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 20mV |  |  |  |  |
| 50mV |  |  |  |  |  |  |  |  |  |

4

3

2

1

0

INPUT VOLTAGE (mV)

100

0

IN+ INPUT VOLTAGE (mV)

MAX921/924-TOC14

RESPONSE TIME  FOR VARIOUS INPUT OVERDRIVES

RESPONSE TIME (ms)

10

1

0.1

0.01

LOAD CAPACITANCE (nF)

MAX921/924-TOC15

MAX934 RESPONSE TIME AT LOW SUPPLY VOLTAGES

20mV OVERDRIVE

100mV OVERDRIVE

-2	2

6	10	14	18

-2	2

6	10	14	18

1.0

1.5

2.0	2.5

RESPONSE TIME ( s)

RESPONSE TIME ( s)

SINGLE-SUPPLY VOLTAGE (V)

100

CURRENT (mA)

10

1

0.1

MAX934  OUTPUT DRIVE AT LOW SUPPLY VOLTAGES

MAX921/924-TOC16

SOURCE CURRENT INTO 0.75V LOAD

SINK CURRENT AT VOUT = 0.4V

200

180

SINK CURRENT (mA)

160

SOURCE CURRENT (mA)

140

120

100

80

60

40

20

0

SHORT-CIRCUIT SOURCE CURRENT

vs. SUPPLY VOLTAGE

MAX121/124-TOC17

OUT CONNECTED TO V-

SHORT-CIRCUIT SINK CURRENT

MAX121/124-TOC18

vs. SUPPLY VOLTAGE

20

10

| 0 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | OUT CONNECTED TO V+ GND CONNECTED TO V- |  |  |  |  |  |  |  |  |  |

1.0

1.5

2.0	2.5

0	1.0

2.0

3.0

4.0

5.0	0

5	10

MAX931-MAX934

IN+ – IN- (V)

SINGLE-SUPPLY VOLTAGE (V)

TOTAL SUPPLY VOLTAGE (V)

TOTAL SUPPLY VOLTAGE (V)

MAX931-MAX934

Pin  Descriptions

| PIN |  |  | NAME | FUNCTION |
| --- | --- | --- | --- | --- |
| MAX931 | MAX932 | MAX933 |  |  |
| 1 | – | – | GND | Ground. Connect to V- for single-supply operation. Output swings from V+ to GND. |
| – | 1 | 1 | OUTA | Comparator A output. Sinks and sources current. Swings from V+ to V-. |
| 2 | 2 | 2 | V- | Negative supply. Connect to ground for single-supply operation (MAX931). |
| 3 | – | – | IN+ | Noninverting comparator input |
| – | 3 | 3 | INA+ | Noninverting input of comparator A |
| 4 | – | – | IN- | Inverting comparator input |
| – | 4 | – | INB+ | Noninverting input of comparator B |
| – | – | 4 | INB- | Inverting input of comparator B |
| 5 | 5 | 5 | HYST | Hysteresis input. Connect to REF if not used. Input voltage range is from VREF to VREF - 50mV. |
| 6 | 6 | 6 | REF | Reference output. 1.182V with respect to V-. |
| 7 | 7 | 7 | V+ | Positive supply |
| 8 | – | – | OUT | Comparator output. Sinks and sources current. Swings from V+ to GND. |
| – | 8 | 8 | OUTB | Comparator B output. Sinks and sources current. Swings from V+ to V-. |

| PIN | NAME | FUNCTION |
| --- | --- | --- |
| MAX934 |  |  |
| 1 | OUTB | Comparator B output. Sinks and sources current. Swings from V+ to GND. |
| 2 | OUTA | Comparator A output. Sinks and sources current. Swings from V+ to GND. |
| 3 | V+ | Positive supply |
| 4 | INA- | Inverting input of comparator A |
| 5 | INA+ | Noninverting input of comparator A |
| 6 | INB- | Inverting input of comparator B |
| 7 | INB+ | Noninverting input of comparator B |
| 8 | REF | Reference output. 1.182V with respect to V-. |
| 9 | V- | Negative supply. Connect to ground for single-supply operation. |
| 10 | INC- | Inverting input of comparator C |
| 11 | INC+ | Noninverting input of comparator C |
| 12 | IND- | Inverting input of comparator D |
| 13 | IND+ | Noninverting input of comparator D |
| 14 | GND | Ground. Connect to V- for single-supply operation. |
| 15 | OUTD | Comparator D output. Sinks and sources current. Swings from V+ to GND. |
| 16 | OUTC | Comparator C output. Sinks and sources current. Swings from V+ to GND. |

Detailed Description

The MAX931-MAX934 comprise various combinations of a micropower 1.182V reference and a micropower comparator.   The Typical Operating Circuit shows the MAX931 configuration,  and Figures 1a-1c show the MAX932/MAX933/MAX934 configurations.

Each comparator continuously sources up to 40mA, and the unique output stage eliminates crowbar glitches during output transitions.  This makes them immune to parasitic feedback  (which can cause instability) and provides excellent performance,  even when circuit- board layout is not optimal.

Internal hysteresis in the MAX931/MAX932/MAX933 provides the easiest method for implementing hysteresis.   It also produces  faster hysteresis action and consumes much less current than circuits using external positive feedback.

Power-Supply and Input Signal Ranges

This family of devices operates from a single +2.5V to

+11V power supply.   The MAX931 and MAX934 have

a separate ground  for the output driver, allowing operation with dual supplies ranging from ±1.25V to

±5.5V.   Connect V- to GND when operating  the MAX931 and the MAX934 from a single supply.   The maximum supply voltage in this case is still 11V.

For proper comparator operation, the input signal can be driven from the negative supply (V-) to within one volt of the positive supply (V+ - 1V).  The guaranteed common-mode input voltage range extends from V- to (V+ - 1.3V).  The inputs can be taken above and below the supply rails by up to 300mV without damage.

Operating the MAX931 and MAX934 at ±5V provides TTL/CMOS compatibility  when monitoring bipolar input signals.  TTL compatibility for the MAX932 and MAX933 is achieved by operation from a single +5V supply.

Low-Voltage Operation: V+ = 1V (MAX934 Only)

The guaranteed minimum operating voltage is 2.5V (or

±1.25V).  As the total supply voltage is reduced below

2.5V, the performance  degrades  and the supply

1    OUTA

2    V-

3    INA+

4    INB+

MAX932

V-

OUTB   8

V+   7

REF    6

HYST    5

1    OUTB OUTA

2

3     V+

MAX934

A	D

OUTC    16

OUTD   15

GND   14

Figure 1a. MAX932 Functional Diagram

INA-

4

INA+

5

IND+

13

IND-

12

1    OUTA

2    V-

3    INA+

4    INB-

MAX933

OUTB   8

V+   7

REF    6

HYST    5

B	C

6    INB-

7    INB+

8    REF

INC+    11

INC-    10

V-   9

V-

MAX931-MAX934

Figure 1b.  MAX933 Functional Diagram

Figure 1c.  MAX934 Functional Diagram

current falls.   The reference will not function below about 2.2V, although the comparators will continue to operate with a total supply voltage as low as 1V.  While the MAX934 has comparators  that may be used at supply voltages below 2V, the MAX931, MAX932, and MAX933 may not be  used  with supply  voltages significantly below 2.5V.

At low supply voltages, the comparators’ output drive is reduced  and the propagation  delay increases (see Typical Operating Characteristics).   The useful input voltage range extends from the negative supply to a

little under 1V below the positive supply,  which is slightly closer to the positive rail than the device operating from higher supply  voltages.   Test your prototype over the full temperature and supply-voltage range if operation below 2.5V is anticipated.

Comparator Output

With 100mV of overdrive, propagation delay is typically

3µs.  The Typical Operating Characteristics show the propagation delay for various overdrive levels.

The MAX931 and MAX934 output swings from V+ to

GND, so TTL compatibility  is assured by using a

+5V ±10% supply.  The negative supply does not affect the output swing, and can range from 0V to -5V ±10%.

The MAX932 and MAX933 do not have a GND pin, and their outputs swing from V+ to V-. Connect V- to ground and V+ to a +5V supply to achieve TTL compatibility.

The MAX931-MAX934’s unique design achieves an output source current of more than 40mA and a sink current of over 5mA, while keeping quiescent currents in the microampere range.   The output can source 100mA (at V+ = 5V) for short pulses, as long as the package's  maximum power dissipation  is not exceeded.  The output stage does not generate crowbar switching currents during transitions, which minimizes feedback through the supplies and helps ensure stability

As the input voltage approaches  the comparator's offset, the output begins to bounce back and forth; this peaks when   VIN = VOS. (The lowpass filter shown on the graph  averages out the bouncing,  making the transfer function easy to observe.) Consequently, the comparator has an effective wideband  peak-to-peak noise of around 0.3mV. The voltage reference has peak-to peak noise approaching  1mV. Thus, when a comparator is used with the reference, the combined peak-to-peak noise is about 1mV. This, of course, is much higher than the RMS noise of the individual components.  Care should be taken in the layout to avoid capacitive  coupling  from any output to the reference pin. Crosstalk can significantly increase the actual noise of the reference.

Applications Information

Hysteresis Hysteresis increases the comparators’ noise margin by increasing the upper threshold and decreasing  the lower threshold (see Figure 2).

Hysteresis (MAX931/MAX932/MAX933) To add hysteresis to the MAX931/MAX932/MAX933, connect  resistor R1 between REF and HYST, and connect resistor R2 between HYST and V- (Figure 3).  If no hysteresis is required, connect HYST to REF. When hysteresis is added,  the upper threshold increases by the same amount that the lower threshold decreases. The hysteresis band (the difference between the upper and lower thresholds, VHB) is approximately equal to twice the voltage between REF and HYST.  The HYST input can be adjusted to a maximum voltage of REF and to a minimum voltage of (REF - 50mV).   The

without bypassing.

Voltage Reference

IN+

THRESHOLDS

The internal bandgap  voltage reference has an output of 1.182V above V-.  Note that the REF voltage is referenced to V-, not to GND.  Its accuracy  is ±2% in the range 0°C to +70°C.   The REF output is typically capable  of sourcing 15µA and sinking 8µA.   Do not bypass the REF output.  For applications that require a

1% precision reference, see the MAX921-MAX924 data sheet.

Noise Considerations Although the comparators have a very high gain, useful gain is limited by noise. This is shown in the Transfer

IN-

VREF - VHYST

OUT

VHB

HYSTERESIS BAND

MAX931-MAX934

Function graph (see Typical Operating Characteristics).

Figure 2. Threshold Hysteresis Band

maximum difference between REF and HYST (50mV) will therefore produce  a 100mV max hysteresis band. Use the following equations to determine R1 and R2:

R1 = 	VHB

2	IREF

VHB

2. Choose the hysteresis voltage (VHB), the voltage between the upper and lower thresholds.   In this example, choose VHB = 50mV.

3. Calculate R1.

R1 =  R3	VHB

V

R2 =

1.182 –

2

IREF

10M	0.05

5

100k

Where IREF (the current sourced  by the reference) should not exceed  the REF source capability,  and should be significantly  larger than the HYST input current.   IREF values between 0.1µA and 4µA are

4. Choose the threshold voltage for VIN rising (VTHR). In this example, choose VTHR = 3V.

5. Calculate R2.

usually   appropriate.   If   2.4M   is   chosen   for

R2 (IREF = 0.5µA), the equation for R1 and VHB can be

R2 =

1

VTHR	1	1

approximated as:

R1 (k  ) =  VHB (mV)

When hysteresis is obtained  in this manner for the MAX932/MAX933, the same hysteresis applies to

(VREF	R1)

3

R1   R3

1

1	1

both comparators.

Hysteresis (MAX934)

(1.182	100k)

65.44k

100k

10M

Hysteresis can be set with two resistors using positive

feedback, as shown in Figure 4.  This circuit generally draws more current than the circuits using the HYST pin on the MAX931/MAX932/MAX933, and the high feedback  impedance  slows hysteresis.   The design procedure is as follows:

1. Choose R3. The leakage current of IN+ is under 1nA (up to +85°C), so the current through R3 can be around 100nA and still maintain good  accuracy. The current through R3 at the trip point is VREF/R3, or 100nA for R3 =  11.8M  .   10M   is a good practical value.

A 1% preferred value is 64.9k.

6. Verify the threshold voltages with these formulas:

VIN rising :

V	V	R1	1	1	1

THR	REF	R1	R2	R3

VIN falling :

R1		V VTHF	VTHR	R3

2.5V TO 11V

7 	R3

IREF

6 		V+ REF

R1	MAX931

MAX932

5	MAX933

HYST

R2	V-

2

R2

VREF

V+

MAX934 V-

GND

OUT

MAX931-MAX934

Figure 3. Programming the HYST Pin

Figure 4. External Hysteresis

Board Layout and Bypassing Power-supply bypass capacitors are not needed if the supply impedance is low, but 100nF bypass capacitors should be used when the supply impedance is high or when the supply leads are long. Minimize signal lead lengths to reduce stray capacitance  between the input and output that might cause instability.  Do not bypass the reference output.

Typical Applications

Auto-Off Power Source Figure 5 shows the schematic for a 40mA power supply that  has  a  timed  auto  power-off  function. 	The comparator output is the switched power-supply output. With a 10mA load, it typically provides a voltage of (VBATT - 0.12V), but draws only 3.5µA quiescent current. This circuit takes advantage of the four key features of the MAX931:   2.5 µA supply  current, an internal reference, hysteresis, and high current output.   Using the component values shown, the three-resistor voltage divider programs the maximum ±50mV of hysteresis and sets the IN- voltage at 100mV.  This gives an IN+ trip threshold of approximately 50mV for IN+ falling.

The RC time constant determines the maximum power- on time of the OUT pin before power-down  occurs. This period can be approximated by:

R x C x 4.6sec

For example:  2M x 10µF x 4.6 = 92sec.  The actual time will vary with both the leakage current of the capacitor and the voltage applied to the circuit.

Window Detector The MAX933 is ideal for making window detectors (undervoltage/overvoltage detectors).   The schematic is shown in Figure 6, with component values selected for an 4.5V undervoltage  threshold, and a 5.5V overvoltage threshold.   Choose different thresholds by changing  the values of R1, R2, and R3.  To prevent chatter at the output when the supply voltage is close to a threshold, hysteresis has been added  using R4 and R5.  OUTA provides an active-low undervoltage indication, and OUTB gives an active-low overvoltage indication.  ANDing the two outputs provides an active- high, power-good signal.

The design procedure is as follows:

1. Choose the required hysteresis level and calculate values for R4 and R5 according  to the formulas in the Hysteresis (MAX931/MAX932/MAX933) section. In this example, ±5mV of hysteresis has been added at the comparator input (VH = VHB/2).  This means that the hysteresis apparent  at VIN will be larger because of the input resistor divider.

2. Select R1.  The leakage current into INB- is normally under 1nA, so the current through R1 should exceed

4.5V TO 6.0V	7

V+

MOMENTARY SWITCH

VIN 	VOTH =  5.5V VUTH = 4.5V

R3

INA+

+5V V+

47k

1.1M

100k

6     REF

5    HYST

4    IN-

V-

MAX931

GND

IN+     3

OUT    8

C 	R

VBATT -0.15V

10mA

R2

R5

10k

R1	R4

HYST REF

INB-

OUTA

OUTB

UNDERVOLTAGE

POWER GOOD

OVERVOLTAGE

2	1	2.4M

V-	MAX933

MAX931-MAX934

Figure 5. Auto-off power switch operates on 2.5µA quiescent current.

Figure 6. Window Detector

100nA for the thresholds to be accurate.  R1 values up to about 10M can be used, but values in the

100k to 1M range are usually easier to deal with. In this example, choose R1 = 294k.

3. Calculate R2 + R3.  The overvoltage threshold should be 5.5V when VIN is rising.   The design equation is as follows:

VOTH

The full-scale threshold (all LEDs on) is given by

VIN = (R1 + R2)/R1 volts.  The other thresholds are at

3/4 full scale, 1/2 full scale, and 1/4 full scale.   The

output resistors limit the current into the LEDs.

Level Shifter Figure 8 shows a circuit to shift from bipolar ±5V inputs to TTL signals.   The 10k   resistors protect  the comparator inputs, and do not materially affect the

R2	R3	R1

294k

1.068M

1

VREF	VH

5.5		1 (1.182	0.005)

operation of the circuit.

Two-Stage Low-Voltage Detector Figure 9 shows the MAX932 monitoring an input voltage in two steps. When VIN is higher than the LOW and FAIL thresholds, outputs are high. Threshold

4. Calculate R2.  The undervoltage threshold should be 4.5V when VIN is falling.  The design equation is as follows:

calculations  are similar to those for the window- detector application.

R2	(R1 + R2 + R3)	(VREF

VUTH

VH )	R1

R1	R2

+5V

3

IN  V

(294k + 1.068M)	(1.182	0.005)

4.5

294k

V+

62.2k

Choose R2	61.9k   (1% standard value).

5. Calculate R3.

R3	(R2 + R3)	R2

1.068M	61.9k

1.006M

Choose R3	1M   (1% standard value).

6. Verify the resistor values.   The equations are as follows, evaluated for the above example.

Overvoltage threshold :

V	(V	V )	(R1	R2	R3)

182k

250k

1.182V

1V

750mV

8       REF

5     INA+

4      INA-

7     INB+

6      INB-

MAX934

V- OUTA

OUTB

9

2

330

1

330

OTH	REF	H 	R1

5.474V.

Undervoltage threshold :

250k

500mV

11     INC+

10     INC-

OUTC

16

330

V	(V	V )	(R1	R2	R3)

UTH	REF	H

4.484V,

(R1 + R2)

250k

250mV

13     IND+

12    IND-

OUTD    15

where the hysteresis voltage V	V	R5 .

330

H	REF	R4

250k

GND

MAX931-MAX934

Bar-Graph Level Gauge The high output source capability of the MAX931 series is useful for driving LEDs.  An example of this is the simple four-stage level detector  shown in Figure 7.

14

Figure 7. Bar-Graph Level Gauge

+5V

V+

VINA

VINB

10k

10k

INA+ INA-

INB+ INB-

MAX934

OUTA

OUTB

0 FOR VINA < 0V

1 FOR VINB > 0V

VIN

R3

R2	R5

INA+ REF

HYST

+5V V+

INPUT VOLTAGE FAIL

VINC

VIND

10k

10k

INC+ INC-

IND+

IND-

OUTC

OUTD

R4

INB+

R1

V-

MAX932

INPUT VOLTAGE LOW

GND

REF

V-

N.C.

MAX931-MAX934

-5V

Figure 8. Level Shifter: ±5V Input to CMOS Output	Figure 9. Two-Stage Low-Voltage Detector

Pin Configurations _Ordering Information (continued) PART TEMP. RANGE PIN-PACKAGE

MAX932CPA 0°C to +70°C 8 Plastic DIP

MAX932CUA 0°C to +70°C 8 µMAX

MAX932EPA -40°C to +85°C 8 Plastic DIP

MAX932ESA -40°C to +85°C 8 SO

MAX933CPA 0°C to +70°C 8 Plastic DIP

MAX933CSA 0°C to +70°C 8 SO

MAX933CUA 0°C to +70°C 8 µMAX

MAX933EPA -40°C to +85°C 8 Plastic DIP

MAX933ESA -40°C to +85°C 8 SO

MAX934CPE 0°C to +70°C 16 Plastic DIP

MAX934CSE 0°C to +70°C 16 Narrow SO

MAX934EPE -40°C to +85°C 16 Plastic DIP

MAX934ESE -40°C to +85°C 16 Narrow SO

TOP VIEW

GND V- IN+ IN-

1	8

2	7

3	MAX931	6

4	5

DIP/SO/  MAX

OUT V+

REF HYST

OUTA V- INA+ INB+

OUTA V- INA+ INB-

1	8

2	7

3	MAX932	6

4	5

DIP/SO/  MAX

1	8

2	7

3	MAX933	6

4	5

DIP/SO/  MAX

OUTB V+

REF HYST

OUTB V+

REF HYST

For similar devices guaranteed over the military temp. range, see the MAX921-MAX924 data sheet. The MAX931, MAX933, and MAX934 are pin-compatible with the 1% accurate MAX921, MAX923, and MAX924, respectively. The MAX932 and MAX922 are not pin-compatible.

OUTB   1

OUTA   2

V+   3

INA-    4

INA+   5

INB-   6

INB+   7

REF  8

MAX934

16    OUTC

15    OUTD

14    GND

13    IND+

12    IND-

11    INC+

10    INC-

9     V-

MAX931-MAX934

DIP/Narrow SO

MAX931-MAX934

SOICN.EPS

PDIPN.EPS

Package Information

MAX931-MAX934

8LUMAXD.EPS

Package Information (continued)

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses are implied. Maxim reserves the right to change the circuitry and specifications without notice at any time.

16    	Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA  94086 (408) 737-7600

©  1997 Maxim Integrated  Products 	Printed USA 	is a registered  trademark of Maxim Integrated  Products.