19-0229; Rev 8; 11/08

High-Speed, Low-Power, 3V/5V, Rail-to-Rail, Single-Supply Comparators

General Description

The MAX941/MAX942/MAX944 are single/dual/quad high-speed  comparators  optimized  for systems pow- ered from a 3V or 5V supply. These devices  combine high speed, low power, and rail-to-rail inputs. Propagation delay is 80ns, while supply current is only

350µA per comparator.

The input  common-mode  range  of the MAX941/ MAX942/MAX944 extends beyond  both power-supply rails. The outputs pull to within 0.4V of either supply rail without external pullup circuitry,  making these devices ideal for interface with both CMOS and TTL logic. All input and output pins can tolerate a continuous short- circuit fault condition to either rail.

Internal hysteresis ensures clean output switching, even with slow-moving input signals. The MAX941 fea- tures latch enable and device shutdown.

The single MAX941 and dual MAX942 are offered in a tiny µMAX® package. Both the single and dual MAX942 are available in 8-pin DIP and SO packages. The quad MAX944 comes in 14-pin DIP and narrow SO packages.

Features

♦ Available in µMAX Package for Automotive Applications

♦ Optimized for 3V and 5V Applications

(Operation Down to 2.7V)

♦ Fast, 80ns Propagation Delay (5mV Overdrive)

♦ Rail-to-Rail Input Voltage Range

♦ Low 350µA Supply Current per Comparator

♦ Low, 1mV Offset Voltage

♦ Internal Hysteresis for Clean Switching

♦ Outputs Swing 200mV of Power Rails

♦ CMOS/TTL-Compatible Outputs

♦ Output Latch (MAX941 Only)

♦ Shutdown Function (MAX941 Only)

Ordering Information PART TEMP RANGE PIN- PACKAGE

MAX941CPA 0°C to +70°C 8 Plastic DIP

MAX941CSA 0°C to +70°C 8 SO

MAX941EPA -40°C to +85°C 8 Plastic DIP

MAX941ESA -40°C to +85°C 8 SO

MAX941EUA-T -40°C to +85°C 8 µMAX

MAX941AUA-T -40°C to +125°C 8 µMAX

Applications

3V/5V Systems

Battery-Powered Systems Threshold Detectors/Discriminators Line Receivers

Zero-Crossing Detectors

Sampling Circuits

Ordering Information continued at end of data sheet.

µMAX is a registered  trademark of Maxim Integrated  Products, Inc.

Pin  Configurations

TOP VIEW

V+    1

IN+    2

MAX941

8   N.C.

7   OUT

OUTA    1

INA-    2

MAX942

8   V+

7   OUTB

OUTA    1

INA-    2

INA+    3

14  OUTD

13  IND-

A	D

12  IND+

IN-  3

SHDN  4

6   GND

5    LATCH

A

INA+    3

GND    4

6   INB-

B

5    INB+

V+  4

INB+  5

INB-   6

MAX944

B 	C

11  GND

10  INC+

9   INC-

DIP/SO/ μ MAX

DIP/SO/ μ MAX

OUTB    7

8   OUTC

MAX941/MAX942/MAX944

DIP/SO

Maxim Integrated Products   1

ABSOLUTE MAXIMUM RATINGS

Power-Supply Ranges

Supply Voltage V+ to GND .............................................+6.5V Differential Input Voltage ..........................-0.3V to (V+ + 0.3V) Common-Mode Input Voltage ..................-0.3V  to (V+ + 0.3V)

–————–

LATCH Input (MAX941 only) ....................-0.3V  to (V+ + 0.3V)

———–

SHDN Control Input (MAX941 only) .........-0.3V to (V+ + 0.3V)

Current Into Input Pins ..................................................±20mA Continuous Power Dissipation  (TA = +70°C)

8-Pin Plastic DIP (derate 9.09mW/°C above +70°C) ...727mW

8-Pin SO (derate 5.88mW/°C above +70°C)................471mW

8-Pin µMAX (derate 4.1mW/°C above +70°C) .............330mW

14-Pin Plastic DIP (derate 10.00mW/°C above +70°C)..800mW

14-Pin SO (derate 8.33mW/°C above +70°C)..............667mW Operating  Temperature  Ranges

MAX94_C_ _ .......................................................0°C to +70°C MAX94_E_ _.....................................................-40°C to +85°C MAX94_AUA ..................................................-40°C to +125°C MAX942MSA..................................................-55°C to +125°C Storage Temperature  Range .............................-65°C to +150°C Lead Temperature  (soldering,  10s) .................................+300°C

MAX941/MAX942/MAX944

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions  beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS

(V+ = 2.7V to 5.5V, TA = TMIN to TMAX, unless otherwise noted. Typical values are at TA = +25°C.) (Note 14)

A 	MIN

| A MIN PARAMETER | SYMBOL | CONDITIONS |  |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- | --- |
| Positive Supply Voltage | V+ |  |  |  | 2.7 5.5 | V |
| Input Voltage Range | VCMR | (Note 1) |  |  | -0.2 V+ + 0.2 | V |
| Input-Referred Trip Points | VTRIP | VCM = 0 or VCM = V+ (Note 2) | TA = +25°C | MAX94_C_ _, MAX94_EP_, MAX94_ES_, MAX942MSA | 1 3 | mV |
|  |  |  |  | MAX941_UA/MAX942_UA | 1 4 |  |
|  |  |  | T = T to TMAX | MAX94_C_ _, MAX94_EP_, MAX94_ES_, MAX942MSA | 4 | mV |
|  |  |  |  | MAX941_UA/MAX942_UA | 6 |  |
| Input Offset Voltage | VOS | VCM = 0 or VCM = V+ (Note 3) | TA = +25°C | MAX94_C_ _, MAX94_EP_, MAX94_ES_, MAX942MSA | 1 2 | mV |
|  |  |  |  | MAX941_UA/MAX942_UA | 1 3 |  |
|  |  |  | T = T to TMAX | MAX94_C_ _, MAX94_EP_, MAX94_ES_, MAX942MSA | 3 | mV |
|  |  |  |  | MAX941_UA/MAX942_UA | 5.5 |  |
| Input Bias Current | I | VIN = VOS, VCM = 0 or VCM = V+ (Note 4) |  | MAX94_C | 150 300 | nA |
|  |  |  |  | MAX94_E/A, MAX942MSA | 150 400 |  |
| Input Offset Current | IOS | VIN = VOS, VCM = 0 or V+ |  |  | 10 150 | nA |
| Input Differential Clamp Voltage | VCLAMP | Force 100µA into IN+, IN- = GND, measure VIN+ - VIN-, Figure 3 |  |  | 2.2 | V |
| Common-Mode Rejection Ratio | CMRR | (Note 5) |  | MAX94_C_ _, MAX94_EP_, MAX94_ES_, MAX942MSA | 80 300 | µV/V |
|  |  |  |  | MAX941_UA/MAX942_UA | 80 800 |  |
| Power-Supply Rejection Ratio | PSRR | 2.7V ≤ V+ ≤ 5.5V, VCM = 0V |  | MAX94_C_ _, MAX94_EP_, MAX94_ES_, MAX942MSA | 80 300 | µV/V |
|  |  |  |  | MAX941_UA/MAX942_UA | 80 350 |  |
| Output High Voltage | VOH | ISOURCE = 400µA |  |  | V+ - 0.4 V+ - 0.2 | V |
|  |  | ISOURCE = 4mA |  |  | V+ - 0.4 V+ - 0.3 |  |
| Output Low Voltage | VOL | ISINK = 400µA |  |  | 0.2 0.4 | V |
|  |  | ISINK = 4mA |  |  | 0.3 0.4 |  |
| Output Leakage Current | ILEAK | (Note 6) |  |  | 1 | µA |

B

ELECTRICAL CHARACTERISTICS (continued)

(V+ = 2.7V to 5.5V, TA = TMIN to TMAX, unless otherwise noted. Typical values are at TA = +25°C.) (Note 14)

PARAMETER 	SYMBOL 	CONDITIONS 	MIN 	TYP 	MAX 	UNITS

Supply Current per Comparator

ICC

V+ = 3V

V+ = 5V

MAX941

MAX942/MAX944

MAX941

MAX942/MAX944

380 	600

350 	500

430 	700 	µA

400 	600

MAX941 only, shutdown mode (V+ = 3V) 	12 	60

Power Dissipation  per

Comparator 	PD	(Note 7)

MAX941

MAX942/MAX944

1.0 	4.2

mW

1.0 	3.6

ns

200

ns ns

V

| V tPD+, MAX94_C Propagation Delay (Note 8) tPD- MAX94_E/A, MAX942MSA Differential Propagation Delay dtPD (Note 9) Propagation Delay Skew (Note 10) |  |  |  | 80 |  | 150 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 80 |  |  |  |
|  |  |  |  | 10 |  |  |  |
|  |  |  |  | 10 |  |  |  |
| V+ V+ Logic Input Voltage High VIH (Note 11) –––– + 0.4 –––– |  |  |  |  |  |  |  |
| 2 |  |  |  | 2 |  |  |  |
| V+ V+ Logic Input Voltage Low VIL (Note 11) –––– –––– - 0.4 |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  | 2 |  |
| Logic Input Current | IIL, IIH | VLOGIC = 0 or V+ (Note 11) |  |  | 2 | 10 | µA |
| Data-to-Latch Setup Time | tS | (Note 12) |  | 2 | 0 |  | ns |
| Latch-to-Data Hold Time | tH | (Note 12) |  | 3 | 0 |  | ns |
| Latch Pulse Width | tLPW | MAX941 only |  | 5 | 0 |  | ns |
| Latch Propagation Delay | tLPD | MAX941 only |  | 7 | 0 |  | ns |
| Shutdown Time |  | (Note 13) |  |  | 3 |  | µs |
| Shutdown Disable Time |  | (Note 13) |  | 1 | 0 |  | µs |

Note 1: 	Inferred from the CMRR test. Note also that either or both inputs can be driven to the absolute maximum limit (0.3V

beyond either supply rail) without damage  or false output inversion.

Note 2: 	The input-referred trip points are the extremities of the differential  input voltage required  to make the comparator output change state. The difference  between the upper and lower trip points is equal to the width of the input-referred hysteresis zone (see Figure 1).

Note 3: 	VOS is defined as the center of the input-referred hysteresis zone (see Figure 1).

Note 4: 	The polarity of IB reverses direction  as VCM approaches either supply rail. See Typical Operating  Characteristics for more detail.

Note 5:	Specified over the full common-mode range (VCMR).

Note 6:	Applies to the MAX941 only when in shutdown mode. Specification is for current flowing into or out of the output pin for

VOUT driven to any voltage from V+ to GND.

Note 7:	Typical power dissipation specified with V+ = 3V; maximum with V+ = 5.5V.

Note 8:	Parameter is guaranteed by design and specified with VOD = 5mV and CLOAD = 15pF in parallel with 400µA of sink or source current. VOS is added to the overdrive voltage for low values of overdrive (see Figure 2).

Note 9:	Specified between any two channels in the MAX942/MAX944.

Note 10:  Specified as the difference between tPD+ and tPD- for any one comparator.

Note 11:  Applies to the MAX941 only for both –———– and –————– pins.

SHDN

LATCH

Note 12:  Applies to the MAX941 only. Comparator is active with –————– pin driven high and is latched with –————– pin driven low

(see Figure 2).

LATCH

LATCH

Note 13:  Applicable to the MAX941 only. Comparator is active with –———– pin driven high and is in shutdown with –———– pin driven

SHDN

low. Shutdown disable time is the delay when –———– is driven high to the time the output is valid.

SHDN

SHDN

MAX941/MAX942/MAX944

Note 14:  The MAX941_UA and MAX942_UA are 100% production tested at TA = +25°C. Specifications over temperature are guaranteed by design.

Typical Operating Characteristics

(V+ = 3.0V, TA = +25°C, unless otherwise noted.)

100

90

80

PROPAGATION DELAY vs.

MAX941-01

INPUT OVERDRIVE

tPD+

400

350

300

PROPAGATION DELAY vs. SOURCE IMPEDANCE

MAX941-02

tPD+

180

PROPAGATION DELAY (ns)

160

140

PROPAGATION DELAY vs.

MAX941-03

CAPACITIVE  LOAD

70

60

50

40	RS = 10Ω

CLOAD = 15pF

30

0	25	50

tPD-

75

100

250

200

PROPAGATION DELAY (ns)

150

100

50

10

CLOAD = 15pF VOD = 5mV

100	1k

10k

tPD-

100k

120

100

80

60

| 0 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  |  | tPD+ |  |
|  |  | tPD- |  |  |
| RS = 10Ω VOD = 5mV |  |  |  |  |

100

200

300

400

500

INPUT OVERDRIVE (mV)

SOURCE IMPEDANCE (Ω)

CAPACITIVE LOAD (pF)

130

120

PROPAGATION DELAY (ns)

110

PROPAGATION DELAY vs.

TEMPERATURE

MAX941-04

| 100 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RS = 10Ω CLOAD = 15pF VOD = 5mV |  |  |  |  |  |  |  |  |  |  |

90

80

PROPAGATION DELAY vs.

SUPPLY VOLTAGE

MAX941-05

tPD+

| 3.0 |  |  | TA = +125°C |  |
| --- | --- | --- | --- | --- |
|  | TA = +25°C TA = -55°C |  |  |  |

2.9

MAX941-06

OUTPUT HIGH VOLTAGE vs.

SOURCE CURRENT

100

90

tPD-

70

60

VOH (V)

2.8

2.7

80

70

60

-60 -40 -20   0

20  40  60

80  100 120 140

PROPAGATION DELAY (ns)

50

RS = 10Ω

40	CLOAD = 15pF

VOD = 5mV

30

2	3	4	5

2.6

2.5

6	1

10	100

1000

10,000

TEMPERATURE (°C)

SUPPLY VOLTAGE (V)

SOURCE CURRENT (μA)

MAX941/MAX942/MAX944

PROPAGATION DELAY (ns)

| 0.5 |  |  |  |
| --- | --- | --- | --- |
|  | TA = -55°C |  |  |
|  | TA = +25°C |  |  |
|  | TA = +125°C |  |  |

0.4

VOL (V)

0.3

0.2

0.1

0

1

OUTPUT LOW VOLTAGE

vs. SINK CURRENT

10	100	1000	10,000

SINK CURRENT (μA)

MAX941-07

600

500

400

300

SUPPLY CURRENT (μA)

200

100

0

2

MAX941 TOTAL SUPPLY CURRENT

vs. SUPPLY VOLTAGE

TA = +125°C TA = +25°C

TA = -55°C

3	4	5	6

SUPPLY VOLTAGE (V)

MAX941-08

800

700

SUPPLY CURRENT (μA)

600

500

400

300

MAX942 TOTAL SUPPLY CURRENT

MAX941-09

vs. SUPPLY VOLTAGE

|  | TA = +125°C |  |  |
| --- | --- | --- | --- |
|  | TA = +25°C |  |  |
|  | TA = -55°C |  |  |

2	3	4	5	6

SUPPLY VOLTAGE (V)

Typical Operating Characteristics (continued)

(V+ = 3.0V, TA = +25°C, unless otherwise noted.)

2.00

MAX944 TOTAL SUPPLY CURRENT

MAX941-10

vs. SUPPLY VOLTAGE

INPUT VOLTAGE RANGE

SHORT-CIRCUIT OUTPUT CURRENT (mA)

vs. TEMPERATURE

| 4 |  |  |  |  |  |  | V+ = 3.0V |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | VCM+ |  |  |  |  |  |  |
|  |  |  | VCM- |  |  |  |  |  |  |

SHORT-CIRCUIT OUTPUT CURRENT

MAX941-12

vs. TEMPERATURE

| 40 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | OUTPUT SHORTED TO V+ (SINKING) |  |  |  |  |  |  |  |
|  |  | OUTPUT SHORTED TO GND (SOURCING) |  |  |  |  |  |  |  |  |

1.75

TA = +125°C 	3	30

1.50

1.25

2

20

TA = +25°C

1

SUPPLY CURRENT (mA)

1.00

0.75

TA = -55°C

2	3	4	5	6

SUPPLY VOLTAGE (V)

INPUT VOLTAGE RANGE (V)

0

-1

MAX941-11

-60 -40 -20   0   20  40  60  80 100 120 140

TEMPERATURE (°C)

10

0

-60  -40 -20   0    20  40  60  80 100 120 140

TEMPERATURE (°C)

MAX941-13

VOLTAGE TRIP POINTS/INPUT OFFSET VOLTAGE vs. TEMPERATURE

INPUT BIAS CURRENT/INPUT OFFSET CURRENT vs. TEMPERATURE

INPUT BIAS CURRENT (IB+,  IB-)

vs. COMMON-MODE VOLTAGE

500

0

INPUT BIAS / OFFSET CURRENT (nA)

200

MAX941-14

200

150

TA = -55°C TA = +25°C

TA = +125°C

MAX941 TOC15

VIN+ = VIN- NEGATIVE VALUES REPRESENT CURRENT FLOWING INTO THE DEVICE

-500

TRIP POINTS / VOS (μV)

-1000

-1500

| -2000 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | VTRIP+ |  |  |  |  |  |
|  |  |  |  | VOS |  |  |  |  |  |
|  |  |  | VTRIP- |  |  |  |  |  |  |
| VCM = 0 |  |  |  |  |  |  |  |  |  |

-60 -40 -20   0   20  40  60  80 100 120 140

TEMPERATURE (°C)

100

50

| 0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | IB+ |  |  |  |  |  |
|  |  |  |  | IB- |  |  |  |  |  |
|  |  |  |  | IOS |  |  |  |  |  |

-60 -40 -20   0   20  40  60  80 100 120 140

TEMPERATURE (°C)

IB+, IB- (nA)

50

0

-50

-100

V+ = 6V

0	1	2	3	4	5	6

COMMON-MODE VOLTAGE (V)

MAX941 TOC16

MAX941 SHUTDOWN SUPPLY CURRENT

vs. TEMPERATURE

| 40 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | V+ = 6.0V |  |  |  |  |  |  |
|  |  |  | V+ = 2.7V |  |  |  |  |  |  |

10MHz RESPONSE

MAX941 TOC 17

SHUTDOWN SUPPLY CURRENT (μA)

30	VOS

20	V+

10

GND

INPUT

50mV/div

OUTPUT

1V/div

MAX941/MAX942/MAX944

0

-60 -40 -20   0    20  40  60  80 100 120 140

TEMPERATURE (°C)

50ns/div

Typical Operating Characteristics (continued)

(V+ = 3.0V, TA = +25°C, unless otherwise noted.)

VOD

VOS

V+ V+

2

GND

PROPAGATION DELAY (tPD+)

MAX941 TOC18

INPUT

50mV/div

OUTPUT

1V/div

VOS VOD V+

V+

2

GND

PROPAGATION DELAY (tPD-)

MAX941 TOC19

INPUT

50mV/div

OUTPUT

1V/div

20ns/div

tPD+

INPUT STEP = 100mV VOD = +5mV

20ns/div

tPD-

INPUT STEP = 100mV VOD = -5mV

MAX941/MAX942/MAX944

Pin  Description

| PIN |  |  | NAME | FUNCTION |
| --- | --- | --- | --- | --- |
| MAX941 | MAX942 | MAX944 |  |  |
| — | 1 | 1 | OUTA | Comparator A Output |
| — | 2 | 2 | INA- | Comparator A Inverting Input |
| — | 3 | 3 | INA+ | Comparator A Noninverting Input |
| 1 | 8 | 4 | V+ | Positive Supply (V+ to GND must be ≤ 6.5V) |
| — | 5 | 5 | INB+ | Comparator B Noninverting Input |
| — | 6 | 6 | INB- | Comparator B Inverting Input |
| — | 7 | 7 | OUTB | Comparator B Output |
| — | — | 8 | OUTC | Comparator C Output |
| — | — | 9 | INC- | Comparator C Inverting Input |
| — | — | 10 | INC+ | Comparator C Noninverting Input |
| 6 | 4 | 11 | GND | Ground |
| — | — | 12 | IND+ | Comparator D Noninverting Input |
| — | — | 13 | IND- | Comparator D Inverting Input |
| — | — | 14 | OUTD | Comparator D Output |
| 2 | — | — | IN+ | Noninverting Input |
| 3 | — | — | IN- | Inverting Input |
| 4 | — | — | ———– SHDN | Shutdown: MAX941 is active when ———– is driven high; MAX941 is in shutdown ———– SHDN when SHDN is driven low. |
| 5 | — | — | ————– LATCH | The output is latched when ————– is low. The latch is transparent when ————– LATCH LATCH is high. |
| 7 | — | — | OUT | Comparator Output |
| 8 | — | — | N.C. | No Connection. Not internally connected. |

VTRIP+

VHYST

VTRIP-

COMPARATOR OUTPUT

VIN+

V TRIP+  + VTRIP-

2

VOS =

VIN- = 0V

VOH

VOL

oscillation occurs.  Standard comparators  require hys- teresis to be  added  with external resistors. The MAX941/MAX942/MAX944’s fixed internal hysteresis eliminates these resistors and the equations needed to determine appropriate values.

Figure 1 illustrates the case where IN- is fixed and IN+ is varied. If the inputs were reversed, the figure would look the same, except the output would be inverted.

The MAX941 includes an internal latch that allows stor- age of comparison  results. The ————–  pin has a high input impedance. If ————– is high, the latch is transpar-

LATCH

LATCH

ent (i.e., the comparator operates as though the latch is not present). The comparator's  output state is stored

when ————–  is pulled  low. All timing constraints must

LATCH

be met when using the latch function (Figure 2).

Shutdown Mode (MAX941 Only)

The MAX941 shuts down when –———– is low. When shut

SHDN

down, the supply current drops to less than 60µA, and the three-state output becomes high impedance.  The

–———–

S H D N  pin  has  a  high  input  impedance.  Connect

–———–

SHDN  to V+ for normal operation. Exit shutdown with

————–

MAX941/MAX942/MAX944

Figure 1. Input and Output Waveform, Noninverting Input

Varied

Detailed Description

The MAX941/MAX942/MAX944 single-supply compara- tors feature internal hysteresis, high speed,  and low power. Their outputs are guaranteed to pull within 0.4V of either supply rail without external pullup or pulldown circuitry.  Rail-to-rail input voltage range and low-volt- age single-supply  operation make these devices ideal for portable equipment. The MAX941/MAX942/ MAX944 interface directly to CMOS and TTL logic.

Timing Most high-speed  comparators  oscillate in the linear region because  of noise or undesired  parasitic  feed- back.  This tends to occur  when the voltage on one input is at or equal to the voltage on the other input. To counter the parasitic  effects and noise, the MAX941/ MAX942/MAX944 have internal hysteresis.

The hysteresis in a comparator creates two trip points: one for the rising input voltage and one for the falling input voltage (Figure 1). The difference between the trip points is the hysteresis. When the comparator’s  input voltages are equal, the hysteresis effectively causes one comparator input voltage to move quickly past the other, thus taking the input out of the region where

LATCH high; otherwise, the output will be indeterminate.

Input Stage Circuitry The MAX941/MAX942/MAX944 include internal protec- tion circuitry  that prevents damage  to the precision input stage from large differential input voltages. This protection circuitry consists of two back-to-back diodes between IN+ and IN- as well as two 4.1kΩ resistors (Figure 3). The diodes  limit the differential  voltage applied to the internal circuitry of the comparators to be no more than 2VF, where VF is the forward voltage drop of the diode (about 0.7V at +25°C).

For a large differential input voltage (exceeding  2VF), this protection circuitry increases the input bias current at IN+ (source) and IN- (sink).

Input Current = (IN+ - IN-) - 2VF

2 x 4.1kΩ

Input current with large differential  input voltages should not be confused with input bias current (IB). As long as the differential input voltage is less than 2VF, this input current is equal to IB. The protection circuitry also allows for the input common-mode  range of the MAX941/MAX942/MAX944 to extend beyond  both power-supply  rails. The output is in the correct  logic state if one or both inputs are within the common-mode range.

DIFFERENTIAL INPUT VOLTAGE

tS

tH

V+

VOS

0V V+

tLPW

LATCH

V+

2

0V

VOH

tPD

tLPD

OUT

V+

2

VOL

Figure 2. MAX941 Timing Diagram with Latch Operator

MAX941/MAX942/MAX944

Output Stage Circuitry The MAX941/MAX942/MAX944 contain a current-driven output stage as shown in Figure 4. During an output transition, ISOURCE or ISINK is pushed or pulled to the output pin. The output source or sink current is high during  the transition, creating  a rapid slew rate. Once the output voltage reaches VOH or VOL, the source or sink current decreases  to a small value, capable  of maintaining the VOH or VOL static condition. This signifi- cant decrease in current conserves power after an out- put transition has occurred.

One consequence of a current-driven output stage is a linear dependence  between the slew rate and the load capacitance.  A heavy capacitive load will slow down a voltage output transition. This can be useful in noise- sensitive applications  where fast edges  may cause interference.

Applications Information

Circuit Layout and Bypassing The high gain bandwidth  of the MAX941/MAX942/ MAX944 requires design  precautions  to realize the comparators’  full high-speed  capability.  The recom- mended precautions are:

1)   Use a printed circuit  board  with a good, unbro- ken, low-inductance ground plane.

2)   Place a decoupling  capacitor  (a 0.1µF ceramic capacitor  is a good  choice)  as close to V+ as possible.

3)   Pay close attention to the decoupling  capacitor’s bandwidth, keeping leads short.

4)   On the inputs and outputs, keep lead lengths short to avoid  unwanted  parasitic  feedback around the comparators.

5)   Solder the device  directly  to the printed  circuit board instead of using a socket.

IN+

4.1kΩ

TO INTERNAL CIRCUITRY

MAX941

MAX942

MAX944

VCC

ISOURCE

MAX941

MAX942

MAX944

OUTPUT

IN–

4.1kΩ

TO INTERNAL CIRCUITRY

ISINK

0

Figure 3. Input Stage Circuitry 	Figure 4. Output Stage Circuitry

VDD = 3.3V

V+ = 3V

SERIAL DIGITAL

SDI

VREFC

VDD

10kΩ

INPUT

8-BIT DAC

MAX512

DACOUTC

COAX LINE

20kΩ

MAX941

3V

0V CLEAN

DIGITAL SIGNAL

GND

VSS

MAX941

20kΩ

MAX941/MAX942/MAX944

ANALOG IN

Figure 5. 3.3V Digitally Controlled Threshold Detector	Figure 6. Line Transceiver Application

MAX941/MAX942/MAX944

Ordering Information (continued) PART TEMP RANGE PIN- PACKAGE

MAX942MSA/PR -55°C to +125°C 8 SO

MAX942CPA 0°C to +70°C 8 Plastic DIP

MAX942CSA 0°C to +70°C 8 SO

MAX942EPA -40°C to +85°C 8 Plastic DIP

MAX942ESA -40°C to +85°C 8 SO

MAX942EUA-T -40°C to +85°C 8 µMAX

MAX942AUA-T -40°C to +125°C 8 µMAX

MAX944CPD 0°C to +70°C 14 Plastic DIP

MAX944CSD 0°C to +70°C 14 SO

MAX944EPD -40°C to +85°C 14 Plastic DIP

MAX942ESD -40°C to +85°C 14 SO

Chip Information

PROCESS: BiPOLAR

MAX941/MAX942/MAX944

8LUMAXD.EPS

Package Information

(For the latest package outline information and land patterns, go to  www.maxim-ic.com/packages.)

| PACKAGE TYPE | PACKAGE CODE | DOCUMENT NO. |
| --- | --- | --- |
| 8 µMAX | U8-1 | 21-0036 |
| 8 Plastic DIP | P8-1 | 21-0043 |
| 8 SO | S8-2 | 21-0041 |
| 14 Plastic DIP | P14-3 | 21-0043 |
| 14 SO | S14-1 | 21-0041 |

α

α

MAX941/MAX942/MAX944

Package Information (continued)

PDIPN.EPS

| (For the latest package outline information and land patterns, go to www.maxim-ic.com/packages.) |  |  |
| --- | --- | --- |

MAX941/MAX942/MAX944

SOICN .EPS

Package Information (continued)

| (For the latest package outline information and land patterns, go to www.maxim-ic.com/packages.) | INCHES |  | MILLIMETERS |  |
| --- | --- | --- | --- | --- |
| DIM | MIN | MAX | MIN | MAX |
| A | 0.053 | 0.069 | 1.35 | 1.75 |
| A1 | 0.004 | 0.010 | 0.10 | 0.25 |
| B | 0.014 | 0.019 | 0.35 | 0.49 |
| C | 0.007 | 0.010 | 0.19 | 0.25 |
| e | 0.050 BSC |  | 1.27 BSC |  |
| E | 0.150 | 0.157 | 3.80 | 4.00 |
| H | 0.228 | 0.244 | 5.80 | 6.20 |
| L | 0.016 | 0.050 | 0.40 | 1.27 |

|  | INCHES |  | MILLIMETERS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| DIM | MIN | MAX | MIN | MAX | N | MS012 |
| D | 0.189 | 0.197 | 4.80 | 5.00 | 8 | AA |
| D | 0.337 | 0.344 | 8.55 | 8.75 | 14 | AB |
| D | 0.386 | 0.394 | 9.80 | 10.00 | 16 | AC |

| N E H VARIATIONS: 1 TOP VIEW D A C e B A1 0∞-8∞ L FRONT VIEW SIDE VIEW |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | PROPRIETARY INFORMATION |  |  |  |
|  | TITLE: PACKAGE OUTLINE, .150" SOIC |  |  |  |
|  | APPROVAL | DOCUMENT CONTROL NO. 21-0041 | REV. B | 1 1 |

MAX941/MAX942/MAX944

Revision History

| REVISION NUMBER | REVISION DATE | DESCRIPTION | PAGES CHANGED |
| --- | --- | --- | --- |
| 8 | 12/08 | Added SO package diagram and removed transistor count | 10 |

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses  are implied. Maxim reserves the right to change the circuitry and specifications  without notice at any time.

14   	Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA  94086 (408) 737-7600

©  2008 Maxim Integrated  Products 	is a registered  trademark  of Maxim Integrated  Products, Inc.