19-1129; Rev 6; 2/08

Single/Dual/Quad, Ultra-High-Speed, +3V/+5V, Beyond-the-Rails Comparators

General Description

The MAX961–MAX964/MAX997/MAX999 are low-power, ultra-high-speed  comparators  with internal hysteresis. These devices  are optimized  for single +3V or +5V operation. The input common-mode  range extends

100mV Beyond-the-Rails™, and the outputs can sink or source 4mA to within 0.52V of GND and  VCC . Propagation delay is 4.5ns (5mV overdrive), while sup- ply current is 5mA per comparator.

The MAX961/MAX963/MAX964 and MAX997 have a shutdown mode in which they consume only 270µA supply current per comparator.  The MAX961/MAX963 provide complementary outputs and a latch-enable fea- ture. Latch enable allows the user to hold a valid com- parator output. The MAX999 is available  in a tiny SOT23-5 package.  The single MAX961/MAX997 and dual MAX962 are available in space-saving  8-pin

µMAX® packages.

Applications

Single 3V/5V Systems Portable/Battery-Powered Systems Threshold Detectors/Discriminators GPS Receivers

Line Receivers

Zero-Crossing Detectors

High-Speed Sampling Circuits

Features

♦ Ultra-Fast, 4.5ns Propagation Delay

♦ Ideal for +3V and +5V Single-Supply Applications

♦ Beyond-the-Rails Input Voltage Range

♦ Low, 5mA Supply Current (MAX997/MAX999)

♦ 3.5mV Internal Hysteresis for Clean Switching

♦ Output Latch (MAX961/MAX963)

♦ TTL/CMOS-Compatible Outputs

♦ Shutdown Mode

(MAX961/MAX963/MAX964/MAX997)

♦ Available in Space-Saving Packages:

5-Pin SOT23 (MAX999)

8-Pin µMAX (MAX961/MAX962/MAX997)

16-Pin QSOP (MAX964)

Ordering Information TOP PKG PART PIN-PACKAGE MARK CODE

MAX961ESA 8 SO — S8-2

MAX961EUA-T 8 µMAX-8 — U8-1

MAX962ESA 8 SO — S8-2

MAX962EUA-T 8 µMAX-8 — U8-1

MAX963ESD 14 SO — S14-1

MAX964ESE 16 Narrow SO — S16-1

MAX964EEE 16 QSOP — E16-1

MAX997ESA 8 SO — S8-2

MAX997EUA-T 8 µMAX-8 — U8-1

MAX999AAUK+T 5 SOT23-5 +AFEI U5+1

MAX999EUK-T 5 SOT23-5 ACAB U5-1

| Selector Guide PART | NO. OF COMPARATORS | COMPLEMENTARY OUTPUT | SHUTDOWN | LATCH ENABLE | PIN-PACKAGE |
| --- | --- | --- | --- | --- | --- |
| MAX961 | 1 | Yes | Yes | Yes | 8 SO/µMAX |
| MAX962 | 2 | No | No | No | 8 SO/µMAX |
| MAX963 | 2 | Yes | Yes | Yes | 14 SO |
| MAX964 | 4 | No | Yes | No | 16 SO/QSOP |
| MAX997 | 1 | No | Yes | No | 8 SO/µMAX |
| MAX999 | 1 | No | No | No | 5 SOT23 |

Note: All E grade devices are specified over the -40°C to +85°C

operating temperature range. MAX999AAUK is specified over the

-40°C to +125°C operating temperature range.

+Denotes a lead-free package.

Pin  Configurations

TOP VIEW

Q    1	5

VCC

GND   2

MAX999

IN+    3	4

IN-

MAX961–MAX964/MAX997/MAX999

Beyond-the-Rails is a trademark and µMAX is a registered trademark of Maxim Integrated Products, Inc.

SOT23-5

Pin Configurations continued at end of data sheet.

ABSOLUTE MAXIMUM RATINGS

Supply Voltage, VCC to GND................................... -0.3V to +6V All Other Pins..............................................-0.3V to (VCC + 0.3V) Current into Input Pins ......................................................±20mA Duration of Output Short Circuit to GND or VCC .......Continuous Continuous Power Dissipation  (TA = +70°C)

5-Pin SOT23 (derate 7.1mW/°C above +70°C).......571mW/°C

8-Pin SO (derate 5.88mW/°C above +70°C)...........471mW/°C

8-Pin µMAX (derate 4.10mW/°C above +70°C) ......330mW/°C

14-Pin SO (derate 8.33mW/°C above +70°C).........667mW/°C

16-Pin SO (derate 8.70mW/°C above +70°C).........696mW/°C

16-Pin QSOP (derate 8.33mW/°C above +70°C)....667mW/°C Operating  Temperature  Range

MAX96_E/MAX99_E.........................................-40°C to +85°C MAX999AAUK ...............................................-40°C to +125°C Storage Temperature  Range ............................ -65°C to +160°C Lead Temperature  (soldering,  10s) .................................+300°C

MAX961–MAX964/MAX997/MAX999

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions  beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS

(VCC = +2.7V to +5.5V, VCM = 0V, COUT = 5pF, VSHDN = 0V, VLE = 0V, unless otherwise noted. TMIN to TMAX is -40°C to +85°C for all

E grade devices.  For MAX999AAUK only, TMIN to TMAX is -40°C to +125°C.)  (Note 1)

| PARAMETER | SYMBOL | CONDITIONS |  | TA = +25°C MIN TYP MAX | TMIN to TMAX MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- | --- |
| Supply Voltage | VCC | Inferred by PSRR |  | 2.7 5.5 | 2.7 5.5 | V |
| Input Common-Mode Voltage Range | VCMR | (Note 2) |  | VCC + -0.1 0.1 | VCC + -0.1 0.1 | V |
| Input-Referred Trip Points | VTRIP | VCM = -0.1V or 5.1V, VCC = 5V (Note 3) | µMAX, SOT23 | ±2.0 ±3.5 | ±6.5 | mV |
|  |  |  | MAX999AAUK | ±2.0 ±3.5 | ±8.0 |  |
|  |  |  | All other E packages | ±2.0 ±3.5 | ±4.0 |  |
| Input-Referred Hysteresis |  |  |  | 3.5 |  | mV |
| Input Offset Voltage | VOS | VCM = -0.1V or 5.1V, VCC = 5V (Note 4) | µMAX, SOT23 | ±0.5 ±1.5 | ±4.5 | mV |
|  |  |  | MAX999AAUK | ±0.5 ±1.5 | ±6.0 |  |
|  |  |  | All other E packages | ±0.5 ±1.5 | ±2.0 |  |
| Input Bias Current | IB | VIN+ = VIN- = 0V or VCC, VCC = 5V | µMAX, SOT23 | ±15 | ±30 | µA |
|  |  |  | All other E packages | ±15 | ±15 |  |
| Differential Input Clamp Voltage |  | VCC = 5.5V, VIN- = 0V, IIN+ = 100µA |  | 2.1 |  | V |
| Input Capacitance |  |  |  | 3 |  | pF |
| Differential Input Impedance | RIND | VCC = 5V |  | 8 |  | kΩ |
| Common-Mode Input | RINCM | VCC = 5V |  | 130 |  | kΩ |

MAX961–MAX964/MAX997/MAX999

ELECTRICAL CHARACTERISTICS (continued)

(VCC = +2.7V to +5.5V, VCM = 0V, COUT = 5pF, VSHDN = 0V, VLE = 0V, unless otherwise noted. TMIN to TMAX is -40°C to +85°C for all

E grade devices.  For MAX999AAUK only, TMIN to TMAX is -40°C to +125°C.)  (Note 1)

| PARAMETER | SYMBOL | CONDITIONS |  | TA = +25°C | TMIN to TMAX | UNITS |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | MIN TYP MAX | MIN TYP MAX |  |
| Common-Mode Rejection Ratio | CMRR | VCC = 5V, VCM = -0.1V to 5.1V (Note 5) | µMAX, SOT23 | 0.1 0.3 | 1.0 | mV/V |
|  |  |  | All other E packages | 0.1 0.3 | 0.5 |  |
| Power-Supply Rejection Ratio | PSRR | VCM = 0V (Note 6) |  | 0.05 0.3 | 0.3 | mV/V |
| Output High Voltage | VOH | ISOURCE = 4mA | E grade | VCC - 0.52 | VCC - 0.52 | V |
|  |  |  | MAX999AAUK | VCC - 0.52 | VCC - 0.55 |  |
| Output Low Voltage | VOL | ISINK = 4mA | E grade | 0.52 | 0.52 | V |
|  |  |  | MAX999AAUK | 0.52 | 0.55 |  |
| Capacitive Slew Current |  | VOUT = 1.4V, VCC = 2.7V |  | 30 60 |  | mA |
| Output Capacitance |  |  |  | 4 |  | pF |
| Supply Current per Comparator | ICC | MAX961/MAX963, VCC = 5V |  | 7.2 11 | 11 | mA |
|  |  | MAX962/MAX964, VCC = 5V |  | 5 8 | 9 |  |
|  |  | MAX997/MAX999E, VCC = 5V |  | 5 6.5 | 6.5 |  |
|  |  | MAX999AAUK, VCC = 5V |  | 5 6.5 | 7.0 |  |
| Shutdown Supply Current per Comparator | ISHDN | MAX961/MAX963/MAX964/ MAX997, VCC = 5V |  | 0.27 0.5 | 0.5 | mA |
| Shutdown Output Leakage Current |  | MAX961/MAX963/MAX964/ MAX997, VOUT = 5V and VCC - 5V |  | 1 | 20 | µA |
| Rise/Fall Time | tR, tF | VCC = 5V |  | 2.3 |  | ns |
| Logic-Input High | VIH |  |  | VCC/2 + 0.4 | VCC/2 + 0.4 | V |
| Logic-Input Low | VIL |  |  | VCC/2 - 0.4 | VCC/2 - 0.4 | V |
| Logic-Input Current | IIL, IIH | VLOGIC = 0V or VCC |  | ±15 | ±30 | µA |
| Propagation Delay | tPD | 5mV overdrive (Note 7) | E grade | 4.5 7.0 | 8.5 | ns |
|  |  |  | MAX999AAUK | 4.5 7.0 | 10 |  |
| Differential Propagation Delay | tPD | Between any two channels or outputs (Q/Q) |  | 0.3 |  | ns |
| Propagation-Delay Skew | tSKEW | Between tPD- and tPD+ |  | 0.3 |  | ns |

MAX961–MAX964/MAX997/MAX999

ELECTRICAL CHARACTERISTICS (continued)

(VCC = +2.7V to +5.5V, VCM = 0V, COUT = 5pF, VSHDN = 0V, VLE = 0V, unless otherwise noted. TMIN to TMAX is -40°C to +85°C for all

E grade devices.  For MAX999AAUK only, TMIN to TMAX is -40°C to +125°C.)  (Note 1)

| PARAMETER | SYMBOL | CONDITIONS | TA = +25°C | TMIN to TMAX | UNITS |
| --- | --- | --- | --- | --- | --- |
|  |  |  | MIN TYP MAX | MIN TYP MAX |  |
| Data-to-Latch Setup Time | tSU | MAX961/MAX963 (Note 8) | 5 | 5 | ns |
| Latch-to-Data Hold Time | tH | MAX961/MAX963 (Note 8) | 5 | 5 | ns |
| Latch Pulse Width | tLPW | MAX961/MAX963 (Note 8) | 5 | 5 | ns |
| Latch Propagation Delay | tLPD | MAX961/MAX963 (Note 8) | 10 | 10 | ns |
| Shutdown Time | tOFF | Delay until output is high-Z (> 10kΩ) | 150 |  | ns |
| Shutdown Disable Time | tON | Delay until output is valid | 250 |  | ns |

Note 1:  The MAX961EUA/MAX962EUA/MAX997EUA/MAX999EUK are 100% production tested at TA = +25°C; all temperature  specifica- tions are guaranteed by design.

Note 2:  Inferred by CMRR. Either input can be driven to the absolute maximum limit without false output inversion, provided that the other input is within the input voltage range.

Note 3:  The input-referred trip points are the extremities of the differential input voltage required to make the comparator output change

state. The difference  between the upper and lower trip points is equal to the width of the input-referred hysteresis zone. (See

Figure 1.)

Note 4:  Input offset voltage is defined as the mean of the trip points.

Note 5:  CMRR = (VOSL - VOSH) / 5.2V, where VOSL is the offset at VCM = -0.1V and VOSH is the offset at VCM = 5.1V.

Note 6:  PSRR = (VOS2.7 - VOS5.5) / 2.8V, where VOS2.7 is the offset voltage at VCC = 2.7V, and VOS5.5 is the offset voltage at

VCC = 5.5V.

Note 7:  Propagation  delay for these high-speed comparators is guaranteed by design characterization because  it cannot be accurately measured using automatic test equipment. A statistically significant  sample of devices is characterized with a 200mV step and

100mV overdrive over the full temperature  range. Propagation  delay can be guaranteed by this characterization, since DC tests

ensure that all internal bias conditions  are correct. For low overdrive conditions, VTRIP is added  to the overdrive.

Note 8:  Guaranteed  by design.

Typical Operating Characteristics

(VCC = +3.0V, CLOAD = 5pF, 5mV of overdrive, TA = +25°C, unless otherwise noted.)

7.5

7.0

PROPAGATION DELAY (ns)

6.5

6.0

5.5

5.0

4.5

4.0

3.5

PROPAGATION DELAY

MAX997toc01

vs. INPUT OVERDRIVE

MAX997toc0202

| 6.8 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | tPD+ |  |  |  |
|  |  |  | tPD- |  |  |

6.6

6.4

PROPAGATION DELAY (ns)

6.2

6.0

5.8

5.6

5.4

5.2

5.0

PROPAGATION DELAY

vs. TEMPERATURE

PROPAGATION DELAY

MAX997toc03

vs. CAPACITIVE LOAD

| 8 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | tPD- |  |  |  |  |  |
|  | tPD+ |  |  |  |  |  |

PROPAGATION DELAY (ns)

7

6

5

4

| 0 10 20 30 40 50 60 70 80 90 100 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | tPD- |  |  |  |  |
|  |  |  | tPD+ |  |  |  |  |  |  |

-40

-20	0

20	40	60	85

0	20	40

60	80

100

120

INPUT OVERDRIVE (mV)

TEMPERATURE (°C)

CAPACITIVE LOAD (pF)

6.0

PROPAGATION DELAY (ns)

5.5

PROPAGATION DELAY

vs. SUPPLY VOLTAGE

MAX997toc04

2.80

| 2.75 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | tPD- tPD+ |  |  |

2.70

VOH (V)

2.65

2.60

2.55

OUTPUT HIGH VOLTAGE

MAX997toc05

| vs. SOURCE CURRENT TA = +85°C |  |  |  |
| --- | --- | --- | --- |
| TA = +25°C TA = -40°C |  |  |  |

0.5

0.4

VOL (V)

0.3

0.2

0.1

MAX997toc06

OUTPUT LOW VOLTAGE

vs. SINK CURRENT

5.0

2.5

3.0

3.5

4.0

4.5

5.0

5.5

6.0

2.50

1	10	100	1000	10,000

0

| 1 10 100 1000 10,000 |  |  |  |
| --- | --- | --- | --- |
| TA = -40°C |  |  |  |
| TA = +25°C TA = +85°C |  |  |  |

SUPPLY VOLTAGE (V)

SOURCE CURRENT (μA)

SINK CURRENT (μA)

8.0

MAX961/MAX963

MAX997toc07A

SUPPLY CURRENT PER COMPARATOR

vs. SUPPLY VOLTAGE

MAX962/MAX964

MAX997toc07B

SUPPLY CURRENT PER COMPARATOR

vs. SUPPLY VOLTAGE

| 7 |  |  |  |
| --- | --- | --- | --- |
|  | TA = +85°C |  |  |
|  | TA | = +25°C TA = -40°C |  |

MAX997/MAX999

MAX9997toc7C

| SUPPLY CURRENT PER COMPARATOR |  |  |  |
| --- | --- | --- | --- |
| TA = +85°C |  | TA = +25°C |  |
|  |  | TA = -40°C |  |

vs. SUPPLY VOLTAGE

9

8

SUPPLY CURRENT (mA)

SUPPLY CURRENT (mA)

7.5	6

7

| 7.0 5 6 |  |  |  |
| --- | --- | --- | --- |
|  | TA = +85° | C |  |
|  |  | TA = +25°C TA = -40°C |  |

SUPPLY CURRENT (mA)

5

6.5	4

4

MAX961–MAX964/MAX997/MAX999

6.0

2

3	4	5	6

SUPPLY VOLTAGE (V)

3

2	3	4	5	6

SUPPLY VOLTAGE (V)

3

2	3	4	5	6

SUPPLY VOLTAGE (V)

Typical Operating Characteristics (continued)

(VCC = +3.0V, CLOAD = 5pF, 5mV of overdrive, TA = +25°C, unless otherwise noted.)

SHUTDOWN SUPPLY CURRENT (μA)

310

MAX961/MAX963/MAX964/MAX997

MAX997toc08

SHUTDOWN SUPPLY CURRENT

vs. TEMPERATURE

MAX997toc09

| VOLTAGE TRIP POINT/INPUT OFFSET VOLTAGE vs. TEMPERATURE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VOS |  |  |  |  |
|  | VTRIP- |  |  |  |  |

3

MAX997toc10

VOLTAGE TRIP POINT/INPUT  OFFSET VOLTAGE vs. SUPPLY VOLTAGE

4

210

160

60

-40

-20	0

VCC = 2.7V

20	40	60	80

0

TRIP POINT / VOS (mV)

-1

-3

-40

-20	0

20	40	60	85

2

1

0

TRIP POINT / VOS (mV)

-1	VOS

-2

-3

-4

2	3	4

VTRIP+

VTRIP-

5	6

TEMPERATURE (°C)

INPUT BIAS CURRENT (IB+, IB-)

vs. COMMON-MODE  VOLTAGE

8

MAX997toc11

VCC = 5.0V

4.0

TEMPERATURE (°C)

MAX997toc12

| INPUT BIAS CURRENT/INPUT OFFSET CURRENT vs. TEMPERATURE | IB- |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | IB+ |  |  |  |  |
|  |  | IOS |  |  |  |

140

SUPPLY VOLTAGE (V)

SHORT-CIRCUIT OUTPUT CURRENT

vs. TEMPERATURE

MAX997toc13

OUTPUT SHORTED

6

4

IB+, IB- (μA)

2

0

-2    TA = -40°C

-4

-6

-8

TA = +85°C

TA = +25°C

VIN = VOS NEGATIVE VALUES REPRESENT CURRENT FLOWING INTO THE DEVICE

3.5

3.0

2.5

2.0

1.5

1.0

INPUT BIAS/OFFSET CURRENT (μA)

0.5

0

-0.5

120

100

80

60

SHORT-CIRCUIT CURRENT (mA)

40

20

0

TO VCC (SINKING)

OUTPUT SHORTED TO GND (SOURCING)

-1	0	1	2	3	4	5	6

-40

-20	0

20	40	60	80

-40

-20	0

20	40

60	80

VCM (V)

TEMPERATURE (°C)

TEMPERATURE (°C)

MAX997toc14

50MHz  RESPONSE

VIN = 100mVP-P

VOS

INPUT

50mV/div

MAX961–MAX964/MAX997/MAX999

OUTPUT

1V/div

GND

5ns/div

Typical Operating Characteristics (continued)

(VCC = +3.0V, CLOAD = 5pF, 5mV of overdrive, TA = +25°C, unless otherwise noted.)

PROPAGATION DELAY (tPD+)

MAX997toc15

PROPAGATION DELAY (tPD-)

MAX997toc16

GND

INPUT

50mV/div

GND

INPUT

50mV/div

OUTPUT

1V/div

OUTPUT

1V/div

GND

GND

5ns/div

5ns/div

MAX961–MAX964/MAX997/MAX999

Pin  Description

| PIN |  |  |  |  |  | NAME | FUNCTION |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MAX997 | MAX999 | MAX961 | MAX962 | MAX963 | MAX964 |  |  |
| 1, 5 | — | — | — | — | — | N.C. | No Connection. Not internally connected. |
| 2 | 4 | 2 | 2 | 1 | 1 | IN-, INA- | Comparator A Inverting Input |
| 3 | 3 | 1 | 1 | 2 | 2 | IN+, INA+ | Comparator A Noninverting Input |
| — | — | 4 | — | 3, 5 | — | LE, LEA, LEB | Latch-Enable Input. The output latches when LE_ is high. The latch is transparent when LE_ is low. |
| 4 | 2 | 5 | 5 | 4, 11 | 12 | GND | Ground |
| — | — | — | — | — | 16 | N.C. | No Connection. Connect to GND to prevent par- asitic feedback. |
| — | — | — | 4 | 6 | 3 | INB- | Comparator B Inverting Input |
| — | — | — | 3 | 7 | 4 | INB+ | Comparator B Noninverting Input |
| — | — | — | — | — | 5 | INC- | Comparator C Inverting Input |
| — | — | — | — | — | 6 | INC+ | Comparator C Noninverting Input |
| — | — | — | — | — | 7 | IND- | Comparator D Inverting Input |
| — | — | — | — | — | 8 | IND+ | Comparator D Noninverting Input |
| 8 | — | 3 | — | 8 | 9 | SHDN | Shutdown Input. The device shuts down when SHDN is high. |
| — | — | — | 6 | 9 | 14 | QB | Comparator B Output |
| — | — | — | — | — | 11 | QC | Comparator C Output |
| — | — | — | — | — | 10 | QD | Comparator D Output |
| — | — | — | — | 10 | — | QB | Comparator B Complementary Output |
| 7 | 5 | 8 | 8 | 12 | 13 | VCC | Positive Supply Input (VCC to GND must be ≤5.5V) |
| 6 | 1 | 6 | 7 | 13 | 15 | Q, QA | Comparator A TTL Output |
| — | — | 7 | — | 14 | — | Q, QA | Comparator A Complementary Output |

Detailed Description

The MAX961–MAX964/MAX997/MAX999 single-supply comparators  feature internal hysteresis, ultra-high- speed  operation, and low power consumption.  Their outputs are guaranteed to pull within 0.52V of either rail without external pullup  or pulldown circuitry.  Beyond- the-Rails input voltage range and low-voltage, single- supply operation make these devices ideal for portable equipment.  These comparators  all interface directly to CMOS logic.

Timing Most high-speed  comparators  oscillate in the linear region because of noise or undesirable parasitic feed- back. This can occur when the voltage on one input is close to or equal to the voltage on the other input. These devices have a small amount of internal hystere- sis to counter parasitic effects and noise.

The added  hysteresis of the MAX961–MAX964/MAX997/ MAX999 creates two trip points: one for the rising input voltage and one for the falling input voltage (Figure 1). The difference between the trip points is the hysteresis. When the comparator’s  input voltages are equal, the hysteresis effectively causes one comparator  input

voltage to move quickly past the other, thus taking the input out of the region where oscillation occurs. Standard comparators require hysteresis to be added with external resistors. The fixed internal hysteresis eliminates these resistors.

The MAX961/MAX963 include internal latches that allow storage of comparison  results. LE has a high input impedance.  If LE is low, the latch is transparent (i.e., the comparator operates as though the latch is not pre- sent). The comparator’s output state is stored when LE is pulled high. All timing constraints must be met when using the latch function (Figure 2).

Input Stage Circuitry The MAX961–MAX964/MAX997/MAX999 include  inter- nal protection circuitry that prevents damage to the pre- cision input stage from large differential input voltages. This protection circuitry consists of two groups of three front-to-back  diodes between IN+ and IN-, as well as two 200Ω resistors (Figure 3). The diodes limit the dif- ferential voltage applied  to the comparator’s  internal circuitry to no more than 3VF, where VF is the diode’s forward-voltage drop (about 0.7V at +25°C).

VTRIP+

VHYST

VIN+

VOS  =

VTRIP+  +  VTRIP-

2

VTRIP-

VIN- =  0V

MAX961–MAX964/MAX997/MAX999

VOH

Q	VOL

Q	VOH

VOL

Figure 1. Input and Output Waveforms, Noninverting Input Varied

tSU

tH

DIFFERENTIAL INPUT VOLTAGE

VOS

VCC

LE

0V

VCC

2

tLPW

tPD

tLPD

VOH

Q

VOL

Q

VCC

2

VCC

2

tSKEW

tSKEW

Figure 2. MAX961/MAX963 Timing Diagram

IN+

IN-

MAX961-MAX964

MAX997

MAX999

200Ω 	TO INTERNAL CIRCUITRY

TO INTERNAL

For a large differential input voltage (exceeding  3VF), this protection circuitry increases the input bias current at IN+ (source) and IN- (sink).

(IN+ - IN-) - 3VF

Input current = ————————

2 x 200

Input currents with large differential  input voltages should not be confused  with input bias currents (IB). As long as the differential input voltage is less than

3VF, this input current is less than 2IB.

The input  circuitry  allows the MAX961–MAX964/ MAX997/MAX999’s input  common-mode  range  to extend 100mV beyond  both power-supply  rails. The

200Ω

CIRCUITRY

output remains in the correct logic state if one or both

inputs are within the common-mode  range.  Taking either input outside the common-mode  range causes the input to saturate and the propagation  delay to increase.

MAX961–MAX964/MAX997/MAX999

Figure 3. Input Stage Circuitry

VCC

ISOURCE

MAX961-MAX964

MAX997

MAX999

Q, Q

ISINK

GND

Figure 4. Output Stage Circuitry

Figure 5. MAX961 PCB Layout

MAX961–MAX964/MAX997/MAX999

Output Stage Circuitry The MAX961–MAX964/MAX997/MAX999 contain a cur- rent-driven output stage, as shown in Figure 4. During an output transition, ISOURCE or ISINK is pushed or pulled to the output pin. The output source or sink current is high during the transition, creating a rapid slew rate. Once the output voltage reaches VOH or VOL, the source or sink current decreases to a small value, capable of maintain- ing the VOH or VOL in static condition. This decrease in current conserves power after an output transition has occurred.

One consequence of a current-driven output stage is a linear dependence  between the slew rate and the load capacitance.  A heavy capacitive  load slows down the voltage output transition.

Shutdown Mode When SHDN is high, the MAX961/MAX963/MAX964/ MAX997 shut down. When shut down, the supply cur- rent drops to 270µA per comparator,  and the outputs become  high impedance.  SHDN has a high input impedance.  Connect SHDN to GND for normal opera- tion. Exit shutdown with LE low; otherwise, the output is indeterminate.

Applications Information

Circuit Layout and Bypassing The MAX961–MAX964/MAX997/MAX999’s high band- width requires a high-speed layout. Follow these layout guidelines:

1) Use a PCB with a good, unbroken, low-inductance ground plane.

2) Place a decoupling  capacitor  (a 0.1µF ceramic sur- face-mount capacitor  is a good choice) as close to VCC as possible.

3) On the inputs and outputs, keep lead lengths short to avoid unwanted parasitic  feedback  around the comparators. Keep inputs away from outputs. Keep impedance between the inputs low.

4) Solder the device directly to the printed circuit board rather than using a socket.

5) Refer to Figure 5 for a recommended circuit layout.

6) For slow-moving input signals, take care to prevent parasitic  feedback.  A small capacitor  (1000pF or less) placed  between the inputs can help eliminate oscillations in the transition region. This capacitor causes negligible  degradation  to tPD when the source impedance is low.

Pin  Configurations

TOP VIEW

IN+    1

IN-  2

SHDN   3

LE    4

MAX961

SO/μMAX

8    VCC

7     Q

6     Q

5     GND

INA+    1

INA-    2

INB+    3

INB-    4

MAX962

SO/μMAX

8    VCC

7     QA

6     QB

5     GND

INA-    1

14    QA

INA-    1

16   N.C.

8     SHDN

LEA    3

12    VCC

INB-    3

14    QB

MAX997

7    VCC

GND   4

LEB   5

INB-    6

INB+    7

MAX963

SO

11    GND

10    QB

9     QB

8     SHDN

INB+   4

INC-    5

INC+   6

IND-    7

IND+    8

MAX964

SO/QSOP

13   VCC

| 12 GND N.C. | 1 |
| --- | --- |
| IN- | 2 |
| IN+ | 3 |
| GND | 4 |

11   QC

10   QD

9    SHDN

SO/μMAX

6     Q

5     N.C.

MAX961–MAX964/MAX997/MAX999

Chip Information

| MAX961/MAX962 TRANSISTOR COUNT: | 286 |
| --- | --- |
| MAX963/MAX964 TRANSISTOR COUNT: | 607 |
| MAX997/MAX999 TRANSISTOR COUNT: | 142 |

MAX961–MAX964/MAX997/MAX999

8LUMAXD.EPS

| Package Information (The package drawing(s) in this data sheet may not reflect the most current specifications. For the latest package outline information go to www.maxim-ic.com/packages.) | INCHES |  | MILLIMETERS |  |
| --- | --- | --- | --- | --- |
| DIM | MIN | MAX | MIN | MAX |
| A | - | 0.043 | - | 1.10 |
| A1 | 0.002 | 0.006 | 0.05 | 0.15 |
| A2 | 0.030 | 0.037 | 0.75 | 0.95 |
| b | 0.010 | 0.014 | 0.25 | 0.36 |
| c | 0.005 | 0.007 | 0.13 | 0.18 |
| D | 0.116 | 0.120 | 2.95 | 3.05 |
| e | 0.0256 BSC |  | 0.65 BSC |  |
| E | 0.116 | 0.120 | 2.95 | 3.05 |
| H | 0.188 | 0.198 | 4.78 | 5.03 |
| L | 0.016 | 0.026 | 0.41 | 0.66 |
| α | 0° | 6° | 0° | 6° |
| S | 0.0207 BSC |  | 0.5250 BSC |  |

| 4X S 8 8 E H Ø0.50±0.1 0.6±0.1 1 1 0.6±0.1 D BOTTOM VIEW TOP VIEW A2 A1 A c α e b L FRONT VIEW SIDE VIEW |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | PROPRIETARY INFORMATION |  |  |  |
|  | TITLE: PACKAGE OUTLINE, 8L uMAX/uSOP |  |  |  |
|  | APPROVAL | DOCUMENT CONTROL NO. 21-0036 | REV. J | 1 1 |

MAX961–MAX964/MAX997/MAX999

QSOP.EPS

MAX961–MAX964/MAX997/MAX999

SOICN .EPS

| Package Information (continued) (The package drawing(s) in this data sheet may not reflect the most current specifications. For the latest package outline information go to www.maxim-ic.com/packages.) | INCHES |  | MILLIMETERS |  |
| --- | --- | --- | --- | --- |
| DIM | MIN | MAX | MIN | MAX |
| A | 0.053 | 0.069 | 1.35 | 1.75 |
| A1 | 0.004 | 0.010 | 0.10 | 0.25 |
| B | 0.014 | 0.019 | 0.35 | 0.49 |
| C | 0.007 | 0.010 | 0.19 | 0.25 |
| e | 0.050 BSC |  | 1.27 BSC |  |
| E | 0.150 | 0.157 | 3.80 | 4.00 |
| H | 0.228 | 0.244 | 5.80 | 6.20 |
| L | 0.016 | 0.050 | 0.40 | 1.27 |

|  | INCHES |  | MILLIMETERS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| DIM | MIN | MAX | MIN | MAX | N | MS012 |
| D | 0.189 | 0.197 | 4.80 | 5.00 | 8 | AA |
| D | 0.337 | 0.344 | 8.55 | 8.75 | 14 | AB |
| D | 0.386 | 0.394 | 9.80 | 10.00 | 16 | AC |

| N E H VARIATIONS: 1 TOP VIEW D A C e B A1 0∞-8∞ L FRONT VIEW SIDE VIEW |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | PROPRIETARY INFORMATION |  |  |  |
|  | TITLE: PACKAGE OUTLINE, .150" SOIC |  |  |  |
|  | APPROVAL | DOCUMENT CONTROL NO. 21-0041 | REV. B | 1 1 |

MAX961–MAX964/MAX997/MAX999

SOT-23 5L .EPS

MAX961–MAX964/MAX997/MAX999

Revision History

| REVISION NUMBER | REVISION DATE | DESCRIPTION | PAGES CHANGED |
| --- | --- | --- | --- |
| 0 | 9/96 | Initial release | — |
| 1 | 12/96 | Added 8-pin µMAX packages. Correct minor errors. | 1, 2, 3 |
| 2 | 3/97 | Added dual and quad MAX963/MAX964 packages. | 1, 2, 3 |
| 3 | 7/97 | Added new MAX997 and MAX999 parts. | 1, 2, 3 |
| 4 | 3/99 | New wafer fab/process change to CB20. Update specifications and TOCs. | 2, 3, 4, 5, 6 |
| 5 | 2/07 | Added new Current into Input Pins in the Absolute Maximum Ratings. | 2 |
| 6 | 12/07 | Added new automotive grade MAX999AAUK part and specifications. | 1, 2, 3 |

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses  are implied. Maxim reserves the right to change the circuitry and specifications  without notice at any time.

16  	Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA  94086 408-737-7600

©  2008 Maxim Integrated  Products 	is a registered  trademark  of Maxim Integrated  Products, Inc.