19-1229; Rev 4; 1/07

Micropower, Low-Voltage, UCSP/SC70, Rail-to-Rail I/O Comparators

General Description

The MAX985/MAX986/MAX989/MAX990/MAX993/ MAX994 single/dual/quad  micropower  comparators feature low-voltage operation and rail-to-rail inputs and outputs. Their operating  voltages range from 2.5V to

5.5V, making them ideal for both 3V and 5V systems. These comparators also operate with ±1.25V to ±2.75V dual supplies. They consume only 11µA of supply cur- rent while achieving a 300ns propagation delay.

Input bias current is typically  1.0pA, and input offset voltage is typically  0.5mV. Internal hysteresis ensures clean output switching,  even with slow-moving  input signals.

The output stage’s unique design limits supply-current surges while switching,  virtually eliminating the supply glitches  typical  of many other comparators.  The MAX985/MAX989/MAX993 have a push-pull  output stage that sinks as well as sources current. Large inter- nal output drivers allow rail-to-rail output swing with loads up to 8mA. The MAX986/MAX990/MAX994 have an open-drain output stage that can be pulled beyond VCC to 6V (max) above VEE. These open-drain versions are ideal for level translators and bipolar  to single- ended converters.

The single MAX985 is available in a chip-scale  pack- age (UCSP™), significantly  reducing  the required  PC

Features

♦ 11µA Quiescent Supply Current

♦ 2.5V to 5.5V Single-Supply Operation

♦ Common-Mode Input Voltage Range Extends

250mV Beyond the Rails

♦ 300ns Propagation Delay

♦ Push-Pull Output Stage Sinks and Sources

8mA Current (MAX985/MAX989/MAX993)

♦ Open-Drain Output Voltage Extends Beyond VCC

(MAX986/MAX990/MAX994)

♦ Unique Output Stage Reduces Output Switching

Current, Minimizing Overall Power Consumption

♦ 80µA Supply Current at 1MHz Switching

Frequency

♦ No Phase Reversal for Overdriven Inputs

♦ Available in Space-Saving Packages: UCSP (MAX985)

SOT23 (MAX985/MAX986/MAX989/MAX990)

µMAX® (MAX989/MAX990)

Ordering Information

board area. The single MAX985/MAX986 are available in 5-pin SC70 packages and the dual MAX989/MAX990 are available in 8-pin SOT23 packages.

PART 	PIN-PACKAGE

TOP MARK

PKG CODE

| Selector Guide PART | COMPARATORS PER PACKAGE | OUTPUT STAGE |
| --- | --- | --- |
| MAX985 | 1 | Push-Pull |
| MAX986 | 1 | Open-Drain |
| MAX989 | 2 | Push-Pull |
| MAX990 | 2 | Open-Drain |
| MAX993 | 4 | Push-Pull |
| MAX994 | 4 | Open-Drain |

MAX985EBT-T 	6 UCSP-6 	AAY 	B6-1

MAX985EXK-T 	5 SC70-5 	ABK 	X5-1

Note: All devices are specified over the -40°C to +85°C operating temperature  range.

Ordering Information continued at end of data sheet. Typical Application Circuit appears at end of data sheet.

Pin  Configurations

Applications

TOP VIEW

(BUMPS ON BOTTOM)

IN+	B1

A1	VEE

Portable/Battery- Powered Systems

Mobile Communications Zero-Crossing Detectors Window Comparators Level Translators

Threshold Detectors/ Discriminators

Ground/Supply-Sensing

Applications

IR Receivers

Digital Line Receivers

IN- N.C.

MAX985

B2	A2

B3	A3

UCSP

OUT VCC

MAX985/MAX986/MAX989/MAX990/MAX993/MAX994

UCSP is a trademark and µMAX is a registered trademark of

Maxim Integrated Products, Inc.

Pin Configurations continued at end of data sheet.

ABSOLUTE MAXIMUM RATINGS

Supply Voltage (VCC to VEE) ...................................................6V Current into Input Pins ......................................................±20mA IN_-, IN_+ to VEE .......................................-0.3V to (VCC + 0.3V) OUT_ to VEE

MAX985/MAX989/MAX993 ....................-0.3V  to (VCC + 0.3V) MAX986/MAX990/MAX994.....................................-0.3V to 6V OUT_ Short-Circuit  Duration to VEE or VCC ...........................10s

Continuous Power Dissipation  (TA = +70°C)

5-Pin SC70 (derate 3.1mW/°C above +70°C) ...............247mW

5-Pin SOT23 (derate 7.10mW/°C above +70°C)...........571mW

6-Bump UCSP (derate 3.9mW/°C above +70°C)..........308mW

8-Pin SOT23 (derate 9.1mW/°C above +70°C).............727mW

8-Pin µMAX (derate 4.5mW/°C above +70°C) ..............362mW

8-Pin SO (derate 5.88mW/°C above +70°C).................471mW

14-Pin TSSOP (derate 9.1mW/°C above +70°C) ..........727mW

14-Pin SO (derate 8.33mW/°C above +70°C)...............667mW Operating  Temperature  Range ...........................-40°C to +85°C Junction Temperature ......................................................+150°C Storage Temperature  Range .............................-65°C to +150°C Lead Temperature  (soldering,  10s) .................................+300°C Bump Reflow Temperature  .............................................+235°C

MAX985/MAX986/MAX989/MAX990/MAX993/MAX994

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions  beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS

(VCC = 2.7V to 5.5V, VEE = 0V, VCM = 0V, TA = -40°C to +85°C, unless otherwise noted. Typical values are at TA = +25°C.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS |  |  |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Supply Voltage | VCC | Inferred from PSRR test |  |  |  | 2.5 5.5 | V |
| Supply Current per Comparator | ICC | VCC = 5V |  | TA = +25°C |  | 12 20 | µA |
|  |  |  |  | TA = -40°C to +85°C |  | 24 |  |
|  |  | VCC = 2.7V |  | TA = +25°C |  | 11 20 |  |
|  |  |  |  | TA = -40°C to +85°C |  | 24 |  |
| Power-Supply Rejection Ratio | PSRR | 2.5V ≤ VCC ≤ 5.5V |  |  |  | 55 80 | dB |
| Common-Mode Voltage Range (Note 2) | VCMR | TA = +25°C |  |  |  | VEE - VCC + 0.25 0.25 | V |
|  |  | TA = -40°C to +85°C |  |  |  | VEE VCC |  |
| Input Offset Voltage (Note 3) | VOS | Full common-mode range |  | TA = +25°C |  | ±0.5 ±5 | mV |
|  |  |  |  | TA = -40°C to +85°C |  | ±7 |  |
| Input Hysteresis | VHYST |  |  |  |  | ±3 | mV |
| Input Bias Current (Note 4) | IB |  |  |  |  | 0.001 10 | nA |
| Input Offset Current | IOS |  |  |  |  | 0.5 | pA |
| Input Capacitance | CIN |  |  |  |  | 1.0 | pF |
| Common-Mode Rejection Ratio | CMRR |  |  |  |  | 52 80 | dB |
| Output Leakage Current (MAX986/MAX990/ MAX994 only) | ILEAK | VOUT = high |  |  |  | 1.0 | µA |
| Output Short-Circuit Current | ISC | Sourcing or sinking, VOUT = VEE or VCC |  |  | VCC = 5V | 95 | mA |
|  |  |  |  |  | VCC = 2.7V | 35 |  |
| OUT Output Voltage Low | VOL | VCC = 5V, ISINK = 8mA | TA = +25°C |  |  | 0.2 0.4 | V |
|  |  |  | TA = -40°C to +85°C |  |  | 0.55 |  |
|  |  | VCC = 2.7V, ISINK = 3.5mA | TA = +25°C |  |  | 0.15 0.3 |  |
|  |  |  | TA = -40°C to +85°C |  |  | 0.4 |  |

MAX985/MAX986/MAX989/MAX990/MAX993/MAX994

ELECTRICAL CHARACTERISTICS (continued)

(VCC = 2.7V to 5.5V, VEE = 0V, VCM = 0V, TA = -40°C to +85°C, unless otherwise noted. Typical values are at TA = +25°C.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS |  |  |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | VOH | VCC = 5V, ISOURCE = 8mA |  | TA = +25°C |  | 4.6 4.85 | V |
| OUT Output Voltage High (MAX985/MAX989/ MAX993 only) |  |  |  | TA = -40°C to +85°C |  | 4.45 |  |
|  |  | VCC = 2.7V, ISOURCE = 3.5mA |  | TA = +25°C |  | 2.4 2.55 |  |
|  |  |  |  | TA = -40°C to +85°C |  | 2.3 |  |
| OUT Rise Time (MAX985/MAX989/ MAX993 only) | tRISE | VCC = 5.0V |  | CL = 15pF |  | 40 | ns |
|  |  |  |  | CL = 50pF |  | 50 |  |
|  |  |  |  | CL = 200pF |  | 80 |  |
| OUT Fall Time | tFALL | VCC = 5.0V |  | CL = 15pF |  | 40 | ns |
|  |  |  |  | CL = 50pF |  | 50 |  |
|  |  |  |  | CL = 200pF |  | 80 |  |
| Propagation Delay | tPD- | CL = 15pF | MAX985/MAX989/ MAX993 only |  | 10mV overdrive | 450 | ns |
|  |  |  |  |  | 100mV overdrive | 300 |  |
|  |  |  | MAX986/MAX990/ MAX994 only, RPULLUP = 5.1kΩ |  | 10mV overdrive | 450 |  |
|  |  |  |  |  | 100mV overdrive | 300 |  |
|  | tPD+ | MAX985/MAX989/ MAX993 only, CL = 15pF |  |  | 10mV overdrive | 450 |  |
|  |  |  |  |  | 100mV overdrive | 300 |  |
| Power-Up Time | tPU |  |  |  |  | 20 | µs |

Note 1:   All device specifications are 100% production tested at TA = +25°C. Limits over the extended  temperature  range are guar- anteed by design.

Note 2:   Inferred from the VOS test. Both or either inputs can be driven 0.3V beyond either supply rail without output phase reversal.

Note 3:   VOS is defined as the center of the hysteresis band at the input.

Note 4:   IB is defined as the average of the two input bias currents (IB-, IB+).

Typical Operating Characteristics

(VCC = 5V, VCM = 0V, TA = +25°C, unless otherwise noted.)

| 18 VIN+ > VIN- |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | VCC = 5.0V |  |  |  |  |
|  |  |  |  | VCC = 2.7V |  |  |  |

17

16

15

14

13

12

SUPPLY CURRENT (µA)

11

10

9

8

-60

SUPPLY CURRENT

vs. TEMPERATURE

-40   -20    0	20    40    60    80

TEMPERATURE (°C)

SUPPLY CURRENT (µA)

100

1000

100

MAX985-01

10

1

SUPPLY CURRENT vs. OUTPUT TRANSITION  FREQUENCY

VCC = 5.0V

VCC = 2.7V

0.01	0.1	1	10	100	1000

OUTPUT TRANSITION FREQUENCY (kHz)

MAX985-02

10,000

1000

100

OUTPUT LOW VOLTAGE (mV) (VOL)

10

1

OUTPUT LOW VOLTAGE

vs. OUTPUT SINK CURRENT

MAX985-04

VIN+ < VIN-

VCC = 2.7V

VCC = 5.0V

| 0.01 0.1 1 10 100 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

OUTPUT SINK CURRENT (mA)

10,000

1000

OUTPUT HIGH VOLTAGE (VCC - VOH) (mV)

100

10

1

0.1

OUTPUT HIGH VOLTAGE

vs. OUTPUT SOURCE CURRENT

VIN+ > VIN-

VCC = 2.7V

VCC = 5.0V

0.01	0.1	1	10	100

OUTPUT SOURCE CURRENT (mA)

MAX985-08

120

110

OUTPUT SINK CURRENT (mA)

100

90

80

70

60

50

40

30

20

10

0

| -60 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | VCC = 5.0V |  |  |  |  |  |  |
|  |  |  |  | VCC = 2.7V |  |  |  |  |  |

OUTPUT SHORT-CIRCUIT CURRENT vs. TEMPERATURE

-40   -20    0	20    40    60    80

TEMPERATURE (°C)

100

MAX985-06

1.1

0.9

OFFSET VOLTAGE (mV)

0.7

0.5

0.3

0.1

-0.1

-0.3

-60

INPUT OFFSET VOLTAGE

MAX985-07

vs. TEMPERATURE

-40   -20    0	20    40    60    80

TEMPERATURE (°C)

100

600

550

tPD (ns)

500

450

400

PROPAGATION DELAY

vs. CAPACITIVE LOAD (VCC = 3V)

MAX985-05a

VOD = 50mV

TO VOUT  = 50% OF FINAL VALUE

TO VOUT  = 10% OF FINAL VALUE

530

510

tPD (ns)

490

470

450

430

410

390

370

PROPAGATION DELAY

MAX985-05b

vs. CAPACITIVE LOAD (VCC = 5V)

VOD = 50mV

TO VOUT  = 50% OF FINAL VALUE

TO VOUT  = 10% OF FINAL VALUE

350

0	200

400

600

800

1000

350

0	200

400

600

800

1000

MAX985/MAX986/MAX989/MAX990/MAX993/MAX994

CAPACITIVE LOAD (pF)

CAPACITIVE LOAD (pF)

Typical Operating Characteristics (continued)

(VCC = 5V, VCM = 0V, TA = +25°C, unless otherwise noted.)

450

440

430

tPD (ns)

420

410

400

390

380

PROPAGATION DELAY

vs. TEMPERATURE

VOD = 50mV

TO VOUT = 50% POINT OF FINAL VALUE

MAX985-09

TO VOUT = 10% POINT OF FINAL VALUE

| 900 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | VCC = 2.7V |  |  |  |  |  |  |
|  | VCC = 5.0V |  |  |  |  |  |  |  |  |

800

700

600

tPD (ns)

500

400

300

200

100

0

PROPAGATION DELAY

MAX985-10

vs. INPUT OVERDRIVE

-40

-20	0	20	40	60	80

TEMPERATURE (°C)

100

0	40

80	120	160	200

INPUT OVERDRIVE (mV)

IN+

MAX985/MAX989/MAX993

PROPAGATION DELAY (tPD+)

MAX985-11

50mV/

div

IN+

PROPAGATION DELAY (tPD-)

MAX985-12

50mV/

div

IN+

OUT

MAX985/MAX989/MAX993

SWITCHING CURRENT, OUT RISING

MAX985-13

50mV/

div

2V/div

OUT

VOD = 50mV

100ns/div

2V/div

OUT

VOD = 50mV

100ns/div

2V/div

ICC

VOD = 50mV

100ns/div

1mA/div

IN+ OUT

ICC

SWITCHING CURRENT, OUT FALLING

MAX985-14

50mV/

div

2V/div

IN+

OUT

1MHz RESPONSE

MAX985-15

50mV/

div

2V/div

VCC

OUT

POWER-UP DELAY

V

IN-

MAX985-16

= 50mV

MAX985/MAX986/MAX989/MAX990/MAX993/MAX994

VOD = 50mV

100ns/div

1mA/div

VOD = 50mV

200ns/div

5µs/div

VIN+ = 0V

MAX985/MAX986/MAX989/MAX990/MAX993/MAX994

Pin  Description

| PIN |  |  |  |  | NAME | FUNCTION |
| --- | --- | --- | --- | --- | --- | --- |
| MAX985 MAX986 |  |  | MAX989 MAX990 | MAX993 MAX994 |  |  |
| SOT23/ SC70 | SO | UCSP* | SO/µMAX/ SOT23 | SO/ TSSOP |  |  |
| 1 | 6 | A2 | — | — | OUT | Comparator Output |
| 2 | 7 | A3 | 8 | 4 | VCC | Positive Supply Voltage |
| 3 | 3 | B1 | — | — | IN+ | Comparator Noninverting Input |
| 4 | 2 | B2 | — | — | IN- | Comparator Inverting Input |
| 5 | 4 | A1 | 4 | 11 | VEE | Negative Supply Voltage |
| — | — | — | 1 | 1 | OUTA | Comparator A Output |
| — | — | — | 2 | 2 | INA- | Comparator A Inverting Input |
| — | — | — | 3 | 3 | INA+ | Comparator A Noninverting Input |
| — | — | — | 5 | 5 | INB+ | Comparator B Noninverting Input |
| — | — | — | 6 | 6 | INB- | Comparator B Inverting Input |
| — | — | — | 7 | 7 | OUTB | Comparator B Output |
| — | — | — | — | 8 | OUTC | Comparator C Output |
| — | — | — | — | 9 | INC- | Comparator C Inverting Input |
| — | — | — | — | 10 | INC+ | Comparator C Noninverting Input |
| — | — | — | — | 12 | IND+ | Comparator D Noninverting Input |
| — | — | — | — | 13 | IND- | Comparator D Inverting Input |
| — | — | — | — | 14 | OUTD | Comparator D Output |
| — | 1, 5, 8 | B3 | — | — | N.C. | No Connection. Not internally connected. |

*MAX985 only

Detailed Description

The MAX985/MAX986/MAX989/MAX990/MAX993/ MAX994 are single/dual/quad  low-power, low-voltage comparators.  They have an operating supply voltage range between 2.5V and 5.5V and consume only 11µA. Their common-mode input voltage range extends 0.25V beyond each rail. Internal hysteresis ensures clean out- put switching,  even with slow-moving  input signals. Large internal output drivers allow rail-to-rail output swing with up to 8mA loads.

The output stage employs a unique design that mini- mizes supply-current  surges while switching,  virtually eliminating the supply glitches  typical  of many other comparators.  The MAX985/MAX989/MAX993 have a push-pull output structure that sinks as well as sources current. The MAX986/MAX990/MAX994 have an open-

VIN

R3

R1

R2

VREF

VCC

VCC

VEE

OUT

MAX985

MAX989

MAX993

MAX985/MAX986/MAX989/MAX990/MAX993/MAX994

drain output stage that can be pulled beyond VCC to an absolute maximum of 6V above VEE.

Input Stage Circuitry

The devices’ input common-mode range extends from

-0.25V to (VCC + 0.25V). These comparators may oper- ate at any differential input voltage within these limits. Input bias current is typically 1.0pA if the input voltage is between the supply rails. Comparator inputs are pro- tected  from overvoltage by internal body  diodes con- nected to the supply rails. As the input voltage exceeds the supply rails, these body  diodes  become  forward biased and begin to conduct.  Consequently, bias cur- rents increase  exponentially  as the input voltage exceeds the supply rails.

Output Stage Circuitry These comparators  contain a unique output stage capable  of rail-to-rail operation with up to 8mA loads. Many comparators consume orders of magnitude more current during switching than during steady-state oper- ation. However, with this family of comparators,  the supply-current  change  during  an output transition is extremely small. The Typical Operating Characteristics graph Supply Current vs. Output Transition Frequency shows the minimal supply-current  increase as the out- put switching frequency approaches  1MHz. This char- acteristic  eliminates the need for power-supply  filter capacitors  to reduce  glitches  created  by comparator switching currents. Another advantage realized in high- speed,  battery-powered  applications  is a substantial increase in battery life.

Figure 1. Additional  Hysteresis (MAX985/MAX989/MAX993)

Applications Information

Additional Hysteresis

MAX985/MAX989/MAX993

The MAX985/MAX989/MAX993 have ±3mV internal hysteresis. Additional hysteresis can be generated with three resistors using positive feedback  (Figure 1). Unfortunately, this method  also slows hysteresis response time. Use the following procedure  to calcu- late resistor values for the MAX985/MAX989/MAX993.

1) Select R3. Leakage current at IN is under 10nA, so the current through R3 should be at least 1µA to minimize errors caused by leakage current. The cur- rent through R3 at the trip point is (VREF - VOUT) / R3. Considering  the two possible  output states in solving for R3 yields two formulas: R3 = VREF / 1µA or R3 = (VREF - VCC) / 1µA. Use the smaller of the two resulting resistor values. For example, if VREF =

1.2V and VCC = 5V, then the two R3 resistor values

are 1.2MΩ  and 3.8MΩ. Choose a 1.2MΩ  standard

value for R3.

2) Choose the hysteresis band required (VHB). For this example, choose 50mV.

3) Calculate R1 according to the following equation: R1 = R3 x (VHB / VCC)

For this example, insert the values R1 = 1.2MΩ  x

(50mV / 5V) = 12kΩ.

4) Choose the trip point for VIN rising (VTHR; VTHF is

the trip point for VIN falling). This is the threshold

voltage at which the comparator switches its output

from low to high as VIN rises above the trip point. For this example, choose 3V.

5) Calculate R2 as follows. For this example, choose an

8.2kΩ standard value:

R2 = 	1

⎛	VTHR	⎞	1 	1

Use the following  procedure  to calculate  resistor values:

1) Select R3 according  to the formulas R3 = VREF /

500µA or R3 = (VREF - VCC) / 500µA - R4. Use the

⎜	⎟     −	−

⎝  VREF

x R1⎠

R1	R3

smaller of the two resulting resistor values.

2) Choose the hysteresis band required (VHB). For this

R2 = 	1

=  8.03kΩ

example, choose 50mV.

⎛	3.0V	⎞	1 	1

3) Calculate R1 according to the following equation:

⎜	⎟     −	−

⎝ 1.2 x 12kΩ ⎠

12kΩ

2.2MΩ

R1 = (R3 + R4) x (VHB / VCC)

6) Verify trip voltages and hysteresis as follows:

4) Choose the trip point for VIN rising (VTHR; VTHF is the trip point for VIN falling). This is the threshold voltage at which the comparator switches its output

VIN rising: VTHR = VREF x R1 x ⎛    1  +

⎜

⎝ R1

1

R2 +

⎟

1 ⎞

R3 ⎟

⎠

from low to high as VIN rises above the trip point.

5) Calculate R2 as follows:

VIN falling:  V	=  V

⎛ R1 x VCC ⎞

−

R2 = 	1

THF	THR	⎜

⎝

R3	⎠

⎛	VTHR	⎞	1 	1

⎜	⎟     −	−

Hysteresis  =

VTHR  −

VTHF

⎝  VREF  x R1⎠

R1	R3 + R4

MAX986/MAX990/MAX994

The MAX986/MAX990/MAX994 have ±3mV internal hysteresis. They have open-drain  outputs and require an external pullup  resistor (Figure 2). Additional  hys- teresis can be generated using positive feedback,  but

6) Verify trip voltages and hysteresis as follows:

VIN rising: VTHR = VREF x R1 x

⎛    1 	1 	1 	⎞

the   formulas   differ   slightly   from   those   of   the

MAX985/MAX989/MAX993.

+

⎝ R1

⎜

R2 +

R3 + R4 ⎠

VIN falling:  V	=  V

⎛ R1 x VCC ⎞

−

⎟

⎠

THF	THR

⎜

⎝   R3 +

R4 ⎟

VCC

R3

Hysteresis  =

VTHR  −

VTHF

VIN

R1	R4

VCC

Board Layout and Bypassing Power-supply bypass capacitors are not typically need- ed, but use 100nF bypass  capacitors  when supply impedance  is high, when supply  leads  are long,

R2

VREF

VEE

OUT

MAX986

MAX990

MAX994

or when excessive noise is expected  on the supply lines. Minimize signal trace lengths to reduce  stray capacitance.

MAX985/MAX986/MAX989/MAX990/MAX993/MAX994

Figure 2. Additional Hysteresis (MAX986/MAX990/MAX994)

Zero-Crossing Detector Figure 3 shows a zero-crossing detector  application. The MAX985’s inverting input is connected  to ground, and its noninverting input is connected  to a 100mVP-P signal source. As the signal at the noninverting input crosses 0V, the comparator’s output changes state.

Logic-Level Translator Figure 4 shows an application that converts 5V logic lev- els to 3V logic levels. The MAX986 is powered by the 5V supply voltage, and the pullup resistor for the MAX986’s open-drain output is connected to the 3V supply voltage. This configuration  allows the full 5V logic swing without creating overvoltage on the 3V logic inputs. For 3V to 5V logic-level translation, simply connect the 3V supply to VCC and the 5V supply to the pullup resistor.

UCSP  Applications Information

For the latest application details on UCSP construction, dimensions, tape carrier information, PCB techniques, bump-pad  layout, and recommended  reflow tempera- ture profile, as well as the latest information on reliability testing results, refer to the Application  Note: UCSP—A Wafer-Level Chip-Scale Package on Maxim’s web site at www.maxim-ic.com/ucsp.

100mV

4   IN+

VCC

2

VCC

100kΩ

4

IN-

5V (3V)

2

VCC

3V (5V) RPULLUP

3   IN-

VEE

MAX985

OUT   1

100kΩ

3

IN+

VEE

5

MAX986

OUT     1

3V (5V) LOGIC OUT

MAX985/MAX986/MAX989/MAX990/MAX993/MAX994

5	5V (3V) LOGIC IN

Figure 3. Zero-Crossing Detector	Figure 4. Logic-Level Translator

Typical Application Circuit

Ordering Information (continued)

VIN

IN+

IN-

VCC

VCC

OUT

*RPULLUP

VEE

MAX98_ MAX99_

VREF

*MAX986/MAX990/MAX994  ONLY.

THRESHOLD DETECTOR

Note: All devices are specified over the -40°C to +85°C operating temperature range. PART PIN-PACKAGE TOP PKG MARK CODE

MAX985EUK-T 5 SOT23-5 ABYZ U5-1

MAX985ESA 8 SO — S8-2

MAX986EXK-T 5 SC70-5 ABL X5-1

MAX986EUK-T 5 SOT23-5 ABZA U5-1

MAX986ESA 8 SO — S8-2

MAX989EKA-T 8 SOT23-8 AADZ K8-5

MAX989EUA-T 8 µMAX-8 — U8-1

MAX989ESA 8 SO — S8-2

MAX990EKA-T 8 SOT23-8 AAEA K8-5

MAX990EUA-T 8 µMAX-8 — U8-1

MAX990ESA 8 SO — S8-2

MAX993EUD 14 TSSOP — U14-1

MAX993ESD 14 SO — S14-1

MAX994EUD 14 TSSOP — K14-1

MAX994ESD 14 SO — S14-1

Pin  Configurations (continued)

TOP VIEW

OUT

1	5    VEE

MAX985

N.C.

| IN- | MAX985 MAX986 |  |
| --- | --- | --- |
| 1 |  | 8 |
| 2 |  | 7 |
| 3 |  | 6 |
| 4 |  | 5 |

N.C.

VCC

OUTA

INA-

VCC

OUTB

OUTA   1

14   OUTD

VCC    2

MAX986

IN+

OUT

INA+

INB-

INA+    3

VCC    4

MAX993

12   IND+

11   VEE

IN+

3	4

SOT23/SC70

IN-

VEE

N.C.

SO

VEE

SO/µMAX/SOT23

INB+

INB+   5

INB-    6

OUTB   7

| 10 INC+ | MAX989 MAX990 |  |
| --- | --- | --- |
| 1 |  | 8 |
| 2 |  | 7 |
| 3 |  | 6 |
| 4 |  | 5 |

9    INC-

8     OUTC

MAX985/MAX986/MAX989/MAX990/MAX993/MAX994

SO/TSSOP

MAX985/MAX986/MAX989/MAX990/MAX993/MAX994

SOT-23 5L .EPS

Package Information (The package drawing(s)  in this data sheet may not reflect the most current specifications. For the latest package outline information, go to www.maxim-ic.com/packages.)

MAX985/MAX986/MAX989/MAX990/MAX993/MAX994

6L, UCSP.EPS

Package Information (continued) (The package drawing(s)  in this data sheet may not reflect the most current specifications. For the latest package outline information, go to www.maxim-ic.com/packages.)

PACKAGE OUTLINE, 3x2 UCSP

1

21-0097	G	1

MAX985/MAX986/MAX989/MAX990/MAX993/MAX994

8LUMAXD.EPS

| Package Information (continued) (The package drawing(s) in this data sheet may not reflect the most current specifications. For the latest package outline information, go to www.maxim-ic.com/packages.) | INCHES |  | MILLIMETERS |  |
| --- | --- | --- | --- | --- |
| DIM | MIN | MAX | MIN | MAX |
| A | - | 0.043 | - | 1.10 |
| A1 | 0.002 | 0.006 | 0.05 | 0.15 |
| A2 | 0.030 | 0.037 | 0.75 | 0.95 |
| b | 0.010 | 0.014 | 0.25 | 0.36 |
| c | 0.005 | 0.007 | 0.13 | 0.18 |
| D | 0.116 | 0.120 | 2.95 | 3.05 |
| e | 0.0256 BSC |  | 0.65 BSC |  |
| E | 0.116 | 0.120 | 2.95 | 3.05 |
| H | 0.188 | 0.198 | 4.78 | 5.03 |
| L | 0.016 | 0.026 | 0.41 | 0.66 |
| α | 0° | 6° | 0° | 6° |
| S | 0.0207 BSC |  | 0.5250 BSC |  |

| 4X S 8 8 E H Ø0.50±0.1 0.6±0.1 1 1 0.6±0.1 D BOTTOM VIEW TOP VIEW A2 A1 A c α e b L FRONT VIEW SIDE VIEW |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | PROPRIETARY INFORMATION |  |  |  |
|  | TITLE: PACKAGE OUTLINE, 8L uMAX/uSOP |  |  |  |
|  | APPROVAL | DOCUMENT CONTROL NO. 21-0036 | REV. J | 1 1 |

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses  are implied. Maxim reserves the right to change the circuitry and specifications  without notice at any time.

Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA  94086 (408) 737-7600  	13

©  2007 Maxim Integrated Products	is a registered trademark of Maxim Integrated Products, Inc.