19-1266; Rev 2; 1/07

High-Speed, Micropower, Low-Voltage, SOT23, Rail-to-Rail I/O Comparators

General Description

The MAX987/MAX988/MAX991/MAX992/MAX995/ MAX996 single/dual/quad  micropower  comparators feature low-voltage operation and rail-to-rail inputs and outputs. Their operating voltage ranges from +2.5V to

+5.5V, making them ideal for both 3V and 5V systems. These comparators also operate with ±1.25V to ±2.75V dual supplies. They consume only 48µA per compara- tor while achieving a 120ns propagation delay.

Input bias current is typically 1.0pA, and input offset volt- age is typically 0.5mV. Internal hysteresis ensures clean output switching, even with slow-moving input signals.

The output stage’s unique design limits supply-current surges while switching, virtually eliminating the supply glitches  typical  of many other comparators.  The MAX987/MAX991/MAX995 have a push-pull  output stage that sinks as well as sources current. Large inter- nal output drivers allow rail-to-rail output swing with loads up to 8mA. The MAX988/MAX992/MAX996 have an open-drain output stage that can be pulled beyond VCC to 6V (max) above VEE. These open-drain versions are ideal for level translators and bipolar  to single- ended converters.

The single MAX987/MAX988 are available in tiny 5-pin SC70 packages,  while the dual MAX991/MAX992 are available in ultra-small 8-pin SOT23 and µMAX® pack- ages.

| Selector Guide PART | COMPARATORS PER PACKAGE | OUTPUT STAGE |
| --- | --- | --- |
| MAX987 | 1 | Push-Pull |
| MAX988 | 1 | Open-Drain |
| MAX991 | 2 | Push-Pull |
| MAX992 | 2 | Open-Drain |
| MAX995 | 4 | Push-Pull |
| MAX996 | 4 | Open-Drain |

Features

♦ 120ns Propagation Delay

♦ 48µA Quiescent Supply Current

♦ +2.5V to +5.5V Single-Supply Operation

♦ Common-Mode Input Voltage Range Extends

250mV Beyond the Rails

♦ Push-Pull Output Stage Sinks and Sources

8mA Current (MAX987/MAX991/MAX995)

♦ Open-Drain Output Voltage Extends Beyond VCC

(MAX988/MAX992/MAX996)

♦ Unique Output Stage Reduces Output Switching

Current, Minimizing Overall Power Consumption

♦ 100µA Supply Current at 1MHz Switching

Frequency

♦ No Phase Reversal for Overdriven Inputs

♦ Available in Space-Saving Packages:

5-Pin SOT23 (MAX987/MAX988)

8-Pin µMAX (MAX991/MAX992)

Ordering Information

PKG TOP PART PIN-PACKAGE CODE MARK

MAX987EXK-T 5 SC70-5 X5-1 ABM

MAX987EUK-T 5 SOT23-5 U5-1 ABZB

MAX987ESA 8 SO S8-2 —

Ordering Information continued at end of data sheet.

Note: All devices  specified over the -40°C to +85°C  operating temperature  range.

Typical Application Circuit appears at end of data sheet.

Pin  Configurations

TOP VIEW

Applications

OUT

1	5    VEE

Portable/Battery- Powered Systems

Mobile Communications

Threshold Detectors/ Discriminators

Ground/Supply Sensing

VCC    2

MAX987

MAX988

Zero-Crossing Detectors Window Comparators Level Translators

IR Receivers

Digital Line Receivers

IN+

3	4

SOT23/SC70

IN-

MAX987/MAX988/MAX991/MAX992/MAX995/MAX996

µMAX is a registered trademark of Maxim Integrated Products, Inc.

Pin Configurations continued at end of data sheet.

ABSOLUTE MAXIMUM RATINGS

Supply Voltage (VCC to VEE) ...................................................6V IN_-, IN_+ to VEE .......................................-0.3V to (VCC + 0.3V) Current into Input Pins .....................................................±20mA OUT_ to VEE

MAX987/MAX991/MAX995 ....................-0.3V  to (VCC + 0.3V) MAX988/MAX992/MAX996 ..................................-0.3V to +6V OUT_ Short-Circuit  Duration to VEE or VCC ...........................10s

Continuous Power Dissipation  (TA = +70°C)

5-Pin SC70 (derate 3.1mW/°C above +70°C) ...............247mW

5-Pin SOT23 (derate 7.10mW/°C above +70°C)...........571mW

8-Pin SOT23 (derate 9.1mW/°C above +70°C).............727mW

8-Pin SO (derate 5.88mW/°C above +70°C).................471mW

8-Pin µMAX (derate 4.5mW/°C above +70°C) ..............362mW

14-Pin TSSOP (derate 9.1mW/°C above +70°C) ..........727mW

14-Pin SO (derate 8.33mW/°C above +70°C)...............667mW Operating  Temperature  Range ...........................-40°C to +85°C Storage Temperature  Range .............................-65°C to +150°C Lead Temperature  (soldering,  10s) .................................+300°C

MAX987/MAX988/MAX991/MAX992/MAX995/MAX996

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions  beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS (Note 1)

(VCC = +2.7V to +5.5V, VEE = 0V, VCM = 0V, TA = -40°C to +85°C, unless otherwise noted. Typical values are at TA = +25°C.)

| n PARAMETER | SYMBOL | CONDITIONS |  |  |  |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Supply Voltage | VCC | Inferred from PSRR test |  |  |  |  | 2.5 5.5 | V |
| Supply Current per Comparator | ICC | VCC = 5V |  |  | TA = +25°C |  | 53 80 | µA |
|  |  |  |  |  | TA = -40°C to +85°C |  | 96 |  |
|  |  | VCC = 2.7V |  |  | TA = +25°C |  | 48 80 |  |
|  |  |  |  |  | TA = -40°C to +85°C |  | 96 |  |
| Power-Supply Rejection Ratio | PSRR | 2.5V ≤ VCC ≤ 5.5V |  |  |  |  | 55 80 | dB |
| Common-Mode Voltage Range (Note 2) | VCMR | TA = +25°C |  |  |  |  | VEE - VCC + 0.25 0.25 | V |
|  |  | TA = -40°C to +85°C |  |  |  |  | VEE VCC |  |
| Input Offset Voltage (Note 3) | VOS | Full common-mode range |  |  | TA = +25°C |  | ±0.5 ±5 | mV |
|  |  |  |  |  | TA = -40°C to +85°C |  | ±7 |  |
| Input Hysteresis | VHYST |  |  |  |  |  | ±2.5 | mV |
| Input Bias Current (Note 4) | IB |  |  |  |  |  | 0.001 10 | nA |
| Input Offset Current | IOS |  |  |  |  |  | 0.5 | pA |
| Input Capacitance | CIN |  |  |  |  |  | 1.0 | pF |
| Common-Mode Rejection Ratio | CMRR |  |  |  |  |  | 50 80 | dB |
| Output Leakage Current (MAX988/MAX992/ MAX996 only) | ILEAK | VOUT = high |  |  |  |  | 1.0 | µA |
| Output Short-Circuit Current | ISC | Sourcing or si VOUT = VEE or | king, VCC |  |  | VCC = 5V | 95 | mA |
|  |  |  |  |  |  | VCC = 2.7V | 35 |  |
| OUT Output-Voltage Low | VOL | VCC = 5V, ISINK = 8mA |  | TA = +25°C |  |  | 0.2 0.4 | V |
|  |  |  |  | TA = -40°C to +85°C |  |  | 0.55 |  |
|  |  | VCC = 2.7V, ISINK = 3.5mA |  | TA = +25°C |  |  | 0.15 0.3 |  |
|  |  |  |  | TA = -40°C to +85°C |  |  | 0.4 |  |
| OUT Output-Voltage High (MAX987/MAX991/ MAX995 Only) | VOH | VCC = 5V, ISOURCE = 8mA |  | TA = +25°C |  |  | 4.6 4.85 | V |
|  |  |  |  | TA = -40°C to +85°C |  |  | 4.45 |  |
|  |  | VCC = 2.7V, ISOURCE = 3.5mA |  | TA = +25°C |  |  | 2.4 2.55 |  |
|  |  |  |  | TA = -40°C to +85°C |  |  | 2.3 |  |

MAX987/MAX988/MAX991/MAX992/MAX995/MAX996

ELECTRICAL CHARACTERISTICS (continued)

(VCC = +2.7V to +5.5V, VEE = 0V, VCM = 0V, TA = -40°C to +85°C, unless otherwise noted. Typical values are at TA = +25°C.)

| CL = 15pF, PARAMETER | SYMBOL | CONDITIONS |  |  |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| OUT Rise Time (MAX987/MAX991/ MAX995 Only) | tRISE | VCC = 5.0V |  | CL = 15pF |  | 15 | ns |
|  |  |  |  | CL = 50pF |  | 20 |  |
|  |  |  |  | CL = 200pF |  | 40 |  |
| OUT Fall Time | tFALL | VCC = 5.0V |  | CL = 15pF |  | 15 | ns |
|  |  |  |  | CL = 50pF |  | 20 |  |
|  |  |  |  | CL = 200pF |  | 40 |  |
| Propagation Delay | tPD- |  | MAX987/MAX991/ MAX995 only |  | 10mV overdrive | 210 | ns |
|  |  |  |  |  | 100mV overdrive | 120 |  |
|  |  | VCC = 5V | MAX988/MAX992/ MAX996 only, RPULLUP = 5.1kΩ |  | 10mV overdrive | 210 |  |
|  |  |  |  |  | 100mV overdrive | 120 |  |
|  | tPD+ | MAX987/MAX991/MAX995 only, CL = 15pF, VCC = 5V |  |  | 10mV overdrive | 210 |  |
|  |  |  |  |  | 100mV overdrive | 120 |  |
| Power-Up Time | tPU |  |  |  |  | 25 | µs |

Note 1:   All device specifications are 100% production tested at TA = +25°C. Limits over the extended  temperature  range are guar- anteed by design, not production tested.

Note 2:   Inferred from the VOS test. Either or both inputs can be driven 0.3V beyond either supply rail without output phase reversal.

Note 3:   VOS is defined as the center of the hysteresis band at the input.

Note 4:   IB is defined as the average of the two input bias currents (IB-, IB+).

Typical Operating Characteristics

(VCC = +5V, VCM = 0V, TA = +25°C, unless otherwise noted.)

SUPPLY CURRENT (µA)

SUPPLY CURRENT PER COMPARATOR

vs. TEMPERATURE

90

MAX9879 TOC1

VIN+ > VIN-

80

1000

SUPPLY CURRENT PER COMPARATOR

MAX987 TOC2

vs. OUTPUT TRANSITION FREQUENCY

70

VCC = 5.5.V

60

100

VCC = 5.5V

SUPPLY CURRENT (µA)

50	VCC = 2.5.V

VCC = 2.5V

40

30

-60

-40

-20

0	20    40    60    80   100

TEMPERATURE (°C)

10

0.01	0.1	1	10	100    1000  10,000

OUTPUT TRANSITION FREQUENCY (kHz)

10,000

1000

OUTPUT HIGH VOLTAGE

MAX987-04

vs. OUTPUT SOURCE CURRENT

VIN+ > VIN-

10,000

1000

OUTPUT LOW VOLTAGE

vs. OUTPUT SINK CURRENT

MAX987-03a

VIN+ < VIN-

OUTPUT HIGH VOLTAGE (mV) (VCC - VOH)

100

10

VCC = 2.7V

VCC = 5.0V

100

VCC = 2.7V

VCC = 5.0V

OUTPUT LOW VOLTAGE (mV) (VOL)

1	10

0.1

0.01	0.1	1	10	100

MAX987 05

OUTPUT SOURCE CURRENT (mA)

1

0.01	0.1	1	10	100

OUTPUT SINK CURRENT (mA)

MAX987/MAX988/MAX991/MAX992/MAX995/MAX996

120

110

OUTPUT SINK CURRENT (mA)

100

90

80

70

60

50

40

30

20

10

0

-60

OUTPUT SHORT-CIRCUIT CURRENT vs. TEMPERATURE

-40   -20    0	20    40    60    80

TEMPERATURE (°C)

| 100 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | VCC = 5.0V |  |  |  |  |  |  |
|  |  |  |  | VCC = 2.7V |  |  |  |  |  |

1.1

0.9

OFFSET VOLTAGE (mV)

0.7

0.5

0.3

0.1

-0.1

-0.3

| -60 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

INPUT OFFSET VOLTAGE

MAX987 06

vs. TEMPERATURE

-40   -20    0	20    40    60    80

TEMPERATURE (°C)

100

Typical Operating Characteristics (continued)

(VCC = +5V, VCM = 0V, TA = +25°C, unless otherwise noted.)

10,000

PROPAGATION DELAY (ns)

1000

100

PROPAGATION DELAY

vs. CAPACITIVE LOAD

MAX987 TOC8

VOD = 50mV

200

190

PROPAGATION DELAY (ns)

180

170

160

150

140

130

120

110

100

PROPAGATION DELAY

MAX987 TOC9

vs. TEMPERATURE

VOD = 50mV

VCC = 2.5.V

VCC = 5.5.V

0.01	0.1	1

10	100	1000

-60

-40

-20

0	20    40    60    80   100

CAPACITIVE LOAD (nF)

TEMPERATURE (°C)

300

250

200

150

PROPAGATION DELAY (ns)

100

50

PROPAGATION DELAY

MAX987 TOC10

vs. INPUT OVERDRIVE

VCC = 2.5V VCC = 5.5V

IN+

OUT

MAX987/MAX991/MAX995

PROPAGATION DELAY (tPD+)

MAX987-11

VOD = 50mV

50mV/div

2V/div

0

0	20	40	60

80    100

120

140

100ns/div

INPUT OVERDRIVE (mV)

IN+

OUT

PROPAGATION DELAY (tPD-)

MAX987-12

50mV/div

2V/div

IN+

OUT

ICC

MAX987/MAX991/MAX995

SWITCHING CURRENT, OUT RISING

MAX987-13

VOD = 50mV

50mV/div

2V/div

2mA/div

VOD = 50mV

MAX987/MAX988/MAX991/MAX992/MAX995/MAX996

100ns/div

200ns/div

Typical Operating Characteristics (continued)

(VCC = +5V, VCM = 0V, TA = +25°C, unless otherwise noted.)

SWITCHING CURRENT, OUT FALLING

MAX987-14

VOD = 50mV

1MHZ RESPONSE

MAX987-15

IN+

OUT ICC

50mV/div

2V/div

2mA/div

IN+

OUT

50mV/div

2V/div

200ns/div

VOD = 50mV

200ns/div

POWER-UP DELAY

VIN- = 50mV VIN+ = 0V

MAX987-16

VCC

OUT

2V/div

2V/div

MAX987/MAX988/MAX991/MAX992/MAX995/MAX996

5µs/div

MAX987/MAX988/MAX991/MAX992/MAX995/MAX996

Pin  Description

| PIN |  |  |  | NAME |  |
| --- | --- | --- | --- | --- | --- |
| MAX987 MAX988 |  | MAX991 MAX996 | MAX995 MAX996 |  | FUNCTION |
| SOT23/ SC70 | SO | SO/µMAX/ SOT23 | SO/ TSSOP |  |  |
| 1 | 6 | — | — | OUT | Comparator Output |
| 2 | 7 | 8 | 4 | VCC | Positive Supply Voltage |
| 3 | 3 | — | — | IN+ | Comparator Noninverting Input |
| 4 | 2 | — | — | IN- | Comparator Inverting Input |
| 5 | 4 | 4 | 11 | VEE | Negative Supply Voltage |
| — | — | 1 | 1 | OUTA | Comparator A Output |
| — | — | 2 | 2 | INA- | Comparator A Inverting Input |
| — | — | 3 | 3 | INA+ | Comparator A Noninverting Input |
| — | — | 5 | 5 | INB+ | Comparator B Noninverting Input |
| — | — | 6 | 6 | INB- | Comparator B Inverting Input |
| — | — | 7 | 7 | OUTB | Comparator B Output |
| — | — | — | 8 | OUTC | Comparator C Output |
| — | — | — | 9 | INC- | Comparator C Inverting Input |
| — | — | — | 10 | INC+ | Comparator C Noninverting Input |
| — | — | — | 12 | IND+ | Comparator D Noninverting Input |
| — | — | — | 13 | IND- | Comparator D Inverting Input |
| — | — | — | 14 | OUTD | Comparator D Output |
| — | 1, 5, 8 | — | — | N.C. | No Connection. Not internally connected. |

Detailed Description

The MAX987/MAX988/MAX991/MAX992/MAX995/ MAX996 are single/dual/quad  low-power, low-voltage comparators.  They have an operating  supply voltage range between +2.5V and +5.5V and consume only

48µA per comparator, while achieving 120ns propaga- tion delay. Their common-mode  input voltage range extends 0.25V beyond  each rail. Internal hysteresis ensures clean output switching, even with slow-moving input signals. Large internal output drivers allow rail-to- rail output swing with up to 8mA loads.

The output stage employs a unique design that mini- mizes supply-current  surges while switching,  virtually eliminating the supply glitches  typical  of many other comparators.  The MAX987/MAX991/MAX995 have a push-pull output structure that sinks as well as sources current. The MAX988/MAX992/MAX996 have an open- drain output stage that can be pulled beyond VCC to an absolute maximum of 6V above VEE.

Input Stage Circuitry

The devices’ input common-mode range extends from

-0.25V to (VCC + 0.25V). These comparators may oper- ate at any differential input voltage within these limits. Input bias current is typically 1.0pA if the input voltage is between the supply rails. Comparator inputs are pro- tected  from overvoltage by internal body  diodes con- nected to the supply rails. As the input voltage exceeds the supply rails, these body  diodes  become  forward biased and begin to conduct.  Consequently, bias cur- rents increase  exponentially  as the input voltage exceeds the supply rails.

Output Stage Circuitry These comparators  contain a unique output stage capable  of rail-to-rail operation with up to 8mA loads. Many comparators consume orders of magnitude more current during switching than during steady-state oper- ation. However, with this family of comparators,  the supply-current  change  during  an output transition is

Applications Information

Additional Hysteresis

MAX987/MAX991/MAX995

The MAX987/MAX991/MAX995 have ±2.5mV internal hysteresis. Additional hysteresis can be generated with three resistors using positive feedback  (Figure 1). Unfortunately, this method  also slows hysteresis response time. Use the following procedure  to calcu- late resistor values for the MAX987/MAX991/MAX995.

1) Select R3. Leakage current at IN is under 10nA;

therefore, the current through R3 should be at least

1µA to minimize errors caused by leakage current. The current through R3 at the trip point is (VREF - VOUT) / R3. Considering  the two possible  output states and solving for R3 yields two formulas: R3 = VREF / 1µA or R3 = (VREF - VCC) / 1µA. Use the smaller of the two resulting resistor values. For example, if VREF = 1.2V and VCC = 5V, then the two

R3 resistor values are 1.2MΩ  and 3.8MΩ. Choose a

1.2MΩ standard value for R3.

2) Choose the hysteresis band required (VHB). For this

example, choose 50mV.

3) Calculate R1 according to the following equation: R1 = R3 x (VHB / VCC)

For this example, insert the values R1 = 1.2MΩ  x

(50mV / 5V) = 12kΩ.

4) Choose the trip point for VIN rising (VTHR; VTHF is

the trip point for VIN falling). This is the threshold

voltage at which the comparator switches its output

from low to high as VIN rises above the trip point. For this example, choose 3V.

VCC

R3

extremely small. The Typical Operating Characteristics Supply Current vs. Output Transition Frequency graph shows the minimal supply-current  increase as the out- put switching frequency approaches  1MHz. This char- acteristic  eliminates the need for power-supply  filter capacitors  to reduce  glitches  created  by comparator switching currents. Battery life increases substantially in high-speed, battery-powered applications.

VIN

R1

R2

VREF

VCC

VEE

0.1µF OUT

MAX987

MAX991

MAX995

MAX987/MAX988/MAX991/MAX992/MAX995/MAX996

Figure 1. Additional Hysteresis (MAX987/MAX991/MAX995)

5) Calculate R2 as shown. For this example, choose an

8.2kΩ standard value:

R2 = 	1

⎛	VTHR	⎞	1 	1

Use the following  procedure  to calculate  resistor values:

1) Select R3 according to the formulas R3 = VREF / 1µA

⎜	⎟     −	−

R2 =

⎝  VREF

x R1⎠	R1	R3

1

=  8.03kΩ

or R3 = (VREF - VCC) / 1µA - R4. Use the smaller of

the two resulting resistor values.

2) Choose the hysteresis band required (VHB). For this

⎛	3.0V	⎞	1 	1

example, choose 50mV.

⎜	⎟     −	−

⎝ 1.2 x 12kΩ ⎠

12kΩ

2.2MΩ

3) Calculate R1 according to the following equation:

6) Verify trip voltages and hysteresis as follows:

R1 = (R3 + R4) x (VHB / VCC)

4) Choose the trip point for VIN rising (VTHR; VTHF is the trip point for VIN falling). This is the threshold

VIN rising: VTHR = VREF x R1 x ⎛    1  +

⎜

⎝ R1

⎛ R1 x V

1

R2 +

⎞

1 ⎞

R3 ⎟

⎠

voltage at which the comparator switches its output from low to high as VIN rises above the trip point.

5) Calculate R2 as follows:

VIN falling:

VTHF =

VTHR

−    ⎜  	CC ⎟

⎝	R3	⎠

R2 = 	1

Hysteresis  =

VTHR  −

VTHF

⎛	VTHR	⎞	1 	1

⎜	⎟     −	−

MAX988/MAX992/MAX996

⎝  VREF  x R1⎠

R1	R3 + R4

The MAX988/MAX992/MAX996 have ±2.5mV internal

hysteresis. They have open-drain  outputs and require an external pullup  resistor (Figure 2). Additional  hys- teresis can be generated using positive feedback,  but the formulas differ slightly from those of the MAX987/MAX991/MAX995.

6) Verify trip voltages and hysteresis as follows:

VIN rising: VTHR = VREF x R1 x

⎛    1 	1 	1 	⎞

⎜	+	+	⎟

VIN falling:

⎝ R1	R2

V	=  V

R3 + R4 ⎠

⎛ R1 x VCC ⎞

⎠

⎜

−

THF	THR

⎝   R3 +

R4 ⎟

VCC

R3

Hysteresis  =

VTHR  −

VTHF

VIN

R1		R4     	 VCC

OUT

0.1µF

Circuit Layout and Bypassing These comparators’  high-gain  bandwidth  requires design precautions to maximize their high-speed capa- bility. The recommended precautions are:

R2

VREF

VEE

MAX988

MAX992

MAX996

1)  Use a PCB with an unbroken,  low-inductance ground plane.

2)  Place a decoupling  capacitor  (a 0.1µF ceramic capacitor  is a good  choice)  as close to VCC as possible.

3)  On the inputs and outputs, keep lead lengths short to avoid unwanted parasitic  feedback  around the comparators.

MAX987/MAX988/MAX991/MAX992/MAX995/MAX996

Figure 2. Additional Hysteresis (MAX988/MAX992/MAX996)

4)  Solder the devices  directly  to the PCB instead of using a socket.

Zero-Crossing Detector Figure 3 shows a zero-crossing detector  application. The MAX987’s inverting input is connected  to ground, and its noninverting input is connected  to a 100mVp-p signal source. As the signal at the noninverting input crosses 0V, the comparator’s output changes state.

Logic-Level Translator Figure 4 shows an application that converts 5V logic lev- els to 3V logic levels. The MAX988 is powered by the +5V supply voltage, and the pullup resistor for the MAX988’s open-drain output is connected  to the +3V supply volt- age. This configuration allows the full 5V logic swing with- out creating overvoltage on the 3V logic inputs. For 3V to

5V logic-level translation, simply connect the +3V supply to VCC and the +5V supply to the pullup resistor.

VCC

+5V (+3V)

100mV

4   IN+

2

VCC

0.1µF

100kΩ

4

IN-

2

VCC

0.1µF   +3V (+5V)

3   IN-

VEE

5

MAX987

OUT   1

100kΩ

3

IN+

VEE

5

MAX988

OUT     1

3V (5V) LOGIC OUT

5V (3V) LOGIC IN

Figure 3.  Zero-Crossing  Detector 	Figure 4.  Logic-Level Translator

Pin  Configurations (continued)

| TOP VIEW N.C. |  | N.C. | OUTA |  | VCC |
| --- | --- | --- | --- | --- | --- |
| IN- IN+ |  | VCC OUT | INA- INA+ |  | OUTB INB- |

| VEE | MAX987 MAX988 |  |
| --- | --- | --- |
| 1 |  | 8 |
| 2 |  | 7 |
| 3 |  | 6 |
| 4 |  | 5 |

N.C.

| SO | MAX991 MAX992 |  |
| --- | --- | --- |
| 1 |  | 8 |
| 2 |  | 7 |
| 3 |  | 6 |
| 4 |  | 5 |

VEE

SO/µMAX/SOT23

INB+

OUTA   1

INA-    2

INA+   3

VCC    4

INB+   5

INB-    6

OUTB   7

MAX995

MAX996

14   OUTD

13   IND-

12   IND+

11   VEE

10   INC+

9    INC-

8     OUTC

MAX987/MAX988/MAX991/MAX992/MAX995/MAX996

SO/TSSOP

Typical Application Circuit

VCC

Ordering Information (continued)

VIN

0.1µF

IN+

VCC

OUT

*RPULLUP

VREF

IN-

VEE

MAX98_ MAX99_

* MAX988/MAX992/MAX996  ONLY

THRESHOLD DETECTOR

Note: All devices specified over the -40°C to +85°C operating temperature range. PART PIN-PACKAGE PKG TOP CODE MARK

MAX988EXK-T 5 SC70-5 X5-1 ABN

MAX988EUK-T 5 SOT23-5 U5-1 ABZC

MAX988ESA 8 SO S8-2 —

MAX991EKA-T 8 SOT23-8 K8-5 AAEB

MAX991EUA-T 8 µMAX-8 U8-1 —

MAX991ESA 8 SO S8-2 —

MAX992EKA-T 8 SOT23-8 K8-5 AAEC

MAX992EUA-T 8 µMAX-8 U8-1 —

MAX992ESA 8 SO S8-2 —

MAX995EUD 14 TSSOP U14-1 —

MAX995ESD 14 SO S14-4 —

MAX996EUD 14 TSSOP U14-1 —

MAX996ESD 14 SO S14-4 —

Tape-and-Reel Information

4.0 ±0.1

1.5 +0.1/-0.0 DIAMETER

2.0 ±0.05

1.0 ±0.1

A

1.75 ±0.1

2.2 ±0.1

3.5 ±0.05 8.0 ±0.3

0.5 RADIUS TYPICAL

4.0 ±0.1 	A0

Bo

1.0 MINIMUM A

MAX987/MAX988/MAX991/MAX992/MAX995/MAX996

0.30 ±0.05

0.8 ±0.05

0.30R MAX.

Ao = 3.1mm ±0.1

Ko 	Bo = 2.7mm ±0.1

Ko = 1.2mm ±0.1

NOTE:  DIMENSIONS ARE IN MM. AND FOLLOW EIA481-1 STANDARD.

MAX987/MAX988/MAX991/MAX992/MAX995/MAX996

SOT-23 5L .EPS

Package Information (The package drawing(s)  in this data sheet may not reflect the most current specifications. For the latest package outline information go to  www.maxim-ic.com/packages.)

MAX987/MAX988/MAX991/MAX992/MAX995/MAX996

8LUMAXD.EPS

| Package Information (continued) (The package drawing(s) in this data sheet may not reflect the most current specifications. For the latest package outline information go to www.maxim-ic.com/packages.) | INCHES |  | MILLIMETERS |  |
| --- | --- | --- | --- | --- |
| DIM | MIN | MAX | MIN | MAX |
| A | - | 0.043 | - | 1.10 |
| A1 | 0.002 | 0.006 | 0.05 | 0.15 |
| A2 | 0.030 | 0.037 | 0.75 | 0.95 |
| b | 0.010 | 0.014 | 0.25 | 0.36 |
| c | 0.005 | 0.007 | 0.13 | 0.18 |
| D | 0.116 | 0.120 | 2.95 | 3.05 |
| e | 0.0256 BSC |  | 0.65 BSC |  |
| E | 0.116 | 0.120 | 2.95 | 3.05 |
| H | 0.188 | 0.198 | 4.78 | 5.03 |
| L | 0.016 | 0.026 | 0.41 | 0.66 |
| α | 0° | 6° | 0° | 6° |
| S | 0.0207 BSC |  | 0.5250 BSC |  |

| 4X S 8 8 E H Ø0.50±0.1 0.6±0.1 1 1 0.6±0.1 D BOTTOM VIEW TOP VIEW A2 A1 A c α e b L FRONT VIEW SIDE VIEW |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | PROPRIETARY INFORMATION |  |  |  |
|  | TITLE: PACKAGE OUTLINE, 8L uMAX/uSOP |  |  |  |
|  | APPROVAL | DOCUMENT CONTROL NO. 21-0036 | REV. J | 1 1 |

Revision History

Pages changed at Rev 2: 1–6, 8–13

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses  are implied. Maxim reserves the right to change the circuitry and specifications  without notice at any time.

Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA  94086 408-737-7600  	13

©  2007 Maxim Integrated Products	is a registered trademark of Maxim Integrated Products, Inc.